3GPP TSG RAN WG1 Meeting #90							R1-1715239

Prague, Czechia, 21th – 25th August 2017

Source:	ZTE

Title:	Offline discussions on simulation assumption for long PUCCH of more than 2 bits

Agenda Item:	6.1.3.2.2.2

Document for: 	Discussion and Decision

# Introduction

Agreements:

- For a PUCCH format for UCI with large payload with no multiplexing capacity within a slot:

  - If frequency-hopping is enabled,

    - For each frequency-hop with less than X symbols, there is one DMRS symbol.

      - X is not smaller than 4.

    - For each frequency-hop with equal to or more than X symbols, there are two DMRS symbols.

    - For each frequency-hop, at least one DMRS symbol is included.

  - FFS: number of DMRS symbols if frequency-hopping is disabled.

  - Targeting one value for X.

  - FFS: The value of X

  - FFS: DMRS structures

- Note:

  - Continue discussion on simulation assumptions within Thursday

- Further results of offline discussion:

- Simulation assumptions for purpose of identifying the number of DMRS symbols for long PUCCH format without multiplexing capability for more than 2 bits UCI payload

  - System bandwidth = 20Mhz

  - Subcarrier spacing =  {15Khz}

  - TDL-C channel with delay spread = {300nS, 1000nS}

  - # UE Tx =1, # gNB Rx =2 ( Optional: # gNB Rx = 4 and 32)

  - Payload sizes without CRC: 20bits, 60bits, 100 bits (only for 14 symbols long PUCCH)

  - Number of RB = {1 RB}

  - Carrier frequency = 4Ghz

  - Number of UEs = {1}

  - UE speed: 3km/h, 120km/h, 500km/h

  - Polar code with 8+3 CRC (note: TBCC is not precluded)

  - Practical channel estimation and ideal noise estimation