3GPP TSG RAN WG1 Meeting 90bis	          		R1-1718456

Prague, Czech Republic, 9 th – 13th Oct 2017

Agenda item:	7.2.2.6

Source: 	National Instruments

Title: 	Further Details of Beam Failure Handling

Document for:	Discussion and Decision

# Introduction

Significant progress has been made on beam management and beam recovery topics in the last few RAN1 meetings. The beam recovery procedure has been divided into its component parts, namely beam failure detection, identification of new candidate beam, transmission of beam recovery request and the gNB response to the beam recovery request.

In this contribution, we study the relationship between beam failure, beam broadening and beam refinement and techniques to reduce the number of required beam failure recovery request transmissions.

During RAN1#90 meeting, the following agreements have been made [1]:

Agreements:

- Beam failure is declared only when all serving control channels fail.

- When a subset of serving control channels fail, this event should also be handled

  - Details FFS

Agreements:

- After entering RRC CONNECTED mode, UE assumes the DM-RS ports of UE-specific PDCCH to be spatially QCL-ed with the SS block identified during initial access by default unless indicated otherwise

…

- In RRC CONNECTED mode,

  - RRC only or RRC+ MAC CE signaling is used to indicate QCL relation (at least w.r.t spatial RX parameters) between DMRS port(s) of UE-specific PDCCH and either SS Block or P/SP CSI-RS resource(s)

    - FFS: necessity of DCI signalling

    - FFS: Use of Aperiodic CSI-RS

  - FFS: RAN1 to study impact of delay (including determination of Rx beam) and reliability on the beam mgmt scenarios aiming to provide input to RAN2 with regard to higher layer protocols

Agreements:

- Support L1-RSRP reporting of measurements on SS block for beam management procedures

- The following configurations for L1-RSRP reporting for beam management are supported

  - SS block only (with mandatory support by UE)

  - CSI-RS only (with mandatory support by UE)

  - SS block + CSI-RS independent L1 RSRP reporting

    - Joint L1-RSRP using QCL-ed SS-block + CSI-RS is optionally supported by UE (with optionally support by UE)

# Overview of Beam Management & Recovery Procedure

The interplay of beam management, beam recovery and radio link failure can be seen in the Figure 1 below, for more details see [3].

Figure 1 Beam management and Beam recovery

In order to allow flexible integration of beam management into procedures like RLM, RLF or link adaptation algorithms, a set of configurable conditions will be beneficial to trigger different actions such as

- Beam refinement,

- Beam broadening and

- Beam failure recovery.

The actions are supported by various functionalities which are implemented on either the gNB, the UE or can be common to both. In the rest of this section, we will discuss two specific new functionalities to aid in optimal beam management and recovery:

- Multiple threshold settings (Section 2.1) and

- Beam broadening (Section 2.2).

## Threshold Setting for Beam Management

The beam triggering process has to robustly and efficiently handle multiple types of tasks such as beam failure as well as beam tracking, beam refinement and beam broadening while minimizing the latency introduced due to these tasks. We propose to support multiple thresholds at the UE and/or  gNB as shown in Figure 2:

- Beam refinement threshold: This threshold is used to trigger at least the beam refinement through a P2/P3 procedure.

- Control channel beam fallback threshold: This threshold is used to trigger at least the beam broadening procedure as introduced in the next section. Beam broadening is proposed to be attempted before a control channel beam failure is triggered.

- Control channel beam failure threshold:  This threshold is used to trigger at least the beam failure recovery when the control channel beam(s) fails.

Figure 2: Beam management and beam recovery

Observation 1: Scenarios that might lead to beam failure might differ in terms of how fast and how severely the beam quality deteriorates.

Proposal 1: A trigger condition consists of a measurement metric related threshold and a timer for which the measurement metric violates the threshold.

Proposal 2: A set of (at least) three trigger conditions that might have different thresholds and timers is defined.

Proposal 3: The threshold and the timer of each of the trigger conditions can be reconfigured.

## Beam Broadening using Nested RS structure

Nested RS Structure:

The concept of a nested RS structure for beam management has been previously introduced in [2]. An example of the nested structure is shown below in the Figs. 3 and 4.

SS#1

SS#2

SS#N

CSI-RS #1

CSI-RS #4

CSI-RS #8

Fig. 3(a): wide beam RS (i.e SS) 					Fig. 3(b): fine beam RS (i.e CSI-RS)

Fig. 4(a): CSI-RS#1-#4 correspond to SS#1		Fig. 4(b): CSI-RS#5-#8 correspond to SS#2

Beam Broadening:

A symmetric mechanism is to use this nested structure of multiple CSI-RS that are QCLed with a SS beam to allow for a graceful mitigation with low latency from a beam quality deterioration without triggering a beam failure recovery request transmission. For example, when a PDCCH that is configured with a narrow beam CSI-RS satisfies the control channel fallback threshold, the narrow beam is lost due to gradual motion or rotation. In this case, it is quite likely that the neighboring beams will be good enough and we can think of broadening the beam that is serving the UE. The SS within which the CSI-RS is nested can be used. Details of the beam broadening procedure are submitted in our previous contribution [3].

Proposal 4: Support nested RS structure for the beams of CSI-RS nesting within the beams of SS block.

Proposal 5: Support beam broadening to reduce transmission of beam recovery requests.

# Details of Improved Beam Failure Handling

## Control Channel Beam Fallback to Mitigate Beam Failure

It has been agreed in RAN1#90 that in the RRC CONNECTED state, the UE PDCCH DMRS is QCLed with the SS beam identified during initial access by default.  After the UE is in the RRC CONNECTED state, it will be configured with UE-specific CSI-RS beams and it can also be configured with additional control channel links for robustness. This can be optionally followed by a P2 beam refinement for the control channel beam(s). DMRS port(s) of each UE-specific PDCCH will have a QCL relationship with either SS Block or P/SP CSI-RS resource(s).

Assuming a nested RS structure as introduced in the previous section, each CSI-RS resource is nested within a SS Block.  In RAN1#90 significant progress was achieved in defining beam failure wherein beam failure is declared only when all serving control channels fail, and when such a beam failure occurs, it will trigger a beam failure recovery request transmission.

If all serving control channels are QCLed with their respective SS block beams then the above definition of beam failure is applicable in a straightforward manner. But consider the situation where at least some of the serving control channels are QCLed with UE-specific CSI-RS beams. In this case, the control channel may have failed for the narrow CSI-RS beam, but it quite likely that it may not have failed for the broader SS block beam that has a nesting relationship with the narrower CSI-RS beam.  Beam broadening should be considered when applicable on all serving control channel beams before considering it to be a failed control channel.

Observation 2: When the PDCCH is associated with a UE-specific CSI-RS as its DL RS, it should attempt to broaden its serving beam before proceeding with normal beam recovery process to reduce the beam recovery latency.

Proposal 6: Beam broadening should be considered when applicable on all serving control channel beams before considering it to be a failed control channel.

## Roles of UE and gNB in Beam Recovery

In the previous subsection, control channel beam fallback via beam broadening has been introduced to mitigate beam failure and to reduce the number of beam failure recovery request transmissions. Control channel beam fallback can minimize latency and increase reliability of beam failure handling. There are two alternative approaches to control the beam failure handling, namely, UE initiated beam failure recovery and gNB initiated beam failure recovery. Both UE-controlled and gNB-controlled beam failure triggering also affect beam recovery latency in varying ways, see [3].

Observation 3: Beam failure recovery can be initiated by either the UE or gNB.  In some cases, the gNB initiated will actually lower recovery time.

Proposal 7:  Support both UE-controlled and gNB-controlled beam failure triggering in the specification for beam failure recovery.

Proposal 8: Study further the impact of higher layer procedures on latency of beam failure recovery using control channel beam fallback and UE-controlled and gNB-controlled beam failure triggering of beam failure recovery.

# Conclusion

In this contribution the following observations and proposals have been presented:

Observation 1: Scenarios that might lead to beam failure might differ in terms of how fast and how severely the beam quality deteriorates.

Observation 2: When the PDCCH is associated with a UE-specific CSI-RS as its DL RS, it can attempt to broaden its serving beam before proceeding with normal beam recovery process to reduce the beam recovery latency.

Observation 3: Beam failure recovery can be initiated by either the UE or gNB.  In some cases, the gNB initiated will actually lower recovery time.

Proposal 1: A trigger condition consists of a measurement metric related threshold and a timer for which the measurement metric violates the threshold.

Proposal 2: A set of (at least) two trigger conditions that might have different thresholds and timers is defined.

Proposal 3: The threshold and the timer of each of the trigger conditions can be reconfigured.

Proposal 4: Support nested RS structure for the beams of CSI-RS nesting within the beams of SS block.

Proposal 5: Support beam broadening to reduce transmission of beam recovery requests.

Proposal 6: Beam broadening should be considered when applicable on all serving control channel beams before considering it to be a failed control channel.

Proposal 7:  Support both UE-controlled and gNB-controlled beam failure triggering in the specification for beam failure recovery.

Proposal 8: Study further the impact of higher layer procedures on latency of beam failure recovery using control channel beam fallback and UE-controlled and gNB-controlled beam failure triggering of beam failure recovery.

# References

[1]	    Chairman’s Notes, RAN1#90, Final Version, Prague, Aug 2017

[2]    R1-1708270, ”Discussion on Nested Usage of RS for Beam Measurement”, National Instruments

[3]    R1-1714180, ”On remaining aspects of beam recovery”, National Instruments