3GPP TSG RAN WG1 Meeting #91		R1-1720564

Reno, Nevada, USA, 27th November – 1st December 2017

Source:		SoftBank

Title:	Support of new NPRACH power control mechanisms

Agenda Item:	6.1.6

Document for:	Discussion and Decision

Introduction

Power control issue on NPRACH has been long time, and the following working assumption was agreed in RAN1#90bis.

Agreements:

- Working assumption: introduce the following solutions for Rel-14 to reduce the possible uplink interference caused by the NB-IoT UE which has level ramped from NPRACH level 0 in a RACH procedure.

  - For a Rel-14 UE which has level ramped from NPRACH level 0 in a RACH procedure, use the NPRACH level 0 power control  and power ramping procedure in the new NPRACH level, i.e. based on path-loss with power ramping.

  - Network can configure a cell-specific Δ additional to the NRSRP threshold of CE level 0 to indicate that the UE shall not perform level ramping when it measures NRSRP higher than “NRSRP threshold of CE level 0 + Δ”, Δ from the set {0, 10, 20, 30} dB.

    - The UE above which does not perform level ramping shall consider the Random Access procedure unsuccessfully completed when it fails at the maximum number of attempts (i.e. maxNumPreambleAttemptCE) configured in CE level 0.

  - Adopt the solutions above as an optional UE capability.

  - FFS how to fix the Msg3 power control for Rel-14 UE measured in CE level 0.

This contribution shows further proposals on top of the working assumption above.

Proposals

As pointed out by [1] , that the NPRACH power control issue in Rel-13 may cause a blocking issue at the eNB receiver side. This issue is very serious because not only NB-IoT but also LTE in the same band will be affected when this issue happens. Therefore, we prefer to apply the new mechanism to as many cases as possible, and the following proposal is made consequently:

Proposal 1

The new functionalities should apply to (A) both IDLE and CONNECTED mode, and (B) anchor and non-anchor carrier.

RAN1 (and RAN2) to discuss whether these new functionalies can allow early implementation in Rel-13.

It is noted that random access on non-anchor is new in Rel-14. Thus no issue is foreseen for non-anchor carrier even when the new functionalities are mandated for non-anchor carrier. It is also preferable to unify the UE behaviour for the stable operations. We therefore propose the following:

Proposal 2

RAN1 to discuss whether the new functionality can be mandated for random access on non-anchor carrier.

References

- R1-1716984, On Rel-14 NB-IoT RACH power control	Huawei, HiSilicon