3GPP TSG RAN WG1 Meeting #92		     	R1-1802500

Athens, Greece, February 26th – March 2nd, 2018

Source:	NTT DOCOMO, INC.

Title:	Discussion on performance comparison for NOMA

Agenda Item:	7.4.4

Document for: 	Discussion and Decision

- Introduction

In the study item of NR in Rel-14, it was agreed that:

- NR targets to support UL non-orthogonal multiple access, in addition to the orthogonal approach, targeting at least for mMTC.

In the SID of NOMA in Rel-15 [2], NOMA targets not only mMTC but also URLLC and eMBB (small packet).

- Link and system level performance evaluation or analysis for non-orthogonal multiple access continued from performance metrics identified from Rel-14. The benchmark for comparison is OFDM contention based multiple access. Realistic modelling of Tx/Rx impairment including potential PAPR issue, channel estimation error, power control accuracy, collision, etc. should be considered. [RAN1, RAN2]

  - Traffic model and Deployment scenarios of eMBB (small packet), URLLC and mMTC

Note: targeting common solution for mMTC, URLLC and eMBB small packet.

During the Rel-14 study item of NR, the comparison of various NOMA schemes was carried out independently, where the evaluation assumptions and transceiver design depend on each company. For Rel-15 study item of NOMA, to make more valid comparisons among NOMA schemes, the comparison methodology and assumptions should firstly be discussed.

In this contribution, we firstly discuss the usage scenarios for uplink NOMA and then discuss the comparison methodology. Finally, the link/system-level simulation (LLS/SLS) evaluation including assumptions and metrics is discussed.

- Usage scenarios

Based on the SID, the study of NOMA targets not only mMTC, but also URLLC and eMBB. In Rel-14 study item of NR, usage scenarios and corresponding requirements have been discussed. However, most discussions and evaluations focused on the connectivity capability and coverage for mMTC. Therefore, the discussion of NOMA study item in Rel-15 should also consider the usage scenario of mMTC.

For eMBB, system capacity and user throughput are most important requirements. From this point of view, NOMA is promising for eMBB since the capacity region of NOMA is larger than that of OMA as shown in [3]. The concern on NOMA for eMBB is that with the antenna setup in NR, whether NOMA can provide additional gain.

An analysis on spectral efficiency (SE) per user with MU-MIMO and power domain NOMA (PD-NOMA) for two users is provided in Table 1, where M and K denote the number of receive antennas at the gNB and active users transmitting at the same time and frequency resources, respectively, and Pk and Pk+K/2 denote the power of user k locating at cell-edge and user k+K/2 locating at cell-edge, respectively, i.e., Pk>Pk+K/2. In scheme 1, the signals of all the K users are detected by zero-forcing receiver in MU-MIMO as shown in Figure 1. The effective array gain is M-K+1 to cancel the interference from K-1 users as given in RkMIMO and Rk+K/2MIMO.  For scheme 2, the cell-center user k and cell-edge user k+K/2 are paired for PD-NOMA and detected by SIC receiver. In this case, only the interference caused by half users is cancelled by zero-forcing receiver in MU-MIMO and the effective array gain becomes M-K/2+1, which is larger than scheme 1. For conventional SIC receiver without outer iteration, cell-center users are interfered by users with lower powers. In this case, only when the number of users is large or power of cell-edge users is low, the performance of cell-edge users in scheme 2 (MIMO-NOMA) is better than scheme 1 (MU-MIMO). Based on the discussion of receiver, enhanced SIC receiver with outer iteration can be considered for NOMA. In this case, the interference caused by cell-edge users can also be cancelled for cell-center users and the SE of cell-center user in scheme 2 becomes log21+M-K2+1Pkσ2, which is always larger than log21+M-K+1Pkσ2 in scheme 1 as shown in Figure 2. Therefore, NOMA with advanced receivers can be used in eMBB for capacity enhancement and should be further discussed, where NOMA is not limited to power domain NOMA and existing NOMA schemes can also be used for further interference reduction and performance enhancement.

Table 1 Analysis on spectral efficiency per user for MU-MIMO and MIMO-NOMA

| UE location | Scheme 1: MU-MIMO with zero-forcing receiver | Scheme 2: MU-MIMO with zero-forcing receiver + PD-NOMA for 2 UEs with SIC receiver | Condition for Scheme 2 > Scheme 1 |
| --- | --- | --- | --- |
| Cell-center UE | RkMIMO=log21+M-K+1Pkσ2 | RkNOMA=log21+M-K2+1PkPk+K/2+σ2 for SIC receiver without outer iteration | K2M-K+1>Pk+K/2σ2: Large number of users or paired UE with low power |
|  |  | RkNOMA=log21+M-K2+1Pkσ2 for SIC receiver with outer iteration | Always |
| Cell-edge UE | Rk+K/2MIMO=log21+M-K+1Pk+K/2σ2 | Rk+K/2NOMA=log21+M-K2+1Pk+K/2σ2 | Always |

- Scheme 1: MU-MIMO                                                    (b) Scheme 2: MIMO-NOMA

Figure 1 Illustration of MU-MIMO and MIMO-NOMA for eMBB

Figure 2 Numerical results of SE gain, enSIC and SIC denote SIC with and without outer iteration, respectively, and PO denotes power offset between cell-center and cell-edge users

On the other hand, for eMBB (small packet), signalling cost is also one of the important KPI, which can be reduced by grant-free NOMA. Therefore, grant-based NOMA and grant-free NOMA should be both studied for usage scenario of eMBB.

In URLLC, retransmission is needed once the physical resources of multiple users collide. With the help of NOMA, the signals of multiple users in URLLC can transmit in same physical resources and retransmission can be reduced as long as the MA signature and physical resources of multiple users do not collide simultaneously. This indicates that the latency is reduced and the efficiency of resource utilization increases. However, the signals of multiple users interfere with each other, which may reduce the reliability for each successful transmission. Therefore, the reliability of NOMA for URLLC should be further studied.

Proposal 1: mMTC, URLLC and eMBB should be considered for uplink NOMA. Further study on each scenario is needed.

Proposal 2: Grant-based NOMA and grant-free NOMA should be both studied in Rel-15 NOMA SI at least for eMBB.

- Comparison methodology

To make an efficient discussion and performance comparison of NOMA in three scenarios, the comparison methodology should be discussed.

  - Unified structure at the transmitter

In the study item of NR in Rel-14, 15 NOMA schemes were proposed, which are listed in Table 2. The features of each NOMA scheme are also highlighted in Table 2. With the help of advanced receivers, signals of multiple users can be detected successfully.

Table 2 Existing NOMA schemes ordered by proposed time

| Scheme | Features | Reference |
| --- | --- | --- |
| SCMA | Multi-dimensional constellation, sparse symbol-to-RE mapping pattern | R1-162155 |
| RSMA | Symbol-level scrambler | R1-162202 |
| MUSA | Random complex spreading sequences | R1-162226 |
| PDMA | Symbol-to-RE mapping pattern | R1-162306 |
| LCRS | Low code rate, bit-level interleaver | R1-162385 |
| SSMA | Short spreading sequences | R1-166552 |
| NCMA | Grassmannian spreading sequences | R1-162517 |
| NOMA | Power domain, orthogonal spreading sequences | R1-163111 R1-167392 |
| IGMA | Bit-level interleaver, symbol-level grid mapping pattern | R1-163992 |
| LDS-SVE | Signature Vector Extension, RB-based sparse mapping pattern | R1-164329 |
| LSSA | Low code rate, user-specific bit-level permutation pattern, group RS pattern | R1-164869 |
| NOCA | LTE defined sequences for uplink RS | R1-165019 |
| IDMA | Low code rate, bit-level interleaver | R1-165021 |
| RDMA | Cyclic-shift Repetition, symbol-level interleaver | R1-167535 |
| GOCA | Orthogonal sequences in one group and non-orthogonal sequences for different groups | R1-167535 |

Even though various signatures are employed by different NOMA schemes, the principle of trying to distinguish users at the receiver by different MA signatures is common. Therefore, the transmitter of NOMA schemes can be unified into one common structure. In Rel-14 NR SI, NOMA schemes were summarized by a high-level block diagram, where bit level and symbol level operations are included. Starting from this, a unified structure at the transmitter consisting of five modules is shown in Figure 3.

- Bit-level interleaver/scrambler: For NOMA schemes with bit-level operations, user-specific bit-level interleaver/scrambler combined with low code rate channel coding is used to help multi-user detection (MUD) at the receiver.

- Bit-to-symbol mapping: This module maps the coded bits into modulated symbols, where no user-specific design is considered. For NOMA schemes except SCMA, conventional modulation schemes, e.g. BPSK and QPSK etc., are used. To further exploit the gain of this module, multi-dimensional modulation with shaping gain is used in SCMA.

- Symbol stream generation: Most NOMA schemes with symbol-level operations provide user-specific designs in this module, such as symbol-level interleaver and/or scrambler, user-specific spreading sequences and user- or group- specific powers as shown in Table 2.

- Symbol-to-RE mapping: The symbol stream can be mapped into either all the available physical resources (i.e., full mapping) or part of available physical resources (i.e., sparse mapping). Sparse mapping can reduce the collision on each RE at the cost of less transmission resources and user-specific mapping patterns can be designed to facilitate MUD. Full mapping can fully utilize the resources at the cost of more interference.

Existing NOMA schemes in Table 2 usually utilize one or multiple user-specific modules in Figure 3 as its MA signature to help receivers distinguishing multiple users.

Figure 3 Unified structure of NOMA schemes at the transmitter

In addition, NOMA schemes usually assume single-layer structure, i.e., the information bits are processed through the modules in Figure 3. For further performance enhancement, the information bits can be divided into multiple streams, where each stream is encoded with lower coding rate and lower modulation order. By incorporating the multiple streams with different signatures, e.g., power, sequences or scrambler etc., the signals of multiple layers can be detected at the receiver successfully. Besides, if the coding rates of multiple layers are different, the multi-layer structure could provide multiple reliabilities, which can increase the robustness and overall throughput in grant-free transmission.

Observation 1: In addition to single-layer structure, multi-layer structure can be considered for existing NOMA schemes.

  - Module-based comparison methodology

In Rel-14 SI, scheme-based performance comparison was employed and the performance of multiple NOMA schemes were obtained independently under different assumptions. This is possible for preliminary study of uplink NOMA. However, the objective of NOMA SI in Rel-15 is to “further progress on the evaluation of non-orthogonal multiple access schemes focusing on uplink, and provide recommendation on the non-orthogonal multiple access scheme(s) to be specified later.”[2]. To this end, transceiver designs for uplink NOMA should be further studied and evaluated to figure out the optimal combinations of modules at transmitter and receivers in each usage scenarios.

Instead of scheme-based performance comparison in Rel-14 SI, module-based performance comparison can be considered in Rel-15 SI. To investigate the performance gain of NOMA modules, the performance of baseline system without any NOMA schemes should be firstly evaluated. Then, the optimal NOMA scheme can be obtained by exhaustive searching on all the possible combinations of NOMA modules. For further simplification, one of the possible approaches is to compare the NOMA modules in Figure 3 successively based on some pre-defined priorities. The performance gain of each NOMA module is evaluated for all use scenarios. The optimal NOMA module and signature can be found by not only the performance gain but also latency and complexity at the transmitter and receiver, etc.

With the above module-based comparison methodology, the performance gain of each NOMA module and signature can be well studied,

Proposal 3: Module-based comparison methodology should be considered and further discussed in NOMA SI in Rel-15. The comparison metrics include at least the following:

- Performance

- Complexity at both transmitter and receiver

- Latency

- Others are not precluded

- Performance evaluation assumptions

Performance is one of the most important comparison metrics for NOMA study. For a fair comparison, the evaluation assumptions and performance metrics should be further discussed. As the evaluation in Rel-14, both LLS and SLS evaluations are necessary. Through LLS evaluations, the performance of all transceiver designs can be clearly compared. With SLS evaluations, the impacts, such as procedures, uplink power control, user distribution and random packet arrival, etc., on the performance of NOMA can be evaluated.

  - LLS evaluation

At the start of NOMA study item in Rel-15, LLS evaluation on the transceiver design should be prioritized. To make a valid comparison, the evaluation assumptions and performance metrics should be firstly determined.

    - Performance metric of LLS evaluation

In the Rel-14 study item of NR, the following evaluation metrics for LLS are considered [1].

- BLER vs SNR reported for UL and DL calibration

- BS and UE receiver complexity reported

- Sum throughput v.s. SNR at given BLER level under different overloading factor.

  - Overload factor is defined as

    - For spreading case:  number of data layers (users) / spreading length (number of REs)

    - For non-spreading case: number of data layers (users) on each RE

- Link budget (MCL with specific data rate)

Above metrics can be reused for Rel-15 study item of NOMA but the definitions of throughput, data rate and SNR should be defined in a clearer way. Besides, the specific performance metric and targets should be further discussed for each usage scenario.

For a clear and valid comparison, the performance metrics can be refined as:

- BLER v.s. SNR under

  - Given target BLER η, target per UE spectral efficiency R and number of UEs multiplexed in the same allocated bandwidth K  Compare the SNR gain

  - Given target BLER η and target per UE spectral efficiency R  Compare the maximal number of users or overloading factor

- Sum throughput k=1KR∙1-ηk v.s. SNR under

  - Different target per UE spectral efficiency R, different target BLER η, different number of users K  Compare the maximal sum throughput

To this end, the definition of spectral efficiency and the target values of BLER, per UE spectral efficiency for each usage scenario should be carefully discussed.

    - LLS evaluation assumptions

Based on [4], LLS assumptions in three usage scenarios are summarized in Table 4. Some further considerations are listed below.

- Usage scenarios

The evaluation assumptions for mMTC and eMBB are similar as listed in Table 4 except the channel coding. Therefore, the simulator for mMTC and eMBB can be reused to reduce the evaluation workload.

- Waveform

NR supports DFT-s-OFDM based waveform complementary to CP-OFDM waveform at least for uplink transmission without grant and eMBB uplink for up to 40GHz, while only DFT-s-OFDM is used for uplink transmission in LTE.

At the start of NOMA study item, CP-OFDM can be prioritized to reduce the workload of evaluation. However, considering that NR may start from DFT-s-OFDM for a smooth transition from LTE and coverage and low cost are important requirements of mMTC, NOMA with DFT-s-OFDM shouldn’t be precluded at this stage.

- Target BLER

Currently, 10% target BLER is considered for mMTC and eMBB and 0.1% target BLER is considered for URLLC in Table 4.

In the discussion of URLLC, BLER target of 10-5 is considered for NR and two BLER targets of 10-4 and 10-5 are discussed for LTE. Based on that discussion, the URLLC BLER target for NOMA may also change.

Besides, 10-2 BLER target is widely used in existing techniques and systems. To be consistent with URLLC, the mMTC and eMBB BLER target for NOMA may also change. The suggested BLER targets are listed in Table 3.

- Target per UE spectral efficiency

Target per UE spectral efficiency is an important factor which will affect performance. In current version of LLS assumptions, continuous values of target per UE spectral efficiency is considered, which introduces complexity for comparison. To reduce evaluation workload and compare fairly, some discrete values should be selected from the target range. Some example values are listed in Table 3 and can be further discussed.

Table 3 Target BLER and per UE spectral efficiency

| Parameters | mMTC | URLLC | eMBB |
| --- | --- | --- | --- |
| Target BLER for one transmission | 10%, 1% | 0.1%, 0.01% | 10%, 1% |
| Target per UE spectral efficiency | {0.1, 0.25, 0.5} for normal coverage {0.01, 0.05, 0.1} for extended coverage | {0.1, 0.25, 0.5} | {0.1, 0.25, 0.5} |

- Traffic model

For mMTC scenario, random user arrival is an important feature. Therefore, modeling of random user arrival should be considered during link-level simulations, which can evaluate the performance of different NOMA schemes in grant-free transmissions. Even though this can also be evaluated by system-level simulations, there are too many realistic factors, such as inter-cell interference, random user arrival and procedures related algorithms etc., that are twisted together, which makes it too difficult to observe their impacts separately. Therefore, both fixed and random number of users should be evaluated for mMTC scenario during link-level simulations.

To reduce the workload, simplified modeling of random user arrival can be considered. For example, under given distribution of user arrival, the probability of all possible number of users can be calculated. Then, the typical number of users with high probability can be evaluated. Finally, the performance weighted by the probabilities can be used for performance comparison of NOMA with random user arrival.

Proposal 4: BLER and sum throughput vs. SNR should be the performance metrics for link-level evaluations of NOMA in all scenarios but with different BLER targets and spectral efficiency targets. Random user arrival should be considered in link-level evaluations at least for mMTC.

  - SLS evaluation

In Rel-14 study item of NR, for system level simulation, the following were used as evaluation metrics [1]

- eMBB: TRP spectrum efficiency and 5th percentile user spectrum efficiency; user experienced data rate and area traffic capacity; signaling overhead

- mMTC: Connection density with “connection efficiency” reported; latency for infrequent small packets; signaling overhead

- URLLC: Reliability for a target latency

For the SLS evaluation for grant-free UL multiple access schemes applied to mMTC, packet drop rate vs. packet arrival rate per cell curve is used, where grant-free UL multiple access schemes has the following characteristics

- A transmission from UE does not need the dynamic and explicit scheduling grant from eNB

- Multiple UEs can share the same time and frequency resources

and packet drop rate is defined as (Number of packet in outage) / (number of generated packets), where a packet is in outage if this packet failed to be successfully received by destination receiver beyond “Packet dropping timer”.

Different from performance metrics for link-level evaluations, the performance metrics of three usage scenarios for system-level evaluation are different considering their specific requirements. These performance metrics can be reused by system-level evaluations in Rel-15. Besides, the assumptions can also start from the SLS evaluation assumptions in Rel-14.

Proposal 5: The performance metrics and assumptions for system-level evaluations of NOMA in Rel-14 can be reused.

- Conclusion

In this contribution, we discussed the use scenarios, comparison methodology, link-level and system-level performance metrics and assumptions for uplink non-orthogonal multiple access in Rel-15. The following observations and proposals are obtained:

Proposal 1: mMTC, URLLC and eMBB should be considered for uplink NOMA. Further study on each scenario is needed.

Proposal 2: Grant-based NOMA and grant-free NOMA should be both studied in Rel-15 NOMA SI at least for eMBB.

Observation 1: In addition to single-layer structure, multi-layer structure can be considered for existing NOMA schemes.

Proposal 3: Module-based comparison methodology should be considered and further discussed in NOMA SI in Rel-15. The comparison metrics include at least the following:

- Performance

- Complexity at both transmitter and receiver

- Latency

- Others are not precluded

Proposal 4: BLER and sum throughput vs. SNR should be the performance metrics for link-level evaluations of NOMA in all scenarios but with different BLER targets and spectral efficiency targets. Random user arrival should be considered in link-level evaluations at least for mMTC.

Proposal 5: The performance metrics and assumptions for system-level evaluations of NOMA in Rel-14 can be reused.

References

- 3GPP TR 38.802, “Study on New Radio Access Technology Physical Layer Aspects (Release 14)” , June 2017.

- RP-170829, ZTE, CATT, Intel, Samsung, “New Study Item proposal: Study on Non-orthogonal Multiple Access for NR”, Mar. 2017

- R1-1610076, NTT DOCOMO INC, “Initial views and evaluation results on non-orthogonal multiple access for NR uplink”, Oct. 2016

- RP-172386, ZTE, “NOMA Workshop Report”, Dec. 2017

Appendix

Table 4 Link-level evaluation assumptions [4]

| Parameters | mMTC |  | URLLC | eMBB |  | Further specified values reported |
| --- | --- | --- | --- | --- | --- | --- |
| Carrier Frequency | 2 GHz |  | 2 GHz | 2 GHz |  |  |
| Waveform (data part) | CP-OFDM and DFT-s-OFDM |  | CP-OFDM as starting point | CP-OFDM as starting point |  |  |
| Numerology (data part) | SCS = 15 kHz, #OS = 14 |  | SCS = 60 kHz #OS = 7 | SCS = 15 kHz #OS = 14 |  |  |
| Allocated bandwidth | 4 or 6 RB as baseline, single-tone, 1 RB as optional |  | 4 or 6 RB as baseline, 12 RB as optional | 4 or 6 RB as baseline, and 12 RB as optional |  | The same for non-orthogonal MA and baseline OFDMA |
| Target per UE spectral efficiency | [0.1-0.5] for normal coverage, [0.01-0.1] for extended coverage |  | [0.1-0.5] |  | [0.1-0.5] | The same total spectral efficiency (per UE SE * number of UEs) for non-orthogonal MA and OFDMA baseline. Company reports the MCS. Without short-term (per TTI) MCS adaptation. |
| Target BLER for one transmission | 10% |  | 0.1% |  | 10% |  |
| Number of UEs multiplexed in the same allocated bandwidth | To be reported by companies. |  | To be reported by companies |  | To be reported by companies | For OFDMA baseline, either simulate 1 UE per PRB (FDM for multiple UEs) and increase the MCS (per UE SE) accordingly, or keep the same number of UEs and MCS (resource collision is allowed). |
| BS antenna configuration | 2Rx as baseline 4Rx as optional |  | 2Rx as baseline 4Rx as optional |  | 2Rx as baseline 4Rx as optional |  |
| UE antenna configuration | 1Tx |  |  |  |  |  |
| Propagation channel & UE velocity | TDL-A 30ns and TDL-C 300ns in TR38.901, 3km/h |  |  |  |  |  |
| Max number of HARQ transmission | 1 as baseline |  |  |  |  | 1 |
| Channel estimation | Realistic channel estimation, Ideal channel estimation results should also be reported |  |  |  |  |  |
| MA signature allocation (for data) | Fixed/Random |  | Fixed/Random | Fixed/Random |  | Proponents report the detais of random MA signature allocation |
| DMRS allocation | Proponents report the details of DMRS, and whether DMRS is randomly selected by UE or pre-configured by gNB with potential DMRS collision. |  |  |  |  | NR Rel-15 DMRS overhead for the baseline OMA |
| Timing/frequency offset | 0 as starting point, |  | 0 as starting point | 0 as starting point |  | Non-zero timing and/or frequency offset to be considered later |
| Distribution of avg. SNR | Both equal and unequal | Equal |  |  | Both equal and unequal | For example, for unequal case, the long term SNR can have [3] values,30% users with x dB, 40% users with y dB, and 30% users with z dB |
| Receiver algorithm | Proponents provide details of receiver algorithms |  |  |  |  | MMSE-IRC for the baseline OMA |

Note: if 2-step RACH is evaluated, the assumption for TA estimation is that it should be within +/- 5us