3GPP TSG-RAN1 #92bis	R1-1805006

Sanya, China, April 16th – 20th, 2018

Source:	Ericsson

Title:	Evaluation methodology and metrics for NoMA

Agenda Item:	7.4.4

Document for:	Discussion

# Introduction

In this contribution, we discuss the evaluation methodology and performance metrics that are relevant for non-orthogonal multiple access (NOMA).  Remaining details of link level simulation parameters above those agreed in RAN1#92 are addressed.   System level performance metrics, and parameters are also proposed.

# Link-level evaluations

In NOMA evaluations, we should use metrics that are tailored to each use-case. For example, in eMBB, where spectral efficiency is the target, it makes sense to have sum data rate as the target metrics. In mMTC, where a high number of UEs should be supported at a certain BLER level, we could look at the UE capacity (that is, the ‘connection density’) that can be supported given a certain grade of service. This is also a relevant metric for low latency applications, yet for high reliability we believe that orthogonal access should be used.

When selecting which setups shall be considered for evaluation, we need to make sure that we avoid parameterizing the system in a way that could significantly inflate potential NOMA benefits. One such example is channel estimation (CE), whose impact on NOMA is much bigger in NOMA than in OMA. The reason is that for orthogonal transmission, the CE is only impaired by noise whereas in NOMA we further have the effect of pilot contamination; the latter is sensitive to channel dispersion, i.e., the more dispersive the channel, the more interference will be experienced on a UE DMRS sequence by other UE DMRS sequences. Ideal CE would significantly underemphasize these issues and, therefore, should not be considered. Other assumptions that inflate NOMA gains should also be captured; for example, in a grant-free scenario where UL time alignment is not guaranteed for all cell sizes and CP values, we should consider or the impact of solutions such as the added overhead of eCP, if employed for UL-NOMA.

It is important to keep in mind that while link level simulations are of course needed, they are generally not suitable for determining the net performance of multiple access schemes, since they tend to examine fixed sets of channel and interference conditions.  As such, it is not necessary to determine gains over e.g. baseline schemes at the link level, since this can and should be done at the system level.  As agreed in RAN1#92, comparison to baseline schemes such as OMA can be done for calibration purposes only, since this can help align companies’ simulation results.

Observations:

- Impairments such as timing, frequency, and power control error are essential parts of the NOMA study, since these impairments can impact different transmission schemes to different degrees.

- It is not necessary to develop a baseline scheme for link level gain determination.

  - Conclusions on the net benefits of NOMA schemes should only be drawn at the system level.

Based on these considerations, we summarize our proposed LLS evaluation framework in Table 1, covering the different use-cases.

Proposal:

- Link-level evaluation assumptions should be updated according to Table 1.

Table 1: Updates to link level simulation parameters from RAN1#92

| Parameters | Values or assumptions |
| --- | --- |
| CRC | 16 bits, added on top of TBS sizes, e.g. for MMTC resulting in [12, 22, 42, 62, 77] byte payloads |
| UE antenna configuration | 1Tx; 2 Tx may be used for URLLC |
| Power Control | Imperfect power control. |
| Frequency Error | 50 Hz for mMTC; other applications FFS. |
| Timing Error | For mMTC: uniform distribution within [0 Tx], where Tx is the max roundtrip propagation delay according to cell radius. Other applications FFS. |
| HARQ processes | 1, 4 for mMTC & eMBB; 1,2 for URLLC |

## Initial Link Level Simulations

In this we present initial simulation results and make some observations based on the them. The Table below shows the main simulation assumptions.

Table 2: Updates to link level simulation parameters from RAN1#92

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 700 MHz |
| Waveform | OFDM |
| Numerology | 15KHz |
| Modulation | 16-QAM |
| BS antenna configuration | 4 Rx |
| UE antenna configuration | 1Tx, speed 3kmph |
| Power Control | Equal long-term SNR (small variations remain). |
| Channel estimation | Ideal CSI |
| Power budget / UE | 12 subcarriers |
| Transmission Time | 2 slots (144 data OFDM symbols + 14 overhead) (6 data OFDM symbols +1 DMRS OFDM symbols) per slot |
| Transmission Bandwidth | 8 PRBs |
| NOMA Receiver | Ordered MMSE-SIC |
| Transport Block Size (TBS) | [10, 20, 40, 60, 75] bytes |
| Channel coding | Turbo-code with the code-rate matched to meet the above TBS sizes and the total Resource Elements, 16 bits CRC |
| Channel Type | TDL-A, delay spread of 300 ns |

Figure 1 and Figure 2 show WSMA schemes with spreading lengths of L =4, 6, and for different transport block Sizes TBS= 20, 60, 75 bytes, with overloading factors ρ=200%, 300%, 400%. For the sake of comparison conventional orthogonal multiple access (COMA) is also shown in both figures.

Figure 1. Various WSMA schemes with spreading lengths  L =4, 6, Transport Block Sizes TBS= 60, 75 bytes, Overloading factors ρ=200%, 300%, 400%. COMA with a TBS=60 bytes is also shown for reference. The channel-coding code-rate is adapted to the TBS.

Figure 2. Results similar to Figure 1 are shown here but with TBS=20, 60 bytes. This provides insights into the performance at lower TBS where saturation is expected.

From the above results we can make some observations:

- For a given SNR, approximately same rate-per-RE may be achieved for various combinations of TBS, SS length L, number of UEs K and overloading factor ρ. E.g., K=12 may be served with wsma4 (L=4 and ρ=3) or with wsma6 (L=6 and ρ=2).

- It can be observed from Figure 2 that some choices of  L, K, ρ, and TBS have throughput vs. SNR curves that are relatively steep, such that as SNR increases the total system throughput increases quickly.  Therefore, a few sets of combinations may be needed for good performance.

- The deciding factor for selecting the SS length L and the overloading factor ρ, could start with identifying the number of UEs K and the TBS for each UE, which are assumed to be fixed during the implementation.

- Smaller SS lengths can perform better at higher overloading over a range of SNR values. E.g., for the same overloading, say 400%, wsma4 and wsma6 are serving 16 and 24 UEs respectively. Here, the rate-per-RE is lower for the latter (wsma6) when compared to the former (wsma4) for a range of SNR values. At very high SNR, the latter outperforms the former, where wsma4 has a saturated rate-per-RE. So the SS length must be mentioned along with its overloading factor.

Observation: It is desirable to have an overloaded WSMA based NOMA system that supports variable length SS. There is a trade-off between the operating SNR range and the SS length along with its overloading factor.

# System-level evaluations

## System level methodology and parameters

System level evaluations of UL NOMA schemes have been only briefly addressed in the NR NOMA study item so far, and so the evaluation methodology and parameters need further refinement.  We discuss some of the principal aspects that should be updated below:

- Baseline schemes should have comparable receiver complexity and allow spatial separability to be exploited

NOMA schemes are fundamentally MU-MIMO schemes: they enable multiple UE to transmit in the same resources and use multi-antenna reception.  The difference over MU-MIMO is that UE specific transmit signal processing is used in NOMA to make the UEs more separable.  Both MU-MIMO and NOMA can benefit from the use of advanced receivers, and the performance of both schemes can improve with multiple antennas.  Pure ‘OMA’ schemes that can’t benefit from spatial superposition are not likely to be suitable baseline schemes. Schemes supported in Rel-15 that allow non-orthogonal access without UE specific transmit signal processing for PUSCH are the most relevant baselines.

- Evaluations should use a traffic model with variably sized packets as well as with random arrival.

NOMA requires that multiple UEs transmit simultaneously on the same resources.  Therefore, the inter-user arrival and packet size statistics of the traffic model can greatly affect the performance of NOMA schemes.  Evaluated systems should take into account realistic effects such bursty traffic, and unequal resource allocation for NOMA UEs.  Full buffer or fixed packet sizes are not realistic.  This is not to say that full buffer models are not useful for calibration purpose; they should just not be used for the purpose of evaluating a NOMA scheme’s gains.

- Channel estimation must be realistic, take into account interference, and account for overhead

NOMA UE’s reference signals can mutually interfere similarly to their PUSCH, depending on if they can be allocated orthogonal resources or not.  They may also be interfered by other UEs in other cells.  Since NOMA schemes rely on interference canceling receivers to provide gains, the accuracy of the channel estimation is crucial to the scheme. High SINR channel estimates can require high RS overhead, and so there is a natural tradeoff between interference suppression and RS overhead.   Furthermore, RS overhead grows as the packet size decreases, and so is especially important to account for in mMTC applications.

- Antenna configurations should reflect realistic gNB configurations for the targeted use cases

At 700 MHz, two columns with two dual polarized antenna elements is sufficiently compact and is expected to be a common antenna configuration for a variety of scenarios and use cases.  The study of 700 MHz in NOMA is targeted for mMTC applications where cell range is particularly important.  Such use cases naturally employ as many receive antennas as can fit within size constraints, since this provides gains for nearly any type of traffic and independent of UE capability.  By contrast, new capacity enhancing transmission schemes such as NOMA require new UE capabilities and that these capabilities are supported by most of the UEs served by the network.  Therefore, capacity enhancing transmission schemes will tend to be of interest once opportunities to expand receive arrays are expended.  Then at higher frequencies than 700 MHz, the number of antenna elements that can fit into this same form factor will increase proportionately, and so 4 Rx elements per cell should be seen as a minimum number.

- DL traffic related assumptions are not needed

The NOMA study item focuses on the uplink, as agreed in RP-170829.  Moreover, the SI already is highly ambitious, considering mMTC, eMBB, and URLLC, grant based and grant free operation, etc.  Evaluating all of these possibilities with sufficiently realistic assumptions already seems highly challenging.  Therefore, if anything, the scope of the UL investigations should be further reduced.

- 5G channel models should be used, assuming a fixed antenna virtualization

These are the most state of the art and were specifically developed for NR studies.  While some of the models were not completely stable during initial NOMA evaluations; the 5G models are now complete, and are available in TR38.901.  Note that the models in 38.901 support active antenna systems used in FD-MIMO where antennas are composed of a 2D array of antenna elements.  Such antenna systems can have a large number of receive chains, and support high order MU-MIMO.  Since the spectral efficiency of such arrays is already very high, it is not expected arrays with many receive antennas (say more than 8) would benefit from the use of UL NOMA.  Consequently, we assume that fixed antenna virtualization is used on columns of the 2D arrays modeled in 38.901, resulting in the normal 17 dBi gain per antenna column that has been typically assumed in 3GPP.  The models in 38.901 can then use 8 elements per column to form such antennas.  Also, since we assume that FD-MIMO is not used, then the normal cable losses apply; the typical value of 3 dB is used.

- NOMA simulation assumptions for URLLC should be aligned with the URLLC study

The performance studies of Rel-15 URLLC consider a variety of issues, such as PDCCH reliability and interfering eMBB traffic, etc.  The simulation scenarios in the NOMA study should be carefully aligned with that work so that the NOMA results are relevant to what is actually specified in Rel-15 for URLLC.  Therefore, we propose that decisions on evaluation parameters for URLLC not be made in RAN1#92, allowing further progress on URLLC to enable NOMA evaluations to align better to those of URLLC.

Recommendations for detailed system level evaluation parameters are given in section 4.3.

Proposals:

- Baseline schemes have comparable receiver complexity to NOMA and allow spatial separability to be exploited

- Evaluations use a traffic model with variably sized packets as well as with random arrival.

- Channel estimation is realistic, takes into account interference, and accounts for overhead

- Focus evaluations on where the minimum number of receive antennas is no less than 4 in any scenario

  - 8 receive antennas should also be evaluated above 700 MHz

  - 2 receive antennas can be optionally evaluated

- DL traffic related assumptions are not developed

- 5G channel models are used for all system level evaluations

  - A fixed virtualization is used to model a classical (non-FDMIMO) antenna

- The system level simulation parameters given in section 4.3 are used

  - URLLC system level simulation assumptions decisions are deferred from RAN1#92, allowing NOMA URLLC evaluations to better align with those in the Rel-15 URLLC work

## Quality of service requirements for mMTC

TR 38.802 section 9.12  calls for an evaluation of packet drop vs. packet arrival rate per cell, where: “packet drop rate is defined as (Number of packet in outage) / (number of generated packets), where a packet is in outage if this packet failed to be successfully received by destination receiver beyond ‘Packet dropping timer’”.  Typically packet drop rate is modelled on link level, while on system level it is more appropriate to consider outage in terms of percentage non-served users. However, considering that it is expected that mMTC corresponds to small and infrequent data delivery it is our understanding that the packet drop rate requirement actually corresponds to an outage requirement of 1%. This implies that the connection density is to be evaluated at the point where 99% of all users are served by the investigated system.

Proposal

- The TR 38.802 packet drop rate metric is interpreted as an outage requirement of 1%, implying that the connection density (number of generated packets) should be provided at the point where 99% of all users are served by the system.

The meaning of a packet dropping timer depends on at what level in the protocol stack the timer is considered. We propose it to be defined as a timer started at the point where the higher layers in the device or the eNB triggers a connection attempt, and at expiration the user is considered as dropped.

Proposal

- A packet dropping timer should be started when upper layers triggers an attempt to access the system for the purpose of initiating a data transfer. The timer is terminated when the data has been delivered by the receiver.

The actual timer value is dependent on many factors such as agreed path loss and distribution of packet sizes. Massive MTC is also considered to be delay insensitive. With this in mind, we believe it is sufficient to declare the used packet drop timer when along with service latency statistics when presenting the connection density.

Proposal

- The packet drop timer is to be declared and service latency performance is to be presented when presenting the achieved connection density.

## System level simulation parameters

The following tables are based on those of R1-1610531, which were used in initial NOMA system level evaluations.

Table 3: General System Level Simulation Assumptions for mMTC

| Attributes | Values or assumptions |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |
| Inter-BS distance | 1732m (RMa) or 500m (UMa) |
| Carrier frequency | 700MHz |
| Simulation bandwidth | Companies report simulation bandwidth used in evaluation |
| Channel model | 5G RMa or UMa. |
| UE Tx power | Max 23dBm |
| BS antenna configuration | Rx: 4 ports (8 as optional) 2 xpol branches / column; column spacing = 0.5λ 2 columns (4 as optional) |
| BS element pattern | Follow the modeling of TR38.901 |
| BS antenna height | According to TR38.901 |
| BS antenna gain | 17 dBi |
| BS cable loss | 3 dB |
| BS receiver noise figure | 5 dB |
| UE antenna elements | 1Tx |
| UE antenna height | According to TR38.901 |
| UE antenna gain | -4dBi |
| UE distribution | According to TR38.901 |
| Baseline Scheme | MU-MIMO |
| BS receiver | Advanced receiver. Comparable complexity for NOMA and baseline. |
| UL power control | Companies report power control scheme |
| Channel estimation | Realistic |

Table 4: Traffic Model for mMTC

| Attributes | Value or assumptions |
| --- | --- |
| Data packet arrival rate per UE | Poisson arrival with arrival rate λ |
| Number of UEs per cell | Companies report the number of UEs per cell |
| Packet size | Follow TR45.820: 20-200 byte Pareto + {65 or 29} bytes higher layer protocol overhead; 29 is used if ROHC is supported. FFS: whether ROHC is or is not used. |
| Simulation Bandwidth | Companies report the simulation bandwidth |
| Target coverage | 99% of users are served at the target connection density Companies report service latency performance Companies report resource utilization |
|  | Companies report HARQ/retransmission assumption. |
| Packet dropping timer | Companies report value used |

Table 5: General System Level Simulation Assumptions for eMBB

| Attributes | dense urban (macro only) | rural |
| --- | --- | --- |
| Layout | Single frequency layer: Hex. Grid | Single layer: Hex. Grid |
| Inter-BS distance | Macro layer: 500m (UMa) or 200m (UMi) | 1732m |
| Carrier frequency | 4GHz for the single layer | 700MHz |
| Simulation bandwidth | 20MHz Note: For FDD, simulation BW is split equally between UL and DL |  |
| Channel model | 5G UMa and 5G UMi | 5G RMa |
| UE Tx power | 23dBm |  |
| BS antenna configurations | 4, 8, [16] Rx 2, 4, [8] columns | 4 Rx (8 optional) 2 columns (4 columns optional) |
|  | 2 xpol branches / column; column spacing = 0.5λ |  |
| BS antenna height | According to TR38.901 |  |
| BS antenna gain | 17 dBi |  |
| BS Cable Loss | 3 dB |  |
| BS receiver noise figure | 5 dB |  |
| UE antenna configuration | 1 TX |  |
| UE antenna height | According to TR38.901 |  |
| UE antenna gain | Follow the modeling of TR38.901 |  |
| UE receiver noise figure | 9dB |  |
| Traffic model | FTP model 1, 100kB or 500 kB packet size NOTE: Full buffer may be provided for simulation calibration purposes, but is not used for technical scheme down selection. |  |
| Traffic load (Resource utilization) | For baseline scheme: 50%, 75% and 25% (optional) Companies report service latency performance |  |
| UE distribution | According to 38.901 In the case of full buffer simulations for calibration only, 10 users per TRP is the baseline. 20 users or other value is not precluded. | According to 38.901 10 users per TRP for full buffer traffic used for calibration purposes only, other values not precluded |

# Conclusions

In this contribution, we have considered link and system level simulation setups for NOMA, including metrics needed in the evaluations.  We made the following observation and proposals:

Observation from link level results

- It is desirable to have an overloaded WSMA based NOMA system that supports variable length SS. There is a trade-off between the operating SNR range and the SS length along with its overloading factor.

- Proposals for Link-level evaluation

- Link-level evaluation assumptions agreed in RAN1#92 are updated according to Table 1.

Proposals for System Level Methodology and Parameters:

- Baseline schemes have comparable receiver complexity to NOMA and allow spatial separability to be exploited

- Evaluations use a traffic model with variably sized packets as well as with random arrival.

- Channel estimation is realistic, takes into account interference, and accounts for overhead

- Focus evaluations on where the minimum number of receive antennas is no less than 4 (in any scenario)

  - 8 receive antennas should also be evaluated above 700 MHz

  - 2 receive antennas can be optionally evaluated

- DL traffic related assumptions are not developed

- 5G channel models (i.e. those of 38.901) are used for all system level evaluations

  - A fixed virtualization is used to model a classical (non-FDMIMO) antenna

- The system level simulation parameters given in section 4.3 are used

  - URLLC system level simulation assumptions decisions are deferred from RAN1#92, allowing NOMA URLLC evaluations to better align with those in the Rel-15 URLLC work

Proposals for mMTC Quality of Service Requirements:

- The TR 38.802 packet drop rate metric is interpreted as an outage requirement of 1%, implying that the connection density (number of generated packets) should be provided at the point where 99% of all users are served by the system.

- A packet dropping timer should be started when upper layers triggers an attempt to access the system for the purpose of initiating a data transfer. The timer is terminated when the data has been delivered by the receiver.

- The packet drop timer is to be declared and service latency performance is to be presented when presenting the achieved connection density.