3GPP TSG RAN WG1 Meeting #93	R1-1805934

Busan, Korea, May 21 – 25, 2018

Agenda Item:	7.8.2

Source:	Huawei, HiSilicon

Title:	Considerations and evaluation results for IMT-2020 for mMTC connection density

Document for:	Discussion and decision

Introduction

In this contribution, we provide the consideration for the full system level simulation method and procedure for connection density evaluation, and present the initial evaluation results under the most stringent environment.

Connection density definition

According to Report ITU-R M.2412, connection density is said to be C (# of devices per km2), if, under the number of devices, N=C×A (A is the simulation area in terms of km2), that the packet outage rate is less than or equal to 1%, where the packet outage rate is defined as the ratio of

- The number of packets that failed to be delivered to the destination receiver within a transmission delay of less than or equal to 10s

to

- The total number of packets generated by the (N=C×A) devices within the time T.

The transmission delay of a packet is understood to be the delay from the time when uplink packet arrives at the device to the time when the packet is correctly received at the destination (BS) receiver.

In addition, it is encouraged that the self evaluation reports the connection efficiency which is given by

(# of device/Hz/TRxP)							(1)

where C is the connection density (# of devices per km2), A is the simulation area in terms of km2, M is the number of TRxP in the simulation area A, and W is the UL bandwidth (for FDD).

Procedure and delay modeling

To evaluate NB-IoT and eMTC, the procedure needs to be assumed for a packet transmission. Considering the packet arrival rate of a device is very sparse (1 message/day/device to 1 message/2 hours/device), it is appropriate to assume that the devices (both for NB-IoT and eMTC) are within idle mode when an uplink message packet arrives.

In [1], it is shown that legacy procedure and small data transmission procedure are available for both NB-IoT and eMTC. It is noted that ITU-R assumes a L2 packet size of 32 byte (i.e., 256 bit) for mMTC evaluation, which is typically within the range of small data packet. Therefore the small data transmission procedure is considered in this contribution for delay modeling. Besides, it is assumed that the devices are in non-initial state, that is, the SIB information is assumed to have been received by the devices. In this case, the SIB reception is ignored. This procedure is shown in Table 1.

Based on the understanding of transmission delay, Step 1 to Step 4 is considered to be contributing to the total transmission delay.

Table 1 Small data procedure of NB-IoT and eMTC under consideration

| NB-IoT |  |  |  | eMTC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Device |  | BS |  | Device |  | BS |
|  | Step1: Sync + MIB |  |  |  | Step1: Sync + MIB |  |
|  | Step 2: PRACH Msg1 |  |  |  | Step2: PRACH Msg1 |  |
|  | Step 3: NPDCCH + RAR (including UL grant) |  |  |  | Step3: MPDCCH + RAR (including UL grant) |  |
|  | Step 4: UL data transmission |  |  |  | Step4: UL data transmission |  |
|  | Step 5: HARQ Ack |  |  |  | Step5: HARQ Ack |  |

Evaluation method of full system level simulation

Generally, the system level simulation should evaluate each packet’s total delay tpacket. If tpacket > 10s, this packet is regarded as failed to be delivered to the destination receiver.

The total delay consists of the delays from Step 1 to 4,

where tUL_data is the UL data transmission time duration (for step 4), and t1, t2, and t3 are the time delay for step 1~3, respectively.

Conventionally, the value of tUL_data can be well derived in system level simulation.

For derivation of t1, t2, and t3, a full implementation of these steps may result in high complexity simulation. Therefore a simplified model is needed for Step 1 to 3. In the following sub-sections, the considered delay models are presented.

Delay Modeling of Step 1: Sync + MIB

The step 1 delay is given by

where tSS is the delay for synchronization, and tMIB is the delay of MIB reception.

Synchronization delay

For NB-IoT, NPSS and NSSS are transmitted for the device to conduct synchronization. NPSS is transmitted in sub-frame 5 of each frame. NSSS is transmitted in sub-frame 9 of every other frameas illustrated in Figure 1.

Figure 1 NPSS and NSSS for NB-IoT

Denote LNPSS and LNSSS as the repetition times needed to successfully accomplish primary synchronization and secondary synchronization, respectively. In this case, the synchronization delay is given by

tss=tPSS+tSSS

tPSS=tNPSS_0+LNPSS-1× TPSS

tSSS=tNSSS_0+LNSSS-1× TSSS

where tNPSS_0 = tNPSS-t0 is the time interval between the nearest NPSS transmission at tNPSS, and the packet arrival time, t0, TPSS =10ms is the transmission period of NPSS; tNSSS_0 = tNSSS-t0 is the time interval between the nearest NSSS transmission at tNSSS, and the packet arrival time, t0,  and TSSS=20ms is the transmission period for NSSS.

The value of LNPSS and LNSSS can be determined by DL SINR, based on link level simulation using TDL-iii channel model. The corresponding value of LNPSS/ LNSSS for a given SINR can be determined according to the 90th percentile point successful detection of NPSS/NSSS (under this SINR value).

The DL SINR for NPSS and NSSS of a device could be derived from DL wideband SINR (geometry) of that device.

PBCH receiving delay

NPBCH (for MIB) is transmitted in sub-frame 0 in every radio frame on anchor-PRB with at most 64 sub-frame combination as illustrated in Figure 2

Figure 2 NPBCH for NB-IoT

Denote LNPBCH as the repetition times for correctly receiving NPBCH. Hence, the receiving PBCH delay is given by

tnpbch=tNPBCH_0+LNPBCH-1× TNPBCH

where tNPBCH_0 = tNPBCH-t0 is the time interval between the nearest NPBCH transmission at tNPBCH, and the synchronization end time, t0, TNPBCH =10ms is the transmission period of NPBCH.

The value of LNPBCH can be determined by DL SINR, based on link level simulation using TDL-iii channel model (with QPSK, coding rate of 0.25, and code block size of 50bit). The corresponding SNR threshold for a given LNPBCH would guarantee larger than 90% successful reception ratio with LNPBCH times repetition of NPBCH reception.

The DL SINR could be derived from DL geometry of that device.

The example mapping of DL SINR to the value of L is illustrated in Figure 3.

1

k

K

L

DL SINR

SINR threshold 1

SINR threshold 2

Figure 3 Illustration of mapping curve of DL SINR to value of repetition time L

Delay Modeling of Step 2: PRACH Msg1

For PRACH delay model, it is dependent on two aspects. One is the number of collisions encountered by the device, ncollision. The other aspect is the time duration, tPRACH, for correctly receiving PRACH without collision. The latter is depending on UL SINR. Therefore the PRACH delay is given by

t2=f2(ncollision, tPRACH)

If collision happens during the PRACH transmission, all of the collided PRACH transmissions are assumed to be failed, and another round of PRACH transmission for the collided UEs is needed.

For NB-IoT, the UEs will transmit PRACH according to its CE level. The CE level is determined by its RSRP (see TS36.331). For UEs in a specific CE level, the time domain resource for PRACH could be configured according to TS36.331 by the transmission periodicity, transmission duration (or repetition times), and the transmission start time (within the period). If the three CE levels share the same PRACH frequency resource, the UEs with lower CE level could not use the PRACH resource that is overlapped with higher CE level. One illustration is shown in Figure 4.

Device starts the PRACH transmission at available transmission time according to its CE level as shown in colored box in Figure 4, and randomly selects one PRACH channel among the available number of channels (assumed to be 24 in this simulation).

Figure 4 Illustration of PRACH configuration for three CE levels 
(PRACH shares the same frequency) resource)

The NPRACH of each CE level has its own parameter nprach-Periodicity-r14 and nprach-StartTime-r14 to decide the NPRACH candidate opportunity (see Section 6.7.3 in TS36.331). The example configuration of NPRACH resource is shown as in Table 2.

Table 2 Example NPRACH configuration for each CE level

| Coverage Enhancement level | Repetition times of NPRACH transmission | Transmission Duration tPRACH_duration | Periodicity | NPRACH start time |
| --- | --- | --- | --- | --- |
| CE Level 0 | 2 | 11.2ms | 40ms | 8ms |
| CE Level 1 | 8 | 44.8ms | 80ms | 32ms |
| CE Level 2 | 32 | 179.2ms | 1280ms | 8ms |

If only one device occupies the channel at that time, it is assumed that BS could receive the PRACH correctly. If multiple UEs within the same CE level start the PRACH transmission at the same time (say t0), and all of them occupies the same sub-carrier (channel) at t0, these UEs are collided, and their PRACH reception at BS side would be failed.

When collision occurs, a backoff mechanism is used to avoid further collision to next transmission (see Section 5.1.5 in TS36.321). The backoff length (until the next PRACH transmission) consists of two parts: one is the backoff window with length of tbackoff, which is a random value between {0, Twindow}ms; the other part is the RAR window with the minimum length of 2×TPDCCH, where TPDCCH is the transmission period of NPDCCH in Step 3. In this case, the total latency for PRACH is given by

tPRACH=i=1ncollision+1tPRACH_0_i+tPRACH_duration_i+tbackoff_i+2×TPDCCH×i-1

where tPRACH_0_i is from the time when the device is ready to send PRACH to the time of the nearest PRACH transmission opportunity for this device, tPRACH_i is the transmission duration of the i-th PRACH transmission of the device (depending on UL SINR; tPRACH_i = LPRACH×TPRACH, where LPRACH is the NPRACH repetition time required by UL SINR, and TPRACH is the interval of preamble format 0), tbackoff_i is the window length of the i-th back off which is randomly selected in (0, Twindow)ms, where the value of Twindow could be configured as in Table 7.2-2 in TS 36.321, and TPDCCH is the period of common search space (CSS), and ncollision≥0 is number of collisions encountered by the device that is provided by the system level simulation.

Delay Modeling of Step 3: NPDCCH + RAR (including UL grant)

For NB-IoT, the Step 3 transmission (downlink) consists of NPDCCH transmission and RAR transmission through PDSCH,

t3=tNPDCCH+tRAR

where tNPDCCH is the delay for correctly receiving NPDCCH, and tRAR is the delay for correctly receiving RAR.

Scheduling scheme of NPDCCH and RAR

The scheduling of NPDCCH and RAR (transmitted on NPDSCH) are based on system level simulation. For a given time instance, NPDCCH is assumed to have higher scheduling priority over RAR transmission. However, once RAR transmission starts, it is assumed that this RAR transmission has the highest scheduling priority at that time instance.

NPDCCH delay

The NPDCCH transmission delay for a specific device consists of two parts, i.e., the scheduling delay and the transmission duration,

tNPDCCH=tNPDCCH_0+LNPDCCH-1× TTI

where tNPDCCH_0 = tNPDCCH_sche - t0 is the time interval between the available NPDCCH transmission for the specific device at tNPDCCH_sche, and the PRACH end time, t0, LNPDCCH is the repetition times for correctly receiving NPDCCH, and TTI=1ms. The value of tNPDCCH_sche is related to the scheduling scheme of NPDCCH and RAR.

If multiple devices which belong to the same CE level request PDCCH transmission at a specific time instance, they may share the NPDCCH resource. In this case, the value of LNPDCCH for device k will be aligned with the device that requests the largest value of LNPDCCH.

The value of LNDCCH can be determined by DL SINR, based on link level simulation using TDL-iii channel model. The corresponding SNR threshold for a given LNDCCH would guarantee larger than 90% successful reception ratio with LNDCCH times repetition of NPDCCH reception. It is noted that NPDCCH is transmitted using QPSK, coding rate of 0.128, and code block size of 39bit for the case of using 12 sub-carriers.

The DL SINR for NPBCH could be derived from DL geometry of that device.

RAR delay

The RAR transmission delay for a specific device consists of two parts, i.e., the scheduling delay and the transmission duration,

tRAR=tNPDSCH_0+LNPDSCH-1×TRAR

where tNPDSCH_0 = tNPDSCH_sche - t0 is the time interval between the available NPDSCH transmission for the specific device at tNPDCCH_sche, and the NPDCCH end time, t0, LNPDSCH is the repetition times for correctly receiving NPDSCH, and TRAR is the transmission duration for one RAR packet. The value of tNPDSCH_sche is related to the scheduling scheme of NPDCCH and RAR.

For the case of BS scheduling RAR for multiple UEs simultaneously, the RAR packet size would be nS where S=56bits is the size of a single RAR, and n denotes the number of scheduled  devices . The value of LNPDSCH and the MCS for device k will be aligned with the device which requests the largest value of LNPDSCH, and it could be derived from simulation according to RAR packet size and DL SINR, while DL SINR could be derived from DL wideband SINR.

For NB-IoT, one RAR transmission duration is derived by

tRAR=nSSE(MCS,OH)×180kHz×1000

where n is the number of scheduled devices for RAR transmission, S=56bit is the size of RAR for one device, SE is the expected spectral efficiency (bps/Hz) that is related to MCS and overhead OH. MCS is selected based on DL PDSCH SINR, which can be derived from DL wideband SINR. When multiple devices are scheduled (n>1), the MCS is selected based on the device that experiences the worst SINR.

Delay Modeling of Step 4: UL data

The UL data transmission is fully modeled in the system level simulation as in conventional system level simulation. UL data contains two parts, RRC connection request message (88bits) and UL traffic packet (256bits), so the total packet size is 344bits.To facilitate the system level simulation, only single-tone is used for scheduling, while UL data transmission is based on MAC scheduling according to UL resource utilization condition and UL SINR of devices. The transmission delay of step 4 could be depicted as the following.

tUL_data=i=1N+1tSCHED_i+tPUSCH_duration_i

where tSCHED_i is the scheduling delay for the i-th transmission, tPUSCH_duration_i is the transmission duration for i-th PUSCH transmission for the specific device. N is the retransmission times for UL data transmission. These values are derived by system level simulation.

Initial evaluation results

NB-IoT using the small data transmission procedure is evaluated following the above model. Configuration B of Urban Macro – mMTC test environment is selected for evaluation. The stringent traffic model of packet arrival rate of 1 message / 2 hours / device is chosen.

It is assumed that NB-IoT is working in standalone mode on anchor carrier. The initial results are shown in Table 2. The detailed simulation assumption is shown in Annex.

Table 2 Initial evaluation result for NB-IoT

| Devices supported per km2 per 180kHz | 607,000 |
| --- | --- |
| Required bandwidth to support 1,000,000 devices | 360kHz |
| Connection efficiency (#of devices/Hz/TRxP) | 2.92 devices/Hz/TRxP |

Conclusion

In this contribution, the consideration on self evaluation for mMTC is provided. The initial consideration for the full system level simulation, including the evaluation method, related delay modeling for small data procedure, and initial evaluation results under the most stringent environment are presented.

Initial evaluation demonstrates that NB-IoT could fulfill the mMTC connection density requirements under the most stringent environment defined in ITU-R.

It is noted that eMTC evaluation may follow a similar way.

Proposal: Take the procedure and delay models as in Section 3 as starting point for connection density evaluation for NB-IoT.

- References

- R1- 1719421, “Consideration on self evaluation of mMTC for IMT-2020”, Huawei, HiSilicon, November 2017.

- Annex       Simulation assumption

Table A-1 system level simulation assumption

| Urban Macro - mMTC | Config. B |
| --- | --- |
| Carrier frequency for evaluation | 700 MHz |
| BS antenna height | 25 m |
| Total transmit power per TRxP | 43 dBm on 180kHz |
| Device power class | 23 dBm |
| Inter-site distance | 1732 m |
| Number of antenna elements per TRxP | 16 Tx/Rx, (M,N,P,Mg,Ng) = (8,1,2,1,1), (dH,dV) = (N/A, 0.8)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 2TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) |
| Number of device antenna elements | 1Tx/Rx 0° polarization |
| Number of TXRU per device | 1TXRU |
| Device deployment | 80% indoor, 20% outdoor Randomly and uniformly distributed over the area |
| Device mobility model | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction. |
| Device speeds of interest | 3 km/h for indoor and outdoor |
| Inter-site interference modeling | Explicitly modelled |
| BS noise figure | 5 dB |
| Device noise figure | 7 dB |
| BS antenna element gain | 8 dBi |
| Device antenna element gain | 0 dBi |
| Thermal noise level | -174 dBm/Hz |
| Traffic model | With layer 2 PDU (Protocol Data Unit) message size of 32 bytes: 1 message/2 hours/device Packet arrival follows Poisson arrival process |
| Device antenna height | 1.5 m |
| Channel model | Channel model A |
| TRxP number per site | 3 |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | 93° in LCS |
| TRxP boresight | 30 / 150 / 270 degrees |
| UT attachment | Based on RSRP from port 0 |
| Wrapping around method | Geographical distance based wrapping |
| Minimum distance of TRxP and device | d2D_min=10m |
| Polarized antenna model | Model-2 in TR36.873 |
| Simulation bandwidth | 180 kHz |
| Sub-carrier spacing for PDCCH, PDSCH | 15 kHz |
| Sub-carrier spacing for PUSCH | 15 kHz |
| PRACH | 90kHz with 24 sub-carriers (channels) in 180 kHz BW, 3.75kHz sub-carrier spacing for PRACH Back off window Twindow=512ms |
| UL DMRS | 2 symbols per 14 OFDM symbols |
| PUSCH scheduling unit | Single tone (15kHz) |
| NPDCCH period TPDCCH | CE level 0: 24ms, CE Level 1: 48ms CE level 2: 96ms |