3GPP TSG-RAN WG1 Meeting #93	 R1-1806016

Busan, Korea, May 21st – 25th, 2018	Revision of R1-1803933

Agenda Item:	7.2.2

Source:	Ericsson

Title:	Evaluation of UP Latency in NR

Document for:	Information

# Introduction

The ITU target for user plane (UP) latency in IMT 2020 has been set to 1ms [1]. In this paper, we make an evaluation of the UP latency in NR with different configurations, and show that the target can be reached in FDD. In the ITU target for reliability there is also a requirement on latency (1ms) during which the packet should be delivered with a certain probability. For this case, it is useful to evaluate how many retransmissions can be done within the latency limit.

# Discussion

In the following we will analyze the worst-case UP latency in FDD after a first transmission and up to 3 HARQ retransmissions. We will follow the ITU definition [1] of UP latency as being defined from L2/L3 ingress to L2/L3 egress.

## Assumptions

Processing delay

For the UE, the processing delay is assumed to be according to processing delay capability 2, that is 3 symbols for 15 and 30kHz SCS. For 120kHz the capability has been assumed to be 9 symbols. In the gNB we have here assumed the same processing delay as in the UE, for simplicity. It is assumed that data can be delivered to higher layers after processing but before HARQ-ACK feedback is transmitted.

Alignment delay

The alignment delay is the time required after being ready to transmit until transmission can start. We assume the worst-case latency meaning the alignment delay is assumed to the longest possible.

gNB timing

The minimum response timing in the gNB between SR and UL grant, and between DL HARQ and retransmission, is assumed to be n+1 TTI. For higher SCS and fewer symbols in the mini-slot, the TTI is shorter, and more TTIs should be used for processing. The used timing assumptions for gNB are given in Table 1.

UE timing

The minimum response timing in the UE between DL data and DL HARQ, and between UL grant and UL data, is assumed to be n+1 TTI. As for the gNB timing this value is higher for higher SCS and shorter mini-slot according to the processing delay capability 2, see Table 1.

UL scheduling

For UL data, the scheduling can either be based on SR or SPS UL. In both cases it is assumed that the period is set to 1TTI.

Table 1. Timing assumptions.

| Timing | 15/30kHz SCS |  |  | 120kHz SCS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| #TTI | 7os TTI | 4os TTI | 2os TTI | 7os TTI | 4os TTI | 2os TTI |
| gNB processing | 1 | 1 | 2 | 2 | 3 | 5 |
| UL timing | 1 | 1 | 2 | 2 | 3 | 5 |

TTI length and pattern

In this evaluation, we study FDD slot lengths of 14 symbols as well as mini-slots of 7, 4, and 2 symbols.

Figure 1: Illustration of latency components for DL and UL data.

## HARQ retransmissions

For the case of FDD the HARQ RTT is here n+k TTI according to Table 1. The resulting UP latencies are shown in Table 2. As can be seen, the 1ms requirement can be reached for SCS 15kHz and up with a one-shot transmission. With HARQ retransmissions 30kHz SCS or higher is required.

Table 2: FDD UP one-way latency for data transmission with HARQ-based retransmission.

| Latency (ms) | HARQ | 15kHz SCS |  |  |  | 30kHz SCS |  |  |  | 120kHz SCS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI |
| DL data | 1st transmission | 2.4 | 1.4 | 1.0 | 0.71 | 1.2 | 0.71 | 0.50 | 0.36 | 0.41 | 0.29 | 0.23 | 0.20 |
|  | 1 retx | 5.4 | 2.9 | 1.9 | 1.4 | 2.7 | 1.5 | 0.93 | 0.71 | 0.79 | 0.60 | 0.48 | 0.39 |
|  | 2 retx | 8.4 | 4.4 | 2.7 | 2.1 | 4.2 | 2.2 | 1.4 | 1.1 | 1.2 | 0.91 | 0.73 | 0.59 |
|  | 3 retx | 11 | 5.9 | 3.6 | 2.9 | 5.7 | 3.0 | 1.8 | 1.4 | 1.5 | 1.2 | 0.98 | 0.79 |
| UL data (SR) | 1st transmission | 4.5 | 2.5 | 1.6 | 1.4 | 2.3 | 1.3 | 0.82 | 0.68 | 0.67 | 0.54 | 0.46 | 0.38 |
|  | 1 retx | 8.4 | 4.4 | 2.7 | 2.1 | 4.2 | 2.2 | 1.4 | 1.1 | 1.2 | 0.91 | 0.73 | 0.59 |
|  | 2 retx | 11 | 5.9 | 3.6 | 2.9 | 5.7 | 3.0 | 1.8 | 1.4 | 1.5 | 1.2 | 0.98 | 0.79 |
|  | 3 retx | 14 | 7.4 | 4.4 | 3.6 | 7.2 | 3.7 | 2.2 | 1.8 | 1.9 | 1.5 | 1.2 | 0.98 |
| UL data (SPS) | 1st transmission | 2.4 | 1.4 | 1.0 | 0.71 | 1.2 | 0.71 | 0.50 | 0.36 | 0.41 | 0.29 | 0.23 | 0.20 |
|  | 1 retx | 5.4 | 2.9 | 1.9 | 1.4 | 2.7 | 1.5 | 0.93 | 0.71 | 0.79 | 0.60 | 0.48 | 0.39 |
|  | 2 retx | 8.4 | 4.4 | 2.7 | 2.1 | 4.2 | 2.2 | 1.4 | 1.1 | 1.2 | 0.91 | 0.73 | 0.59 |
|  | 3 retx | 11 | 5.9 | 3.6 | 2.9 | 5.7 | 3.0 | 1.8 | 1.4 | 1.5 | 1.2 | 0.98 | 0.79 |

With slot transmission 120kHz is required to fulfill the 1ms target.

With mini-slots NR FDD can fulfill the 1ms UP latency target with SCS 15kHz and higher.

With mini-slots HARQ retransmissions can be done within the latency target with 30kHz SCS or higher.

# Conclusion

In section 2 we made the following observations:

- With slot transmission 120kHz is required to fulfill the 1ms target.

With mini-slots NR FDD can fulfill the 1ms UP latency target with SCS 15kHz and higher.

With mini-slots HARQ retransmissions can be done within the latency target with 30kHz SCS or higher.

# References

ITU-R M.[IMT-2020-TECH PERF REQ] (WP5D-TD-0300)