3GPP TSG RAN WG1 Meeting #93                                                                                R1-1806306

Busan, Korea, April May 21st – 25th, 2018

Source:	CATT

Title:	Discussion on NOMA receivers

Agenda Item:	7.4.2

Document for:	Discussion and Decision

Introduction

In RAN1#92bis, the NOMA receivers were discussed with the following agreements.

Agreements:

Adopt Figure 1 as the general block diagram of multi-user receiver for UL data transmissions.

- The algorithms for the detector block (for data) can be e.g. MMSE, MF, ESE, MAP, MPA, EPA.

- The interference cancellation can be hard, soft, or hybrid, and can be implemented in serial, parallel, or hybrid.

  - Note: the IC block may consist of an input of the received signal for some types of IC implementations

- The interference cancellation block may or may not be used.

  - Note: if not used, an input of interferene estimation to the decoder may be required for some cases.

- The input to interference cancellation may come directly from the Detector for some cases

Figure 1 A high-level block diagram of multi-user receiver

In [1], potential receivers suitable for PDMA are analyzed and compared from complexity perspective.

In this contribution, we provide initial simulation results for PDMA with different receivers.

Potential receivers for NOMA

According to the agreements, the algorithms for the detector block (for data) can be e.g. MMSE, MF, ESE, MAP, MPA, EPA and interference cancellation (IC) can be hard, soft or hybrid implemented in serial, parallel or hybrid.

Figure 2 gives illustration of NOMA receivers with MMSE-SIC, MPA/EPA-PIC with soft IC and MPA/EPA-PIC with hybrid soft IC and hard IC.

- MMSE-SIC                                                   (b) MPA-PIC / EPA-PIC with soft IC

(c) MPA-PIC / EPA-PIC with hybrid soft IC and hard IC

Figure 2: Illustration of NOMA receivers

The computation complexity per modulation symbol are summarized in Table 1, where denotes the size of modulation constellation,  is the product of the number of receiving antennas and spreading factor (SF), ,   and  represent inner iteration number of MPA/EPA, and maximum row weight of PDMA pattern matrix, respectively.

Table 1: Computation complexity per modulation symbol per outer loop iteration

| Receiver algorithm | Complexity order (only dominant part is considered) |
| --- | --- |
| MMSE-SIC |  |
| MPA- PIC |  |
| EPA -PIC |  |

If the number of receiving antennas and spreading factor (SF) is large, , EPA-PIC has lower computation complexity than MMSE-SIC.

LLS results of PDMA

As shown in the Appendix, given the updated simulation conditions agreed in RAN#92, LLS performance of different PDMA receivers are compared. In this section, we give the comparison of EPA-PIC and MMSE-SIC receivers. For those varying parameters for simulation, usually TBS per UE and overload factor (OF) are regarded as key factors affecting receivers which should be given full consideration. While other factors like application scenario and channel type can be evaluated with representatives, to avoid a redundancy huge amount of simulation. Here, mMTC and eMBB are selected as representatives of the scenarios, and TDL-A channel is used.

According to [2], different PDMA pattern matrix have nearly no effect on LLS performance, so only one type of PDMA pattern matrix is selected for each overload case, as shown in Table 2.

Table 2: Examples of PDMA pattern matrix

| Overload factor | Type | PDMA pattern matrix |
| --- | --- | --- |
| 150% | Type 1 |  |
| 300% | Type 1 |  |

mMTC scenario

- TBS=40 bytes, OF=150%

Figure 3: mMTC scenario, TDL-A channel, TBS=40 bytes, OF=150%

- TBS=40 bytes, OF=200%

Figure 4: mMTC scenario, TDL-A channel, TBS=40 bytes, OF=200%

eMBB  scenario

- TBS=20 bytes, OF=150%

Figure 5: eMBB scenario, TDL-A channel, TBS=20 bytes, OF=150%

- TBS=80 bytes, OF=150%

Figure 6: eMBB scenario, TDL-A channel, TBS=80 bytes, OF=150%

- TBS=80 bytes, OF=300%

Figure 7: eMBB scenario, TDL-A channel, TBS=80 bytes, OF=300%

Based on the above evaluation results, we have the following observation.

Observation: Both EPA-PIC and MMSE-SIC receivers are applicable to PDMA.

Conclusion

In this contribution, we provide initial simulation results with different PDMA receivers with the following observation.

Observation: Both EPA-PIC and MMSE-SIC receivers are applicable to PDMA.

References

- RP-1801754, Discussion on NOMA receivers, CATT, 3GPP RAN1#92

- RP-1803370, PDMA transmitter side signal processing, CATT, 3GPP RAN1#92bis

- Appendix: LLS simulation conditions agreed in RAN1#92

| Parameters | mMTC | URLLC | eMBB |
| --- | --- | --- | --- |
| Carrier Frequency | 700 MHz | 700 MHz or 4 GHz | 4 GHz, 700 MHz as optional |
| Waveform (data part) | CP-OFDM and DFT-s-OFDM | CP-OFDM as starting point | CP-OFDM as starting point |
| Channel coding | URLLC: NR LDPC eMBB: NR LDPC mMTC: NR LDPC |  |  |
| Numerology (data part) | SCS = 15 kHz, #OS = 14 | Case 1: SCS = 60 kHz, #OS = 7 (normal CP), optionally 6 (ECP) Case 2: SCS = 30 kHz, #OS = 4 | SCS = 15 kHz #OS = 14 |
| Allocated bandwidth | 6 as the starting point | 12 as the starting point | 12 as the starting point |
| TBS per UE | At least five TBS that are [10, 20, 40, 60, 75] bytes. Other values higher than 10 bytes are not precluded. Lower than 0.1 bits/RE is optional | At least five TBS that are [10, 20, 40, 60, 75] bytes. Other values higher than 10 bytes are not precluded. | At least five TBS that are [20, 40, 80, 120, 150] bytes. Other values higher than 20 bytes are not precluded. |
| Target BLER for one transmission | 10% | 0.1% | 10% |
| Number of UEs multiplexed in the same allocated bandwidth | To be reported by companies. |  |  |
| BS antenna configuration | 2 Rx or 4 Rx for 700MHz, 4Rx or 8 Rx for 4 GHz 8Rx as optional |  |  |
| UE antenna configuration | 1Tx |  |  |
| Propagation channel & UE velocity | TDL-A 30ns and TDL-C 300ns in TR38.901, 3km/h, CDL optional |  |  |
| Max number of HARQ transmission | 1 as starting point. | 1 as starting point. More values, 2 for URLLC can be used. | 1 as starting point. |
| Channel estimation | Realistic channel estimation |  |  |
| MA signature allocation (for data and DMRS) | Fixed/Random |  |  |
| Distribution of avg. SNR | Both equal and unequal | Equal | Both equal and unequal |
| Timing offset | 0 as starting point. For grant-free without perfect TA, value is TBD |  |  |
| Frequency error | 0 as starting point. The value(s) is TBD. |  |  |
| Traffic model for link level | Full buffer as starting point. Non-full-buffer model (like Poisson arrival of fixed packet size) is optional. |  |  |
| For link level calibration purpose only | OMA single user whose spectral efficiency is the same as per UE SE in NOMA. AWGN curves can be provided also. |  |  |