3GPP TSG RAN WG1 Meeting #93                                                                       R1-1806330

Busan, Korea, May 21st – 25th, 2018

Source:	CATT

Title:	Initial self evaluation for Rural-eMBB

Agenda Item:	7.8.2

Document for:	Discussion and Decision

Introduction

At RAN#75 meeting, new Study Item on Self Evaluation towards IMT-2020 submission was approved [1], the self evaluation will provide the performances towards all the ITU-R IMT-2020 requirements. The key minimum technical performance requirements are provided in Report ITU-R M.2410 [2] for the test environment of Rural-eMBB.

TABLE 1

5th percentile user spectral efficiency

| Test environment | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Rural – eMBB | 0.12 | 0.045 |

The performance requirement for Rural-eMBB is not applicable to Rural-eMBB LMLC (low mobility large cell) which is one of the evaluation configurations under the Rural- eMBB test environment.

TABLE 2

Average spectral efficiency

| Test environment | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Rural – eMBB | 3.3 | 1.6 |

The performance requirement for Rural-eMBB is also applicable to Rural-eMBB LMLC which is one of the evaluation configurations under the Rural- eMBB test environment.

According to the evaluation methodology of Report ITU-R M.2412 [3], the average spectral efficiency and the 5th percentile user spectral efficiency need system-level simulation evaluation to demonstrate the achievement for the ITU-R technical performance requirements.

In our companion contribution [4], some initial evaluation results including the average spectral efficiency and the 5th percentile user spectral efficiency were provided. In this document, more updated evaluation results on the spectral efficiencies in Rural-eMBB based on the agreed technical features and system configurations in [5] and updated simulation assumptions, e.g. overhead in our companion contribution [6] are provided to meet the technical performance requirements.

Evaluation methodology and assumption

System-level simulation with proper link to system interface based on link-level simulation results could be used to evaluate average spectral efficiency and 5th percentile user spectral efficiency.

In this contribution, different antenna configurations are evaluated to observe the proper antenna configurations to fulfil the ITU minimum performance requirement and to observe the performance advantage of NR system, including different number of antenna elements and different TXRU mapping. The evaluation assumptions used in this document refer to [5] [6] and are detailed summarized in Annex, which will be updated when 3GPP has further discussions and/or agreements.

Initial downlink evaluation results

Based on the above information, the initial evaluation results for downlink average spectral efficiency and 5th percentile spectral efficiency are provided under Rural-eMBB. Evaluation assumptions can be found in Annex.

Initial downlink evaluation results

The initial evaluation results for DL average spectral efficiency and DL 5th percentile spectral efficiency under Rural-eMBB are provided in Table 1 and Table 2.

Note: Impacts from channel estimation error modeling, PRB bundling, updated subband size and scheduler unit size and overhead are updated.

Table 1 Initial DL average spectral efficiency evaluation (Rural-eMBB)

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at transmitter) | Element configuration(No. at transmitter/receiver, configuration at transmitter) | Requirement (bit/s/Hz/TRxP) | SEavg (bit/s/Hz/TRxP) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| Rural-eMBB Config. A, 700 MHz, RMa-B | FDD | 4T/2R, (1,2,2,1,1) | 4T/2R, (1,2,2,1,1) | 3.3 | 4.67 | 41.52% |
|  |  | 8T/2R, (1,4,2,1,1) | 8T/2R, (1,4,2,1,1) |  | 6.87 | 108.18% |
|  |  | 16T/2R, (1,8,2,1,1) | 16T/2R, (1,8,2,1,1) |  | 9.52 | 188.48% |
|  |  | 32T/2R, (2,8,2,1,1) | 32T/2R, (2,8,2,1,1) |  | 9.8 1 | 197.27% |
|  |  |  | 64T/2R, (4,8,2,1,1) |  | 10.46 | 216.97% |
|  |  | 32T/4R, (2,8,2,1,1) | 64T/4R, (4,8,2,1,1) |  | 13.99 | 323.94% |
|  | TDD 15KHz SCS | 4T/2R, (1,2,2,1,1) | 4T/2R, (1,2,2,1,1) |  | 5.11 | 54.85% |
|  |  | 8T/2R, (1,4,2,1,1) | 8T/2R, (1,4,2,1,1) |  | 7.66 | 132.12% |
|  |  | 16T/2R, (1,8,2,1,1) | 16T/2R, (1,8,2,1,1) |  | 11.02 | 233.94% |
|  |  | 32T/2R, (2,8,2,1,1) | 32T/2R, (2,8,2,1,1) |  | 11.22 | 240.00% |
|  |  |  | 64T/2R, (4,8,2,1,1) |  | 11.90 | 260.61% |
|  |  | 32T/4R, (2,8,2,1,1) | 64T/4R, (4,8,2,1,1) |  | 15.49 | 369.39% |
|  |  | 64T/4R, (2,8,2,1,1) | 64T/4R, (4,8,2,1,1) |  | 15.84 | 380.00% |
|  | TDD 30KHz SCS | 4T/2R, (1,2,2,1,1) | 4T/2R, (1,2,2,1,1) |  | 4.79 | 45.15% |
|  |  | 8T/2R, (1,4,2,1,1) | 8T/2R, (1,4,2,1,1) |  | 7.21 | 118.48% |
|  |  | 16T/2R, (1,8,2,1,1) | 16T/2R, (1,8,2,1,1) |  | 10.45 | 216.67% |
|  |  | 32T/2R, (2,8,2,1,1) | 32T/2R, (2,8,2,1,1) |  | 10.69 | 223.94% |
|  |  |  | 64T/2R, (4,8,2,1,1) |  | 11.46 | 247.27% |
|  |  | 32T/4R, (2,8,2,1,1) | 64T/4R, (4,8,2,1,1) |  | 14.87 | 350.61% |
|  |  | 64T/4R, (2,8,2,1,1) | 64T/4R, (4,8,2,1,1) |  | 15.16 | 359.39% |
| Rural-eMBB Config. B, 4GHz, RMa-B | FDD | 8T/2R, (1,4,2,1,1) | 8T/2R, (1,4,2,1,1) |  | 6.19 | 87.58% |
|  |  | 16T/2R, (1,8,2,1,1) | 16T/2R, (1,8,2,1,1) |  | 8.57 | 159.70% |
|  |  | 32T/2R, (2,8,2,1,1) | 32T/2R, (2,8,2,1,1) |  | 8.85 | 168.18% |
|  |  |  | 64T/2R, (4,8,2,1,1) |  | 9.51 | 188.18% |
|  |  |  | 128T/2R, (8,8,2,1,1) |  | 10.65 | 222.73% |
|  | TDD 15KHz SCS | 8T/4R, (1,2,2,1,1) | 8T/2R, (1,4,2,1,1) |  | 6.57 | 99.09% |
|  |  | 16T/2R, (1,8,2,1,1) | 16T/2R, (1,8,2,1,1) |  | 9.03 | 173.64% |
|  |  | 32T/2R, (2,8,2,1,1) | 32T/2R, (2,8,2,1,1) |  | 9.32 | 182.42% |
|  |  |  | 64T/2R, (4,8,2,1,1) |  | 10.25 | 210.61% |
|  |  |  | 128T/2R, (8,8,2,1,1) |  | 11.79 | 257.27% |
|  | TDD 30KHz SCS | 8T/2R, (1,4,2,1,1) | 9T/2R, (1,4,2,1,1) |  | 6.17 | 86.97% |
|  |  | 16T/2R, (1,8,2,1,1) | 16T/2R, (1,8,2,1,1) |  | 8.94 | 170.91% |
|  |  | 32T/2R, (2,8,2,1,1) | 32T/2R, (2,8,2,1,1) |  | 9.15 | 177.27% |
|  |  |  | 64T/2R, (4,8,2,1,1) |  | 9.98 | 202.42% |
|  |  |  | 128T/2R, (8,8,2,1,1) |  | 11.38 | 244.85% |

Table 2 Initial DL 5th percentile spectral efficiency evaluation (Rural-eMBB)

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at transmitter) | Element configuration(No. at transmitter/receiver, configuration at transmitter) | Requirement (bit/s/Hz) | SEuser (bit/s/Hz) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| Rural-eMBB Config. A, 700 MHz, RMa-B | FDD | 4T/2R, (1,2,2,1,1) | 4T/2R, (1,2,2,1,1) | 0.12 | 0.13 | 8.33% |
|  |  | 8T/2R, (1,4,2,1,1) | 8T/2R, (1,4,2,1,1) |  | 0.20 | 66.67% |
|  |  | 16T/2R, (1,8,2,1,1) | 16T/2R, (1,8,2,1,1) |  | 0.28 | 133.33% |
|  |  | 32T/2R, (2,8,2,1,1) | 32T/2R, (2,8,2,1,1) |  | 0.28 | 133.33% |
|  |  |  | 64T/2R, (4,8,2,1,1) |  | 0.30 | 150.00% |
|  |  | 32T/4R, (2,8,2,1,1) | 64T/4R, (4,8,2,1,1) |  | 0.54 | 350.00% |
|  | TDD 15KHz SCS | 4T/2R, (1,2,2,1,1) | 4T/2R, (1,2,2,1,1) |  | 0.14 | 16.67% |
|  |  | 8T/2R, (1,4,2,1,1) | 8T/2R, (1,4,2,1,1) |  | 0.21 | 75.00% |
|  |  | 16T/2R, (1,8,2,1,1) | 16T/2R, (1,8,2,1,1) |  | 0.30 | 150.00% |
|  |  | 32T/2R, (2,8,2,1,1) | 32T/2R, (2,8,2,1,1) |  | 0.31 | 158.33% |
|  |  |  | 64T/2R, (4,8,2,1,1) |  | 0.32 | 166.67% |
|  |  | 32T/4R, (2,8,2,1,1) | 64T/4R, (4,8,2,1,1) |  | 0.57 | 375.00% |
|  |  | 64T/4R, (2,8,2,1,1) | 64T/4R, (4,8,2,1,1) |  | 0.59 | 391.67% |
|  | TDD 30KHz SCS | 4T/2R, (1,2,2,1,1) | 4T/2R, (1,2,2,1,1) |  | 0.12 | 0.00% |
|  |  | 8T/2R, (1,4,2,1,1) | 8T/2R, (1,4,2,1,1) |  | 0.19 | 58.33% |
|  |  | 16T/2R, (1,8,2,1,1) | 16T/2R, (1,8,2,1,1) |  | 0.27 | 125.00% |
|  |  | 32T/2R, (2,8,2,1,1) | 32T/2R, (2,8,2,1,1) |  | 0.28 | 133.33% |
|  |  |  | 64T/2R, (4,8,2,1,1) |  | 0.30 | 150.00% |
|  |  | 32T/4R, (2,8,2,1,1) | 64T/4R, (4,8,2,1,1) |  | 0.53 | 341.67% |
|  |  | 64T/4R, (2,8,2,1,1) | 64T/4R, (4,8,2,1,1) |  | 0.56 | 366.67% |
| Rural-eMBB Config. B, 4GHz, RMa-B | FDD | 8T/2R, (1,4,2,1,1) | 8T/2R, (1,4,2,1,1) |  | 0.18 | 50.00% |
|  |  | 16T/2R, (1,8,2,1,1) | 16T/2R, (1,8,2,1,1) |  | 0.27 | 125.00% |
|  |  | 32T/2R, (2,8,2,1,1) | 32T/2R, (2,8,2,1,1) |  | 0.29 | 141.67% |
|  |  |  | 64T/2R, (4,8,2,1,1) |  | 0.31 | 158.33% |
|  |  |  | 128T/2R, (8,8,2,1,1) |  | 0.33 | 175.00% |
|  | TDD 15KHz SCS | 8T/2R, (1,4,2,1,1) | 8T/2R, (1,4,2,1,1) |  | 0.19 | 58.33% |
|  |  | 16T/2R, (1,8,2,1,1) | 16T/2R, (1,8,2,1,1) |  | 0.27 | 125.00% |
|  |  | 32T/2R, (2,8,2,1,1) | 32T/2R, (2,8,2,1,1) |  | 0.27 | 125.00% |
|  |  |  | 64T/2R, (4,8,2,1,1) |  | 0.30 | 150.00% |
|  |  |  | 128T/2R, (8,8,2,1,1) |  | 0.33 | 175.00% |
|  | TDD 30KHz SCS | 8T/2R, (1,4,2,1,1) | 8T/2R, (1,4,2,1,1) |  | 0.18 | 50.00% |
|  |  | 16T/2R, (1,8,2,1,1) | 16T/2R, (1,8,2,1,1) |  | 0.27 | 125.00% |
|  |  | 32T/2R, (2,8,2,1,1) | 32T/2R, (2,8,2,1,1) |  | 0.27 | 125.00% |
|  |  |  | 64T/2R, (4,8,2,1,1) |  | 0.28 | 133.33% |
|  |  |  | 128T/2R, (8,8,2,1,1) |  | 0.31 | 158.33% |

Observation 1: For both 700MHz and 4GHz, NR can fulfill the DL spectral efficiency requirements for the Rural-eMBB test environment in both TDD and FDD mode.

Observation 2: For both 700MHz and 4GHz, the performance can be over three times than the DL spectral efficiency requirements for the Rural-eMBB test environment in both TDD and FDD mode when the same assumptions in this contribution are applied.

Observation 3: In Rural-eMBB DL, configuration A at 700MHz overperformances the case of configuration B at 4GHz assuming the same antenna configuration, although more antenna elements are more feasible at 4GHz

Observation 4: Evaluation results on downlink spectral efficiency with 30KHz SCS are slightly worse than 15KHz SCS with the assumption that the simulation bandwidth is not scaled with SCS.

Based on the assumptions in this contribution and [5][6], according to the above initial evaluation results, the sufficient antenna configurations to meet the minimum requirement are summarized in the following Table 3.

Proposal 1: For downlink spectral efficiency evaluation to meet the requirements, the sufficient antenna configurations can refer to the configurations in Table 3 when the same assumptions in this contribution are applied.

Table 3 Sufficient antenna configurations for Rural-eMBB

| Test environment | Duplex scheme | Sufficient antenna configuration |  |
| --- | --- | --- | --- |
|  |  | TxRU configuration(No. at transmitter/receiver, configuration at transmitter) | Element configuration(No. at transmitter/receiver, configuration at transmitter) |
| Rural-eMBB, Config. A, 700 MHz | FDD | 4T/2R, (1,2,2,1,1) | 4Tx/2Rx, (1,2,2,1,1) |
|  | TDD 15KHz SCS | 4T/2R, (1,2,2,1,1) | 4Tx/2Rx, (1,2,2,1,1) |
|  | TDD 30KHz SCS | 4T/2R, (1,2,2,1,1) | 4Tx/2Rx, (1,2,2,1,1) |
| Rural-eMBB, Config. B, 4 GHz | FDD | 8T/2R, (1,4,2,1,1) | 8Tx/2Rx, (1,4,2,1,1) |
|  | TDD 15KHz SCS | 8T/2R, (1,4,2,1,1) | 8Tx/2Rx, (1,4,2,1,1) |
|  | TDD 30KHz SCS | 8T/2R, (1,4,2,1,1) | 8Tx/2Rx, (1,4,2,1,1) |

Initial uplink evaluation results

In RAN1#92 and RAN1#92bis meeting, simulation assumption about FDD and TDD for uplink is discussed and updated agreements are achieved. To align with latest assumptions as much as possible, we evaluate TDD with non-codebook transmission and polish power control parameter for both FDD and TDD to satisfy ITU IoT requirement better which targets IoT smaller than 10dB.

The initial evaluation results for UL average spectral efficiency and UL 5th percentile spectral efficiency under Rural-eMBB are provided in Table 4 and Table 5. As initial evaluation, SU-MIMO can be as a baseline MIMO transmission technology. From initial results, it can be observed that spectrum efficiency is a relaxed requirement in Rural-eMBB. Coverage may be a more important evaluation metric for Rural-eMBB.

Note: Non-ideal evaluation assumptions and overhead may be updated in future.

Table 4 Initial UL average spectral efficiency evaluation (Rural-eMBB at 700MHz)

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at receiver) | Element configuration(No. at transmitter/receiver, configuration at receiver) | Requirement (bit/s/Hz/TRxP) | SEavg (bit/s/Hz/TRxP) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| Rural-eMBB Config. A, 700 MHz, RMa-A | FDD | 2T/8R, (1,4,2,1,1) | 2Tx/64Rx, (8,4,2,1,1) | 1.6 | 7.10 | 343.50% |
| Rural-eMBB Config. A, 700 MHz, RMa-A | TDD,15KHz SCS | 2T/8R, (1,4,2,1,1) | 2Tx/64Rx, (8,4,2,1,1) |  | 5.24 | 227.19% |

Table 5 Initial UL 5th percentile spectral efficiency evaluation (Rural-eMBB at 700MHz)

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at receiver) | Element configuration(No. at transmitter/receiver, configuration at receiver) | Requirement (bit/s/Hz) | SEuser (bit/s/Hz) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| Rural-eMBB Config. A, 700 MHz, RMa-A | FDD | 2T/8R, (1,4,2,1,1) | 2Tx/64Rx, (8,4,2,1,1) | 0.045 | 0.25 | 449.73% |
| Rural-eMBB Config. A, 700 MHz, RMa-A | TDD,15KHz SCS | 2T/8R, (1,4,2,1,1) | 2Tx/64Rx, (8,4,2,1,1) |  | 0.135 | 200.00% |

Observation 5: For 700MHz, NR can fulfill the UL spectral efficiency requirements for the Rural-eMBB test environment in both FDDand TDD mode.

Observation 6: Even SU-MIMO can fulfil the requirement for uplink. It is possible that reduced number of TxRU at receiver (eNB) can be sufficient to meet the requirement considering antenna size and complexity at eNB.

Proposal 2: Further investigation is needed to find proper antenna configuration for rural uplink considering antenna size, complexity and performance.

Conclusion

In this contribution, we provide our preliminary evaluation results of Rural-eMBB, we have the following observation:

Observation 1: For both 700MHz and 4GHz, NR can fulfill the DL spectral efficiency requirements for the Rural-eMBB test environment in both TDD and FDD mode.

Observation 2: For both 700MHz and 4GHz, the performance can be over three times than the DL spectral efficiency requirements for the Rural-eMBB test environment in both TDD and FDD mode when the same assumptions in this contribution are applied.

Observation 3: In Rural-eMBB DL, configuration A at 700MHz over performances the case of configuration B at 4GHz assuming the same antenna configuration, although more antenna elements are more feasible at 4GHz

Observation 4: Evaluation results on downlink spectral efficiency with 30KHz SCS are slightly worse than 15KHz SCS with the assumption that the simulation bandwidth is not scaled with SCS.

Observation 5: For 700MHz, NR can fulfill the UL spectral efficiency requirements for the Rural-eMBB test environment in both FDDand TDD mode.

Observation 6: Even SU-MIMO can fulfil the requirement for uplink. It is possible that reduced number of TxRU at receiver (eNB) can be sufficient to meet the requirement considering antenna size and complexity at eNB.

Proposal 1: For downlink spectral efficiency evaluation to meet the requirements, the sufficient antenna configurations can refer to the configurations in Table 3 when the same assumptions in this contribution are applied.

Proposal 2: Further investigation is needed to find proper antenna configuration for rural uplink considering antenna size, complexity and performance.

- References

  - RP-170832, “New Study Item on Self Evaluation towards IMT-2020 submission”, Huawei, Ericsson.

  - Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

  - Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

  - R1-1803789, “Initial self evaluation for Rural-eMBB”, CATT.

  - RAN1#92 Chairman’s notes.

  - R1-1806326, “Discussions on system level simulation configurations and technical features for self evaluation”, CATT

- Annex: Evaluation assumptions

In this contribution, the evaluation assumptions for Rural-eMBB are provided in the following table.

Table A-1 Evaluation assumptions for DL and UL

| Rural - eMBB | Config. A | Config. B |
| --- | --- | --- |
| Carrier frequency for evaluation | 700 MHz | 4 GHz |
| Simulation bandwidth | FDD: 10M+10M TDD: 20M | FDD: 10M+10M TDD: 20M |
| Slots structure | FDD: paired spectrum, DL + UL TDD: unpaired spectrum, DSUUD | FDD: paired spectrum, DL + UL TDD: unpaired spectrum, DSUUD |
| Subcarrier spacing | 15KHz, 30KHz(TDD only) | 15KHz, 30KHz(TDD only) |
| Symbols number per slot | 14 | 14 |
| BS antenna height | 35 m | 35 m |
| Total transmit power per TRxP | FDD: 46 dBm TDD: 49 dBm | FDD: 46 dBm TDD: 49 dBm |
| UE power class | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | 100% low loss (applies to Channel model B) | 100% low loss (applies to Channel model B) |
| Inter-site distance | 1732 m | 1732 m |
| Number of antenna elements per TRxP | 4 Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), 8 Tx/Rx, (M,N,P,Mg,Ng) = (1,4,2,1,1), 16 Tx/Rx, (M,N,P,Mg,Ng) = (1,8,2,1,1), 32 Tx/Rx, (M,N,P,Mg,Ng) = (2,8,2,1,1), 64 Tx/Rx, (M,N,P,Mg,Ng) = (8,4,2,1,1) / (4,8,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization | 4 Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), 8 Tx/Rx, (M,N,P,Mg,Ng) = (1,4,2,1,1), 16 Tx/Rx, (M,N,P,Mg,Ng) = (1,8,2,1,1), 32 Tx/Rx, (Mp,Np,P,Mg,Ng) = (2,8,2,1,1) 64 Tx/Rx, (M,N,P,Mg,Ng) = (8,4,2,1,1) / (4,8,2,1,1) 128 17s document, we ution that in [] Tx/Rx, (M,N,P,Mg,Ng) = (8,8,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) 16TXRU, (Mp,Np,P,Mg,Ng) = (1,8,2,1,1) 32TXRU, (Mp,Np,P,Mg,Ng) = (2,8,2,1,1) | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) 16TXRU, (Mp,Np,P,Mg,Ng) = (1,8,2,1,1) 32TXRU, (Mp,Np,P,Mg,Ng) = (2,8,2,1,1) |
| Number of UE antenna elements | 2Tx/Rx, (M,N,P,Mg,Ng) = (1,1,2,1,1) 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1) 0°,90° polarization | 2Tx/Rx, (M,N,P,Mg,Ng) = (1,1,2,1,1) 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1) 0°,90° polarization |
| Number of TXRU per UE | 2TXRU (1-to-1 mapping) 4TXRU (1-to-1 mapping) | 2TXRU (1-to-1 mapping) 4TXRU (1-to-1 mapping) |
| Device deployment | 50% indoor, 50% outdoor (in car) Randomly and uniformly distributed over the area | 50% indoor, 50% outdoor (in car) Randomly and uniformly distributed over the area |
| UE speeds of interest | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; |
| BS noise figure | 5 dB | 5 dB |
| UE noise figure | 7 dB | 7 dB |
| BS antenna element gain | 8 dBi | 8 dBi |
| BS antenna element pattern | See Table 9 in Report ITU-R M.2412 | See Table 9 in Report ITU-R M.2412 |
| UE antenna element gain | 0 dBi | 0 dBi |
| UE antenna element pattern | Omni-directional | Omni-directional |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz |
| Traffic model | Full buffer | Full buffer |
| UE density | 10 UEs per TRxP | 10 UEs per TRxP |
| UE antenna height | 1.5 m | 1.5 m |
| Channel model variant | Channel model B | Channel model B |
| TRxP number per site | 3 | 3 |
| Mechanic tilt | 90° in GCS | 90° in GCS |
| Electronic tilt | 100° in LCS | 100° in LCS |
| TRxP boresight | 30 / 150 / 270 degrees | 30 / 150 / 270 degrees |
| UT attachment | Based on RSRP from port 0 | Based on RSRP from port 0 |
| Wrapping around method | Geographical distance based wrapping | Geographical distance based wrapping |
| Minimum distance of TRxP and UE | d2D_min=10m | d2D_min=10m |
| Polarized antenna model | Model-2 in TR36.873 | Model-2 in TR36.873 |
| TX mode | SU/MU-MIMO | SU/MU MIMO |
| scheduler | PF | PF |
| Receiver | MMSE-IRC | MMSE-IRC |
| DL CSI feedback | 10ms period with delay | 10ms period with delay |
| DL CSI method | FDD: CSI Type II codebook based TDD: full reciprocity based | FDD: CSI TypeII codebook based TDD: full reciprocity based |
| NR Type II codebook parameters | 4 beam basis, WB+SB and 8PSK quantification | 4 beam basis, WB+SB and 8PSK quantification |
| HARQ ACK delay | the next available UL slot | the next available UL slot |
| HARQ retransmission delay | the next available DL slot after ACK report | the next available DL slot after ACK report |
| Channel estimation | Non-ideal | Non-ideal |
| Interference estimation | ideal | ideal |
| UE precoder scheme | Codebook based for FDD; Non-codebook based for TDD. | -- |
| UL CSI derivation | SRS based with delay | -- |
| UL Waveform | DFT-s-FDMA | -- |
| IoT | <10dB | -- |
| Overhead | Refer to our companion contribution [6], for DL without NZP CSI-IS IMR | Refer to our companion contribution [6], for DL without NZP CSI-IS IMR |