3GPP TSG RAN WG1 Meeting #93		R1-1806347

Busan, Korea, May 21st –  25th,  2018

Agenda item:	7.1.1.2.1

Source: 	Lenovo, Motorola Mobility

Title: 	Correction on QCL of SS blocks

Document for:	Discussion and decision

Introduction

In the previous meeting [1], the following agreements on QCL assumptions of SSBs were made:

Agreements:

- A UE may assume that antenna ports used for transmissions of SS/PBCH blocks with same index recurring with the SS/PBCH burst set periodicity are quasi-collocated with respect to spatial, average gain, delay, and Doppler parameters

- By default, a UE may not assume that antenna ports used for transmissions of SS/PBCH blocks with different indexes are quasi-collocated with respect to spatial, average gain, delay and Doppler parameters

- FFS: Whether the network indication of SS block repetition and the impact on QCL assumption

- Note: In the above agreements, gNB has the flexibility to use different physical beam(s) and its mapping  for the SS/PBCH block transmission

  - Note: The case where antenna ports used for transmissions of SS/PBCH blocks with same index recurring with the SS/PBCH burst set periodicity are not quasi-collocated with respect to spatial, average gain, delay, and [Doppler] parameters is not specified

Agreements:

- UE assumes the DMRS of NR-PDCCH transmitted in the CORESET for RMSI and the DMRS of NR-PDSCH for RMSI/broadcast OSI is QCLed with the corresponding  SS/PBCH block

- FFS: On the details on the associations between SS blocks and monitoring windows (if introduced) for RMSI CORESETs/broadcast OSI .

In 3GPP RAN1#91[2], the agreements on QCL assumptions were made as following:

Agreement:

QCL types A: Doppler shift, Doppler spread, average delay, delay spread

QCL types B: Doppler shift, Doppler spread

QCL types C: average delay, Doppler shift

QCL types D: Spatial Rx

| QCL linkage for below 6GHz before RRC | signalling |
| --- | --- |
| SSB DMRS for PDSCH w.r.t Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters (spatial RX parameters are used only for above 6GHz) |  |
| SSB DMRS for PDCCH w.r.t Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters (spatial RX parameters are used only for above 6GHz) |  |

| QCL linkage for below 6GHz after RRC | signalling |
| --- | --- |
| SSB TRS: Doppler shift, average delay | QCL type: C |
| TRS CSI-RS for CSI acquisition: Doppler shift, Doppler spread, average delay, delay spread | QCL type: A |
| TRS DMRS: Doppler shift, Doppler spread, average delay, delay spread | QCL type: A |
| TRS CSI-RS for CSI acquisition: Doppler shift, Doppler spread | QCL type: B |
| CSI-RS DMRS: Doppler shift, Doppler spread, average delay, delay spread | QCL type: A |

| QCL linkage for above 6GHz after RRC | signalling |
| --- | --- |
| SSB TRS w.r.t average delay, Doppler shift, spatial RX parameters | QCL type: C + D |
| TRS CSI-RS for BM w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation | QCL type: A |
| TRS CSI-RS for CSI w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation | QCL type: A |
| TRS DMRS for PDCCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation | QCL type: A |
| TRS DMRS for PDSCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation | QCL type: A |
| SSB CSI-RS for BM w.r.t. average delay, Doppler shift, spatial RX parameters | QCL type: C+D |
| SSB CSI-RS for CSI w.r.t, spatial RX parameters | QCL type: D |
| SSB DMRS for PDCCH (before TRS is configured) w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters | QCL type: A+D |
| SSB DMRS for PDSCH (before TRS is configured) w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters | QCL type: A+D |
| CSI-RS for BM DMRS for PDCCH w.r.t. spatial RX parameters | QCL type: D |
| CSI-RS for BM DMRS for PDSCH w.r.t., spatial RX parameters | QCL type: D |
| CSI-RS for CSI DMRS for PDSCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters; Note: QCL parameters may not be derived directly from CSI-RS for CSI | QCL type: A+D |
| CSI-RS for BM CSI-RS for TRS/BM/CSI w.r.t. spatial RX parameters | QCL type: D |

In this contribution we propose to correct the QCL assumptions related to SSB.

Discussion

According to the agreements in Section 1, the spatial Rx parameters are used only for above 6GHz. We propose to correct the QCL assumptions related to SS blocks as following in 3GPP TS 38.211[3]:

- 7.4.1.1.2	Mapping to physical resources

***************** Start of the changed part *****************

In absence of CSI-RS configuration, and unless otherwise configured, the UE may assume PDSCH DM-RS and SS/PBCH block to be quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and spatial Rx parameters when applicable. The UE may assume that the PDSCH DM-RS within the same CDM group are quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and spatial Rx parameters when applicable.

***************** End of the changed part *****************

- 7.4.1.3.2	Mapping to physical resources

***************** Start of the changed part *****************

In absence of CSI-RS configuration, and unless otherwise configured, the UE may assume PDCCH DM-RS and SS/PBCH block to be quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and spatial Rx parameters when applicable.

***************** End of the changed part *****************

- 7.4.3.1	Time-frequency structure of an SS/PBCH block

***************** Start of the changed part *****************

The UE may assume that SS/PBCH blocks transmitted with the same block index on the same center frequency location are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and spatial Rx parameters when applicable. The UE shall not assume quasi co-location for any other SS/PBCH block transmissions.

***************** End of the changed part *****************

Conclusions

In this contribution, we propose to correct the QCL assumptions related to SS block as in Section 2.

References

[1] Chairman’s notes, RAN1 #90.

[2] Chairman’s notes, RAN1#91.

[3] 3GPP TS38.211