3GPP TSG-RAN WG1 Meeting #93	R1-1806437

Busan, Korea, May 21st –25th, 2018

Source:	Ericsson

Title:	IMT-2020 self-evaluation: mMTC full buffer connection density

Agenda Item:	7.8.2

Document for:	Discussion

# Introduction

3GPP has as part of the Study Item on Self Evaluation towards IMT-2020 Submission [1] agreed to “Provide self evaluation results against technical performance requirements for mMTC as per defined in Report ITU-R M.[IMT-2020. TECH PERF REQ]  [RAN1, RAN2], including Connection density”.

A submission time plan has also been agreed [2] where the 3GPP meetings for submission of description and compliance templates according to report IMT-2020.SUBMISSION [3] are set. The Compliance template for technical performance contains the minimum technical performance requirement item Connection density. To fulfil this requirement 3GPP must show that it’s candidate Radio Interface Technologies (RITs) submitted to ITU-R supports a connection density of 1 000 000 devices per km2. The evaluation is to be performed in accordance to test environment Urban Macro-mMTC as described in report IMT-2020.EVAL [4].

In this contribution, we present results on Connection Density for NR, for LTE Bandwidth reduced Low complexity (BL) UEs operating in Coverage Enhanced (CE) modes A and B, and for NB-IoT. The LTE BL/CE operation is hereafter referred to as LTE-M operation. It is shown that NR, LTE-M and NB-IoT meets the IMT-2020 requirement and should be part of the 3GPP submission to ITU-R.

The contribution also presents the connection density requirement definition and the full buffer system simulation procedure that is followed in this contribution.

# IMT-2020 Connection density

## Requirement definition

The connection density requirement requires a RIT to provide a certain Quality of Service (QoS) to 1 000 000 devices per km2 at a grade of service (GoS) of 99 percent. Service is considered provided when a message latency of less than 10 seconds is supported for a user attempting to send an uplink data packet of 32 bytes defined at layer 2. Besides the supported connection density, it is encouraged to report the connection efficiency which is defined as the connection density normalized by the required system bandwidth.

The connection density requirement requires 99% grade of service where acceptable quality of service is defined by a message latency of 10 seconds or less.

## System simulation procedures

Report IMT-2020.EVAL [4] outlines two system simulator procedures for evaluating connection density. The first is a non-full buffer system level simulation that requires a state of the art system simulator to perform the evaluations. The second approach is for a full buffer system simulation that allows input based on a more rudimentary system simulator combined with post processing supported by link level simulations. In this paper, we have followed the full buffer approach which is described in detail by the below table. We provide input for the non-full buffer methodology in [6].

Table 1 Full buffer system level simulation procedure [4].

## Test environment

### System level

Report IMT-2020.EVAL specifies the test environment to be used in the first step of the evaluations according to Table 2 below. The simulations presented in this contribution are to a large extent following this set of assumptions. Further detailed assumptions are outlined in section 3.

Table 2: Urban Macro-mMTC test environment definition [4].

| Parameters | Config. A | Config. B |
| --- | --- | --- |
| Carrier frequency for evaluation | 700 MHz | 700 MHz |
| BS antenna height | 25 m | 25 m |
| Total transmit power per TRxP | 46 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | 20% high loss, 80% low loss Note: Applies only to Channel model B. | 20% high loss, 80% low loss Note: Applies only to Channel model B. |
| Inter-site distance | 500 m | 1732 m |
| Number of antenna elements per TRxP | Up to 64 Tx/Rx | Up to 64 Tx/Rx |
| Number of UE antenna elements | Up to 2 Tx Up to 2 Rx | Up to 2 Tx Up to 2 Rx |
| Device deployment | 80% indoor, 20% outdoor Note: Randomly and uniformly distributed over the area | 80% indoor, 20% outdoor Note: Randomly and uniformly distributed over the area |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction. | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction. |
| UE speeds of interest | 3 km/h for indoor and outdoor Note: Corresponds to 2 Hz Doppler | 3 km/h for indoor and outdoor Note: Corresponds to 2 Hz Doppler |
| Inter-site interference modeling | Explicitly modelled | Explicitly modelled |
| BS noise figure | 5 dB | 5 dB |
| UE noise figure | 7 dB | 7 dB |
| BS antenna element gain | 8 dBi | 8 dBi |
| UE antenna element gain | 0 dBi | 0 dBi |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz |
| Traffic model | With layer 2 PDU(Protocol Data Unit) message size of 32 bytes: 1 message/day/device or 1 message/2 hours/device Note: Not modelled in the Full buffer system level simulation. | With layer 2 PDU(Protocol Data Unit) message size of 32 bytes: 1 message/day/device or 1 message/2 hours/device Note: Not modelled in the Full buffer system level simulation. |
| Simulation bandwidth | Up to 10 MHz | Up to 50 MHz |
| UE density | For full buffer system level simulation followed by link level simulation, 10 UEs per TRxP for SINR CDF distribution derivation | For full buffer system level simulation followed by link level simulation, 10 UEs per TRxP for SINR CDF distribution derivation |
| UE antenna height | 1.5 m | 1.5 m |

### Link level

The second step of the full buffer evaluation requires link level evaluations. The link level assumptions for the Urban Macro-mMTC test environment presented in the below table.

Table 3: Urban Macro-mMTC link level definition [4].

| Parameters | Configuration |
| --- | --- |
| Evaluated service profiles | Full buffer best effort |
| Simulation bandwidth | For ISD = 500 m, up to 10 MHz; For ISD = 1732 m, up to 50 MHz |
| Number of users in simulation | 1 |
| Packet size | 32 bytes at Layer 2 PDU |
| Inter-packet arrival time | 1 message/day/device or 1 message/2 hours/device |
| Link level Channel model | NLOS: TDL-iii LOS: TDL-v Note: Only NLOS is evaluated. |
| Delay spread | 363.1 ns |

# Simulation configuration

Table 4 presents the herein investigated NR, LTE-M and NB-IoT system level configurations, that overrides or comes in addition to those presented in Table 2.

Table 4: System level simulation configuration.

| Parameter | NR | LTE-M | NB-IoT |
| --- | --- | --- | --- |
| System bandwidth | 180kHz | 180kHz | 180 kHz |
| BS transmit power per PRB | 29 dBm |  |  |
| Number of BS antenna elements per TRxP | 16 Tx/Rx, (M,N,P,Mg,Ng) = (8,1,2,1,1), (dH,dV) = (N/A, 0.8)λ +45°, -45° polarization [5] |  |  |
| Number of TXRU per TRxP | 2TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) [5] |  |  |
| Number of UE antenna elements | 1Tx/Rx 0° polarization [5] |  |  |
| Number of TXRU per UE | 1TXRU [5] |  |  |
| UE antenna element pattern | Omni-directional [5] |  |  |
| Channel model variant | Alt. 1: Channel model A Alt. 2: Channel model B |  |  |
| TRxP per site | 3 [5] |  |  |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) |  |  |
| Electronic tilt | 99° for configuration A 93° for configuration B |  |  |
| Handover margin (dB) | 2 dB |  |  |
| TRxP boresight | 30 / 150 / 270 degrees [5] |  |  |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 [5] |  |  |
| Wrapping around method | Radio distance based wrapping |  |  |
| Minimum distance of TRxP and UE | 10 m [5] |  |  |
| Polarized antenna model | Model-2 in TR36.873 [5] |  |  |
| System layout | 7 sites, 3 sectors per site |  |  |
| UL Power control SINR target | 3 dB | 3 dB | Config A: 10 dB Config B: 3 dB |

Table 5 presents the investigated NR, LTE-M and NB-IoT link level configurations, that overrides or comes in addition to those presented in Table 3.

Table 5: Link level simulation configuration.

| Parameter | NR | LTE-M | NB-IoT |
| --- | --- | --- | --- |
| Physical channel | PUSCH | PUSCH | NPUSCH F1 |
| Channel bandwidth | 180 kHz | 180 kHz | 15 kHz |
| Subcarrier spacing | 15 kHz | 15 kHz | 15 kHz |
| TBS | 32, 42, 48, 64, 80, 104, 144, 168, 184 bits | 144, 256 bits | 256 bits |
| Modulation | QPSK | 16QAM/QPSK | QPSK |
| # Resource units | - | - | 2, 3, 4, 5, 6, 8, 10 |
| # Repetitions | - | 1, 2, 4, 8, 16, 32 | 1, 2, 4, 8, 16 |
| HARQ | 3 transmissions | - | - |
| Scenario | Sensitivity limited | Sensitivity limited | Sensitivity limited |
| Channel estimation | Realistic | Realistic | Realistic |
| SNR range | -15…15 dB | -15…15 dB | -15…15 dB |

# Connection density evaluation

## Full buffer system level performance

Figure 1, Figure 2 and Figure 3 presents the UL SINR distributions required in Step 1 in the Full buffer system level simulation. The NR and LTE-M distributions are identical due to the similar channel bandwidth and uplink power control target.

Figure 1 NR UL SINR CDFs.

Figure 2 LTE-M UL SINR CDFs.

Figure 3 NB-IoT UL SINR CDFs.

## Full buffer link level performance

Figure 4 presents the LTE-M, NR and NB-IoT UL link spectral efficiencies required in Step 2 in the Full buffer evaluation methodology.. The SNR is defined for 15 kHz in the case of NB-IoT and 180 kHz for LTE-M and NR.

For NB-IoT up to 16 repetitions was simulated to reach adequate performance in the low SINR domain. A fixed TBS of 256 bits was simulated, which exactly matches the agreed packet size of 32 bytes.

For LTE-M up to 32 repetitions were used. For the TBS of 256 bits 16QAM modulation is used which explains the high SPEFF for LTE-M at good SINRs. The QPSK TBS of 144 bits contributes to the LTE-M link adaptation envelope for SINR < ~4 dB.

For NR a range of TBSs up to size of 184 bits was simulated. This explains why the NR link adaptation curve is smoother than the LTE-M and NB-IoT curves. The limit of 184 bits also explains why the NR spectral efficiency flattens between SNR 5-10 dB. For NR 3 HARQ transmission was applied to secure performance in the low SNR domain. Note that no consideration was given to the control channel performance in this SNR region.

The performance difference shown in Figure 4 can be attributed to the different assumptions used when running the simulations for the three technologies. It should also be noticed that three different simulators were used to derive the results. Before comparing the relative performance, it is necessary to first calibrate the simulators, which has not yet been done.

Figure 4 LTE-M, NR and NB-IoT link spectral efficiency (SPEFF).

## Performance

Based on the link and system level performance presented in sections 4.1 and 4.2 the performance shown in Table 6 and Table 7 were derived. The results are presented for a bandwidth of 180 kHz.

All three investigated solutions meet the IMT-2020 connection density target with margin. NB-IoT offers the highest capacity due to the support for sub-carrier UL scheduling. The difference in delay and capacity between LTE-M and NR can be explained by the link level performance difference shown in  Figure 4.

Table 6 99th percentile delay D recorded in Step 3.

|  | Conf A, UMA A | Conf A, UMA B | Conf B, UMA A | Conf B, UMA B |
| --- | --- | --- | --- | --- |
| NR | 0,006 | 0,007 | 0,067 | 0,063 |
| LTE-M | 0,009 | 0,010 | 0,061 | 0,057 |
| NB-IoT | 0,077 | 0,090 | 0,099 | 0,126 |

Table 7 Connection density C recorded in Step 7.

|  | Conf A, UMA A | Conf A, UMA B | Conf B, UMA A | Conf B, UMA B |
| --- | --- | --- | --- | --- |
| NR | 35 426 393 | 35 212 804 | 1 637 184 | 2 004 071 |
| LTE-M | 25 674 701 | 25 524 579 | 1 231 947 | 1 476 635 |
| NB-IoT | 46 058 578 | 45 272 306 | 2 237 326 | 2 103 872 |

# Conclusions

In this contribution, we have presented NR, LTE-M and NB-IoT IMT-2020 connection density performance for the full buffer evaluation procedure. It has been shown that all three technologies with margin meets the requirement of 1.000.000 devices/km2.

# References

RP-171451, “New Study Item on Self Evaluation towards IMT-2020 Submission”, RAN#75, Ericsson et al

RP-172098, “3GPP submission towards IMT-2020”, ITU-R Ad-Hoc Contact Person

[PRELIMINARY] DRAFT NEW REPORT ITU-R M. [IMT-2020.SUBMISSION], 27th meeting of WP5D, ITU-R WP5D

[PRELIMINARY] DRAFT NEW REPORT ITU-R M. [IMT-2020.EVAL], 27th meeting of WP5D, ITU-R WP5D

RP-172728, LS on Summary of email discussion “[ITU-R AH 01] Calibration for self-evaluation”, RAN#78, Huawei

R1-1806436, “IMT-2020 self evaluation: mMTC non-full buffer connection density”, RAN1#93, Ericsson

R1-1805232, “IMT-2020 self evaluation: mMTC full buffer connection density”, RAN1#92b, Ericsson