3GPP TSG RAN WG1 Meeting #93			R1-1806535

Busan, Korea, May 21st - May 25th, 2018

Source:	Intel Corporation

Title:	Initial Link Level Evaluation for NOMA - mMTC

Agenda item:	7.4.5

Document for:	Discussion and Decision

# Introduction

In RAN1 #92, we have agreed on link level evaluation methodology including evaluation parameters and performance metrics. This contribution provides initial link level results of the proposed NOMA Tx/Rx schemes by following the agreed evaluation assumptions focusing on mMTC use case.

# LLS Performance

## Evaluation assumption

Table 1 illustrates the link level evaluation assumptions agreed in RAN1#92. Shaded parts are for mMTC.

Table 1: Link-level evaluation assumptions

| Parameters | mMTC | URLLC |  | eMBB |  | Further specified values |
| --- | --- | --- | --- | --- | --- | --- |
| Carrier Frequency | 700 MHz | 700 MHz or 4 GHz |  | 4 GHz, 700 MHz as optional |  |  |
| Waveform (data part) | CP-OFDM and DFT-s-OFDM | CP-OFDM as starting point |  | CP-OFDM as starting point |  |  |
| Channel coding | URLLC: NR LDPC eMBB: NR LDPC mMTC: NR LDPC |  |  |  |  | The choice of channel coding here is only for the performance evaluation purpose for NOMA study |
| Numerology (data part) | SCS = 15 kHz, #OS = 14 | Case 1: SCS = 60 kHz, #OS = 7 (normal CP), optionally 6 (ECP) Case 2: SCS = 30 kHz, #OS = 4 |  | SCS = 15 kHz #OS = 14 |  |  |
| Allocated bandwidth | 6 as the starting point | 12 as the starting point |  | 12 as the starting point |  | For high payload such as 75 bytes, larger number of RBs can be considered. |
| TBS per UE | At least five TBS that are [10, 20, 40, 60, 75] bytes. Other values higher than 10 bytes are not precluded. Lower than 0.1 bits/RE is optional | At least five TBS that are [10, 20, 40, 60, 75] bytes. Other values higher than 10 bytes are not precluded. |  | At least five TBS that are [20, 40, 80, 120, 150] bytes. Other values higher than 20 bytes are not precluded. |  | #bits per RE calculation does not include DMRS overhead (e.g., REs of one every 7 symbols for DMRS would not be used to carry the data) |
| Target BLER for one transmission | 10% | 0.1% |  | 10% |  |  |
| Number of UEs multiplexed in the same allocated bandwidth | To be reported by companies. |  |  |  |  | Companies are encouraged to perform evaulations with various number of UEs Note: refined set of numbers of UEs should be further discussed in the next meeting. |
| BS antenna configuration | 2 Rx or 4 Rx for 700MHz, 4Rx or 8 Rx for 4 GHz 8Rx as optional |  |  |  |  | CDL model in 38.901 should be considered for 8Rx |
| UE antenna configuration | 1Tx |  |  |  |  |  |
| Propagation channel & UE velocity | TDL-A 30ns and TDL-C 300ns in TR38.901, 3km/h, CDL optional |  |  |  |  |  |
| Max number of HARQ transmission | 1 as starting point. |  | 1 as starting point. More values, 2 for URLLC can be used. |  | 1 as starting point. |  |
| Channel estimation | Ideal channel estimation results should be reported for calibration Realistic channel estimation |  |  |  |  |  |
| MA signature allocation (for data and DMRS) | Fixed/Random |  |  |  |  | Proponents report the details of random MA signature allocation (whether without or with collision) |
| Distribution of avg. SNR | Both equal and unequal | Equal |  | Both equal and unequal |  | Uniform discrete values for unequal case,, range [x - a, x + a] (dB) with 1 dB step, where x is the average SNR among UEs, and the deviation [a=3] |
| Timing offset | 0 as starting point. For grant-free without perfect TA, value is TBD |  |  |  |  |  |
| Frequency error | 0 as starting point. The value(s) is TBD. |  |  |  |  |  |
| Traffic model for link level | Full buffer as starting point. Non-full-buffer model (like Poisson arrival of fixed packet size) is optional. |  |  |  |  |  |
| For link level calibration purpose only | OMA single user whose spectral efficiency is the same as per UE SE in NOMA. AWGN curves can be provided also. |  |  |  |  |  |

## Initial Link Level Evaluation Results

In this section, we evaluated our proposed NOMA scheme, LCRS as introduced in [2], under the evaluation assumptions for mMTC use case. For the receiver structure, we used the modified SIC scheme as explained in [3]. For the practical channel estimation, we used 2-dimensional MMSE channel estimation with inter-PRB sliding window filtering. Evaluations are based on the assumptions in Table 1. (we used the ones in red color in the Table 1 for the parameter with multiple options)

### Performance with Ideal channel estimation

Figure 1: Ideal CE, TDL-A 30ns

Figure 2: Ideal CE, TDL-C 300ns

### Performance with practical channel estimation

Figure 3: Practical CE, TDL-A 30ns

Figure 4: Practical CE, TDL-C 300ns

# Conclusions

In this contribution, we provide some initial link level evaluation based on LCRS NOMA scheme under the LLS assumptions for mMTC scenario. It is proposed that

- LLS results with Ideal CE can be used for the calibration purpose

- LLS results with practical CE can be used mainly for the performance evaluation between different schemes, e.g., different NOMA tx schemes, different receiver schemes, different DMRS schemes, etc.

# References

- RP-171043, “Revision of Study on 5G Non-orthogonal Multiple Access”, ZTE, West Palm Beach, USA, June, 2017

- R1-1806531, “NOMA transmission scheme: LCRS”, Intel Corporation

- R1-1804744, “Receiver structure for NOMA”, Intel Corporation