3GPP TSG-RAN WG1 Meeting #93															R1-1807185

Busan, Korea, May21st - May25th, 2018

Agenda item:		7.1.2.2.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining Details on Beam Recovery

Document for:		Discussion and Decision

# 1	Introduction

In this contribution we discuss on the remaining details on beam recovery after RAN1#92bis.

# 2	Discussion

## 2.1 Explicit configuration of SS block index to resource set q0

Currently in 38.213 chapter 6 Link reconfiguration reads following:

“A UE can be configured, for a serving cell, with a set  of periodic CSI-RS resource configuration indexes by higher layer parameter Beam-Failure-Detection-RS-ResourceConfig and with a set  of CSI-RS resource configuration indexes and/or SS/PBCH block indexes by higher layer parameter Candidate-Beam-RS-List for radio link quality measurements on the serving cell. “

The sentence states that only periodic CSI-RS resources can be explicitly configured to be included in set q0. RAN1 has not explicitly made any decision to particularly exclude other configurations and especially we don’t see any technical reason why the SS/PBCH block index should note be explicitly configurable in set q0 as it is anyway possible to configure them implicitly.

Observation 1: RAN1 has not made an explicit agreement to exclude configuration of SS/PBCH Block indexes to set q0

It should be noted that the SS/PBCH block indexes were still included in the 2017-10 version of 38.213 but left out in succeeding versions.

Also, RAN2#101 agreement, included in the earlier RAN2 LS (R1-1803577) to RAN1 indicates that one list (RRC) can be used to indicate whether an RS can be used for both beam- and cell-RLM, where the beam-RLM refers to beam failure detection. As it has been agreed already in RLM track that both RS types, CSI-RS and SS blocks, can be configured for radio link monitoring.

Agreements

1	Introduce one list of RSs and indicate for each whether it is used for beam- and/or cell-RLM.

1a	If no RSs are provided for Beam-Monitoring, the UE performs Beam-Monitoring based on the TCI-State for PDCCH (as agreed by RAN1)

2	If no RSs are provided in this list at all (neither for Cell- nor for Beam-RLM), the UE performs Cell-RLM based on TCI-State of PDCCH

Observation 2: UE can be configured explicitly with both RS types, CSI-RS and SS/PBCH Block indexes, for radio link monitoring

The RAN2 agreed signaling mechanism allows network to efficiently configure i.e. the same signals for beam failure detection and RLM using a single list, but currently due to limitations of explicit q0 configuration in 38.213 6 Link Reconfiguration, it is not possible to include SS/PBCH block indexes explicitly to set of q0, thus, this limits the efficient configuration of failure detection RS using a single list. We see no reason for this, and would respectfully ask RAN1 to reinstate the earlier agreement to allow both periodic CSI-RS and SS/PBCH block to be explicitly configured to set of q0.

Proposal 1: SS/PBCH block indexes can be explicitly configured to set of q0

Proposal 2: Adopt the text proposal to 38.213 Link Reconfiguration Procedure in annex A and inform RAN2 on the decision to capture it in RRC specification.

## 2.2 Maximum Number of Beam Failure Detection Resources

RAN1#92

Agreement (RRC parameter update):

maxNrofFailureDetectionResources is 2 per BWP

As per agreement in the RAN1#92, the maximum number of beam failure detection resources is limited to X=2 per BWP. In contrast, UE can be configured with maximum of 3 CORESET, each with associated active TCI State for PDCCH. This mismatch introduces ambiguity for UE that which resources is should monitor for beam failure detection (set of q0) in case of implicit configuration. Also, in case of explicit configuration of q0, network is not able to configure UE with 3 BFD-RS.

Observation 3: With current maximum number of BFD-RS the implicit q0 configuration has ambiguity

Thus, we propose to increase the maximum number of beam failure detection resources to be 3 to match the maximum number of CORESETs. In case the proposal is not accepted, UE should have clear rule to determine which DL RS are included in the set of in case of implicit configuration. Also, network should be allowed to configure UE with corresponding number of BFD-RS and CORESETs with active TCI States for PDCCH.

Proposal 3: Increase the maximum number of beam failure detection resources per BWP to be X =3.

Proposal 4: If proposal 3 is not accepted, define how UE selects subset of failure detection resources when implicit BFD-RS configuration is used.

## 2.3. Discussion on RAN2 Reply LS to RAN1 on beam failure recovery

In RAN2 LS to RAN1 on beam failure recovery (R1-1805802), four questions (Q4-Q7) were provided to RAN1 regarding the SCell beam failure recovery. We propose the following answers to be included in the LS.

Q4: The purpose of BFR on SCell is to support a rapid recovery mechanism in scenarios where the PCell typically is on sub 6 GHz, but SCell is on mmW, according to R1-1803397 (RAN1 e-mail discussion, referenced in the LS). In this scenario it is not unlikely that the SCell will be downlink-only to make use of the better uplink coverage in the PCell. Shall BFR on SCells with downlink only be supported, in addition to SCells with downlink and uplink?

A4: Yes, to have support for wider range of deployment for NR, the DL only SCell should be supported as well.

Q5: RAN2 has so far identified the following broad solutions for BFR on SCell:

- CFRA BFR on SCell UL and SCell DL. The CORESET-BFR for BFR response monitoring should be configured in USS.

- CFRA BFR on SCell UL and PCell DL, using the same CORESET-BFR as BFR on SpCell.

- CFRA BFR on PCell UL and PCell DL, using same resources as BFR on SpCell but different preambles.

- MAC CE transmission on PCell to indicate the new beams.

Other solutions are not precluded. Given that RAN1 concluded that "there is no additional RAN1 specification impact", is there any solution which should be avoided, or which is preferred from a RAN1 perspective?

A5: MAC CE option provides most flexibility for SCell candidate indication (q_new) and can be considered also as fallback solution when CFRA signaling cannot be used i.e. UE cannot indicate any candidates using the dedicated signals, or network has not configured CFRA at all.

Q6: RAN2 understand that the UE may be configured with several SCells in mmW. What is the reason to limit the support of BFR to one SCell?

A6: From RAN1 perspective the number of SCells to support could be determined based on SCell group, where one group shares failure detection properties i.e. the failure of set of SCells could be determined based on the failure of a single SCell in the group. In this way multiple SCells could be supported without increasing the beam failure detection complexity. From UE perspective it could perform failure detection for only one SCell out of set of SCells that share the failure detection signals.

Q7: What is the intended UE behavior of BFR (e.g. whether to stop beam failure detection/recovery) on the deactivated SCell?

A7: Stop SCell Recovery upon deactivation either by command or timer.

Above answers are provided also in the draft response in document R1-1807180.

Proposal 5: Adopt the draft LS R1-1807180 as RAN1 reply on RAN2 LS on Beam failure recovery

## 2.4. On the issues regarding CBRA for BFR

In BFR summary document, companies were requested to provide views on CBRA BFR.

### 2.4.1 Indicating BFR purpose to network in CBRA

Currently RAN2 has not specified any explicit indication for beam failure recovery when CBRA is used. It can be assumed, that NW is able to detect the BFR when it identifies UE during the contention based random access procedure (UE still has valid C-RNTI). At least this should be the approach for release-15 (this was also agreed by RAN2#101) and potential enhancements, if needed, can be discussed later.

### 2.4.2 gNB response for CBRA BFR

The gNB response to CBRA BFR could be considered to be the completion of random access procedure. This may be either successful or unsuccessful. In case of unsuccessful completion of BFR using CBRA (in fact using CFRA or CBRA) causes MAC layer to indicate RRC the random access problem.

### 2.4.3 Implicit TCI State after successful completion of CBRA BFR

An implicit TCI State configuration for PDCCH based on the indicated reference signal (SSB) could also be used in case of CBRA based beam failure recovery. When UE indicates SSB for recovery and receives network response (in practice after successful completion of RACH procedure) UE assumes the TCI State for PDCCH and PDSCH to be the indicated SSB until reconfigured by network.

Proposal 6: Upon successful completion of CBRA BFR, UE assumes the TCI State for PDCCH and PDSCH to be the indicated candidate SSB until reconfigured by network

## 2.5. Implicit BFD-RS configuration

In previous meeting the issue of implicit configuration of BFD-RS when TCI State associated with multiple RS was discussed. Similar discussion was also held in radio link monitoring track. In our view, if UE is configured to determine the BFD-RS resources implicitly based PDCCH TCI State activation (indication) and the TCI state is associated with multiple RS, the RS that has the configuration for QCL type-d should be used as BFD-RS.

Proposal 7: In implicit configuration of BFD-RS, when indicated TCI State for PDCCH is associated to multiple RS, UE selects the RS that has type-D QCL configuration.

If the TCI state has no type-D QCL association, e.g. in FR1, it is up to UE to selects the periodic RS.

In related issue, when the TCI state (or the selected BFD-RS based on type-D QCL condition) indicates the aperiodic or semi-persistent RS TCI state and implicit configuration of BFD-RS is used by UE, it should select the periodic source RS as BFD-RS since the beam failure detection is based on periodic signals.

Proposal 8: when the TCI state indicates the aperiodic or semi-persistent RS as TCI state for PDCCH and implicit configuration of BFD-RS is used by UE, it selects the periodic source RS of TCI state given in NZP-CSI-RS-Resource as BFD-RS.

## 2.6. gNB Response Monitoring Window

Currently, in CFRA BFR, UE needs to wait 4 slots before the monitoring window for beam failure recovery starts. In Random access procedure UE starts monitoring the gNB response starting from the next PDCCH occasion after transmitting msg.1 regardless of CBRA (for initial access or for beam recovery) or CFRA when the CFRA is used for other purposes that beam failure recovery. In our view, the random access procedure and BFR should be harmonized since they are essentially part of same procedure and there is no clear reason why the monitoring window for BFR has 4 slot delay. In fact, it may only delay the potential recovery since network is not able to respond any earlier. Such additional and artificial increase in latency for BFR is not desirable.

Proposal 9: For beam failure recovery using dedicated signals, (CFRA) start gNB response monitoring window on the next PDCCH occasion after transmission of msg.1.

## 2.7. Resource Set for Beam Failure Detection

Currently in 38.213 v15.0.1 the chapter 6 for link reconfiguration procedures reads:

If the UE is not provided with higher layer parameter Beam-Failure-Detection-RS-ResourceConfig, the UE determines the set  to include SS/PBCH block indexes and periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by the TCI states for respective control resource sets that the UE is configured for monitoring PDCCH

Although it can be understood that “monitoring PDCCH” refers to TCI states that UE is currently configured for PDCCH reception hence, considered as active TCI states, it would be preferred if this is indicated more clearly in the text.

Proposal 10: Adopt the text proposal to link reconfiguration procedures in annex B.

## 2.8. Beam Failure Detection Resource before PDCCH TCI state configuration and activation

Currently the 38.213 the of support of implicit configuration of BFD-RS (q0) based on PDCCH TCI state allows beam failure detection to be performed after UE has been provided with RRC configuration of TCI table for PDCCH and activation of TCI state using MAC CE. Similar procedure has also been agreed for RLM-RS configuration.

As the TCI state configuration and activation requires RRC+MAC signaling and in practice UE needs to be in connected mode, there is an ambiguity or actually lack of specified behavior of how UE performs beam failure detection the during time between initial access (msg.1/msg.2 onwards) and before the activation of TCI state for PDCCH./PDSCH

Observation 4: UE is not able to perform beam failure detection before the activation of TCI state.

One proposed option would be defining in the 38.213 the assumption of implicit TCI State configuration for PDCCH and PDSCH based on the indicated reference signal (SS/PBCH Block) in initial access. In more general view this approach could be used also in case of handover/beam failure recovery using dedicated signaling.

Proposal 11: Assume implicit TCI-State for PDCCH/ configuration based on the associated downlink RS (SS/PBCH block) with PRACH preamble. Assuming implicit TCI State for PDCCH allows Beam Failure Detection before the explicit configuration and activation of TCI state by RRC+MAC CE signaling.

An implicit TCI State configuration for PDCCH based on the indicated reference signal (SSB) could also be used in case of CBRA based beam failure recovery. When UE indicates SSB for recovery and receives network response (in practice after successful completion of RACH procedure) UE assumes the TCI State for PDCCH and PDSCH to be the indicated SSB until reconfigured by network

## 2.9. CORESET for Monitoring gNB Response for BFRQ

Agreements: Upon receiving gNB response for beam failure recovery request transmission,

- UE shall monitor CORESET-BFR for dedicated PDCCH reception until one of the following conditions is met:

  - Reconfigured by gNB to another CORESET for receiving dedicated PDCCH and activated by MAC-CE a TCI state if the configured CORESET has K>1 configured TCI states

    - FFS: if a default TCI state can be assumed for PDCCH after reconfiguration without MAC-CE activation

  - Re-indicated by gNB to another TCI state(s) by MAC-CE ‘of CORESET(s) before beam failure

As per RAN1 agreement, for beam failure recovery request using dedicated resources UE monitors dedicated Beam-failure-Recovery-Response-CORESET (CORESET-BFR) for gNB response until reconfigured with new/another TCI state. Beam failure detection is done on the active TCI state(s) associated with configured CORESET for PDCCH reception (or multiple CORESETs).

Observation 5: Beam failure detection is performed on active TCI states configured for CORESET for PDCCH reception

When declaring beam and selecting new candidate beam, UE may be able to indicate another TCI state of the same CORESET which has a signal quality above candidate beam threshold. In this case it would not be optimal to switch the monitoring to CORESET-BFR for receiving gNB response as UE already has a configuration for CORESET that has still TCI states above candidate beam quality threshold.

If UE switches the monitoring to CORESET-BFR, it means in turn that network has to transmit the BFRQ (beam failure recovery request) response using the CORESET-BFR although the response to recovery request could be transmitted using the CORESET that is associated to the indicated TCI state.

Observation 6: When beam failure has been declared and UE indicates inactive TCI state (CSI-RS / SS/PBCH Block) of a current CORESET (for PDCCH) as new candidate, it is not beneficial switch the monitoring to alternative CORESET(-BFR) when monitoring gNB response.

We propose a small change to the current beam failure recovery procedure which would speed up the recovery and reduce the signaling overhead in cases where a candidate beam is an inactive TCI configured for CORESET currently monitored for PDCCH reception.

If multiple candidate beams that can be indicated using dedicated resources are above the configured candidate beam threshold it has been agreed that the selection is up to UE but considering the potential benefits of not having to switch monitoring to CORESET-BFR and obtain new TCI configurations, UE should if not prioritize, prefer the recovery on inactive TCI states when determined to have adequate signal quality.

Proposal 12: Adopt the text proposal on link reconfiguration in annex B.

# 3	Conclusions

Observation 1: RAN1 has not made an explicit agreement to exclude configuration of SS/PBCH Block indexes to set q0

Observation 2: UE can be configured explicitly with both RS types, CSI-RS and SS/PBCH Block indexes, for radio link monitoring

Observation 3: With current maximum number of BFD-RS the implicit q0 configuration has ambiguity

Observation 4: UE is not able to perform beam failure detection before the activation of TCI state.

Observation 5: Beam failure detection is performed on active TCI states configured for CORESET for PDCCH reception

Observation 6: When beam failure has been declared and UE indicates inactive TCI state (CSI-RS / SS/PBCH Block) of a current CORESET (for PDCCH) as new candidate, it is not beneficial switch the monitoring to alternative CORESET(-BFR) when monitoring gNB response.

Proposal 1: SS/PBCH block indexes can be explicitly configured to set of q0

Proposal 2: Adopt the text proposal to 38.213 Link Reconfiguration Procedure in annex A and inform RAN2 on the decision to capture it in RRC specification.

Proposal 3: Increase the maximum number of beam failure detection resources per BWP to be X =3.

Proposal 4: If proposal 3 is not accepted, define how UE selects subset of failure detection resources when implicit BFD-RS configuration is used.

Proposal 5: Adopt the draft LS R1-1807180 as RAN1 reply on RAN2 LS on Beam failure recovery

Proposal 6: Upon successful completion of CBRA BFR, UE assumes the TCI State for PDCCH and PDSCH to be the indicated candidate SSB until reconfigured by network

Proposal 7: In implicit configuration of BFD-RS, when indicated TCI State for PDCCH is associated to multiple RS, UE selects the RS that has type-D QCL configuration.

Proposal 8: when the TCI state indicates the aperiodic or semi-persistent RS as TCI state for PDCCH and implicit configuration of BFD-RS is used by UE, it selects the periodic source RS of TCI state given in NZP-CSI-RS-Resource as BFD-RS.

Proposal 9: For beam failure recovery using dedicated signals, (CFRA) start gNB response monitoring window on the next PDCCH occasion after transmission of msg.1.

Proposal 10: Adopt the text proposal to link reconfiguration procedures in annex B.

Proposal 11: Assume implicit TCI-State for PDCCH/ configuration based on the associated downlink RS (SS/PBCH block) with PRACH preamble. Assuming implicit TCI State for PDCCH allows Beam Failure Detection before the explicit configuration and activation of TCI state by RRC+MAC CE signaling.

Proposal 12: Adopt the text proposal on link reconfiguration in annex B.

# References

- RAN1#92 Chairman Notes

- 38.213 v15.1.0

- 38.331 15.1.0 March 2018 draft

- R2-1804066 (R1-1803577), LS to RAN1 on Beam- and Cell- Radio-Link-Monitoring, RAN2

- R1-1807180 RAN1 reply to RAN2 LS on Beam failure recovery

- R1-1805730, Summary 3 on Remaining issues on Beam Failure Recovery, Mediatek Inc.

# Annex A. Text Proposal for 38.213 on Link Reconfiguration Procedure

=== Text Proposal Starts 38.213 v15.1.0 ===

### 6 Link reconfiguration procedures

A UE can be configured, for a serving cell, with a set  of SS/PBCH block indexes and/or periodic CSI-RS resource configuration indexes by higher layer parameter Beam-Failure-Detection-RS-ResourceConfig and with a set  of CSI-RS resource configuration indexes and/or SS/PBCH block indexes by higher layer parameter Candidate-Beam-RS-List for radio link quality measurements on the serving cell. If the UE is not provided with higher layer parameter Beam-Failure-Detection-RS-ResourceConfig, the UE determines the set  to include SS/PBCH block indexes and periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by the TCI states for respective control resource sets that the UE is configured for monitoring PDCCH. The UE expects single port RS in the set .

--- parts that are not affected are omitted ---

=== Text Proposal Ends 38.213 v15.1.0 ===

# Annex B. Text Proposal for 38.213 on Link Reconfiguration Procedure

=== Text Proposal Starts 38.213 v15.1.0 ===

### 6 Link reconfiguration procedures

--- parts that are not affected are omitted ---

If the UE is not provided with higher layer parameter Beam-Failure-Detection-RS-ResourceConfig, the UE determines the set  to include SS/PBCH block indexes and periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by the active TCI states for respective control resource sets that the UE is configured for monitoring PDCCH

--- parts that are not affected are omitted ---

=== Text Proposal Ends 38.213 v15.1.0 ===

# Annex C. Text Proposal for 38.213 on Link Reconfiguration Procedure

=== Text Proposal Starts ===

### 6 Link reconfiguration procedures

--- parts that are not affected are omitted ---

A UE is configured with one control resource set by higher layer parameter Beam-failure-Recovery-Response-CORESET. The UE may receive from higher layers, by parameter Beam-failure-recovery-request-RACH-Resource, a configuration for a PRACH transmission as described in Subclause 8.1. After 4 slots from the slot of the PRACH transmission and according to an antenna port quasi co-location associated with periodic CSI-RS configuration or SS/PBCH block with index  in set , the UE monitors PDCCH for detection of a DCI format 1_0 or a DCI format 1_1 with CRC scrambled by C-RNTI within a window configured by higher layer parameter Beam-failure-recovery-request-window, and in the control resource set configured by higher layer parameter Beam-failure-Recovery-Response-CORESET. If periodic CSI-RS configuration or SS/PBCH block with index  is included in the TCI-StatesPDCCH UE monitors PDCCH on CORESET associated with index.

The UE determines the index  based on a mapping between the PRACH preamble index and the SS/PBCH block index provided by higher layer parameter SSB-PRACH-CFRA-association or based on a mapping between the PRACH preamble index and a periodic CSI-RS configuration index provided by higher layer parameter CSI-RS-PRACH-association.

--- parts that are not affected are omitted ---

=== Text Proposal Ends ===