3GPP TSG-RAN WG1 Meeting #93	R1-1807285

Busan, Korea, 21st - 25th May 2018

Agenda item:		7.8.2

Source:	Nokia, Nokia Shanghai Bell

Title:	IMT-2020 Reliability Evaluation Results (URLLC)

Document for:		Discussion and Decision

# 1	Introduction

In this contribution, we discuss the evaluation of the reliability performance indicator for the self-evaluation of NR for IMT-2020 and provide performance results.

# Discussion of Evaluation Methods

For the IMT-2020 self-evaluation, the reliability performance indicator is evaluated in the Dense Urban – URLLC test environment using the following steps [2]:

Run downlink or uplink full buffer system-level simulations of candidate RITs/SRITs using the evaluation parameters of Urban Macro-URLLC test environment see § 8.4.1 below, and collect overall statistics for downlink or uplink SINR values, and construct CDF over these values.

Use the CDF for the Urban Macro-URLLC test environment to save the respective 5th percentile downlink or uplink SINR value.

Run corresponding link-level simulations for either NLOS or LOS channel conditions using the associated parameters in the Table 8-3 of this Report, to obtain success probability, which equals to (1-Pe), where Pe is the residual packet error ratio within maximum delay time as a function of SINR taking into account retransmission.

The proposal fulfils the reliability requirement if at the 5th percentile downlink or uplink SINR value of Step 2 and within the required delay, the success probability derived in Step 3 is larger than or equal to the required success probability. It is sufficient to fulfil the requirement in either downlink or uplink, using either NLOS or LOS channel conditions.

The required delay and required success probability referred to in Step 4 are 1 ms and 1-10-5, respectively, using transmission of 32 byte packets (100 byte packets are encouraged) [1].  Equivalently, the required error probability is 10-5 under the same delay requirements.  The delay is defined in [1] as the time from the packet arrival at the transmitter layer 2/3 interface (ingress) to its departure at the receiver layer 2/3 interface (egress).

# 3	System Simulation Results

The system simulations follow the Urban Macro – URLLC configurations given in [2] with additional parameters given in Table 3 .  Full buffer simulations are run for both Configuration A at a carrier frequency of 4 GHz and Configuration B at a carrier frequency of 700 MHz, both using channel model A (UMa_A).  All simulations were run using a bandwidth of 10 MHz.  Distributions of the post-processing SINR’s were collected from the simulation results and are shown in Figure 1.  The 5th percentile SINR’s were extracted from the distributions and are shown in Table 1.

The 5th percentile (cell edge) SINR’s in the Urban Macro – URLLC environment for Configuration A (4 GHz) are 14.52 dB in DL and 3.20 dB in UL.  The 5th percentile SINR’s for Configuration B (700 MHz) are 9.41 dB in DL and 8.04 dB in UL.

Figure 1.  Full Buffer post-processing SINR distributions using UMa_A.

Table 1.  Summary of Results

| Configuration | 5th %-ile SINR [dB] |
| --- | --- |
| Config. A (4 GHz), DL, FDD | 14.52 |
| Config. A (4 GHz), UL, FDD | 3.20 |
| Config. B (700 MHz), DL, FDD | 9.41 |
| Config. B (700 MHz), UL, FDD | 8.04 |

# 4	Link Simulation Results

Link simulations were run for the PDCCH, PUCCH, and data (PDSCH/PUSCH) channels in order to assess the overall transmission reliability which depends on the successful transmission of both control and data signals.  The link simulation assumptions are found in [2] with additional details in Table 4.  Simulations are performed using 1 transmit antenna and 1 receive antenna with omnidirectional antennas because antenna effects are taken into account in the system level simulations.

The link performance of the PDCCH is shown in Figure 2 where the performance for aggregation levels of 4, 8, and 16 are shown.  The simulations use a 39 bit payload on 1 OFDM symbol with precoder cycling at 4 GHz.  There is no performance difference at a carrier frequency of 700 MHz.

For PUCCH, link performance results are shown in Figure 3 with results given for NACK2DTX and NACK2ACK.  The simulations assume PUCCH format 0 with a 1 RB allocation using 2 OFDM symbols with frequency hopping.

Data channel link performance (PDSCH/PUSCH) was simulated for mini-slots with 4 and 7 OFDM symbols per slot using 30kHz subcarrier spacing using the TDL-C NLoS channel (363 ns delay spread).  Results are shown in Figure 4.  The number of data bits per slot was 256 bits (32 bytes, CRC overhead not considered) for 4 symbols per slot and 288 bits (36 bytes, with CRC overhead) for 7 symbols per slot.  The impact of the slightly larger packet for 7 symbols is small.  The 1ms latency requirement with URLLC can be met with these four and seven symbol mini-slots.  From these results, we can see that an SINR of at least 5.4 dB is required to achieve a BLER of less than 10-5 with 7 symbols/slot.  With 4 symbols/slot, an SINR of at least 7.6 dB is required.  These values provide a lower bound for data channel reliability as control channel reliability is also taken into account.

Since the 5th percentile SINR for the 4 GHz UL case does not meet the SINR requirements for the NLoS channel, link simulations were also performed for the data channel in the TDL-E LoS channel (93 ns delay spread).  Results are shown in Figure 5. The data channel performance in the TDL-E channel is significantly better and the required SINR to achieve a BLER of less than 10-5 is  -1.7 dB and 0.6 dB for 7 symbols/slot and 4 symbols/slot, respectively.

URLLC service with FR1 is limited to 15kHz and 30kHz subcarrier spacing for the mobile terminals which are studied here.  As noticed in [3,4], 30kHz is better suited for lower latency communication due to the shorter symbol duration.  Table 2 shows the latency for FDD with 30kHz subcarrier spacing using similar assumptions to those in [3,4].  In particular, semi-persistent scheduling (SPS) or blind grant is used for the UL. We see as in [3,4] that 2, 4, 7 and 14 symbols provide acceptable latency for URLLC service.

Table 2.  DL and UL latency analysis for 30kHz subcarrier spacing (UL with SPS or blind grant).

|  | Latency [ms], symbols |  |  |  |
| --- | --- | --- | --- | --- |
| #trans. | 14 | 7 | 4 | 2 |
| 1 | 0.93 | 0.54 | 0.39 | 0.29 |
| 2 | 2.14 | 1.27 | 0.89 | 0.64 |
| 3 | 3.36 | 1.98 | 1.39 | 1.00 |

Figure 2.  PDCCH performance

Figure 3.  PUCCH performance

Figure 4.  PDSCH/PUSCH performance on TDL-C NLoS channel

Figure 5.  PDSCH/PUSCH performance on TDL-E LoS channel

The total reliability is calculated using the methodology described in [3] where the effects of control and data channel reliability are combined over one or more HARQ transmission attempts.  A retransmission gain of 2 dB is assumed with soft combining of all transmissions and using 4 OFDM symbols per slot.  For both downlink and uplink, one front loaded DMRS is used as overhead and a full band (10 MHz) allocation is used for PDSCH and PUSCH. An aggregation level of 16 is selected for the PDCCH and the payload is 39 bits.  Results are shown in Figure 6 through Figure 9 as a function of the percentile, where the BLER must be less than or equal to 10-5 at the 5th percentile.  The TDL-C channel has been used for all cases except the 4 GHz UL where the TDL-E channel has been used for data channel performance.  In all four cases, NR can meet the IMT-2020 requirements.

NR can meet the IMT-2020 reliability requirements for Configuration A (4 GHz) DL with a single transmission attempt with a NLoS channel.

NR can meet the IMT-2020 reliability requirements for Configuration A (4 GHz) UL with a single transmission attempt with a LoS channel.

NR can meet the IMT-2020 reliability requirements for Configuration B (700 MHz) DL and UL with a single transmission attempt with a NLoS channel.

Figure 6.  Total DL Reliability for Configuration A (4 GHz)

Figure 7.  Total UL Reliability for Configuration A (4 GHz)

Figure 8.  Total DL Reliability for Configuration B (700 MHz)

Figure 9.  Total UL Reliability for Configuration B (700 MHz)

# 5	Conclusions

In this contribution, we have discussed the evaluation methods to use for evaluating the reliability performance indicator for the IMT-2020 self-evaluation of NR and provided performance results.  The observations in this contribution may be summarized as:

- The 5th percentile (cell edge) SINR’s in the Urban Macro – URLLC environment for Configuration A (4 GHz) are 9.41 dB in DL and 8.04 dB in UL.  The 5th percentile SINR’s for Configuration B (700 MHz) are 14.52 dB in DL and 10.61 dB in UL.

- NR can meet the IMT-2020 reliability requirements for Configuration A (4 GHz) DL with a single transmission attempt with a NLoS channel.

- NR can meet the IMT-2020 reliability requirements for Configuration A (4 GHz) UL with a single transmission attempt with a LoS channel.

- NR can meet the IMT-2020 reliability requirements for Configuration B (700 MHz) DL and UL with a single transmission attempt with a NLoS channel.

# References

- “Minimum requirements related to technical performance for IMT-2020 radio interface(s),” Report ITU-R M.2410‑0, November 2017.

- “Guidelines for evaluation of radio interface technologies for IMT-2020,” Report ITU-R M.2412-0, October 2017.

- R1-1805238, “IMT-2020 self-evaluation: Reliability in NR”, Ericsson, RAN1#92bis, Sanya, China, 16-20 April 2018.

- R1-1807288, “IMT-2020 self-evaluation: User plane latency”, Nokia, RAN1#93, Busan, Korea, 21st - 25th May 2018.

# Appendix

Table 3.  System Simulation Parameters

| Parameter | Value |
| --- | --- |
| Environment | Urban Macro - URLLC |
| Channel model | UMa_A |
| Carrier frequencies | 4 GHz (Configuration A), 700 MHz (Configuration B) |
| System bandwidth | 4 GHz DL and UL: 10MHz 700 MHz DL and UL: 10MHz |
| Subcarrier spacing | 15kHz |
| Duplex mode | FDD |
| Traffic model | Full buffer |
| gNB transmit power | 46 dBm |
| UE transmit power | 23 dBm |
| gNB antenna element configuration | 4 GHz: (M,N,P) = (8,16,2), (dV,dH) = ( 0.8, 0.5 ) λ, 700 MHz: (M,N,P) = (4,8,2) , (dV,dH) = ( 0.8, 0.5 ) λ +45°, −45° polarization and 90° mechanical downtilt (GCS) in both cases |
| gNB antenna virtualization | 4 GHz: (M,N,P) = (1,16,2) 700 MHz: (M,N,P) = (1,8,2) 99° electronic downtilt (LCS) in both cases |
| UE antenna config. | 4 GHz: (M,N,P) = (1,4,2), dH = 0.5 λ 700 MHz: (M,N,P) = (1,2,2), dH = 0.5 λ 1-to-1 mapping and 0°, 90° polarization in both cases |
| UE antenna pattern | Omni |
| Receiver | MMSE with channel estimation error and interference modeling. |
| MIMO Transmission scheme | Rank 1 SU-MIMO with ideal EBF |
| Scheduler | PF with wideband scheduling |
| Uplink power control | 4 GHz: P0 = −113 dBm, α = 1.0 700 MHz: P0 = −95 dBm, α = 0.8 |

Table 4.  Link Simulation Parameters

| Parameter | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| Subcarrier spacing | 30 kHz |
| Waveform | CP-OFDM |
| Bandwidth | 10 MHz (26 PRB’s for data, 1RB for PUCCH), 20MHz for PDCCH |
| Slot structure | 4 symbols (1xDMRS+3xData) 7 symbols (1xDMRS+6xData) |
| Codec | LDPC |
| HARQ | No HARQ |
| Data bits/slot | PDSCH/PUSCH: 256 bits (4 symbols), 288 bits (7 symbols), PDCCH: 39 bits, PUCCH: 1 bit |
| Antennas | 1Tx/1Rx |
| DMRS | 1xDMRS, Config. 1, 3.0 dB power boost |
| Channel estimator | 2x1D (FD: Wiener, TD: sample-and-hold) |
| Channel model | TDL-C, 363ns at 3km/h, TDL-E, 93ns at 3km/h |