3GPP TSG-RAN WG1 Meeting 93	R1-1807332

Busan, Korea 21st – 25th May 2018

Agenda item:	7.1.1.4.1

Source: 	Qualcomm Incorporated

Title: 	Remaining Details on RACH Formats

Document for:	Discussion/Decision

# Introduction

This article focuses on confirming the previous working assumption that handles conflicts between SSB and DL parts with RACH occasions.

# Handling conflicts between DL/SSB and RACH occasions

RAN1 made the following working assumption in the last meeting.

R1-1805738 Agreements: - For both FR1 and FR2, the previous working assumption is replaced with the following working assumption - Note: The value with yellow background is not final - For the Semi-static UL/DL configuration in RMSI, only PRACH occasions within the UL part and X part are valid as long as it does not precede or collide with an SSB in the RACH slot, and it is more than at least N symbols after DL part and/or the last symbol of an SSB. - N = 0 for Msg1 SCS = 1.25kHz, 5kHz - N = 2 for Msg1 SCS = 15kHz - N = 2 for Msg1 SCS = 30kHz - N = 2 for Msg1 SCS = 60kHz - N = 2 or 3 for Msg1 SCS = 120kHz - Note for format B4, N = 0 irrespectively of Msg1 SCS - UE is not expected to receive DL signals during any valid RACH occasion - The SSB to RO association pattern period shall repeat at most every 160 ms - If there are leftover ROs after an integer number of SSB to RO association period(s) within the SSB to RO association pattern period. Those leftover ROs are invalid. - Note: In the UL/DL semi-static configuration, 0 UL slots and 0 UL symbols can be configured

Proposal 1: RAN1 confirms above working assumption.

A higher gap between SSB/DL part and RACH occasion is needed to support higher cell radius. But networks with higher cell radius can avoid inter-cell interference from SSB to RACH through proper implementation, e.g., by selecting PRACH configuration indices in such a way so that the gap between actually transmitted SSB and available RACH occasion remains greater than N symbols. That’s why, RAN1 selected the value of N to be zero symbol for 1.25 kHz and 5 kHz Msg1 although these tone spacings are used in networks with higher cell radius.

On the other hand, if value of N is high, the gap between actually transmitted SSB and actually used RACH occasion is mandated to be high even in a network that supports smaller cell radius.

Observation 1: Networks with relatively higher cell radius can use N = 2 for 120 kHz Msg1 SCS and still avoid inter-cell interference from SSB to RACH by selecting PRACH configuration indices in such a way so that the gap between actually transmitted SSB and configured RACH occasion remains greater than 3 symbols.

Observation 2: If N = 3 is used for 120 kHz Msg1 SCS, the gap between actually transmitted SSB and actually used RACH occasion is mandated to be three symbols even in a network that supports relatively smaller cell radius.

# Conclusion

Observation 1: Networks with relatively higher cell radius can use N = 2 for 120 kHz Msg1 SCS and still avoid inter-cell interference from SSB to RACH by selecting PRACH configuration indices in such a way so that the gap between actually transmitted SSB and configured RACH occasion remains greater than 3 symbols.

Observation 2: If N = 3 is used for 120 kHz Msg1 SCS, the gap between actually transmitted SSB and actually used RACH occasion is mandated to be three symbols even in a network that supports relatively smaller cell radius. This mandated gap of three symbols is unnecessary for networks with smaller cell radius.

Proposal 1: RAN1 confirms the following working assumption.

- For the Semi-static UL/DL configuration in RMSI, only PRACH occasions within the UL part and X part are valid as long as it does not precede or collide with an SSB in the RACH slot, and it is more than at least N symbols after DL part and/or the last symbol of an SSB.

  - N = 0 for Msg1 SCS = 1.25kHz, 5kHz

  - N = 2 for Msg1 SCS = 15kHz

  - N = 2 for Msg1 SCS = 30kHz

  - N = 2 for Msg1 SCS = 60kHz

  - N = 2 or 3 for Msg1 SCS = 120kHz

  - Note for format B4, N = 0 irrespectively of Msg1 SCS

- UE is not expected to receive DL signals during any valid RACH occasion

- The SSB to RO association pattern period shall repeat at most every 160 ms

  - If there are leftover ROs after an integer number of SSB to RO association period(s) within the SSB to RO association pattern period. Those leftover ROs are invalid.

Note: In the UL/DL semi-static configuration, 0 UL slots and 0 UL symbols can be configured

# Reference

- Chairman’s Notes RAN1 92bis final, 3GPP TSG WG RAN1 92bis.