3GPP TSG RAN WG1 #93	R1-1807384

May 21st – May 25th, 2018

Busan, Korea

Agenda item:	7.6.1

Source: 	Qualcomm Incorporated

Title: 	Summary of email discussion on NR-U indoor sub7 GHz simulation calibration

Document for:	Discussion and Decision

# Background

In RAN1 #92bis, the following agreements were reached:

Agreement:

- For sub7 indoor simulation evaluation:

  - Scenario: Option 2 (3+3) with indoor mixed office model

    - Target to reach 10%-15% serving links below -72dBm

    - Further layout parameter fine tuning may be needed. An example procedure for fine tuning is the following sequence.

      - Currently a-b-a=15-20-15

      - If not reaching target, try a-b-a=15-30-15 and a-b-a=20-40-20

      - If not reaching target, apply a scaling factor to the layout with a-b-a=20-40-20

  - Other parameters: Default is NR parameters in 38.901 and 38.802 with the exception of the following

| Parameters | Indoor Sub-7GHz |
| --- | --- |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz baseline , 80MHz optional |
| Number of carriers | 1 |
| Number of users per operator | 5 per gNB per 20MHz |
| SCS | To be reported together simulation results |
| Channel Model | NR InH Mixed Office model |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| UE receiver | MMSE-IRC as the baseline receiver |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Baseline Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ Optional Tx/Rx: (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1. Note: Results based on the mixed traffic models can be used to determine the design. |
| UE/STA to UE/STA link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |
| gNB to gNB link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |

An email discussion was assigned to further fine tune the parameters on the layout of the simulation, targeting May 3rd for a decision. This paper summarizes the email discussion Discussions. A companion spreadsheet contains the raw data for the cdf curves provided by different companies.

# Discussion

The agreed layout is shown in figure below:

Figure 1. Indoor sub7 simulation office layout

The agreement describes the following cases to try:

- (a,b,c,d)=(15,20,20,40)

- (a,b,c,d)=(15,30,20,40)

- (a,b,c,d)=(20,40,20,40)

- (a,b,c,d)=α× (20,40,20,40) with α>1

  - α=1.25  (a,b,c,d)=(25,50,50,25)

The following figures show the cdf of UE received signal power from serving cell. Note -82dBm re-dropping is not enforced in this set of simulation.

Figure 2. UE serving cell received power cdf (15,20,20,40)

Figure 3. UE serving cell received power cdf (15,30,20,40)

Figure 4. UE serving cell received power cdf (20,40,20,40)

Figure 5. UE serving cell received power cdf (25,50,25,50)

The percentile at -72dBm for each layout options are summarized in the next table.

Table 1. Percentile of -72dBm point

|  | (15,20,20,40) | (15,30,20,40) | (20,40,20,40) | (25,50,25,50) |
| --- | --- | --- | --- | --- |
| Qualcomm | 4.2% | 6.7% | 10.9% | 19.3% |
| LG Electronics | 4% | 6.4% | 10.7% | 20.5% |
| Broadcom | 4% | 6.3% | 10.1% | 19.1% |
| Intel | 5.7% | 6.9% | 11.2% | 21.2% |
| Samsung | 5.5% | 8.4% | 12.1% | 20.4% |
| Nokia, Nokia Shanghai Bell | 5.2% | 7.6% | 11.7% | - |
| Mediatek | 4.4% | 6.1% | 11.9% | 21.4% |
| Ericsson | 4% | 6.4% | 10.3% | 19.2% |
| Huawei | 4% | 6.7% | 10.5% | 20% |
| InterDigital | 3.8% | 6.6% | 10.0% | 18.7% |

| Company | Comments |
| --- | --- |
| Qualcomm | From our observation, the layout with parameter (20,40,20,40) can meet the target 10%-15% probability with UE received power lower than -72dBm. This implies a layout with office size 80x120 meters |
| LG Electronics | We agree with Qualcomm’s comments. In our observation, it seems that layout with parameter (20,40,20,40) is enough to satisfy the target 10% -15% serving links below -72dBm. |
| Broadcom | We agree that configuration (20,40,20,40) achieves the agreed target for the percentage of serving links below -72 dBm. We used the following UE dropping method: With a uniform distribution of 15 same-operator UEs in the entire layout, it is not possible to ensure on average that each BS has 5 UEs connected to it while also being their strongest BS among the 3 same operator BSs. This is due to the placement topology of the same-operator BSs. Hence, we redrop UEs until we get 5 UEs connected to each BS, this BS being their strongest BS among the same-operator BSs and with an RSSI >= -82 dBm. Our results include shadowing and redropping of UEs with <-82 dBm RSSI. |
| Intel | Based upon our simulation results, we agree that the configuration (20,40,20,40) satisfies the target 10%-15% probability that the RSRP for the serving links is lower than the ED threshould of -72dBm. As a clarification, all our results include shadowing and, as per the assumptions agreed, we performed redropping of UEs for which the RSRP is below -82dBm. |
| Samsung | Our simulations (-82 dBm UE re-dropping is applied) have indicated that the layout with parameters (a,b,c,d) = (20,40,20,40) can meet the target 10%-15% probability for the UE serving cell received power to be below -72 dBm. In this case, the office size is 120 x 80 meters. |
| Nokia, Nokia Shanghai Bell | We find that with the agreed simulation methodology (including re-dropping of users below -82 dBm), layout (20,40,20,40) meets the target probability of at least 10% of the users below -72 dBm. Given this, we did not run the simulations for the case (25,50,25,50) anymore. |
| Mediatek | The layout with parameter (20,40,20,40) can meet the target 10%-15% UE serving links with received power smaller than -72dBm. All our results include shadowing. However, the re-dropping of UEs with <-82 dBm RSSI is not applied. It seems that the re-dropping does not effect the outcome. |
| Ericsson | Based on our simulation results, we agree that the (20,40,20,40) layout meets the design target of 10 – 15% of serving links with RSSI < -72 dBm. This results in a building size of 80 x 120 m. We do not see a need to scale the layout beyond this size. |
| Huawei | From the simulation result, we found that indoor sub7GHz layout configuration with parameters (20, 40, 20, 40) achieved the target that 10%-15% of serving links have RSRP below -72dBm. It should be clarified that there is no correlation of shadow fading assumed in the simulation. -82dBm UE redropping threshold was applied |
| InterDigital | From our simulation, we observe that (20,40,20,40) meets the target probability. Note that the above results is with re-dropping of RSSI<-82dBm. Without re-dropping, the outcome is (3.8%, 6.8%, 10.6%, 20.8%) and it still leads to the same observation that (20,40,20,40) meets the target probability. |

For the calibration over email, 10 companies provided simulation results and all indicate the layout with parameters (20,40,20,40) satisfies the agreed goal on UE serving link received power distribution. Thus we propose to adopt that layout for the indoor sub7 GHz evaluation.

Proposal: Adopt layout as in Figure 1 with a=20 meters, b=40 meters, c=20 meters, and d=40 meters for indoor sub7GHz NR-U evaluation.

# Conclusion

For the calibration over email, 10 companies provided simulation results and all indicate the layout with parameters (20,40,20,40) satisfies the agreed goal on UE serving link received power distribution. The following agreement was proposed and agreed for the layout for the indoor sub7 GHz evaluation.

Agreement:

- Adopt layout as in Figure 1 with a=20 meters, b=40 meters, c=20 meters, and d=40 meters for indoor sub7GHz NR-U evaluation.