3GPP TSG-RAN WG1 Meeting #93	R1-1807429

Busan, Korea, May 21st – 25th 2018

Source: 	Ericsson

Title:	Feature lead summary NPRACH reliability and range enhancements

Agenda Item:	6.2.7.2.1

Document for:	Information

# Introduction

In this meeting in total 14 documents are submitted regarding NPRACH enhancements, 7 for reliability enhancements and 7 for range enhancements for NPRACH. A list of contributions can be found in the Reference section. This document summarizes the views from different sourcing companies and their proposals regarding NPRACH enhancements for NB-IoT.

# Reliability enhancements for NPRACH

## Summary

A list of contributions as well as their observations and proposals are summarized as follows.

R1-1805860 NPRACH false alarm reduction for NB-IoT Ericsson - Observation 1: The NPRACH Release 15 scrambling design should seek to optimize the number of scrambling patterns and their properties in terms of auto- and cross correlation. - Observation 2: For 3.75 kHz numerology a new frequency hopping pattern in Release 15 will increase the total number of hopping patterns that needs to be hypothesized by an eNB receiver. This may negatively impact the eNB implementation complexity. - Observation 3: Symbol group level scrambling is less effective in mitigating inter-cell interference than symbol level pairwise scrambling. - Observation 4: Using pairwise (symbol level) scrambling with symbol-by-symbol processing of NPRACH preamble does not cause inter-subcarrier interference. It minimizes the impact on base station receiver implementation. - Observation 5: It is possible to leverage a longer scrambling sequence to further improve the false alarm rate. For example, using length-12 scrambling sequences, we may construct a codebook of length 24 codewords for the entire repetition unit (4 symbol groups with 6 symbols per symbol group). Another option is to use a cover code on top of the basic codewords defined for a symbol group (Codeword set X). - Proposal 1: The NPRACH Release 15 design should support the flexibility to use dedicated time-frequency resources as well as to reuse the resources configured for Release 13/14 NPRACH. - Proposal 2: The NPRACH Release 15 scrambling design should be applicable to all Release 13/14/15 NPRACH formats. - Proposal 3: Improved frequency hopping pattern is introduced for 1.25 kHz NPRACH to enhance reliability. - Proposal 4: Symbol level pairwise scrambling is proposed for NPRACH reliability enhancement. R1-1805975 On NPRACH false alarm reduction due to inter-cell interference, Huawei, HiSilicon - Observation 1: Inter-cell interference causes NPRACH false alarm rates of over 1% for 164 dB MCL across a range of SIR values. - Observation 2: Orthogonal symbol-level scrambling codes applied to Rel-13 NPRACH significantly decrease the occurrence of NPRACH false alarm rates of over 1% for 164 dB MCL and 0.5% for 144 dB MCL. - Observation 3: For intra-cell multi user scenarios, the proposed symbol level scrambling can achieve good ToA estimation accuracy. - Observation 4: The alternative solution of pairwise symbol level scrambling has significant NPRACH preamble performance impact on both Rel-15 and legacy UEs due to coverage loss. - Observation 5: Symbol group level scrambling applied to Rel-13 NPRACH does not improve the reliability performance properly. - Proposal 1: For NPRACH reliability enhancement, the preamble is scrambled by a symbol-level scrambling code. - Proposal 2: For NPRACH formats 0/1, the symbol level scrambling code reuses the 5-length code set from LTE in TS 36.211 Table 5.4.2A-1 (i.e. the orthogonal sequences for ). The symbol level scrambling code selection is as follows: nocc=NIDNcell+nscRA0mod5, where NIDNcell is narrowband physical layer cell identity, nscRA(0) is frequency location of the first symbol group of a NPRACH signal. Sequence index nocc; Orthogonal sequence. 0. 1. 2. 3. 4. - Proposal 3: For FDD NPRACH format 2, symbol level scrambling is applied to improve reliability. The symbol level scrambling code selection is as follows: nocc=NIDNcell+nscRA0mod3, where NIDNcell is narrowband physical layer cell identity, nscRA(0) is frequency location of the first symbol group of a NPRACH signal. Sequence index nocc; Orthogonal sequence. 0; [1 1 1]. 1; [1 ej2π3 ej4π3]. 2; [1 ej4π3 ej8π3]. R1-1806197 NPRACH reliability enhancement for NB-IoT ZTE, - Observation 1: Symbol level scrambling will cause a significant increase in computational complexity for NPRACH detection in eNB. - Observation 2: Nearly 50% of number of symbols in symbol group will be treated as CP for symbol level pairwise scrambling. - Observation 3: Rel-15 NPRACH with symbol level scrambling will cause strongly interference to legacy NPRACH. - Observation 4: For a cell with large radius (e.g. 35 km), in the worst situation, the FAP performance of length-2 scrambling code will fall back to the same as legacy NRPACH and has no improvement on FAP. - Observation 5: For a cell with much smaller radius than 35 km, the length-2 scrambling code can effectively reduce FAP. - Proposal 1: Symbol level scrambling code is not suitable for NPRACH reliability enhancement. - Proposal 2: Length-2 symbol group level scrambling code could be used for NPRACH reliability enhancement. R1-1806164 NPRACH reliability enhancement in NB-IoT Nokia, Nokia Shanghai Bell - Proposal 1: Use symbol-group level scrambling to enhance NPRACH reliability. R1-1806692 Reliability enhancements for NPRACH Samsung - Observation 1: For symbol level scrambling, when a length-6 scrambling sequence is used for one symbol group, inter-tone interference will be introduced and have an impact on detection performance. - Observation 2: For symbol level scrambling, when a shorter scrambling sequence is used, and each value in the scrambling sequence is adopted to multiple symbols within the same symbol group (e.g. “AAABBB”-type), the inter-tone issue can be solved at the expense of significantly increased overhead. - Proposal: For enhancement of NPRACH reliability, symbol group level scrambling is preferred due to no additional inter-tone interference. R1-1806593 Remaining issues on reliability enhancements for NPRACH LG Electronics - Proposal 1: Adopt symbol group-level scrambling for NPRACH reliability enhancement. - Proposal 2: The partitioning of legacy NPRACH resource is supported for NPRACH reliability enhancements and the new resource allocation parameter for enhanced preamble should be broadcasted as system information. - Proposal 3: A UE supporting enhanced preamble transmission shall select the legacy NPRACH preamble only if there is no NPRACH resource for the enhanced preamble in the corresponding CE level. R1-1807114 NPRACH Reliability Enhancement Qualcomm Incorporated - Observation 1: With symbol-level scrambling, it’s unclear how to maintain FFT processing without significant loss in signal energy and NPRACH signals at different tones within a cell are no longer orthogonal. - Observation 2: symbol group level scrambling can effectively reduce the false alarm rate for CE levels 0 and 1. - The proposal is: - Proposal 1: Adopt the following enhancement for NPRACH: - A phase rotation of m*p/2 with m=0,1,2,3 is applied to the 4th symbol group of each repetition. - Exact signaling phase rotation is FFS: by SIB or by specification as cell ID dependent.

In RAN1#92b, the following agreements were made that

“

Agreement

- Sharing the same NPRACH resources as Rel-13 NPRACH formats, with symbol or symbol-group level scrambling; maintaining feasibility of FFT processing and orthogonality of preambles on different tones.

- Down-select the following alternatives at RAN1#93:

  - Symbol level scrambling

  - Symbol group scrambling

“.

## Way forwards

From the submitted contributions, two sourcing companies [1][2] support symbol level scrambling, and other sourcing companies support symbol group scrambling. However, there is no convergence of the detailed design.

In total, three sourcing companies provide simulation results in terms of false alarm rate and midsection rate [1][2][4]. Two sourcing companies provide simulation results in terms of ToA estimation performance [2][7]. Furthermore, simulation results proved in [1][2] show that the symbol-level scrambling gives a much superior false alarm reduction than symbol group level scrambling, but simulation results in [4] show otherwise.

## Way forwards

Based on the provided simulation results, symbol level scrambling is preferred, though a majority sourcing companies prefer various schemes based on symbol group scrambling. Based on the understanding of the feature lead, RAN1 has to conclude the study in this meeting. It is proposed by the feature lead that

Step 1

Down-select the following alternatives at RAN1#93:

- Symbol level scrambling (supported by simulation results)

- Symbol group scrambling (supported by majority sourcing companies)

Step 2

If symbol level scrambling is chosen, further down selection among the following

- Length-5 scrambling proposed in [2]

- Pair-wise scrambling proposed in [1]

If symbol group scrambling is chosen, further down selection among following

- Length-2 symbol group level scrambling code [4] [5]

- Length-4 sequences symbol group level scrambling code [6]

- A phase rotation of m*p/2 with m=0,1,2,3 is applied to the 4th symbol group of each repetition [7]

Updated proposal:

- For 3.75 kHz NPRACH formats 0/1, the following scheme is used for NPRACH reliability enhancements:

  - The symbol level scrambling code reuses the 5-length code set from LTE in TS 36.211 Table 5.4.2A-1 (i.e. the orthogonal sequences for ). The symbol level scrambling code selection is as follows:

nocc=NIDNcell+nscRA0mod5, where NIDNcell is narrowband physical layer cell identity, nscRA(0) is frequency location of the first symbol group of a NPRACH signal.

| Sequence index nocc | Orthogonal sequence |
| --- | --- |
| 0 |  |
| 1 |  |
| 2 |  |
| 3 |  |
| 4 |  |

- For 1.25 kHz NPRACH format 2, symbol level pair-wise scrambling is used for NPRACH reliability enhancements:

  - The symbol-level NPRACH scrambling sequence is  based on a length-12 sequence generated by repeating the first scrambling symbol of the length-11 Zadoff-Chu (ZC) sequence at the end.

  - The length-11 Zadoff-Chu (ZC) is defined as ZCRn=e-jRπn(n+1)N, n=0,1,…,N-1,  where N=11. Different cells use different root of the ZC sequence R, and R =  {1, 2, 3, …, 10} are used.

  - The length-12 sequence is repeated pairwise to generate a length-24 sequence scrambling code applied on each NPRACH preamble repetition.

# Range enhancements for NPRACH

A list of contributions as well as their observations and proposals are summarized as follows.

R1-1806594 Resource configuration for NPRACH range enhancement LG Electronics - Proposal 1: An enhanced NPRACH structure that is characterized below is proposed as an enhanced NPRACH structure. - Number of symbol groups per preamble equals 6. - Support Alt.2 frequency hopping (3-level -/+ 1.25 kHz -/+ 3.75 kHz 22.5 kHz). - Frequency hopping pattern - Symmetric 1.25 kHz frequency hopping (-/+ 1.25 kHz) between the 1st and the 2nd hopping - Symmetric 3.75 kHz frequency hopping (-/+ 3.75 kHz) between the 3rd and the 4th hopping - 22.5 kHz frequency hopping in the 5th hopping (22.5 kHz) - Minimum distance between the -/+ 1.25 kHz frequency hoppings (1st and 2nd) - Minimum distance between the -/+ 3.75 kHz frequency hoppings (3rd and 4th) - Proposal 2: The frequency hopping rule that enables full utilization of all the subcarriers in the enhanced NPRACH bandwidth is proposed. - For frequency hopping distance=J, the subcarrier index of the i-th symbol group ki is determined by the following rule: ki= ki-1+J if ki-1mod 2*J < J ki= ki-1-J otherwise , where ki-1 is subcarrier index of the (i-1)-th symbol group - Proposal 3: For resource overlapping between the legacy and the enhanced NPRACH format resources, RAPID is allocated first to the legacy NPRACH format resources as defined in Rel-13/14 spec, and then subsequently allocated to the enhanced NPRACH format resources. - Proposal 4: Support overlapping of the legacy and the enhanced NPRACH format resources in the legacy contention-based and the contention-free NPRACH time/frequency regions. - When the enhanced NPRACH format resources are overlapped with the legacy contention-based NPRACH format resources, the enhanced NPRACH format resources are configured in subcarrier locations with 1.25 kHz offsets from the frequency locations of the legacy NPRACH resources with a 3.75 kHz subcarrier spacing. - When the enhanced NPRACH format resources are overlapped with the legacy contention-free NPRACH format resources, the enhanced NPRACH format resources are configured in 1.25 kHz subcarrier spacing. - Proposal 5: For the enhanced NPRACH format, the collision problem between the last repetition of the enhanced NPRACH format and the following uplink subframe should be resolved. As a simple solution, puncturing of the last part of the enhanced NPRACH format to provide the required guard time is proposed. - Proposal 6: If both the legacy and enhanced NPRACH format resources are configured in same NB-IoT carrier and the same CE level, and if the DCI format N1 for NPDCCH order is detected, the UE capable of enhanced NPRACH selects the same NPRACH format (either the legacy or the enhanced) used in previous RACH procedure. R1-1805974 NPRACH enhancement for cell radius extension Huawei, HiSilicon - Observation 1: for the frequency hopping design within one NPRACH preamble, - After coarse estimation by using small hopping gap value, the residual timing error should be able distinguished by using another large frequency hopping to get much finer ToA estimation. - For legacy NPRACH, the reason no other hopping gap value was introduced between 3.75 kHz and 22.5 kHz is because the residual timing error after using 3.75 kHz hopping is within the capable range of using 22.5 kHz hopping gap. - The residual timing error of using 1.25 kHz hopping gap is within the range [-125 us, 125 us], which requires another hopping gap value of less than 4 kHz to get finer ToA. - Observation 2: Three-level hopping with 6 symbol groups outperforms the other alternatives at same MCL and at same equivalent transmission duration. - Proposal 1: For frequency hopping within one NPRACH preamble, Alt 2: 3-level -/+ 1.25 kHz -/+ 3.75 kHz 22.5 kHz is adopted. - Proposal 2: The new NPRACH format has: - 6 symbol groups per preamble. - Symbol-group level frequency hopping within one preamble - 1.25 kHz hopping gap for 1st to 2nd symbol group and with opposite direction for 5th to 6th symbol group. - 3.75 kHz hopping gap for 2nd to 3rd symbol group and with opposite direction for 4th to 5th symbol group. - 22. 5 kHz hopping gap for 3rd to 4th symbol group. - Proposal 3: Symbol level scrambling is applied on top of FDD NPRACH format 2to further improve reliability. The symbol-level scrambling code set is: - Proposal 4: For NPRACH format 2, the following RRC parameters are signaled: - The following NPRACH resource configuration parameters and value ranges can be reused from NPRACH format 0/1: nprach-StartTime, numRepetitionsPerPreambleAttempt, nprach-Periodicity, nprach-SubcarrierMSG3-RangeStart, npdcch-NumRepetitions-RA, npdcch-StartSF-CSS-RA, npdcch-Offset-RA, npdcch-CarrierIndex . - The following frequency domain parameters have new value ranges for NPRACH format 2: - nprach-CP-Length: 800 us - nprach-SubcarrierOffset: {0, 36, 72, 108, 6, 54, 102, spare} subcarriers - nprach-NumCBRA-StartSubcarriers: {24, 30, 33, 36, 60, 66, 69, 72, 96, 102, 105, 108, 120, 132, 138, 144} subcarriers - nprach-NumSubcarriers: {36, 72, 108, 144} subcarriers - Proposal 5: For Rel-13/14 UEs, the network is assumed to avoid collisions between NPRACH format 2 and NPUSCH. - Proposal 6: The range of timing advance in RAR is extended to be from 0 to 1536. R1-1805861 NPRACH range enhancements for NB-IoT Ericsson - Observation 1 With 1.25 kHz numerology, eNB complexity is increased since the FFT operation needs to be carried out on a 3x finer grid. - Observation 2 Pairwise symbol-level scrambling maintains the feasibility of FFT processing and orthogonality of preambles on different tones. - Proposal 1 With 1.25 kHz subcarrier spacing, the frequency hopping pattern consists of both inner layer fixed size hopping and outer layer pseudo-random hopping. - Proposal 2 With 1.25 kHz subcarrier spacing, the range of the outer layer pseudo-random hopping can scale with the number of configured NPRACH subcarriers. - Proposal 3 With 1.25 kHz subcarrier spacing, a basic NPRACH repetition unit consists of 5 symbol groups. - Proposal 4 With 1.25 kHz subcarrier spacing, adopt the hopping pattern in Figure 1 for the hopping of symbol groups within a NPRACH repetition unit. - Proposal 5 The parameters and their values for the type2-CSS configurations of RAR corresponding to the Rel-15 NPRACH resources remain the same as in legacy NPRACH. - Proposal 6 Reuse the Rel-13 values for nprach-StartTime, numRepetitionsPerPreambleAttemp, and nprach-Periodicity for Rel-15 NPRACH format 2 resources configurations. - Proposal 7 Set nprach-SubcarrierOffset to {0, 36, 72, 108, 6, 54, 102, 12, 24, 48, 84, 120, 18, 66, 144, spare1} subcarriers. - Proposal 8 When a minimum 36 subcarrier is used, with 1.25 kHz subcarrier spacing, finer FDM is supported. - Proposal 9 If Option A or Option B is adopted, indicate the which subset of preamble(s) of the configured NPRACH format 2 resources are used. The indication can be either signaled explicitly or derived from cell ID. - Proposal 10 Set nprach-SubcarrierOffset to {2, 4, 6, 8, 10, 11, 12, 14, 16, 20, 22, 23, 24, 28, 30 32, 34, 35, 36, 38, 40, 42, 44, 46, 48, 50, 52, 55, 56, 58, 60, 62, 64, 66, 69, 70, 72, 76, 78, 80, 82, 88, 89, 90, 92, 96, 98, 108, 110, 111, 112, 114, 116, 120, 122, 123, 124, 132, 134, 135, 136, 140, 142, 144} subcarriers. - Proposal 11 With 1.25 kHz subcarrier spacing, pairwise symbol-level scrambling is proposed for NPRACH reliability enhancement. R1-1806198 Remaining issues on NPRACH range enhancement ZTE, - Observation 1: Alt 1 with 5 symbol groups cannot meet the TA detection requirement in TS36.104 (-3.645 us，+3.645 us). - Observation 2: Enhanced scheme of Alt 1 with 7 symbol groups can meet the TA detection requirement in TS36.104 (-3.645 us，+3.645 us). - Observation 3: TA detection performance of Alt 2 is seriously affected by Residual CFO. - Observation 4: Due to only +22.5 kHz frequency hopping is supported for Alt 2, NPRACH resource efficiency of Alt 2 is significantly reduced comparing to Alt 1. - Proposal 1: The two symbol groups of one hopping level should be consecutive in time domain. - Proposal 2: Two-level hopping is used for frequency hopping within one NPRACH preamble which consists of 7 symbol groups. - +1.25 kHz hopping distance is used for the hopping of symbol group 0, 1 and 2; - -1.25 kHz hopping distance is used for the hopping of symbol group 2, 3 and 4; - +22.5 kHz hopping distance is used for the hopping between symbol group 4 and 5; - -22.5 kHz hopping distance is used for the hopping between symbol group 5 and 6. R1-1806165 NPRACH cell range enhancement in NB-IoT Nokia, Nokia Shanghai Bell Proposal 1: For NPRACH design, the followings are proposed – 3-level frequency hopping: -/+ 1.25 kHz -/+ 3.75 kHz -/+22.5 kHz 6 symbol groups per repetition R1-1806693 Range enhancements for NPRACH Samsung - Proposal #1: 2-level hopping with hopping distance of -/+ 1.25 kHz and -/+ 22.5 kHz is supported. - Proposal #2: Reuse legacy hopping pattern with 4 symbol groups in a NPRACH preamble, with the minimum hopping distance replaced by 1.25 KHz. - Proposal #3: the following RRC parameters are defined for Rel-15 NPRACH range enhancement. How to capture the parameters is up to editor. - numRepetitionsPerPreambleAttempt: {n1, n2, n4, n8, n16, n32, n64, n128} - nprach-Periodicity: {ms80, ms160, ms240, ms320, ms640, ms1280, ms2560, ms5120} - nprach-StartTime: {ms16, ms32, ms64, ms128, ms256, ms512, ms1024, ms2048} - nprach-SubcarrierOffset: {n0, n36, n72, n108, n6, n54, n102, spare1} - nprach-NumSubcarriers: {n36, n72, n108, n144} - nprach-NumCBRA-StartSubcarriers: {n24, n30, n33, n36, n60, n66, n69, n72, n96, n102, n105, n108, n120, n132, n138, n144} - nprach-SubcarrierMSG3-RangeStart: invalid - Proposal #4: Do not support NPRACH resource partition to indicate the support of multi-tone, i.e., Multi-tone transmission is mandatory for Rel-15 NB-IoT UEs capable of NPRACH range enhancement. R1-1807113 NPRACH support for large cell access Qualcomm Incorporated Proposal 1: For NPRACH for large cells, adopt the NPRACH numerology as given in Table 1. CP duration; Symbol duration; No. of symbols/group; No. of symbol groups/repetition; Hop between groups 1 and 2; Hop between groups 2 and 3; Hop between groups 3 and 4. 800 us; 800us; 3; 4; 1.25 kHz; 3*1.25 kHz; 18*1.25 kHz.

In RAN1#92bis, the following agreements were made for NPRACH range enhancement.

“

Agreement:

- Configurable NPRACH bandwidth is integer multiple of 45 kHz.

- For Rel-15 UEs, in case NPUSCH transmission collides with any configured NPRACH resources in the cell, the NPUSCH transmission is postponed.

- For one NPRACH symbol group for subcarrier spacing of 1.25 kHz, one NPRACH symbol group consists of CP+3 Symbols

- FFS the number of symbol groups per NPRACH preamble repetition.

- For frequency hopping within one NPRACH preamble, down-select the following alternatives at RAN1#93 meeting

- Alt 1: 2-level  -/+ 1.25 kHz -/+ 22.5 kHz

- Alt. 2: 3-level -/+ 1.25 kHz -/+ 3.75 kHz 22.5 kHz

- Specification supports that the new NPRACH format resouces can be configured to overlap or not overlap with the NPRACH resources of Rel-13/Rel-14.

“.

Current agreement

Agreement

Number of symbol groups per NPRACH preamble repetition is 6 symbol groups per NPRACH preamble repetition.

Agreement

Frequency hopping within one NPRACH preamble is 3-level -/+ 1.25 kHz -/+ 3.75 kHz 22.5 kHz

Agreement

The parameters and their values for the type2-CSS configurations of RAR corresponding to the Rel-15 NPRACH resources remain the same as in legacy NPRACH.

Agreement

- Reuse the Rel-13 values for nprach-StartTime, numRepetitionsPerPreambleAttemp, npdcch-CarrierIndex, for Rel-15 NPRACH format 2 resources configurations

- Values for NPRACH periodicity are 40, 80, 160, 320, 640, 1280, 2560, 5120 ms

Agreement

- The new NPRACH format has:

  - 6 symbol groups per preamble.

  - Symbol-group level frequency hopping within one preamble

  - 1.25 kHz hopping gap for 1st to 2nd symbol group and with opposite direction for 5th to 6th symbol group.

  - 3.75 kHz hopping gap for 2nd to 3rd symbol group and with opposite direction for 4th to 5th symbol group.

  - 22. 5 kHz hopping gap for 3rd to 4th symbol group.

Agreement

- nprach-NumSubcarriers: {n36, n72, n108, n144}

- nprach-SubcarrierMSG3-RangeStart: same as legacy

Updated proposal

- nprach-SubcarrierOffset: {0, 36, 72, 108, 6, 54, 102, 42, 78, 90, 12, 24, 48, 84, 120, 18} subcarriers.

- nprach-NumCBRA-StartSubcarriers: { 3, 6, 9,12, 15, 18, 21, 24, 27, 30, 33, 36, 39, 42, 45, 48, 51, 54    57, 60,  63, 66, 69, 72, 75, 78, 81, 84, 87, 90, 93, 96, 99, 102, 105, 108, 111, 114, 117, 120, 123, 126,  129, 132, 135, 138, 141,144, spares} subcarriers.

## Way forwards

For the number of symbol groups per NPRACH preamble repetition, we have three sourcing companies proposeing 6 (total length 19.2 ms) [9][10][12], two sourcing companies proposing 4 (total length 12.8 ms) [13][14], one sourcing company proposing 5 (total length 16 ms) [8], and one sourcing company proposing 7 (total length 22.4 ms) [11] symbol groups per preamble repetition unit.

Based on the understanding of the feature lead and the views of majority  of supporting sourcing companies, it is proposed that

- Number of symbol groups per NPRACH preamble repetition is down selected among

  - 4 symbol groups per NPRACH preamble repetition

  - 6 symbol groups per NPRACH preamble repetition.

In total, four out of seven sourcing companies support alternative 2 [9][10][12][14], i.e., 3-level -/+ 1.25 kHz -/+ 3.75 kHz 22.5 kHz. However, the detailed hopping pattern differs from company to company. From the discussions, it is observed that reversed hopping direction can help improve the performance, as it cancels out the frequency errors within a repetition unit. Therefore, it is proposed that

- Frequency hopping within one NPRACH preamble is

Alt. 2: 3-level -/+ 1.25 kHz -/+ 3.75 kHz 22.5 kHz

- Further discuss the detailed hopping pattern based on alternative 2, and the number of symbol groups per NPRACH preamble repetition.

For NPRACH resource configurations, three sourcing companies expressed opinions about the parameters for NPRACH setting [8][9][13], i.e., numRepetitionsPerPreambleAttempt, nprach-Periodicity, nprach-StartTime, nprach-NumSubcarriers, nprach-NumCBRA-StartSubcarriers, nprach-SubcarrierMSG3-RangeStart, etc.

Based on the understanding of the feature lead, the following proposal can be easily agreed.

- The parameters and their values for the type2-CSS configurations of RAR corresponding to the Rel-15 NPRACH resources remain the same as in legacy NPRACH.

- Reuse the Rel-13 values for nprach-StartTime, numRepetitionsPerPreambleAttemp, npdcch-CarrierIndex, and nprach-Periodicity for Rel-15 NPRACH format 2 resources configurations.

- RAN1 decides whether signal tone UE indication is supported by NPRACH format 2.

- RAN1 further discuss the values of the following parameters:

  - nprach-SubcarrierOffset

  - nprach-NumCBRA-StartSubcarriers

For NPRACH format 2 reliability enhancement, two sourcing companies have provided inputs [8][9]. Based on the inputs, it is proposed that

- RAN1 further discuss whether NPRACH format 2 reliability enhancement should be considered. If not, identical symbols should be used across all the repetitions.

- If the NRAHC format 2 reliability enhancement is needed, choose from the following alternatives:

  - Length-3 scrambling proposed in [9]

  - Pair-wise scrambling proposed in [8][1]

## Other issues

Proposed to treat these issues during the CR phases:

- Which NPRACH format to use for PDCCH ordered PRACH.

- Further FDM of the NPRACH format 2 in the case when 36 preambles are configured.

- Extend the range of timing advance in RAR.

- Insufficient RAPID

  - Assume 6-bit RAPID for Rel.15? Or consider extending to 8 bits to cover all the 144 subcarriers within 180 kHz bandwidth? (may be not relevant for RAN1 discussion)

  - RAPID allocation

    - How to allocate RAPID between the legacy and enhanced NPRACH format resources when NPRACH format resources are overlapped

    - We have a proposal on this topic:

- For resource overlapping between the legacy and the enhanced NPRACH format resources, RAPID is allocated first to the legacy NPRACH format resources as defined in Rel-13/14 spec, and then subsequently allocated to the enhanced NPRACH format resources

- Guard time

How to provide the required guard time of ~800 us needs to be discussed [12]

For the enhanced NPRACH format, the collision problem between the last repetition of the enhanced NPRACH format and the following uplink subframe should be resolved. As a simple solution, puncturing of the last part of the enhanced NPRACH format to provide the required guard time is proposed

- UL transmission gap for resynchronization

  - Same UL transmission gap when the enhanced NPRACH format resources are overlapped with the legacy NPRACH format resources?

    - Should consider impact on the legacy UEs

  - Different UL transmission gap for non-overlapping enhanced NPRACH format resources?

    - After X repetitions of the enhanced NPRACH, transmission gap Y is introduced

    - Should consider the following conditions:

      - X*Tp,e <= 64*Tp,l (legacy NPRACH transmission gap)

          - Tp,e : time duration (in ms) of one repetition of the enhanced preamble

          - Tp,l : time duration (in ms) of one repetition of the legacy preamble

# References

- R1-1807479	NPRACH false alarm reduction for NB-IoT, source Ericsson (revision of R1-1805860)

- R1-1805975	On NPRACH false alarm reduction due to inter-cell interference, source Huawei, HiSilicon

- R1-1806164	NPRACH reliability enhancement in NB-IoT, source Nokia, Nokia Shanghai Bell

- R1-1806197	NPRACH reliability enhancement for NB-IoT, source ZTE

- R1-1806593	Remaining issues on reliability enhancements for NPRACH, source LG Electronics

- R1-1806692	Reliability enhancements for NPRACH, source Samsung

- R1-1807114	NPRACH Reliability Enhancement, source Qualcomm Incorporated

- R1-1805861	NPRACH range enhancements for NB-IoT, source Ericsson

- R1-1805974	NPRACH enhancement for cell radius extension, source Huawei, HiSilicon

- R1-1806165	NPRACH cell range enhancement in NB-IoT, source Nokia, Nokia Shanghai Bell

- R1-1806198	Remaining issues on NPRACH range enhancement, source ZTE

- R1-1806594	Remaining issues on NPRACH range enhancement, source LG Electronics

- R1-1806693	Range enhancements for NPRACH, source Samsung

- R1-1807113	NPRACH support for large cell access, source Qualcomm Incorporated