3GPP TSG RAN WG1 Meeting #93	R1-1807618

Busan, South Korea, May 21 – 25, 2018

Agenda item:		7.1.7

Source:	Nokia, Nokia Shanghai Bell

Title:	On selected MIMO UE features

Document for:		Discussion and Decision

# Introduction

In this document shares Nokia views on a number of MIMO category UE features on the post-RAN1#92bis UE feature list [1]. This document is an update to R1-1807615 fixing an error in the feature 2-20.

2-20 Beam correspondence – mandatory/optional

2-21 Periodic beam report – mandatory/optional

2-22 Aeriodic beam report – mandatory/optional

2-24 SSB/CSI-RS for beam measurement – mandatory/optional

2-37 Support Semi-open loop CSI – mandatory/optional

2-38 CSI report without PMI – mandatory/optional

2-39 CSI report with CRI – mandatory/optional

# Beam reporting and CSI reporting

| # | FG | Components | Consequence if not supported by UE | RAN WG recommendation | Nokia comment |
| --- | --- | --- | --- | --- | --- |
| 2-20 | Beam correspondence | 1. Support Beam correspondence | Beam correspondence is not supported | [Mandatory at least for FR2] | [Mandatory for FR2 without capability signalling] |
| 2-21 | Periodic beam report | 1. Support report on PUCCH formats over 1 – 2 OFDM symbols once per slot 2. Support report on PUCCH formats over 4 – 14 OFDM symbols once per slot | No support of periodic L1-RSRP report | Note: Beam correspondence means each Tx port can be beamformed in a desirable direction but does not imply setting phase across ports Mandatory with UE capability at least for FR2 FFS: for FR1 | Mandatory with UE capability for FR1&FR2 |
| 2-22 | Aperiodic beam report | 1. Support report on PUSCH | No support of aperiodic L1-RSRP report | Mandatory with UE capability at least for FR2 | Mandatory with UE capability for FR1&FR2 |
| 2-24 | SSB/CSI-RS for beam measurement | 1. The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP within a slot shall not exceed MB_1 2. The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP within a slot shall not exceed MB_2 3. Supported density of CSI-RS | RSRP measurement is not supported | Component-1, candidate value set for MB_1 is {8, 16, 32, 64} Support MB_1 =8 is mandatory for at least for >6Ghz bands Component-2, candidate value set for MB_2 is {0, 4, 8, 16, 32, 64} Component-3: candidate value set: {“1 only”, “3 only”, “both 1 and 3”} At least density of CSI-RS =3 is mandatory at least for FR2 | Support MB_1 =8 is mandatory for FR1&FR2 Support MB_2 =8 is mandatory for FR1&FR2 At least density of CSI-RS =3 is mandatory for FR1&FR2 |
| 2-37 | Support Semi-open loop CSI | 1. Support Semi-open loop CSI report | Semi-Open loop CSI report is not supported | FFS | Mandatory without UE capability |
| 2-38 | CSI report without PMI | Support CSI report without PMI | CSI report without PMI is not supported | FFS | Mandatory without UE capability |
| 2-39 | CSI report with CRI | Support CSI report with CRI | CSI report with CRI is not supported | FFS | Mandatory without UE capability |

# References

- R1-1805728, NR UE feature list, NTT DOCOMO, INC., AT&T