3GPP TSG RAN WG1 Meeting #93   			           	            R1-1807688

- Busan, Korea, May 21st – 25th, 2018

______________________________________________________________________Agenda item: 7.5.2

Source: LG Electronics

Title: Summary of discussion for NR V2X channel model

Document for: Discussion and decision

- Introduction

This contribution summarizes discussion aspects of NR V2X channel model based on submitted contributions for AI 7.5.2.

- Remaining issues for remaining issues for antenna model

  - gNB

    - Antenna placement

Intel (R1-1806539)

Reuse gNB cell layout from TR 36.885 including antenna boresight angle definition

Apply gNB antenna tilts of 102 and 96 degree for Urban and Freeway scenarios accordingly

Proposal: The above proposal is agreed.

    - Antenna array configuration

Intel (R1-1806539)

|  | gNB (Macro gNB) |  |
| --- | --- | --- |
|  | Low band (2-4 GHz) | High band (30 GHz) |
| TXRU mapping | Up to proponents | Up to proponents |
| Number of antenna elements across all panels | Up to 256 Tx /Rx antenna elements | Up to 256 Tx /Rx antenna elements |
| Antenna array configuration (M, N, P, Mg, Ng) | (8, 8, 2, 1, 1) (TR 38.802, Baseline gNB antenna configuration for UMa scenario) | (4, 8, 2, 2, 2) (TR 38.802, Baseline gNB antenna configuration for UMa scenario) |
| Antenna array spacing (dH,dV,dH,g,dV,g) | (dH, dV) = (0.5, 0.8)λ | (dH, dV) = (0.5, 0.5)λ (dH,g, dV,g) = (4.0, 2.0)λ |
| Others | TXRUs within a panel can be assumed to be synchronized and phase-calibrated (at least to the same level as in LTE) |  |

Ericsson (R1-1806152)

|  | Below 6 GHz | Above 6 GHz |
| --- | --- | --- |
| TXRU mapping | Per panel, per polarization | Per panel, per polarization. Consider the following a TXRU to antenna elements mapping as examples 30GHz and 70GHz: Option 1: a single TXRU is mapped per panel per polarization. Option 2: a single TXRU is mapped per panel per subarray per polarization, - E.g., where a subarray consists of consecutive M/2 vertical antennas and N/2 horizontal antennas with the same polarization. - Other subarray configurations are not precluded. Option 3: Fully connected TXRU mapping within a panel per polarization. - Other Fully connected TXRU mapping is not precluded. For evaluating multi beam based approaches at 30GHz and 70GHz, consider the following: - TXRU to antenna mapping weights are adjustable and used to steer the panel beam direction in multi beam based approaches in time domain. |
| Number of antenna elements across all panels | Up to 256 Tx/Rx antenna elements | 30GHz: Up to 256 Tx /Rx antenna elements 70GHz: Up to 1024 Tx /Rx antenna elements |
| Antenna array configuration (M, N, P, Mg, Ng) | Dense urban and Urban macro: - Baseline: (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1). | 30GHz: Dense urban and Urban macro: - Baseline: (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2). 70GHz: Dense urban: - Baseline: (M, N, P, Mg, Ng) = (8, 16, 2, 2, 2) |
| Antenna array spacing (dH,dV,dH,g,dV,g) | Dense urban and Urban macro: - Baseline: (dH, dV) = (0.5, 0.8)λ | 30GHz: Dense urban and Urban macro: - Baseline: (dH, dV) = (0.5, 0.5)λ. (dg,H,dg,V) = (4.0, 2.0)λ 70GHz: Dense urban: - Baseline: (dH, dV) = (0.5, 0.5)λ. (dg,H, dg,V) = (8.0, 4.0) λ. |
| Antenna tilt, deg | No mechanical tilt | No mechanical tilt |

Samsung (R1-1806756)

Antenna array configuration for Macro BS

|  | For 4 GHz | For 30 GHz |
| --- | --- | --- |
| TXRU mapping | Up to proponents decision | Up to proponents decision |
| Number of antenna elements across all panels | Up to 256 TX/RX antenna elements | Up to 256 TX/RX antenna elements (the same as TR38.802) |
| Antenna array configuration (M, N, P, Mg, Ng) | (8, 8, 2, 1, 1) (reuse value for High Speed Train or Dense urban and Urban macro in TR38.802) | (4,8,2,2,2) (reuse value for Dense urban and Urban macro in TR38.802) |
| Antenna array spacing (dH,dV,dH,g,dV,g) | (dH, dV) = (0.5, 0.8)λ (reuse value for Dense urban and Urban macro in TR38.802) | (dH, dV) = (0.5, 0.5)λ. (dg,H,dg,V) = (4.0, 2.0)λ (reuse value for Dense urban and Urban macro in TR38.802) |

Proposal: Proposal from R1-1806539 is agreed.

  - gNB-type RSU

    - Antenna placement

Intel (R1-1806539)

gNB-type RSUs which are deployed along the roads in Highway and Urban grid scenarios utilize directional antenna patterns.

gNB-Type RSU deployed at Urban grid intersections utilize omni-directional antenna element pattern.

    - Antenna element pattern

Intel (R1-1806539)

gNB-type RSUs which are deployed along the roads in Highway and Urban grid scenarios utilize directional antenna patterns.

| Parameter | For both below and above 6GHz |
| --- | --- |
| Antenna element gain vertical pattern |  |
| Antenna element gain horizontal pattern |  |
| Pattern combining method for 3D |  |
| Max direct. gain of the antenna element | 8 dBi |

gNB-Type RSU deployed at Urban grid intersections utilize omni-directional antenna element pattern.

| Parameter | For both below and above 6GHz |
| --- | --- |
| Antenna element gain vertical pattern |  |
| Antenna element gain horizontal pattern |  |
| Pattern combining method for 3D |  |
| Max direct. gain of the antenna element | 8 dBi |

    - Antenna array configuration

Intel (R1-1806539)

|  | gNB-Type RSU (Micro gNB) |  |
| --- | --- | --- |
|  | Low band (2-4 GHz) | High band (30 GHz) |
| TXRU mapping | Up to proponents | Up to proponents |
| Number of antenna elements across all panels | Up to 8 Tx /Rx antenna elements | Up to 32 Tx /Rx antenna elements |
| Antenna array configuration (M, N, P, Mg, Ng) | Along the roads: (1, 2, 2, 1, 2) Panel bearing angle: Ω0,1=Ω0,0+180 Directional antenna elements Intersection: (1, 2, 2, 2, 1) Panel bearing angle: Ω0,1=Ω0,0+90 Omni antenna elements | Along the roads: (2, 4, 2, 1, 2) Panel bearing angle: Ω0,1=Ω0,0+180 Directional antenna elements Intersection: (1, 4, 2, 4, 1) Panel bearing angle: Ω1,0=Ω0,0+90°; Ω2,0=Ω0,0+180°; Ω3,0=Ω0,0+270°; Directional antenna elements |
| Antenna array spacing (dH,dV,dH,g,dV,g) | (dH, dV) = (0.5, 0.8)λ (TR 38.802, Section A.2.1, Dense Urban Micro cell TRP configuration) | (dH, dV) = (0.5, 0.5)λ |
| Others | TXRUs within a panel can be assumed to be synchronized and phase-calibrated (at least to the same level as in LTE) |  |

Proposal: Discuss this after the discussion on the micro BS deployment.

  - UE-type RSU

    - Antenna placement

Intel (R1-1806539)

Same as gNB-type RSU deployment

Proposal: Discuss this after the discussion on the UE-type RSU deployment.

    - Antenna element pattern

Intel (R1-1806539)

|  | Low band (6 GHz) |
| --- | --- |
| Antenna element gain vertical pattern |  |
| Antenna element gain horizontal pattern |  |
| Pattern combining method for 3D |  |
| Max direct. gain of the antenna element | 5 dBi |

|  | High band (63 GHz) |
| --- | --- |
| Antenna element gain vertical pattern |  |
| Antenna element gain horizontal pattern |  |
| Pattern combining method for 3D |  |
| Max direct. gain of the antenna element | 5 dBi |

Ericsson (R1-1806152)

UE-type RSU antenna element radiation pattern model for below 6GHz

| Parameter | Values |
| --- | --- |
| Antenna element gain vertical pattern | Half spherically uniform distribution with bottom direction |
| Antenna element gain horizontal pattern |  |
| Pattern combining method for 3D |  |
| Max direct. gain of the antenna element | 3 dBi |

UE-type RSU antenna element radiation pattern model for above 6GHz

| Parameter | Values |
| --- | --- |
| Antenna element gain vertical pattern |  |
| Antenna element gain horizontal pattern |  |
| Pattern combining method for 3D |  |
| Max direct. gain of the antenna element | 5dBi |

Samsung (R1-1806756)

Antenna element pattern for UE-Type RSU

|  | For 6 GHz | For 63 GHz |
| --- | --- | --- |
| Antenna element gain vertical pattern | Omni-directional |  |
| Antenna element gain horizontal pattern |  |  |
| Pattern combining method for 3D |  |  |
| Max direct. gain of the antenna element | 0 dBi | 5 dBi |

Proposal: Select one out of three proposals.

    - Antenna array configuration

Intel (R1-1806539)

|  | Low band (2-6 GHz) | High band (30-63 GHz) |
| --- | --- | --- |
| TXRU mapping | Up to proponents decision | Up to proponents decision |
| Number of antenna elements across all panels | Up to 8 Tx /Rx antenna elements | Up to 32 Tx /Rx antenna elements |
| Antenna array configuration (M, N, P, Mg, Ng) | Along the roads: (1, 4, 2, 1, 1) Omni antenna elements Intersection: (1, 2, 2, 2, 1) Panel bearing angle: Ω1,0=Ω0,0+90°; Omni antenna elements | Along the roads: (2, 4, 2, 1, 2) Panel bearing angle: Ω0,1=Ω0,0+180 Directional antenna elements Intersection: (1, 4, 2, 4, 1) Panel bearing angle: Ω1,0=Ω0,0+90°; Ω2,0=Ω0,0+180°;Ω3,0=Ω0,0+270°; Directional antenna elements |
| Antenna array spacing (dH,dV,dH,g,dV,g) | (dH, dV) = (0.5, 0.8)λ TR 38.802, Section A.2.1, Dense Urban micro cell TRP configuration | (dH, dV) = (0.5, 0.5)λ |
| Antenna tilt, deg | 96 | 96 |

Ericsson (R1-1806152)

Antenna array configuration for UE-type RSU

|  | UE-type RSU |  |
| --- | --- | --- |
|  | Below 6 GHz | Above 6 GHz |
| TXRU mapping | A single TXRU is mapped per panel per polarization | A single TXRU is mapped per panel per polarization |
| Number of antenna elements across all panels | Up to 8 TX/RX antenna elements | Up to 32 Tx /Rx antenna elements |
| Antenna array configuration (M, N, P, Mg, Ng) | Collocated: (1, 4, 2, 1, 1) | Collocated: (2, 4, 2, 1,2) |
| Antenna array spacing (dH,dV,dH,g,dV,g) | (0.5, 0.5, -, -) λ | (0.5, 0.5, 0, 0) λ |

Samsung (R1-1806756)

Antenna array configuration for UE-Type RSU

|  | For 6 GHz | For 63 GHz |
| --- | --- | --- |
| TXRU mapping | Up to proponents decision | Up to proponents decision |
| Number of antenna elements across all panels | Up to 8 Tx /Rx antenna elements | Up to 32 Tx /Rx antenna elements |
| Antenna array configuration (M, N, P, Mg, Ng) | (1, 2, 2, 1, 1) TR 38.802, Table A.1.6-1 | (2, 4, 2, 1, 2) Panel bearing angle: Ω0,1=Ω0,0+180° TR 38.802, Table A.1.6-2 |
| Antenna array spacing (dH,dV,dH,g,dV,g) | (dH, dV) = (0.5, 0.5)λ TR 38.802, Table A.1.6-1 | (dH, dV) = (0.5, 0.5)λ; (dH,g, dV,g) = (0, 0)λ TR 38.802, Table A.1.6-2 |
| Antenna tilt, deg | 0 | 90 |

Proposal: Discuss this after the discussion on the UE-type RSU deployment.

  - Vehicle UE

    - Antenna placement

It was agreed “For both below and above 6 GHz, an option for “collocated antenna case” is supported. Note that this can be revised based on input from other organizations.” In addition, some companies discussed details of the antenna placement for vehicle UEs.

- For vehicle type 1,

  - One panel at the front bumper and one panel at the rear bumper

    - Supported by Intel, LGE, GM

- For vehicle type 2,

  - Collocated antenna at the center of the rooftop

    - Supported by Intel

  - One panel at the front edge and one panel at the rear edge of the rooftop

    - Supported by LGE, GM

- For vehicle type 3,

  - One panel at the front edge and one panel at the rear edge of the rooftop

    - Supported by Intel, LGE, GM

- Collocated antenna for calibration and distributed antenna for technical solution evaluation

  - Supported by LGE, GM

Proposal: Define both collocated antenna case and distributed antenna case for vehicle UEs. Discuss the usage of each case.

    - Antenna element pattern

Intel (R1-1806539)

For VT1 and VT3 use 20dB self-attenuation value.

For coupling between rear and front antennas, use distance dependent pathloss (e.g. free space) and 20dB self–attenuation.

Consider two types of antenna element radiation patterns:

- Type A – Reference radiation patterns

- Type B – Coupled radiation patterns

Further study how to model coupled antenna radiation models including the following aspects:

- Amplitude/phase characteristics in azimuth and zenith angles

- Parameters and procedure for antenna array modeling

|  | Low band (6 GHz) |
| --- | --- |
| Antenna element gain vertical pattern |  |
| Antenna element gain horizontal pattern | Vehicle Type 2: Vehicle Type 1 and Type 3: |
| Pattern combining method for 3D |  |
| Max direct. gain of the antenna element | 5 dBi |

|  | High band (63 GHz) |
| --- | --- |
| Antenna element gain vertical pattern |  |
| Antenna element gain horizontal pattern |  |
| Pattern combining method for 3D |  |
| Max direct. gain of the antenna element | 5 dBi |

Ericsson (R1-1806152)

Vehicle UE antenna element radiation pattern model for below 6GHz

| Parameter | Values |
| --- | --- |
| Antenna element gain vertical pattern | Half spherically uniform distribution with top direction |
| Antenna element gain horizontal pattern |  |
| Pattern combining method for 3D |  |
| Max direct. gain of the antenna element | 3 dBi |

Vehicle UE antenna element radiation pattern model for above 6GHz

| Parameter | Values |
| --- | --- |
| Antenna element gain vertical pattern |  |
| Antenna element gain horizontal pattern |  |
| Pattern combining method for 3D |  |
| Max direct. gain of the antenna element | 5dBi |

Samsung (R1-1806756)

Antenna element pattern for vehicle UE

|  | Vehicle UE |  |
| --- | --- | --- |
|  | For 6 GHz | For 63 GHz |
| Antenna element gain vertical pattern | Omni-directional |  |
| Antenna element gain horizontal pattern |  |  |
| Pattern combining method for 3D |  |  |
| Max direct. gain of the antenna element | 0 dBi | 5 dBi |

GM (R1-1807613)

Front bumper for below 6GHz

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AvdB"θ,ϕ=0°=k=14aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 27.855; 0.9281; -1.3062. 2; 31.967; 2.7336; 1.4674. 3; 23.665; 3.0514; -1.8167. 4; 1.8441; 7.7354; -0.6126. AvdB"θ,ϕ=π=k=14aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 146.47; 1.095; -1.6043. 2; 131.87; 1.192; 1.4566. 3; 5.2383; 3.9996; -0.8389. 4; 1.9429; 9.0248; 1.6705. |
| Horizontal cut of the radiation power pattern (dB) | AhdB"θ=π2,ϕ=a0+k=18akcosϕk+⁡bksin(ϕk), ϕ∈[0,2π] K; ak; bk. 0; -20.793; -. 1; -3.4026; 0.0721. 2; 1.3366; 0.1213. 3; 2.1764; -0.0595. 4; 0.5881; 0.0102. 5; -2.4068; 0.0675. 6; 1.7561; -0.1513. 7; 0.7899; -0.0133. 8; -1.0144; 0.0177. |
| 3D radiation power pattern (dB) | Gθ1=GVθ=0°,ϕ=0°,Gθ2=GHθ=π2,ϕ,Gϕ1=GVθ,ϕ=0,Gϕ2=GVθ,ϕ=π AdB"θ,ϕ=ϕ1Gϕ2+ϕ2Gϕ1θ1θ2(θ1+θ2)2+θ1Gθ2+θ2Gθ1ϕ1ϕ2(ϕ1+ϕ2)2ϕ1+ϕ2θ1θ2(θ1+θ2)2+[θ1+θ2]ϕ1ϕ2(ϕ1+ϕ2)2 |
| Maximum directional gain of an antenna element GE,max | 13dBi |

Rear bumper for below 6GHz

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AvdB"θ,ϕ=0°=k=16aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 28.64; 0.6822; -1.6819. 2; 17.941; 1.0841; 0.8466. 3; 10.034; 4.5559; -2.317. 4; 9.9931; 4.9505; 0.8248. 5; 103.05; 9.2847; 0.5153. 6; 102.49; 9.3113; -2.6065. AvdB"θ,ϕ=π=k=18aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 29.045; 0.2614; -0.7172. 2; 7.9836; 2.8649; 2.965. 3; 5.1881; 3.3343; -0.1509. 4; 0.4384; 7.4621; 1.8565. 5; 1.0833; 47.882; 2.0727. 6; 1.131; 35.828; -0.901. 7; 1.0114; 33.576; -0.1873. 8; 1.0014; 39.562; -1.0305. |
| Horizontal cut of the radiation power pattern (dB) | AhdB"θ=π2,ϕ=a0+k=18akcosϕk+⁡bksin(ϕk), ϕ∈[0,2π] k; ak; bk. 0; -17.964; -. 1; 4.1952; 0.0073. 2; -0.3746; -0.0202. 3; 1.2325; 0.005. 4; 0.2862; -0.0764. 5; -0.2796; 0.0252. 6; -0.063; -0.0114. 7; -1.0205; 0.0298. 8; -0.2964; 0.0095. |
| 3D radiation power pattern (dB) | Gθ1=GVθ=0°,ϕ=0°,Gθ2=GHθ=π2,ϕ,Gϕ1=GVθ,ϕ=0,Gϕ2=GVθ,ϕ=π AdB"θ,ϕ=ϕ1Gϕ2+ϕ2Gϕ1θ1θ2(θ1+θ2)2+θ1Gθ2+θ2Gθ1ϕ1ϕ2(ϕ1+ϕ2)2ϕ1+ϕ2θ1θ2(θ1+θ2)2+[θ1+θ2]ϕ1ϕ2(ϕ1+ϕ2)2 |
| Maximum directional gain of an antenna element GE,max | 11dBi |

Front rooftop for below 6GHz

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AvdB"θ,ϕ=0°=k=16aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 27.156; 0.8354; -0.9866. 2; 10.637; 1.2651; 1.9323. 3; 2.1018; 3.7081; -1.6452. 4; 0.9267; 12.057; -2.9006. 5; 0.8573; 8.5269; 3.5787. 6; 0.7456; 15.27; -2.9591. AvdB"θ,ϕ=π=k=14aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 18.648; 1.0206; -0.9611. 2; 11.099; 2.2755; 1.7808. 3; 213.68; 3.7367; -1.6448. 4; 208.11; 3.7558; 1.492. |
| Horizontal cut of the radiation power pattern (dB) | AhdB"θ=π2,ϕ=a0+k=18akcosϕk+⁡bksin(ϕk), ϕ∈[0,2π] k; ak; bk. 0; -11.719; -. 1; -2.4504; 0.0053. 2; -0.9825; -0.0508. 3; -0.0419; -0.0225. 4; 0.2384; 0.0065. 5; 0.2406; -0.0023. 6; 0.1532; 0.006. 7; 0.1285; 0.0062. 8; 0.1407; 0.0038. |
| 3D radiation power pattern (dB) | Gθ1=GVθ=0°,ϕ=0°,Gθ2=GHθ=π2,ϕ,Gϕ1=GVθ,ϕ=0,Gϕ2=GVθ,ϕ=π AdB"θ,ϕ=ϕ1Gϕ2+ϕ2Gϕ1θ1θ2(θ1+θ2)2+θ1Gθ2+θ2Gθ1ϕ1ϕ2(ϕ1+ϕ2)2ϕ1+ϕ2θ1θ2(θ1+θ2)2+[θ1+θ2]ϕ1ϕ2(ϕ1+ϕ2)2 |
| Maximum directional gain of an antenna element GE,max | 3dBi |

Rear rooftop for below 6GHz

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AvdB"θ,ϕ=0°=k=15aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 21.785; 0.7541; -2.5975. 2; 122; 2.0393; -0.3421. 3; 4.5185; 4.4006; 2.4657. 4; 25.257; 3.2885; -0.6558. 5; 125.16; 2.3309; 2.7194. AvdB"θ,ϕ=π=k=14aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 89.841; 0.5245; -0.2707. 2; 64.942; 0.6058; 2.9391. 3; 2.4779; 4.1726; -1.4849. 4; 1.097; 7.9882; -1.7998. |
| Horizontal cut of the radiation power pattern (dB) | AhdB"θ=π2,ϕ=a0+k=18akcosϕk+⁡bksin(ϕk), ϕ∈[0,2π] k; ak; bk. 0; -12.399; -. 1; -0.6788; -0.0034. 2; -1.025; -0.0364. 3; -0.0755; -0.0404. 4; 0.4216; 0.014. 5; -0.6139; -0.0053. 6; 0.2297; 0.0063. 7; -0.4418; -0.0038. 8; 0.0055; -0.0141. |
| 3D radiation power pattern (dB) | Gθ1=GVθ=0°,ϕ=0°,Gθ2=GHθ=π2,ϕ,Gϕ1=GVθ,ϕ=0,Gϕ2=GVθ,ϕ=π AdB"θ,ϕ=ϕ1Gϕ2+ϕ2Gϕ1θ1θ2(θ1+θ2)2+θ1Gθ2+θ2Gθ1ϕ1ϕ2(ϕ1+ϕ2)2ϕ1+ϕ2θ1θ2(θ1+θ2)2+[θ1+θ2]ϕ1ϕ2(ϕ1+ϕ2)2 |
| Maximum directional gain of an antenna element GE,max | 3dBi |

Front bumper for above 6GHz

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AvdB"θ,ϕ=0°=k=13aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 103.4; 1.2756; -1.4588. 2; 93.054; 1.4402; 1.6919. 3; 1.2433; 16.391; -0.2701. AvdB"θ,ϕ=π=k=13aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 97.485; 1.2777; -1.5428. 2; 88.914; 1.4458; 1.5626. 3; 1.6315; 6.6824; -1.4228. |
| Horizontal cut of the radiation power pattern (dB) | AhdB"θ=π2,ϕ=a0+k=18akcosϕk+⁡bksin(ϕk), ϕ∈[0,2π] k; ak; bk. 0; -14.712; -. 1; -0.814; -0.0126. 2; 4.5025; -0.4615. 3; 0.6572; -0.0404. 4; 0.3252; 0.5985. 5; -2.0187; 0.0372. 6; -1.6555; 0.2589. 7; -0.843; 0.6858. 8; -0.2954; -0.2211. |
| 3D radiation power pattern (dB) | Gθ1=GVθ=0°,ϕ=0°,Gθ2=GHθ=π2,ϕ,Gϕ1=GVθ,ϕ=0,Gϕ2=GVθ,ϕ=π AdB"θ,ϕ=ϕ1Gϕ2+ϕ2Gϕ1θ1θ2(θ1+θ2)2+θ1Gθ2+θ2Gθ1ϕ1ϕ2(ϕ1+ϕ2)2ϕ1+ϕ2θ1θ2(θ1+θ2)2+[θ1+θ2]ϕ1ϕ2(ϕ1+ϕ2)2 |
| Maximum directional gain of an antenna element GE,max | 16.6dBi |

Rear bumper for above 6GHz

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AvdB"θ,ϕ=0°=k=14aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 87.458; 0.0103; -2.9955. 2; 4.1908; 2.1185; 1.8618. 3; 1.0935; 127.65; 2.3653. 4; 1.2773; 16.837; 1.1214. AvdB"θ,ϕ=π=k=16aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 17.631; 0.823; -1.5005. 2; 12.155; 1.9599; 1.2643. 3; 15.226; 5.6698; 2.2669. 4; 15.495; 5.7995; -0.9411. 5; 0.3371; 10.294; -3.8515. 6; 0.5965; 11.41; 0.4383. |
| Horizontal cut of the radiation power pattern (dB) | AhdB"θ=π2,ϕ=a0+k=18akcosϕk+⁡bksin(ϕk), ϕ∈[0,2π] k; ak; bk. 0; -12.742; -. 1; -1.2007; 0.2451. 2; 4.1916; -0.5651. 3; 0.1901; 0.453. 4; 0.4235; 0.6884. 5; -1.3848; -0.4145. 6; -0.2381; 0.2875. 7; 0.4488; 0.5441. 8; 0.1606; -0.2555. |
| 3D radiation power pattern (dB) | Gθ1=GVθ=0°,ϕ=0°,Gθ2=GHθ=π2,ϕ,Gϕ1=GVθ,ϕ=0,Gϕ2=GVθ,ϕ=π AdB"θ,ϕ=ϕ1Gϕ2+ϕ2Gϕ1θ1θ2(θ1+θ2)2+θ1Gθ2+θ2Gθ1ϕ1ϕ2(ϕ1+ϕ2)2ϕ1+ϕ2θ1θ2(θ1+θ2)2+[θ1+θ2]ϕ1ϕ2(ϕ1+ϕ2)2 |
| Maximum directional gain of an antenna element GE,max | 13.6dBi |

Front rooftop for above 6GHz

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AvdB"θ,ϕ=0°=k=16aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 53.19; 1.0466; -0.9824. 2; 171.45; 2.3513; 2.2593. 3; 154.83; 2.4609; -0.8176. 4; 2.166; 6.2024; 0.5122. 5; 1.2277; 9.599; 0.5604. 6; 1.1363; 14.311; -0.4448. AvdB"θ,ϕ=π=k=18aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 34.284; 1.0869; -1.1043. 2; 36.721; 2.2823; 1.4488. 3; 4.2902; 5.2423; 1.1692. 4; 1.7681; 7.7706; -3.641. 5; 14.094; 2.7398; -1.5731. 6; 1.2267; 10.465; 2.0934. 7; 1.0796; 14.285; 2.8759. 8; 0.8846; 31.578; -1.1871. |
| Horizontal cut of the radiation power pattern (dB) | AhdB"θ=π2,ϕ=a0+k=18akcosϕk+⁡bksin(ϕk), ϕ∈[0,2π] k; ak; bk. 0; -16.085; -. 1; -7.903; 0.1101. 2; -0.2972; -1.4639. 3; 0.4083; -0.0866. 4; 0.6706; 1.0598. 5; -0.9804; -2.5714. 6; -0.9085; 0.1326. 7; 0.1433; -0.1372. 8; -0.1282; -0.2697. |
| 3D radiation power pattern (dB) | Gθ1=GVθ=0°,ϕ=0°,Gθ2=GHθ=π2,ϕ,Gϕ1=GVθ,ϕ=0,Gϕ2=GVθ,ϕ=π AdB"θ,ϕ=ϕ1Gϕ2+ϕ2Gϕ1θ1θ2(θ1+θ2)2+θ1Gθ2+θ2Gθ1ϕ1ϕ2(ϕ1+ϕ2)2ϕ1+ϕ2θ1θ2(θ1+θ2)2+[θ1+θ2]ϕ1ϕ2(ϕ1+ϕ2)2 |
| Maximum directional gain of an antenna element GE,max | 11.5dBi |

Rear rooftop for above 6GHz

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AvdB"θ,ϕ=0°=k=17aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 55.839; 1.0927; -1.1273. 2; 95.402; 2.0245; 2.1133. 3; 2.9533; 6.0111; 1.5482. 4; 1.5696; 12.462; 3.2683. 5; 1.2073; 10.016; 2.2276. 6; 70.244; 2.1361; -0.8181. 7; 1.4093; 13.127; 0.6659. AvdB"θ,ϕ=π=k=17aksin(bkϕ+ck), θ∈[-π2,π2] k; ak; bk; ck. 1; 42.818; 0.9167; -0.9757. 2; 18.065; 1.895; 1.5513. 3; 8.8444; 3.6653; -0.1862. 4; 9.5301; 7.6263; 0.1674. 5; 15.018; 9.0456; 0.2995. 6; 20.098; 8.565; -2.8308. 7; 1.5077; 12.14; -1.3725. |
| Horizontal cut of the radiation power pattern (dB) | AhdB"θ=π2,ϕ=a0+k=18akcosϕk+⁡bksin(ϕk), ϕ∈[0,2π] k; ak; bk. 0; -18.471; -. 1; 3.6106; -0.1824. 2; -1.4888; 1.3614. 3; 0.5772; 0.5331. 4; 1.7125; 1.3205. 5; -0.0573; -2.5904. 6; 0.0628; -1.0024. 7; -1.0526; -1.2366. 8; 0.0433; -0.1443. |
| 3D radiation power pattern (dB) | Gθ1=GVθ=0°,ϕ=0°,Gθ2=GHθ=π2,ϕ,Gϕ1=GVθ,ϕ=0,Gϕ2=GVθ,ϕ=π AdB"θ,ϕ=ϕ1Gϕ2+ϕ2Gϕ1θ1θ2(θ1+θ2)2+θ1Gθ2+θ2Gθ1ϕ1ϕ2(ϕ1+ϕ2)2ϕ1+ϕ2θ1θ2(θ1+θ2)2+[θ1+θ2]ϕ1ϕ2(ϕ1+ϕ2)2 |
| Maximum directional gain of an antenna element GE,max | 12.5dBi |

LG (R1-1807612)

For modelling the radiation pattern of collocated antenna, the followings can used for all the type of vehicles.

|  | For 6 GHz | For 63 GHz |
| --- | --- | --- |
| Antenna element gain vertical pattern | Half spherically uniform distribution with top direction |  |
| Antenna element gain horizontal pattern |  |  |
| Pattern combining method for 3D |  |  |
| Max direct. gain of the antenna element | 3 dBi | 5dBi |

Proposal: Proposal from R1-1806152 is agreed for the antenna element pattern in the collocated case.

Proposal: Proposal from R1-1807612 is agreed for the antenna element pattern in the distributed case with the assumption that separate channel modeling between different antenna panels is applied.

    - Antenna array configuration

Intel (R1-1806539)

|  | Vehicle UE |  |
| --- | --- | --- |
|  | Low band (2-6 GHz) | High band (30-63 GHz) |
| TXRU mapping | Up to proponents decision | Up to proponents decision |
| Number of antenna elements across all panels | Up to 8 Tx /Rx antenna elements | Up to 256 Tx /Rx antenna elements |
| Antenna array configuration (M, N, P, Mg, Ng) | Vehicle Type 1 and Type 3: Front and rear antennas: (1, 2, 2, 1, 1) Vehicle Type 2: Rooftop antenna: (1, 2, 2, 1, 1) | Vehicle Type 1: Two single panel antenna arrays: (4, 8, 2, 1, 1) Front antenna array bearing angle: ΩFront = 0° Rear antenna array bearing angle: ΩRear = 180° Vehicle Type 2: Multi-panel: (2, 8, 2, 1, 4) Panel bearing angle: Ω0,1=Ω0,0+90°; Ω0,2=Ω0,0+180°; Ω0,3=Ω0,0+270°; Vehicle Type 3: (2, 8, 2, 1, 2) Front antenna array panels bearing angle: Ω0,0 = 45°; Ω0,1=315° Rear antenna array panels bearing angle: Ω0,0 = 135°; Ω0,1= 225° |
| Antenna array spacing (dH,dV,dH,g,dV,g) | (dH, dV) = (0.5, 0.5)λ | (dH, dV) = (0.5, 0.5)λ |
| Antenna tilt, deg | 90 | 90 |

Ericsson (R1-1806152)

Antenna array configuration for vehicle UE

|  | Vehicle UE |  |
| --- | --- | --- |
|  | Below 6 GHz | Above 6 GHz |
| TXRU mapping | A single TXRU is mapped per panel per polarization | A single TXRU is mapped per panel per polarization |
| Number of antenna elements across all panels | Up to 8 TX/RX antenna elements | Up to 32 Tx /Rx antenna elements |
| Antenna array configuration (M, N, P, Mg, Ng) | Collocated: (1, 4, 2, 1, 1) | Collocated: (2, 4, 2, 1,2) |
| Antenna array spacing (dH,dV,dH,g,dV,g) | (0.5, 0.5, -, -) λ | (0.5, 0.5, 0, 0) λ |

Samsung (R1-1806756)

Antenna array configuration for vehicle UE

|  | Vehicle UE |  |
| --- | --- | --- |
|  | For 6 GHz | For 63 GHz |
| TXRU mapping | Up to proponents decision | Up to proponents decision |
| Number of antenna elements across all panels | Up to 8 Tx /Rx antenna elements | Up to 32 Tx /Rx antenna elements |
| Antenna array configuration (M, N, P, Mg, Ng) | (1, 2, 2, 1, 1) TR 38.802, Table A.1.6-1 | (2, 4, 2, 1, 2) Panel bearing angle: Ω0,1=Ω0,0+180° TR 38.802, Table A.1.6-2 |
| Antenna array spacing (dH,dV,dH,g,dV,g) | (dH, dV) = (0.5, 0.5)λ TR 38.802, Table A.1.6-1 | (dH, dV) = (0.5, 0.5)λ; (dH,g, dV,g) = (0, 0)λ TR 38.802, Table A.1.6-2 |
| Antenna tilt, deg | 0 | 90 |

Proposed antenna model below 6 GHz

| Parameters | Urban grid for eV2X | Highway for eV2X |
| --- | --- | --- |
| UE antenna height | Vehicle UE: 1.5m | Vehicle UE: 1.5m |
| UE antenna gain | Vehicle UE: 3dBi | Vehicle UE: 3dBi |

Proposal: Discuss this after concluding the issue of collocated and distributed antenna.

  - Pedestrian UE

    - Antenna array configuration

The antenna height and antenna panel configuration are open issues.

- 1.5 meter pedestrian UE antenna height

  - Supported by Samsung, Ericsson

- A single panel array with (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1)

  - Supported by Intel, Samsung

Proposal: Pedestrian antenna height is 1.5 m. A single panel array with (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) is used.

- Remaining issues for remaining issues for blockage modeling

Decision is needed between two options for determining whether a link is in NLOSv state:

- Option 1: if a line connecting the antennas of the two vehicles in the same street intersects any vehicle (including either of the two vehicles) in the 3-dimensional space.

  - Supported by Intel, LGE, DOCOMO

- Option 2: with a probability

  - Supported by Huawei, Ericsson, Samsung, ZTE, CATT, InterDigital, OPPO

Proposal: Take option 2 as the baseline and continue discussion on the detailed probability based on R1-1805914.

Decision is needed on the additional loss to be added to the LOS pathloss equation when the link is in NLOSv state. Two contributions proposed how to generate this loss value.

Huawei (R1-1805914)

For V2V side link, use the random variables in the following table for NLOSv blockage loss

| scenarios |  | Option 3-6-2a |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A The main blocker height is definitely lower than both the TX and Rx height. | B The main blocker height is between or similar to the Tx and Rx height. | C The main blocker height is definitely higher than the TX and RX height. | Additional loss due to second blocker of the same height as the main blocker | Additional loss due to second blocker of lower height as the main blocker | Additional loss due to third or any further blocker of any size |
| distribution |  |  | Normal distribution N(Mu, Sigma) |  |  |  |  |
| Urban grid | NLOSv | 0 | Mu = 5.86 dB Sigma = 3.08 dB | Mu = 10.24 dB Sigma = 4.29 dB | Mu = 6 dB Sigma = 0 dB | Mu = 1 dB Sigma = 0 dB | Mu = 1 dB Sigma = 0 dB |
| highway | NLOSv | 0 | Mu = 4.77 dB sigma = 4.26 dB | Mu = 15.39 dB sigma = 5.02dB | Mu = 6 dB Sigma = 0 dB | Mu = 1 dB Sigma = 0 dB | Mu = 1 dB Sigma = 0 dB |

LG (R1-1806640)

In case of NLOSv, the following statistics can be used for deriving the random variable that is added to the pathloss equation, and different statistics are selected depending on whether the blocker height is larger than, equal to, or less than the maximum antenna height value of TX and RX.

- Case 1: Maximum antenna height value of TX and RX = Blocker height

  - Carrier frequency of 5.9 GHz

    - Mean [dB]: 4.1736

    - Standard deviation [dB]: 0.8076

  - Carrier frequency of 60 GHz

    - Mean [dB]: 5.2439

    - Standard deviation [dB]: 0.5957

- Case 2: Maximum antenna height value of TX and RX < Blocker height

  - Carrier frequency of 5.9 GHz

    - Mean [dB]: 10.6265

    - Standard deviation [dB]: 3.5784

  - Carrier frequency of 60 GHz

    - Mean [dB]: 19.4635

    - Standard deviation [dB]: 4.7466

- Case 3: Maximum antenna height value of TX and RX > Blocker height

  - No blockage between TX and RX

Proposal: Define three statistics of the additional loss value depending on the maximum antenna height of TX/RX and the blocker height. Decide the exact distribution based on R1-1805914 and R1-1806640.

- Remaining issues for remaining issues for pathloss model

  - V2B/P2B/B2R

An FFS point is the LOS pathloss equation of V2B/P2B/B2R links in highway scenario above 6 GHz. It was observed in RAN1#92bis that TR 38.901 does not define fast fading for gNB-UE link using RMa channel at 30 GHz.

- Use RMa of TR 38.901

  - Supported by Intel, Ericsson, Samsung, AT&T

- Use UMa of TR 38.901

  - Supported by LGE, InterDigital

Proposal: Use UMa of TR 38.901 with the option of the highway overlaid with 500 m ISD cell layout (Figure A.1.3-3 of TR 36.885).

R1-1806539 mentioned that the effective environment height hE is currently set to 1 meter which is higher than the antenna height of the vehicle type 1 (0.75 meters).

Proposal: Discuss setting hE to 0.25m.

  - V2V/P2V/P2P

The NLOS pahtloss equation for V2V needs to be decided as mentioned in the existing agreement:

| NLOS | FFS including Option 1: where and Option 2: NLOS pathloss equation in R1-1803671 Other options are not precluded. |
| --- | --- |

- Option 1

  - Supported by Intel (a variant of option 1), LGE, Ericsson, Huawei (if modified the gap between minimum and maximum path loss is reduced)

- Option 2

  - Supported by Huawei (revised pathloss equation)

Proposal: Option 1 (or its variant if justified) is agreed.

Proposal: Pathloss equation of V2V is reused for that of V2P, P2P, V2R, R2R.

- Summary of remaining issues for shadowing model

Shadowing model is an open issue:

- Use Rel-14 shadowing model for sidelink

  - Supported by Samsung, LGE

- Use the shadowing model corresponding to the pathloss model for Uu

  - Supported by InterDigital

Proposal: For V2V, V2P, P2P, V2R, R2R links, the shadowing model in TR 36.885 is used. For B2V, B2P, B2R links, the shadowing model associated with the used pathloss model in TR 38.901 is used.

- Summary of remaining issues for fast fading model

The agreement is “For sidelink in Urban and freeway, the fast fading parameters of “UMi-Street Canyon in TR 38.901” is the starting point and to be modified considering sidelink characteristics.” The following contribution proposed modification to the fast fading patameters.

Huawei (R1-1805914 with errors corrected)

| Scenarios |  | Urban |  |  | Highway |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | NLOSv | LOS | NLOSv |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | -0.2 log10(1+ fc) – 7.4 | -0.24 log10(1+ fc) - 6.83 | -0.3 log10(1+ fc) – 7 | -8.16 | -8.04 |
|  | lgDS | -0.04 log10(1+ fc) – 8 | 0.16 log10(1+ fc) + 0.28 | -0.2 log10(1+ fc) – 8 | -8.53 | -8.08 |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | -0.1 log10(1+ fc) + 1.5 | -0.08 log10(1+ fc) + 1.81 | -0.1 log10(1+ fc) + 1.7 | 1.39 | 1.29 |
|  | lgASD | -0.03 log10(1+ fc) + 0.9 | 0.05 log10(1+ fc) + 0.3 | -0.3 log10(1+ fc) + 1.4 | 0.59 | 0.82 |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | -0.1 log10(1+ fc) + 1.5 | -0.08 log10(1+ fc) + 1.81 | -0.1 log10(1+ fc) + 1.7 | 1.39 | 1.29 |
|  | lgASA | -0.03 log10(1+ fc) + 0.9 | 0.05 log10(1+ fc) + 0.3 | -0.3 log10(1+ fc) + 1.4 | 0.59 | 0.82 |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | lgZSA | -0.1 log10(1+ fc) + 0.73 | -0.04 log10(1+ fc) + 0.92 | -0.04 log10(1+ fc) + 0.92 | -0.1 log10(1+ fc) + 0.73 | -0.04 log10(1+ fc) + 0.92 |
|  | lgZSA | -0.04 log10(1+ fc) + 0.34 | -0.07 log10(1+ fc) + 0.41 | -0.07 log10(1+ fc) + 0.41 | -0.04 log10(1+ fc) + 0.34 | -0.07 log10(1+ fc) + 0.41 |
| ZOD spread (ZSD) lgZSD=log10(ZSD/1) | lgZSD | -0.1 log10(1+ fc) + 0.73 | -0.04 log10(1+ fc) + 0.92 | -0.04 log10(1+ fc) + 0.92 | -0.1 log10(1+ fc) + 0.73 | -0.04 log10(1+ fc) + 0.92 |
|  | lgZSD | -0.04 log10(1+ fc) + 0.34 | -0.07 log10(1+ fc) + 0.41 | -0.07 log10(1+ fc) + 0.41 | -0.04 log10(1+ fc) + 0.34 | -0.07 log10(1+ fc) + 0.41 |
| Shadow fading (SF) [dB] | SF | 2 | 3 |  | 4 |  |
| K-factor (K) [dB] | K | 3.48 | N/A | N/A | 9 | N/A |
|  | K | 1.71 | N/A | N/A | 3.5 | N/A |
| Cross-Correlations | ASD vs DS | 0.6 | 0.8 | 0.8 | 0.8 | 0.3 |
|  | ASA vs DS | 0.6 | 0.8 | 0.8 | 0.8 | 0.3 |
|  | ASA vs SF | -0.3 | -0.3 | -0.4 | -0.4 | -0.4 |
|  | ASD vs SF | -0.3 | -0.3 | -0.4 | -0.5 | 0 |
|  | DS vs SF | -0.5 | -0.5 | -0.5 | -0.4 | -0.7 |
|  | ASD vs ASA | 0.4 | 0.1 | 0.1 | 0.8 | 0.3 |
|  | ASD vs  | -0.3 | N/A | N/A | -0.2 | N/A |
|  | ASA vs  | -0.3 | N/A | N/A | -0.3 | N/A |
|  | DS vs  | -0.2 | N/A | N/A | -0.7 | N/A |
|  | SF vs  | 0.1 | N/A | N/A | 0.5 | N/A |
| Cross-Correlations | ZSD vs SF | 0 | 0 | 0 | 0 | 0 |
|  | ZSA vs SF | 0 | 0 | 0 | 0 | 0 |
|  | ZSD vs K | 0 | N/A | N/A | 0 | N/A |
|  | ZSA vs K | 0 | N/A | N/A | 0 | N/A |
|  | ZSD vs DS | 0 | -0.5 | -0.5 | 0 | -0.5 |
|  | ZSA vs DS | 0.2 | 0 | 0 | 0.2 | 0 |
|  | ZSD vs ASD | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
|  | ZSA vs ASD | 0.3 | 0.5 | 0.5 | 0.3 | 0.5 |
|  | ZSD vs ASA | 0 | 0 | 0 | 0 | 0 |
|  | ZSA vs ASA | 0 | 0.2 | 0.2 | 0 | 0.2 |
|  | ZSD vs ZSA | 0 | 0 | 0 | 0 | 0 |
| Delay scaling parameter r |  | 3 | 2.1 | 2.1 | 3 | 2.1 |
| XPR [dB] | XPR | 9 | 8.0 | 8.0 | 9 | 8.0 |
|  | XPR | 3 | 3 | 3 | 3 | 3 |
| Number of clusters |  | 12 | 19 | 19 | 12 | 19 |
| Number of rays per cluster |  | 20 | 20 | 20 | 20 | 20 |
| Cluster DS () in [ns] |  | 5 | 11 | 11 | 5 | 11 |
| Cluster ASD () in [deg] |  | 3 | 10 | 10 | 3 | 10 |
| Cluster ASA () in [deg] |  | 17 | 22 | 22 | 17 | 22 |
| Cluster ZSA () in [deg] |  | 7 | 7 | 7 | 7 | 7 |
| Per cluster shadowing std  [dB] |  | 4 | 4 | 4 | 4 | 4 |
| Correlation distance in the horizontal plane [m] | DS | 7 | 10 | 10 | 7 | 10 |
|  | ASD | 8 | 10 | 10 | 8 | 10 |
|  | ASA | 8 | 9 | 9 | 8 | 9 |
|  | SF | 10 | 13 | 13 | 10 | 13 |
|  |  | 15 | N/A | N/A | 15 | N/A |
|  | ZSA | 12 | 10 | 10 | 12 | 10 |
|  | ZSD | 12 | 10 | 10 | 12 | 10 |
| fc is carrier frequency in GHz. Procedure for generating both ZOA and ZOD is the same and based on the ZOA procedure in 3GPP TR38.901. |  |  |  |  |  |  |

Proposal: The fading parameters in the above table are agreed.

- Summary of remaining issues for Doppler model

The agreement is “Rel-14 dual mobility is modified such that a random Doppler shift is added to each reflected path.” The following contribution proposes details of the additional random Doppler shift.

Huawei (R1-1805914)

Use the Doppler model for dual mobility with moving scatterers for V2X sidelink channel according to equations (1) -(4).

- Doppler for the LOS path:

(1)

(2)

(3)

- Doppler for the delayed paths:

vn,m=rrx,n,mT∙vrx+rtx,n,mT∙vtx+2αn,mDn,mλ0    (4)

where Dn,m is a random variable with uniform distribution from -vscatt to vscatt,  vscatt is the maximum speed of a scatterer, and αn,m (0≤αn,m≤1) is a parameter to account for fixed scatterers. In case when αn,m=0, all scatterers are static (similar to with Rel-14).

Proposal: The dual mobility model in R1-1805914 is agreed. Discuss the statistics of αn,m.

- Channel model for the evaluation related to the cellular UE

For evaluations of UE-network relaying or coexistence of sidelink and Uu, channel model for the links involving cellular UEs need to be decided.

Proposal: Discuss the following proposal:

- Reuse antenna model for UE-type RSU as baseline for the vehicle UE’s antenna inside a vehicle which will be used for communication with cellular UEs inside the vehicle.

- Reuse V2P channel model as baseline for the channel model between vehicle UE acting as relay and cellular UE.

  - [1] meter is assumed for the link between antenna of a cellular UE inside a vehicle and the antenna of the same vehicle UE.

- Cellular UE uses the antenna model of pedestrian UE.

Reference

[1] R1-1805914, “V2X sidelink channel model”, Huawei, HiSilicon,

[2] R1-1806028, “Further discussion on eV2X Channel Modeling”, ZTE Wistron Telecom AB, Sanechips

[3] R1-1806103, “Remaining issues on eV2X channel modeling”, ZTE

[4] R1-1806152, “Remaining issues on channel models and antenna models for V2X”, Ericsson

[5] R1-1806204, “Vehicle UE Antenna Radiation Patterns”, OnStar Europe

[6] R1-1806312, “Remaining details on channel models of new V2X use cases”, CATT

[7] R1-1806539, “Channel models for eV2X evaluation methodology”, Intel Corporation

[8] R1-1806640, “Discussion on channel model for new V2X use cases”, LG Electronics

[9] R1-1806756, “Channel model for NR V2X”, Samsung

[10] R1-1806832, “On vehicle blocking in channel modelling”, OPPO

[11] R1-1806985, “Channel model for V2X”, AT&T

[12] R1-1807032, “On the Remaining Details of V2X Channel Model”, InterDigital, Inc.

[13] R1-1807079, “Channel model for V2X phase 3”, NTT DOCOMO, INC.

[14] R1-1807612, “Discussion on antenna radiation pattern for vehicle UE”, LG Electronics

[15] R1-1807613, “Vehicle UE Antenna Radiation Patterns”, OnStar Europe