3GPP TSG RAN WG1 Meeting #94                                                                       R1-1808421

Gothenburg, Sweden, August 20th – 24th, 2018

Source:	CATT

Title:	Self-evaluation on Spectrum Efficiency and User Experienced Data Rate in Dense Urban-eMBB

Agenda Item:	7.2.7.2

Document for:	Discussion and Decision

Introduction

At RAN#75 meeting, new Study Item on Self Evaluation towards IMT-2020 submission was approved [1], the self evaluation will provide the performance towards all the ITU-R IMT-2020 requirements. The key minimum technical performance requirements are provided in Report ITU-R M.2410 [2] for the test environment of Dense Urban-eMBB.

TABLE 1

5th percentile user spectral efficiency

| Test environment | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Dense Urban – eMBB | 0.225 | 0.15 |

TABLE 2

Average spectral efficiency

| Test environment | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Dense Urban – eMBB (Note 1) | 7.8 | 5.4 |

NOTE 1 – This requirement applies to Macro TRxP layer of the Dense Urban – eMBB test environment as described in Report ITU-R M.2412-0.

According to the evaluation methodology of Report ITU-R M.2412 [3], the average spectral efficiency and the 5th percentile user spectral efficiency need system-level simulation evaluation to demonstrate the achievement for the ITU-R technical performance requirements, while user experienced data rate can be calculated by 5th percentile user spectral efficiency SEuser through equation in [2] (this KPI is only applied to Dense Urban-eMBB).

Ruser = W × SEuser

In our companion contribution [4], some initial evaluation results including the average spectral efficiency, the 5th percentile user spectral efficiency and the user experienced data rate were provided. In this document, more updated evaluation results in Dense Urban-eMBB based on the updated overhead [5] and the agreed technical features and system configurations in [6-8] are provided to meet the technical performance requirements. Also, the evaluation results of above 6GHz are provided.

Evaluation methodology and assumption

System-level simulation with proper link to system interface based on link-level simulation results could be used to evaluate average spectral efficiency and 5th percentile user spectral efficiency.

The user experienced data rate can be calculated based on proper bandwidth to demonstrate the achievement for the requirement of the ITU-R.

In this contribution, different antenna configurations are evaluated to observe the proper antenna configurations to fulfil the ITU minimum performance requirement and to observe the performance advantage of NR system, including different number of antenna elements and different TXRU mapping. Also, the evaluation results based on different SCS and DL CSI measurement reference signal are provided.

Evaluation results

The evaluation results are provided for average spectral efficiency, 5th percentile spectral efficiency and user experienced data rate under Dense Urban-eMBB, the overhead assumptions refer to our companion contribution [5]. The detailed evaluation assumptions can be found in Annex.

Downlink evaluation results

The evaluation results for DL average spectral efficiency and DL 5th percentile spectral efficiency under Dense Urban-eMBB are provided in Table 1~4. The DL user experienced data rates results are provided in Table 5 which are calculated based on the results of the DL 5th percentile spectral efficiency.

Spectral efficiency:

Note: Impacts from channel estimation error modeling, PRB bundling, interference error modeling and overhead are updated. Evaluation results in TDD mode based on non-precoded CSI-RS are also provided. The evaluation results of Configuration B of 30GHz are also provided.

Table 1 DL average spectral efficiency evaluation (Dense Urban-eMBB at 4GHz)

| Test environment | Duplex scheme | DL CSI measurement | TRxP and UE antenna Config. (M,N,P,Mg,Ng; Mp,Np) | Requirement (bit/s/Hz/TRxP) | SEavg (bit/s/Hz/TRxP) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| Dense Urban-eMBB Config. A, 4GHz UMa-B | FDD | Non-precoded CSI-RS based | 16T/4R, (8,8,2,1,1;1,8) | 7.8 | 8.34 | 6.92% |
|  |  |  | 32T/4R, (8,8,2,1,1;2,8) |  | 11.22 | 43.85% |
|  |  |  | 32T/4R, (16,8,2,1,1;2,8) |  | 12.08 | 54.87% |
|  | TDD 15KHz SCS | Non-precoded CSI-RS based | 16T/4R, (8,8,2,1,1; 1,8) |  | 11.03 | 41.41% |
|  |  |  | 32T/4R, (8,8,2,1,1; 2,8) |  | 14.36 | 84.10% |
|  |  |  | 32T/4R, (16,8,2,1,1; 2,8) |  | 15.67 | 100.90% |
|  |  | Precoded CSI-RS based | 16T/4R, (8,8,2,1,1; 1,8) |  | 10.71 | 37.31% |
|  |  |  | 32T/4R, (8,8,2,1,1; 2,8) |  | 14.22 | 82.31% |
|  |  |  | 32T/4R, (16,8,2,1,1; 2,8) |  | 15.52 | 98.97% |
|  |  |  | 64T/4R, (8,8,2,1,1; 4,8) |  | 17.10 | 119.23% |
|  |  |  | 64T/4R, (12,8,2,1,1; 4,8) |  | 18.35 | 135.26% |
|  |  |  | 64T/4R, (16,8,2,1,1; 4,8) |  | 19.15 | 145.51% |
|  | TDD 30KHz SCS | Non-precoded CSI-RS based | 16T/4R, (8,8,2,1,1; 1,8) |  | 10.64 | 36.41% |
|  |  |  | 32T/4R, (8,8,2,1,1; 2,8) |  | 13.95 | 78.85% |
|  |  |  | 32T/4R, (16,8,2,1,1; 2,8) |  | 15.13 | 93.97% |
|  |  | Precoded CSI-RS based | 16T/4R, (8,8,2,1,1; 1,8) |  | 10.32 | 32.31% |
|  |  |  | 32T/4R, (8,8,2,1,1; 2,8) |  | 13.79 | 76.79% |
|  |  |  | 32T/4R, (16,8,2,1,1; 2,8) |  | 15.03 | 92.69% |
|  |  |  | 64T/4R, (8,8,2,1,1; 4,8) |  | 16.62 | 113.08% |
|  |  |  | 64T/4R, (12,8,2,1,1; 4,8) |  | 17.80 | 128.21% |
|  |  |  | 64T/4R, (16,8,2,1,1; 4,8) |  | 18.57 | 138.08% |

Table 2 DL 5th percentile spectral efficiency evaluation (Dense Urban-eMBB at 4GHz)

| Test environment | Duplex scheme | DL CSI measurement | TRxP and UE antenna Config. (M,N,P,Mg,Ng; Mp,Np) | Requirement (bit/s/Hz) | SEuser (bit/s/Hz) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| Dense Urban-eMBB Config. A, 4GHz UMa-B | FDD | Non-precoded CSI-RS based | 16T/4R, (8,8,2,1,1;1,8) | 0.225 | 0.288 | 28.00% |
|  |  |  | 32T/4R, (8,8,2,1,1;2,8) |  | 0.402 | 78.67% |
|  |  |  | 32T/4R, (16,8,2,1,1;2,8) |  | 0.448 | 99.11% |
|  | TDD 15KHz SCS | Non-precoded CSI-RS based | 16T/4R, (8,8,2,1,1; 1,8) |  | 0.356 | 58.22% |
|  |  |  | 32T/4R, (8,8,2,1,1; 2,8) |  | 0.504 | 124.00% |
|  |  |  | 32T/4R, (16,8,2,1,1; 2,8) |  | 0.548 | 143.56% |
|  |  | Precoded CSI-RS based | 16T/4R, (8,8,2,1,1; 1,8) |  | 0.341 | 51.56% |
|  |  |  | 32T/4R, (8,8,2,1,1; 2,8) |  | 0.501 | 122.67% |
|  |  |  | 32T/4R, (16,8,2,1,1; 2,8) |  | 0.547 | 143.11% |
|  |  |  | 64T/4R, (8,8,2,1,1; 4,8) |  | 0.669 | 197.33% |
|  |  |  | 64T/4R, (12,8,2,1,1; 4,8) |  | 0.725 | 222.22% |
|  |  |  | 64T/4R, (16,8,2,1,1; 4,8) |  | 0.825 | 266.67% |
|  | TDD 30KHz SCS | Non-precoded CSI-RS based | 16T/4R, (8,8,2,1,1; 1,8) |  | 0.345 | 53.33% |
|  |  |  | 32T/4R, (8,8,2,1,1; 2,8) |  | 0.505 | 124.44% |
|  |  |  | 32T/4R, (16,8,2,1,1; 2,8) |  | 0.525 | 133.33% |
|  |  | Precoded CSI-RS based | 16T/4R, (8,8,2,1,1; 1,8) |  | 0.330 | 46.67% |
|  |  |  | 32T/4R, (8,8,2,1,1; 2,8) |  | 0.500 | 122.22% |
|  |  |  | 32T/4R, (16,8,2,1,1; 2,8) |  | 0.526 | 133.78% |
|  |  |  | 64T/4R, (8,8,2,1,1; 4,8) |  | 0.642 | 185.33% |
|  |  |  | 64T/4R, (12,8,2,1,1; 4,8) |  | 0.725 | 222.22% |
|  |  |  | 64T/4R, (16,8,2,1,1; 4,8) |  | 0.797 | 254.22% |

Table 3 DL average spectral efficiency evaluation (Dense Urban-eMBB at 30GHz)

| Test environment | Duplex scheme | TRxP and UE antenna Config. (M,N,P,Mg,Ng; Mp,Np) | Requirement (bit/s/Hz/TRxP) | SEavg (bit/s/Hz/TRxP) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Ideal Chan&Intf Esti | Non-ideal Chan&Intf Esti |
| Dense Urban-eMBB Config. B, 30GHz UMa-B | TDD 60KHz SCS | 32T/32R, (8,16,2,1,1; 1,16)/(2,8,2,1,1;2,8) | 7.8 | 12.11 | 9.96 |
|  |  | 64T/32R, (8,16,2,1,1; 2,16)/(2,8,2,1,1;2,8) |  | 13.23 | 11.28 |

Table 4 DL 5th percentile spectral efficiency evaluation (Dense Urban-eMBB at 30GHz)

| Test environment | Duplex scheme | TRxP and UE antenna Config. (M,N,P,Mg,Ng; Mp,Np) | Requirement (bit/s/Hz/TRxP) | SEuser (bit/s/Hz) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Ideal Chan&Intf Esti | Non-ideal Chan&Intf Esti |
| Dense Urban-eMBB Config. B, 30GHz UMa-B | TDD 60KHz SCS | 32T/32R, (8,16,2,1,1; 1,16)/(2,8,2,1,1;2,8) | 0.225 | 0.29 | 0.019 |
|  |  | 64T/32R, (8,16,2,1,1; 2,16)/(2,8,2,1,1;2,8) |  | 0.44 | 0.052 |

Observation 1: For sub 6 GHz, NR can fulfill the DL spectral efficiency requirements for the Dense Urban-eMBB test environment in both TDD and FDD mode.

Observation 2: Evaluation results on downlink spectral efficiency with 30KHz SCS are slightly worse than 15KHz SCS with the assumption that the simulation bandwidth is not scaled with SCS.

Observation 3: For TDD, the evaluations results based on non-precoded CSI-RS and precoded CSI-RS are similar. The evaluations results based on precoded CSI-RS are slightly worse due to the larger overhead.

Observation 4: For above 6 GHz with 60KHz SCS

- With ideal channel and interference estimation, NR can fulfill the DL spectral efficiency requirements for the Dense Urban-eMBB test environment in TDD mode.

- With non-ideal channel and interference estimation, NR can not fulfill the DL 5th percentile user spectral efficiency requirements for the Dense Urban-eMBB test environment.

User experienced data rate:

Considering various bandwidth at 4GHz, DL user experienced data rates are provided in Table 5.

Table 5 DL user experienced data rate evaluation (Dense Urban-eMBB at 4GHz)

| Test environment | Duplex scheme | TRxP and UE antenna Config. (M,N,P,Mg,Ng; Mp,Np) at TRxPs | Bandwidth W (MHz) | Ruser (Mbit/s) | Requirement (Mbit/s) |
| --- | --- | --- | --- | --- | --- |
| Dense Urban-eMBB Config. A, 4GHz UMa-B | FDD | 16T/4R, (8,8,2,1,1;1,8) | 300 | 86.4 | 100 |
|  |  |  | 350 | 100.8 |  |
|  |  |  | 400 | 115.2 |  |
|  |  | 32T/4R, (16,8,2,1,1;2,8) | 200 | 89.6 |  |
|  |  |  | 250 | 112 |  |
|  | TDD 15KHz SCS | 16T/4R,(8,8,2,1,1; 1,8) | 250 | 85.25 |  |
|  |  |  | 300 | 102.3 |  |
|  |  |  | 350 | 119.35 |  |
|  |  | 64T/4R,(16,8,2,1,1; 4,8) | 100 | 82.5 |  |
|  |  |  | 150 | 123.75 |  |
|  | TDD 30KHz SCS | 16T/4R,(8,8,2,1,1; 1,8) | 300 | 99 |  |
|  |  |  | 350 | 115.5 |  |
|  |  |  | 400 | 132 |  |
|  |  | 64T/4R,(16,8,2,1,1; 4,8) | 100 | 79.7 |  |
|  |  |  | 150 | 119.55 |  |

Observation 5: The ITU requirement on DL user experienced data rate can be met by NR.

Note that it is good to list both results from minimum required antenna configuration and a reasonable configuration which can provide required bandwidth respectively to meet DL user experienced data rate requirement.

Proposal 1: Further discuss which antenna configuration(s) are applied to DL user experienced data rate evaluation, which leads to different presentation of required bandwidth.

Uplink evaluation results

Based on simulation assumptions in RAN1#92 and RAN1#92bis meeting, non-ideal channel estimation, non-ideal interference estimation, and power backoff for CP-OFDM are modeled. The initial evaluation results for UL average spectral efficiency and UL 5th percentile spectral efficiency under Dense-Urban eMBB are provided in Table 6 and Table 7. Further polish power control parameter and scheduler for both FDD and TDD to balance cell average spectral efficiency and 5th percentile spectral efficiency is performed. From initial results, it can be observed using non-ideal modeling on channel estimation and interference estimation, NR can also satisfy ITU requirement in FR1(Configuration A) while it is very hard to satisfy the requirement in FR2 (Configuraiton B). In this contribution, non-ideal modeling is applied to FR1 and ideal modeling is applied to FR2.

Spectral efficiency:

Table 6 UL average spectral efficiency evaluation

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at receiver) | Element configuration(No. at transmitter/receiver, configuration at receiver) | Requirement (bit/s/Hz) | SEavg (bit/s/Hz) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| Dense Urban-eMBB Config. A, 4GHz UMa-B | FDD | 4T/16R, (1,8,2,1,1) | 4Tx/128Rx, (8,8,2,1,1) | 5.4 | 6.40 | 18.5% |
|  | TDD,15KHz | 4T/16R, (1,8,2,1,1) | 4Tx/128Rx, (8,8,2,1,1) |  | 7.01 | 29.8% |
| Dense Urban-eMBB Config. B, 30GHz UMa-B | TDD,60KHz | 16T/32R, (1,4,2,1,2)/(2,8,2,1,1) | 32Tx/256Rx, (2,4,2,1,2)/ (8,16,2,1,1) |  | 8.13 | 50.6% |

Note: for FR2, ideal channel estimation and interference estimation is assumed.

Table 7 UL 5th percentile spectral efficiency evaluation

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at receiver) | Element configuration(No. at transmitter/receiver, configuration at receiver) | Requirement (bit/s/Hz) | SEedge (bit/s/Hz) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| Dense Urban-eMBB Config. A, 4GHz UMa-B | FDD | 4T/16R, (1,8,2,1,1) | 4Tx/128Rx, (8,8,2,1,1) | 0.15 | 0.349 | 132.8% |
|  | TDD,15KHz | 4T/16R, (1,8,2,1,1) | 4Tx/128Rx, (8,8,2,1,1) |  | 0.321 | 113.8% |
| Dense Urban-eMBB Config. B, 30GHz UMa-B | TDD,60KHz | 16T/32R, (1,4,2,1,2)/(2,8,2,1,1) | 32Tx/256Rx, (2,4,2,1,2)/ (8,16,2,1,1) |  | 0.160 | 6.35% |

Note: for FR2, ideal channel estimation and interference estimation is assumed.

Observation 6: For sub 6 GHz, NR can fulfill the UL spectral efficiency requirements for the Dense Urban-eMBB test environment in both FDD and TDD mode.

Observation 7: For above 6 GHz with 60KHz SCS, with ideal channel and interference estimation, NR can fulfill the UL spectral efficiency requirements for the Dense Urban-eMBB test environment in TDD mode.

User experienced data rate:

Considering various bandwidth at 4GHz, UL user experienced data rates are provided in Table 7.

Table 8 UL user experienced data rate evaluation at 4GHz

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at receiver) | Element configuration(No. at transmitter/receiver, configuration at receiver) | Bandwidth W (MHz) | Ruser (Mbit/s) | Requirement (Mbit/s) |
| --- | --- | --- | --- | --- | --- | --- |
| Dense Urban-eMBB Config. A,4GHz UMa-B | FDD | 4T/16R, (1,8,2,1,1) | 4Tx/128Rx, (8,8,2,1,1) | 100 | 34.93 | 50 |
|  |  |  |  | 150 | 52.39 |  |
|  | TDD | 4T/16R, (1,8,2,1,1) | 4Tx/128Rx, (8,8,2,1,1) | 150 | 48.09 |  |
|  |  |  |  | 200 | 64.12 |  |

Observation 8: The ITU requirement on UL user experienced data rate can be met by NR.

Conclusion

In this contribution, we provide our preliminary evaluation results of Dense Urban -eMBB, we have following observations and proposals:

Observation 1: For sub 6 GHz, NR can fulfill the DL spectral efficiency requirements for the Dense Urban-eMBB test environment in both TDD and FDD mode.

Observation 2: Evaluation results on downlink spectral efficiency with 30KHz SCS are slightly worse than 15KHz SCS with the assumption that the simulation bandwidth is not scaled with SCS.

Observation 3: For TDD, the evaluations results based on non-precoded CSI-RS and precoded CSI-RS are similar. The evaluations results based on precoded CSI-RS are slightly worse due to the larger overhead.

Observation 4: For above 6 GHz with 60KHz SCS

- With ideal channel and interference estimation, NR can fulfill the DL spectral efficiency requirements for the Dense Urban-eMBB test environment in TDD mode.

- With non-ideal channel and interference estimation, NR can not fulfill the DL 5th percentile user spectral efficiency requirements for the Dense Urban-eMBB test environment.

Observation 5: The ITU requirement on DL user experienced data rate can be met by NR.

Observation 6: For sub 6 GHz, NR can fulfill the UL spectral efficiency requirements for the Dense Urban-eMBB test environment in both FDD and TDD mode.

Observation 7: For above 6 GHz with 60KHz SCS, with ideal channel and interference estimation, NR can fulfill the UL spectral efficiency requirements for the Dense Urban-eMBB test environment in TDD mode.

Observation 8: The ITU requirement on UL user experienced data rate can be met by NR.

Proposal 1: Further discuss which antenna configuration(s) are applied to DL user experienced data rate evaluation, which leads to different presentation of required bandwidth.

- References

  - RP-170832, “New Study Item on Self Evaluation towards IMT-2020 submission”, Huawei, Ericsson.

  - Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

  - Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

  - R1-1806329, “Initial self evaluation for Dense Urban-eMBB”, CATT.

  - R1-1808417,  “Discussions on overhead for spectrum efficiency”, CATT

  - RAN1#92 Chairman’s notes.

  - RAN1#92 Chairman’s notes.

  - RAN1#93 Chairman’s notes.

  - R1-1806322, “Discussions on evaluation methodology for self-evaluation”, CATT.

- Annex: Evaluation assumptions

In this contribution, the evaluation assumptions for Dense Urban-eMBB are provided in the following table.

Table A-1 Evaluation assumptions for DL and UL

| Dense Urban - eMBB | Config. A | Config. B |
| --- | --- | --- |
| Carrier frequency for evaluation | 1 layer (Macro) with 4 GHz | 1 layer (Macro) with 30 GHz |
| Simulation bandwidth | FDD: 10M+10M TDD: 20M | TDD: 80M |
| Slots structure | FDD: paired spectrum, DL + UL TDD: unpaired spectrum, DSUUD | TDD: unpaired spectrum, DSUUD |
| Subcarrier spacing | 15KHz,30KHz(TDD only) | 60 KHz |
| Symbols number per slot | 14 | 14 |
| BS antenna height | 25 m | 25 m |
| Total transmit power per TRxP | FDD: 41 dBm TDD: 44 dBm | 40 dBm |
| UE power class | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | 20% high loss, 80% low loss (applies to Channel model B) | 20% high loss, 80% low loss (applies to Channel model B) |
| Inter-site distance | 200 m | 200 m |
| Number of antenna elements per TRxP | 128Tx/Rx, (M,N,P,Mg,Ng) = (8,8,2,1,1), 192Tx/Rx, (M,N,P,Mg,Ng) = (12,8,2,1,1), 256Tx/Rx, (M,N,P,Mg,Ng) = (16,8,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization | 256Tx/Rx, (M,N,P,Mg,Ng) = (8,16,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 16TXRU, (Mp,Np,P,Mg,Ng) = (1,8,2,1,1) 32TXRU, (Mp,Np,P,Mg,Ng) = (2,8,2,1,1) 64 TXRU, (Mp,Np,P,Mg,Ng) = (4,8,2,1,1) | 32TXRU, (Mp,Np,P,Mg,Ng) = (1,16,2,1,1) 32TXRU, (Mp,Np,P,Mg,Ng) = (2,8,2,1,1) 64 TXRU, (Mp,Np,P,Mg,Ng) = (2,16,2,1,1) |
| Number of UE antenna elements | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ 0°,90° polarization | 32Tx/Rx, (M,N,P,Mg,Ng) = (2,8,2,1,1), (dH,dV) = (0.5, N/A)λ 32Tx/Rx, (M,N,P,Mg,Ng) = (2,4,2,1,2), (dH,dV) = (0.5, N/A)λ 0°,90° polarization |
| Number of TXRU per UE | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) (1-to-1 mapping) | 32TXRU, (Mp,Np,P,Mg,Ng) = (2,8,2,1,1) 16TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,2) |
| Device deployment | 80% indoor, 20% outdoor (in car) Randomly and uniformly distributed over the area under Macro layer | 80% indoor, 20% outdoor (in car) Randomly and uniformly distributed over the area under Macro layer |
| UE speeds of interest | Indoor users: 3km/h Outdoor users (in-car): 30 km/h | Indoor users: 3km/h Outdoor users (in-car): 30 km/h |
| BS noise figure | 5 dB | 5 dB |
| UE noise figure | 7 dB | 7 dB |
| BS antenna element gain | 8 dBi | 8 dBi |
| BS antenna element pattern | See Table 9 in Report ITU-R M.2412 | See Table 9 in Report ITU-R M.2412 |
| UE antenna element gain | 0 dBi | 5 dBi |
| UE antenna element pattern | Omni-directional | See Table 11 in Report ITU-R M.2412 |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz |
| Traffic model | Full buffer | Full buffer |
| UE density | 10 UEs per TRxP | 10 UEs per TRxP |
| UE antenna height | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |
| Channel model | Channel model B | Channel model B |
| TRxP number per site | 3 | 3 |
| Mechanic tilt | 90° in GCS | 90° in GCS |
| Electronic tilt | 102° in LCS | / |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Wrapping around method | Geographical distance based wrapping | Geographical distance based wrapping |
| Polarized antenna model | Model-2 in TR36.873 | Model-2 in TR36.873 |
| Beam set at TRxP | / | For direction of TRxP analog beam steering (in LCS): Zenith angle θj : [7*pi/12, 2*pi/3, 3*pi/4, 5*pi/6] |
| Beam set at UE | / | / |
| TX mode | SU/MU-MIMO | SU/MU-MIMO |
| scheduler | PF | PF |
| Receiver | MMSE-IRC | MMSE-IRC |
| DL CSI feedback | 10ms period with delay | 10ms period with delay |
| DL CSI method | FDD: CSI TypeII codebook based TDD: SRS and non-precoded CSI-RS / precoded CSI-RS based channel reciprocity | TDD: SRS and precoded CSI-RS based channel reciprocity |
| NR Type II codebook parameters | 4 beam basis, WB+SB and 8PSK quantification | / |
| HARQ ACK delay | the next available UL slot | the next available UL slot |
| HARQ retransmission delay | the next available DL slot after ACK report | the next available DL slot after ACK report |
| Channel estimation | Non-ideal | Ideal / Non-ideal |
| Interference estimation | Non-ideal | Ideal / Non-ideal |
| UE precoder scheme | Codebook based for FDD; Non-codebook based for TDD | Non-codebook based for TDD |
| UL CSI derivation | SRS based with delay | SRS based with delay |
| UL Waveform | OFDM | OFDM |
| IoT | <10dB | <10dB |
| PAPR model | Non-continuous MPR is supported[9] | / |
| Overhead | Refer to our companion contribution [5]. | Refer to our companion contribution [5]. |