3GPP TSG-RAN WG1 Meeting #94	R1-1808975

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda Item:	7.2.1.1

Source:	Ericsson

Title:	TX schemes for NoMA

Document for:	Discussion

# 1	Introduction

In this contribution, we discuss aspects of NOMA transmission scheme design, including signature design for WSMA, issues related to flexibility of the scheme for varying number of users, and the PAPR/CM to PA efficiency in NOMA design.

# 2	Discussion

In NOMA, each UE spreads its QAM information symbols using an N-length spreading sequence {sk}. Let K denote the number of simultaneously active UEs. For a single antenna BS, the received signal vector y∈CN, where N is the number of REs spanned by the signature vectors and carry the same QAM information symbols, at the gNB can be written as

y=k=1Ksk⊙hkbk+w,

where hk is the channel vector between UEk and the gNB, bk is the QAM symbol of UEk and the operator ⊙ stands for the pointwise multiplication/product of two vectors. For multiple RX antennas, we can form the received signal corresponding to a single QAM symbol per UE, simply by concatenating the N-length received vector y from each RX antenna. From a system performance point-of-view, it is optimal to jointly choose the transmit strategies for all UEs and then employ a joint MUD detector. In some NoMA schemes, the QAM symbols are spread using sequences that are designed to have certain desired correlation properties. The differences between various schemes lie in how the sequences {sk} are constructed.

In Welch bound equality (WBE) based spreading, the design metric, for the signature vectors is the total squared cross-correlation Tc≜i,j|siHsj|2. The lower bound on the total squared cross-correlation of any set of K vectors of length N, is K2/N≤Tc. The WBE sequences are designed to meet the bound on the total squared cross-correlations of the vector set with equality BWelch≜K2/N. We call such sequences Welch bound equality spread multiple access (WSMA).

Figure 1 shows a simplified block diagram of WSMA transmitter. As it can be seen the spreading operation is done on the QAM modulated symbols.

Channel  encoder

Modulator

Spreading

Resource mapping

IFFT

info

bits

Figure 1: Simplified block diagram of WSMA transmitter.

It has been shown that WSMA sequences are capacity-optimal ‎[1], hence they are excellent candidates for achieving high spectral efficiency (eMBB) and high UE capacity (mMTC).

There are known methods to iteratively construct such codes for arbitrary N, K. Further details can be found in [2], where Equiangular subset of WBE sequences and an extension to the WBE sequences in multidimension were also discussed. In this contribution, PAPR and overloading factor of NoMA are studied.

## 2.1	PAPR of NoMA

Our understanding of the intention of the agreement to use PAPR/CM is that PA efficiency should be considered in the NOMA study. PAPR/CM is not a sufficient metric in of itself to determine the amount of PA backoff needed. For example, PA backoff depends on the number of PRBs used in the transmission [3].

Also, the net benefit of low PAPR/CM waveforms should be determined. This is a function of the conditions the UE experiences over the entire cell, rather than for example at the cell edge. Furthermore, low PAPR/CM waveforms can have worse performance by making equalization more difficult, and so some of the gains from lower CM may be offset by having to power the UE up more.

Therefore, metrics such as PAPR/CM should not be considered in isolation, but take into account the required transmit power and occupied bandwidth for the transmission.

Observation:

PA backoff does not depend only on PAPR/CM, but also on the number of allocated PRBs and on the required transmit power for the transmission scheme.

Proposal:

PAPR/CM is not considered in isolation, but takes into account the required transmit power and the occupied bandwidth for the transmission.

## 2.2	Overloading factor of NOMA

WSMA based NOMA offers an additional degree of freedom (DoF) in the form of overloading the system, i.e., the number of users that can simultaneously be scheduled over the same occupied resources. This is achieved by allocating low correlation spreading sequences (or vectors) to the active users and is quantified by a performance indicator  overloading factor given as K/N. For a given combination of spread length (N), #PRBs, TBS, MCS, #Rx antennas and SNR-range within the system, analysing the effect of overloading helps in identifying its limitations.

In this section, we perform some initial studies on behavior as a function of overloading. The overloading characteristics are shown separately with respect to the average BLER per user and sum-throughput metrics. Table-I provides the assumptions made for the LLS evaluation.

| Channel model | TDL-C, 700MHz carrier frequency, 300ns RMS delay spread |
| --- | --- |
| Antennas | 1 Tx, 2 or 4 Rx |
| Modulation | QPSK |
| Channel coding/decoding | Rate matched LDPC encoder, Layered normalized min-sum 25 iterations |
| SNR Variation per UE | Uniform over [-5, 5], [0, 10], or [5, 15]. |
| Channel Estimation | Ideal |
| Total TBS | Given #bytes + 2 bytes CRC |
| Transmission Bandwidth | 10MHz |
| #OFDM symbols | CP-OFDM with 12 data OFDM symbols + 2 DMRS symbols |
| Subcarrier spacing | 15KHz |
| #PRBs | 6 PRBs with 12 subcarriers per PRB |
| Receiver Structure | joint space-frequency symbol level MMSE-SIC |
| WSMA spread Length N | 4 |
| #slots | 10,000 |

Table-I. Assumptions for the Link Level Simulations in Figures 1-6

### 2.2.1 Interference limited nature of NOMA

Figures 1 and 3 show the BLER performance vs the %overloading, with two different TBS (10 and 30 bytes, 2bytes CRC in addition for each) and three different SNR [dB] operating regions ([-5,5], [0,10], [5,15]). The SNR value for every user at each overloading is chosen from the uniformly distributed SNR operating interval. Figures 2 and 4 show the corresponding sum-throughput [Mbps] vs the %overloading. Let the target BLER be assumed at 10%.

Figure 1. Average BLER per user vs % overloading. 2Rx.              Figure 2. Average Sum Throughput [Mbps] vs %overloading. 2Rx

For the smaller TBS (10 bytes + 2 bytes CRC), the sum-throughput gradually increases from 100% (K=4) overloading up to 250% (K=10 ) overloading and then saturates for overloading beyond 300% (K=12) in all the considered SNR operating regions. The corresponding BLER is below 10% for overloading <=250% in the operating regions [0,10]dB and [5,15]dB, while for [-5,5]dB it is for <=200%. For a relatively higher TBS (30 bytes + 2 bytes CRC), the operating region [-5,5]dB is not suitable for any overloading since BLER>10%. In the [5,15]dB SNR region, the overloading when compared to the 10bytes case is reduced from 250% to 200%. In the [0,10]dB SNR region this further reduces to 150% overloading (K=6). The corresponding sum-throughput plot shows an interesting result. With an increase in overloading, the system becomes interference-limited, i.e., Multi-User Interference (MUI) for each user’s received signal increases (or dominates), resulting in a drop in the overall throughput. For the 30bytes case, the sum-throughput drops for overloading >200% and falls below the 10bytes case at higher overloading.  This clearly shows that there is a limitation on the tolerable overloading.

Figure 3. Average BLER per user vs % overloading. 4Rx.             Figure 4. Average Sum Throughput [Mbps] vs %overloading. 4Rx

Figures 3 and 4 show a similar set of results as Figures 1 and 2, but with the number of Rx antennas increased to 4. Spreading in the frequency domain by a factor N at the Tx will increase the overall receive DoF by the same factor. This will further aid the joint space-frequency combining at the receiver, thereby significantly improving the BLER performance even at a high overloading such as 400% (K=16). For the simulated #slots, only the 400% overloading has a small BLER value in the [5,15]dB interval. The sum-throughput plots do not show an interference-limited nature even at 400% overloading for all the considered SNR ranges and almost increase linearly with the increasing number of users for most of the considered conditions. This gives a possibility to further overload the system. The target BLER of 10% is not met only by the 30byte curves in the [-5,5]dB region beyond 300% overloading . So, an increase in the number of Rx antennas may bring the BLER values to well below the target BLER, while at the same time not compromising on the sum-throughput. The supported overloading factor (or the number of supported users K ) that is expected for NOMA depends on the operating parameters and operating conditions. While deciding the allowed overloading factor, both the BLER and sum-throughput plots together provide a limit on the total number of admissible users that can share the same set of resources.

Observation:

The supported overloading factor (or the number of supported users K) that is expected for NOMA depends on the operating parameters and operating conditions.

  - While deciding the allowed overloading factor (K/L), both the BLER and sum-throughput plots must be analysed to identify the interference-limited nature.

### 2.2.2 Dependence on the total number of available signature vectors

Let S denote the pool of signature vectors at the BS. The cardinality of S, which is the total number of available signature vectors, is |S|. Since the system is expected to support varying number of users (K), there must exist enough number of signature vectors at the BS, i.e., |S|>= K. Further, under grant-based signature allocation, the BS is expected to assign the vectors to the active users in a non-colliding manner. As a result, the set of allocated vectors (K in number) may not meet the required correlation (or orthogonality) property defined by the Welch Bound (WB), i.e., the operating point shifts away from the WB. With decreasing number of non-allocated vectors at the BS ((|S|- K) in number), there is a possibility that the allocated vectors operate close to the WB making the dependency on |S| insignificant. This situation is closely related to the pre-assigned case, where each UE has its own pre-assigned vector (that does not change) and vector assignment is not a design criterion. In other words, it is a configured grant-based transmission and not dynamic grant-based scheduling [4]. To meet the performance requirement, additional receiver processing may be required to overcome the lack of this WB correlation property in the received composite signal. Of course, the target BLER and the interference limited nature of the curves with increased overloading are implicit.

Figures 5 and 6 show BLER vs the number of available signature vectors |S| for TBS a of 30bytes, with 2Rx and 4Rx antennas and two different SNR operating regions. As an example, consider |S|=20 and 100% (K=4) overloading. The 20 signature vectors together satisfy the WB and if any 4 vectors are chosen at random, the WB does not hold. Ideally |S|=4 when K=4, but as |S| increases from 4 to 20, the BLER also increases. Though this is at a small BLER value, it could be an influencing factor for higher N, TBS and MCS. So with increasing (|S|- K) for a low overloading, the BLER performance can be sensitive to the value (|S|- K). This could also move the BLER plots beyond the assumed target BLER of 10%. Further, as (|S|- K) tends to |S|, the impact on the BLER curves is negligible in both the figures, where the operating point approaches the WB.

Figure 5. BLER vs |S|, 2Rx, SNR~[0,10]dB        Figure 6. BLER vs |S|, 4Rx, SNR~[-5,5]dB

Observation:

Under configured grant-based transmission, the BLER performance can be sensitive to the fraction of unused (or unassigned) signature vectors at low overloading.

# Conclusion

In this contribution, we discussed aspects of NOMA transmission scheme design, including signature design for WSMA, issues related to flexibility of the scheme for varying number of users, and the PAPR/CM to PA efficiency in NOMA design.  We made the following observations and proposal:

Observations:

PA backoff does not depend only on PAPR/CM, but also on the number of allocated PRBs and on the required transmit power for the transmission scheme.

The supported overloading factor (or the number of supported users K) that is expected for NOMA depends on the operating parameters and operating conditions.

  - While deciding the allowed overloading factor (K/L), both the BLER and sum-throughput plots must be analysed to identify the interference-limited nature.

Under configured grant-based transmission, the BLER performance can be sensitive to the fraction of unused (or unassigned) signature vectors at low overloading.

Proposal:

PAPR/CM is not considered in isolation, but takes into account the required transmit power and the occupied bandwidth for the transmission.

# References

S. Ulukus and R. Yates, “Iterative construction of optimum signature sequence sets in synchronous CDMA systems,” IEEE Trans. Inf. Theory, vol. 47, no. 5, pp. 1989 - 1998, July 2001.

R1-1806241, “Signature Design for NoMA”, Ericsson, RAN1 #93, Busan, South Korea, 21st – 25th May 2018.

R1-060144, “UE power management for E-UTRA”, Motorola, RAN1 LTE Ad-Hoc, Helsinki, Finland, 23rd – 25th January 2006.

R1-1808977, “Procedures related to NoMA”, Ericsson, RAN1 #94, Gothenburg, Sweden, 20th – 24th August 2018.