3GPP TSG RAN WG1 Meeting #94			R1-1809067

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda Item:	7.2.4.1.2

Source:	AT&T

Title:	V2X sidelink PHY structure and procedures

Document for:	Discussion/Approval

# Introduction

In RAN#80, V2X SI was approved, [1]. In this paper, we give our view on the sidelink design for NR.

# Design principle for NR sidelink

When looking at sidelink and acceslink, a fundamental difference is the structure of communication: accesslink is designed to support hierarchical structure where base-station is the centre of wireless communication while sidelink needs to support more flexible structure such as peer-2-peer structure. Meanwhile, advanced V2X use case also require sidelink to support hierarchical structure, e.g. vehicle platooning is a typical advanced use case where a platoon head is needed to manage a group of vehicles, essentially, it requires sidelink to support hierarchical structure based communications.  Naturally, NR sidelink should be able to support both hierarchical and peer-2-peer communication.

NR access link has been designed to support great flexibility through rich configurability. Many of the design can be reused in NR sidelink design especially for hierarchical structure based communication. Also reuse the access link can minimize the standardization effort as well maximize the possibility of shared hardware between access-link and sidelink. Therefore we have the following two proposals:

Proposal-1: NR sidelink should target to work on both peer-2-peer and hierarchical network structure.

Proposal-2: NR sidelink design should be based on access link and target to have common PHY procedures as much as possible.

Another important question for V2X service is on which band it operates on. In multiple countries, Intelligent Transportation System (ITS) band has been allocated to vehicle communication which is a possible spectrum for C-V2X. Another possibility is on the spectrum licensed for cellular service, which includes both FR1 and FR2. Of course, it’s rather expensive to have a dedicated band for C-V2X service, the best way is to share band with cellular service.

Proposal-3: NR sidelink must be able to operate on wireless licensed spectrum and co-exist with wireless service.

# Three-Party based communication framework

Typical communication is between two nodes, for example, cellular communication is between base-station and UE(s). And it has been further categorized into downlink and uplink based on the direction of data packet flow. No matter which direction it is, base-station serves two roles: as a data packet transmitter/receiver and as a scheduler which decides the radio resource to be used for every data packet, see below figure.

Figure.1 Two-Parties communication diagram

In order to support more flexible communication structure, it’s better to have a separate node dedicated as scheduler node, see figure.2. The function of Node-S is to coordinate the resource usage within a local area. Thus we also call Node-S as local manager. Similar to two parties’ communication, local manager sends DCI to Node-T and Node-R to convey the scheduling decision. Three parties communication is much flexible than traditional two parties counterpart mainly because it doesn’t need formal association between the Node-T/Node-R with Node-S. In C-V2X, a vehicle UE can serve as local manager to surrounding vehicles. That can effectively reduce transmission collision.

With the present of local manager, sidelink communication is in hierarchical structure. In another words, vehicle UEs need to associate with local manager somehow. However, there is no need to have full association just like UE-BS in cellular network since local manager essentially is another UE. It simply helps to coordinate the resource utilization in a certain area.  In our opinion, UE can have “diet” connection to local manager, which means UE synchronizes to local manager and blindly decode control channel following the search space configured for it. Also, when UE switch to a different local manager, no need to have normal handover procedure, UE simply just send an SR to another local manager to ask for new resources.

Figure.2 Three Parties communication diagram

Proposal-4: NR sidelink should support vehicle UE serves as a local manager to coordinate resource for the communication between other UEs within a local area. Local manager is granted with a resource pool from the cellular network.

# Local manager selection

In cellular coverage: cellular network can appoint a UE as local manager based on its capability report. Then cellular network will grant a resource pool for the local manager to use.

Out of cellular coverage: An election process should be conducted between UEs. Each UE announces its candidacy to be a local manager. Then based on the received signal quality or other factors, every UE can cast its vote. Then the local manager can be elected. Elected local manager can only use the pre-configured resource pool.

# PHY procedures for local manager

As per definition, local manager is a pure scheduler to coordinate the resource in a certain local area. That makes it different from a regular base-station. As a result, UE doesn’t need to fully associate to a local manager, instead, all the C-plane signaling can be done through cellular network. In this section, we show several key procedures that local manager needed.

Synchronization and scheduling request procedure: Local manager needs to broadcast synchronization and system information periodically for other vehicle to discover the existence of the local manager. When detected, other UEs can send scheduling request to the local manager to ask for resource. Note, here SR may only include one bit, the full BSR, including the QoS, buffer status, traffic type etc. ,can be relayed by cellular network to local manager. Once UE sends scheduling request to a local manager, it shall monitor the control channel configured for that local manager. It’s possible that one UE “connects” to multiple local manager.

Discovery: In order to let other UEs knows the existence of Node-T, Local manager may trigger Node-T to transmit A-DRS to be received by surrounding UEs. Those UEs need to report RSRP back to local manager, thus local manager can have a list of UEs which can be reached by the Node-T.

CSI acquisition: in order to have close-loop MIMO transmission and better link adaption, CSI acquisition procedure can obtain the CSI for the local manager. The procedure is similar to the CSI acquisition in cellular system, local manager can send a DCI to trigger Node-T to transmit A-CSI-RS, and meanwhile, local manager sends a DCI to trigger Node-R to measure and report the CSI back to local manager.

Data transmission: Local manager should run the scheduling procedure then determine which UE can transmit on which radio resource. Then it should covey the scheduling decision by a DCI-T to the node-T and possibly a DCI-R to Node-R. Possibly the DCI-T and DCI-R can be jointly encoded and received by Node-T and Node-R. Upon reception the DCI-T, node-T should transmit data packet on PSSCH and possibly feedback an ACK on the DCI-T back to Node-T. And Node-R should receive and decode the data packet and feedback the ACK/NAK back to Local manager for further HARQ process.

# Conclusion

In this contribution, we proposed the phy structure and procedures for NR sidelink. Details can be found in section-5.

Proposal-1: NR sidelink should target to work on both peer-2-peer and hierarchical network structure.

Proposal-2: NR sidelink design should be based on access link and target to have common PHY procedures as much as possible.

Proposal-3: NR sidelink must be able to operate on wireless licensed spectrum and co-exist with wireless service.

Proposal-4: NR sidelink should support vehicle UE serves as a local manager to coordinate resource for the communication between other UEs within a local area. Local manager is granted with a resource pool from the cellular network.

# Reference

[1]  	RP-181480   	New SID: Study on NR V2X

[2]	R1-1806986	Autonomous topology management, AT&T