3GPP TSG RAN WG1 Meeting #94   	  R1-1809291

Gothenburg, Sweden, August 20th – 24th, 2018

Source:	Xiaomi

Title: 	Discussion on the PDCCH repetition for NR URLLC

Agenda Item:	7.2.6.1

Document for:	Discussion and decision

Introduction

In the 3GPP RAN #80 meeting, a study item about enhancement for NR URLLC was agreed. Following items need to be taken into account,

URLLC L1 improvements (RAN1) for further improved reliability/latency and for other requirements related to the use cases identified,

- PDCCH enhancements. Study focus on Compact DCI, PDCCH repetition, increased PDCCH monitoring capability

- UCI enhancements. Study focus on Enhanced HARQ feedback methods (increased number of HARQ transmission possibilities within a slot), CSI feedback enhancements

- PUSCH Enhancements. Study focus on mini-slot level hopping & retransmission/repetition enhancements.

- Enhancements to scheduling/HARQ/CSI processing timeline (UE and gNB), (for existing TTI durations)

Enhanced multiplexing considering different latency and reliability requirements (RAN1):

UL inter UE Tx prioritization/multiplexing

Enhanced UL configured grant (grant free) transmissions, with study focusing on improved configured grant operation, example methods such as explicit HARQ-ACK, ensuring K repetitions and mini-slot repetitions within a slot. (RAN1/RAN2)

The following items have been identified to have relationship URLLC but are covered in other study items:

- Multi-TRP transmission

- Mobility improvements for higher reliability

- Beam Management

In this contribution, we present the evaluation results and analyze the design for the PDCCH repetition.

PDCCH repetition

For release 16 URLLC, higher reliability within o.5ms or1ms is up to 1E-6 level. In order to achieve the target BLER, the PDCCH repetition is proposed. Basically, there are several options to perform PDCCH repetition as follows:

- Option 1a: the PDCCH repetition is performed in the frequency domain within the same  CORESET

For this option, the PDCCH repetition is quite similar to increase the aggregation level. It is beneficial from the decoding latency perspective. However, the PDCCH blocking probability is increased.

- Option 1b: the PDCCH repetition is performed in the frequency domain with different  CORESETs

This option is similar with option 1a with additional diversity gain achieved by different CORESET configurations.

Figure 1: Illustration of option 1

- Option 2a: the PDCCH repetition is performed in the time domain within the same CORESET

For this option, the UE can be configured with multiple monitoring occasions to receive the repeated PDCCH transmission, or a fast feedback mechanism can be introduced to reduce the PDCCH overhead. The PDCCH blocking problem can be alleviated with the cost of the increased decoding latency.

- Option 2b: the PDCCH repetition is performed in the time domain with different CORESETs

This option is similar with option 2a with additional diversity gain achieved by different CORESET configurations.

Figure 2: Illustration of option 2

Proposal 1: The PDCCH repetition have impact on the decoding latency and/or PDCCH blocking performance.

Performance evaluation

In this section, we present the evaluation results for both frequency and time domain PDCCH repetition. The following two cases are simulated:

Case 1: PDCCH repetition with AL=8 for a two-symbols CORESET

- Reference: PDCCH transmission with AL=16 without repetition

- Case 1-1: PDCCH repetition with AL=8 in frequency domain

- Case 1-2: PDCCH repetition with AL=8 in time domain

Case 2: PDCCH repetition with AL=4 for a one-symbol CORESET

- Reference: PDCCH transmission with AL=8 without repetition

- Case 2-1: PDCCH repetition with AL=4 in frequency domain

- Case 2-2: PDCCH repetition with AL=4 in time domain

The performance of the PDCCH repetition is illustrated in Figure 3, detailed simulation setting can be found in the appendix:

Figure 3: PDCCH repetition performance

Observation:

- Frequency domain PDCCH repetition with AL=4 or 8 have a similar performance with a PDCCH transmission with AL=8 or 16.

- Time domain PDCCH repetition have slight performance loss

- PDCCH repetition does not provide significant performance gain

Proposal 2: PDCCH repetition does not provide significant performance gain from the PDCCH reliability perspective.

Conclusions

In this contribution, we present the discussion on the PDCCH repetition design and the initial evaluation results. Based on our analysis, we have the following proposals:

Proposal 1: The PDCCH repetition have impact on the decoding latency and/or PDCCH blocking performance.

Proposal 2: PDCCH repetition does not provide significant performance gain from the PDCCH reliability perspective.

References

- RP 181477 New SID on Physical Layer Enhancements for NR URLLC

- 3GPP RAN1 92 chairman’s notes

- Appendix

Table 1: Link level simulation assumptions[2]

| Parameters | Value |
| --- | --- |
| DCI payload (excluding 24bits CRC) | 40bits |
| System bandwidth | 20MHz |
| Carrier Frequency | 4GHz, |
| Number of symbols for CORESET | 1(for AL8), 2(for AL16) |
| CORESET BW (contiguous PRB allocation) | 20MHz |
| Subcarrier spacing | 30KHz |
| Aggregation level | 8, 16. |
| Transmission type | Interleaved |
| REG bundling size | 6 |
| Modulation | QPSK |
| Channel coding | Polar code (DCI) |
| Transmission scheme | 1-port precoder cycling |
| Channel estimation | Realistic, LMMSE |
| Detection method | MMSE |
| Channel model | TDL-A (delay spread: 30ns) |
| UE speed | 3 km/h |
| Number of BS antennas | 2Tx |
| Number of UE antennas | 4Rx |