3GPP TSG RAN WG1 Meeting #94	R1-1809352

Gothenburg, Sweden, August 20th- 24th, 2018

Agenda Item:	6.1.2.3

Source:	Huawei, HiSilicon

Title:	Corrections on HARQ-ACK feedback for DL SPS

Document for:	Discussion and decision

Introduction

In this contribution, we clarify the procedure of HARQ-ACK feedback for DL SPS.

Discussion

For HARQ-ACK feedback, we have agreed that DL SPS should be supported for sPDSCH transmission. However, in current spec, there are some ambiguities in the timing and PUCCH resource of HARQ-ACK feedback. To make them clear, the following is proposed:

Proposal 1: Endorse the following text proposal:

------------------------------------------------Start of Text Proposal to 36.213-----------------------------------------

10.1.2.1	FDD HARQ-ACK procedure for one configured serving cell

<Unchanged parts are omitted>

For slot-PUCCH, and for transmission of up to 2 HARQ-ACK bits in slot , when at least one HARQ-ACK bit is sent in response to a PDSCH transmission indicated by the detection of a corresponding PDCCH/SPDCCH with DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G or a semi-persistently scheduled slot-PDSCH transmission or for a PDCCH/SPDCCH indicating downlink SPS release in

- slot  or

- when scheduling request is sent in slot on a resource configured by higher layer parameter sr-slotSPUCCH-IndexFH or sr-slotSPUCCH-IndexNoFH,

the UE shall use PUCCH resource  for transmission of HARQ-ACK in slot  for  mapped to antenna port p for slot-PUCCH transmission using format 1a/1b [3], where the value of  is determined according to higher layer configuration and Table 10.1.2.1-2. For a UE configured for two antenna port transmission for PUCCH format 1a/1b, a PUCCH resource value in Table 10.1.2.1-2 maps to two PUCCH resources with the first PUCCH resource  for antenna port  and the second PUCCH resource  for antenna port , otherwise, the PUCCH resource value maps to a single PUCCH resource  for antenna port .

Table 10.1.2.1-2: PUCCH Resource Value for HARQ-ACK Resource for slot-PUCCH

| Value of 'SPUCCH resource indication' field as defined in section 5.3.3.1.17 of [4] | or |
| --- | --- |
| '00' | The 1st PUCCH resource value configured by the higher layers |
| '01' | The 2nd PUCCH resource value configured by the higher layers |
| '10' | The 3rd PUCCH resource value configured by the higher layers |
| '11' | The 4th PUCCH resource value configured by the higher layers |
| NOTE: are respectively determined from the first and second PUCCH resource lists configured by n1SlotSPUCCH-hoppingAN-List in [11] for slot-PUCCH when hopping is enabled, and by n1SlotSPUCCH-nohoppingAN-List in [11] for slot-PUCCH when hopping is disabled. |  |

For subslot-PUCCH, and for transmission of up to 2 HARQ-ACK bits, in subslot , when at least one HARQ-ACK bit is sent in response to a PDSCH transmission indicated by the detection of a corresponding PDCCH/SPDCCH with DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G in subslot  or a semi-persistently scheduled subslot-PDSCH transmission in subslot or for a PDCCH/SPDCCH indicating downlink SPS release in subslot  or when scheduling request is sent in slot on a resource configured by higher layer parameter sr-subslotSPUCCH-Resource, the UE shall use PUCCH resource  for transmission of HARQ-ACK in subslot  for  mapped to antenna port p for subslot-PUCCH transmission using format 1a/1b [3], where

-	for a PDSCH transmission indicated by the detection of a corresponding PDCCH/SPDCCH with DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G in subslot or for a PDCCH/SPDCCH indicating downlink SPS release in subslot , PUCCH resource group is determined by higher layer configuration and Table 10.1.2.1-3, and  is determined according to higher layer configuration and Table 10.1.2.1-4 for PUCCH format 1a, and according to higher layer configuration and Table 10.1.2.1-5 for PUCCH format 1b.

<Unchanged parts are omitted>

10.1.2.2.2	PUCCH format 3 HARQ-ACK procedure

<Unchanged parts are omitted>

-	for transmission of up to 2 HARQ-ACK bits in slot , when at least one HARQ-ACK bit is sent in response to a PDSCH transmission indicated by the detection of a corresponding PDCCH/SPDCCH with DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G or a semi-persistently scheduled slot-PDSCH transmission or  for a PDCCH/SPDCCH indicating downlink SPS release in

-	slot  or

-	in a subslot that is  subslots before slot or

-	when scheduling request is sent in slot on a resource configured by higher layer parameter sr-slotSPUCCH-IndexFH or sr-slotSPUCCH-IndexNoFH,

<Unchanged parts are omitted>

-	for transmission of 3-11 HARQ-ACK bits on slot-PUCCH when at least one HARQ-ACK bit is sent in response to a PDSCH transmission indicated by the detection of a corresponding PDCCH/SPDCCH with DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G or a semi-persistently scheduled subslot-PDSCH transmission or for a PDCCH/SPDCCH indicating downlink SPS release in

-	slot  or

-	any of the subslots given in Table 10.1-1 according to the value of  when the slot-PUCCH is transmitted in subframe m or

-	when scheduling request is sent in slot on a resource configured by higher layer parameter sr-slotSPUCCH-IndexFH or sr-slotSPUCCH-IndexNoFH,

the UE shall use PUCCH format 3, if configured by higher layers, and PUCCH resource is determined according to higher layer configuration n3SPUCCH-AN-List and Table 10.1.2.2.2-1.

<Unchanged parts are omitted>

10.1.2.2.3	PUCCH format 4 HARQ-ACK procedure

<Unchanged parts are omitted>

-	for transmission of up to 2 HARQ-ACK bits,

-	in slot for slot-PUCCH when at least one HARQ-ACK bit is sent in response to a PDSCH transmission indicated by the detection of a corresponding PDCCH/SPDCCH with DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G or a semi-persistently scheduled slot-PDSCH transmission or for a PDCCH/SPDCCH indicating downlink SPS release in

-	slot  or

-	any of the subslots given in Table 10.1-1 according to the value of  when the slot-PUCCH is transmitted in subframe m

-	when scheduling request is sent in slot on a resource configured by higher layer parameter sr-slotSPUCCH-IndexFH or sr-slotSPUCCH-IndexNoFH,

the UE shall use PUCCH format 1a/1b and PUCCH resource  where the value of  is determined according to higher layer configuration and Table 10.1.2.1-2. For a UE configured for two antenna port transmission for PUCCH format 1a/1b, a PUCCH resource value in Table 10.1.2.1-2 maps to two PUCCH resources with the first PUCCH resource  for antenna port  and the second PUCCH resource  for antenna port , otherwise, the PUCCH resource value maps to a single PUCCH resource  for antenna port .

-	in subslot for subslot-PUCCH when at least one HARQ-ACK bit is sent in response to a PDSCH transmission indicated by the detection of a corresponding PDCCH/SPDCCH with DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G in subslot  or a semi-persistently scheduled subslot-PDSCH transmission in subslot  or for a PDCCH/SPDCCH indicating downlink SPS release in subslot  or when scheduling request is sent in slot on a resource configured by higher layer parameter sr-subslotSPUCCH-Resource, the UE shall use PUCCH resource  for transmission of HARQ-ACK in subslot  for  mapped to antenna port p for subslot-PUCCH transmission using format 1a/1b [3], where PUCCH resource group is determined by higher layer configuration and Table 10.1.2.1-3, and  is determined according to higher layer configuration and Table 10.1.2.1-4 for PUCCH format 1a, and according to higher layer configuration and Table 10.1.2.1-5 for PUCCH format 1b.

<Unchanged parts are omitted>

for slot-PUCCH and when the UE is not configured with PUCCH format 3, and at least one HARQ-ACK bit is sent in response to a PDSCH transmission indicated by the detection of a corresponding PDCCH/SPDCCH with DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G or a semi-persistently scheduled slot-PDSCH transmission or for a PDCCH/SPDCCH indicating downlink SPS release in

-	slot  or

-	any of the subslots given in Table 10.1-1 according to the value of  when the slot-PUCCH is transmitted in subframe m or

-	when scheduling request is sent in slot on a resource configured by higher layer parameter sr-slotSPUCCH-IndexFH or sr-slotSPUCCH-IndexNoFH,

-	for subslot-PUCCH and when at least one HARQ-ACK bit is sent in response to a PDSCH transmission indicated by the detection of a corresponding PDCCH/SPDCCH with DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G in subslot  or a semi-persistently scheduled subslot-PDSCH transmission in subslot  or for a PDCCH/SPDCCH indicating downlink SPS release in subslot  or when scheduling request is sent in slot on a resource configured by higher layer parameter sr-subslotSPUCCH-Resource

-	The UE is not expected to transmit PUCCH corresponding to 21 or 22 HARQ-ACK bits in a subslot if higher layer parameter n4numberOfPRBSubslot indicates single resource block for PUCCH transmission.

-	for slot-PUCCH and for transmission of 12 or more HARQ-ACK bits, the UE shall use PUCCH format 4, and PUCCH resourceis determined according to higher layer configuration and Table 10.1.2.2.3-1, when the UE is configured with PUCCH format 3, and at least one HARQ-ACK bit is sent in response to a PDSCH transmission indicated by the detection of a corresponding PDCCH/SPDCCH with DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G or a semi-persistently scheduled slot-PDSCH transmission or for a PDCCH/SPDCCH indicating downlink SPS release in

-	slot  or

-	any of the subslots given in Table 10.1-1 according to the value of  when the slot-PUCCH is transmitted in subframe m or

-	when scheduling request is sent in slot on a resource configured by higher layer parameter sr-slotSPUCCH-IndexFH or sr-slotSPUCCH-IndexNoFH.

------------------------------------------------End of Text Proposal to 36.213-----------------------------------------