3GPP TSG RAN WG1 Meeting #94			                                                                        R1-1809461

August 20th – 24th, 2018

Gothenburg, Sweden

Agenda item:	7.2.7.2

Source: 	Qualcomm Incorporated

Title: 	IMT-2020 eMBB spectral efficiency evaluations

Document for:	Discussion/Decision

# Introduction

This contribution presents system-level simulation results on spectral efficiency for eMBB for downlink and uplink for the NR self-evaluation study for IMT-2020. The evaluation has been performed in accordance with the evaluation guidelines from [2].

# Downlink TDD FR1 Results

Table 1 provides the simulation assumptions used to evaluate the performance of downlink for FR1.

Table 1: Simulation Assumptions for Downlink TDD FR1

| Parameter | Dense Urban (4 GHz) | Rural (4 GHz) | Rural (700 MHz) |
| --- | --- | --- | --- |
| BS antenna configuration | 256 elements (M, N, P) = (8, 16, 2) dH = 0.5 λ, dV = 0.8 λ 128 TXRU (2 V elems combined) | 256 elements (M, N, P) = (4, 32, 2) dH = 0.5 λ, dV = 0.8 λ 128 TXRU (2 V elems combined) | 64 elements (M, N, P) = (4, 8, 2) dH = 0.5 λ, dV = 0.8 λ 16 TXRU (4 V elems combined) |
| UE antenna configuration | 4 elements (M,N,P) = (2,1,2) dH = 0.5 λ |  |  |
| Precoder at gNB | SRS-based with non-ideal channel estimation at gNB |  |  |
| Channel Estimation | Non-ideal estimation for both channel and interference for both CSI and Data |  |  |
| TDD Config | DDSU: D = 2 PDCCH, 12 PDSCH S = 2 PDCCH, 8 PDSCH, 2 Gap, 1 SRS, 1 PUCCH U = 12 PUSCH, 1 SRS, 1 PUCCH |  |  |
| Processing delay | UE Decoding Delay: 1 slot gNB Processing Delay: 3 slots |  |  |
| Scheduling | MU-MIMO with Proportional Fair Scheduling |  |  |
| UE SRS Tx Power | 23 dBm max over system BW |  |  |
| Traffic | Full Buffer w/10 UEs/cell |  |  |
| Bandwidth | System BW = 100 MHz, Simulated BW = 20 MHz (54 RBs at 30 kHz SCS) |  |  |

The results are shown in the table below. We observed that the requirements specified in the ITU Report [1] for downlink can be achieved by NR TDD (FR1).

Table 2: Downlink TDD FR1 Spectral Efficiency Results

| Scenario | Average Cell SE (b/s/Hz/TRxP) |  | 5th %ile UE SE (b/s/Hz) |  |
| --- | --- | --- | --- | --- |
|  | Simulation | Requirement | Simulation | Requirement |
| Dense Urban (4 GHz) | 11.25 | 7.8 | 0.378 | 0.225 |
| Rural (4 GHz) | 10.82 | 3.3 | 0.135 | 0.12 |
| Rural (700 MHz) | 6.90 | 3.3 | 0.137 | 0.12 |

Observation 1: NR can meet the IMT average cell spectral efficiency and 5th percentile UE spectral efficiency requirements for downlink in the dense-urban and rural scenarios.

# Uplink TDD FR1 Results

Table 3 provides the simulation assumptions used to evaluate the performance of uplink for FR1. The evaluation was performed for 3 scenarios:

- Dense urban layout over 4 GHz

- Rural layout over 4 GHz

- Rural layout over 700 MHz

Table 3: Simulation Assumptions for Uplink TDD FR1

| Parameter | Dense Urban (4 GHz) | Rural (4 GHz) | Rural (700 MHz) |
| --- | --- | --- | --- |
| BS antenna configuration | 256 elements (M, N, P) = (8, 16, 2) dH = 0.5 λ, dV = 0.8 λ 128 TXRU (2 V elems combined) | 256 elements (M, N, P) = (4, 32, 2) dH = 0.5 λ, dV = 0.8 λ 128 TXRU (2 V elems combined) | 64 elements (M, N, P) = (4, 8, 2) dH = 0.5 λ, dV = 0.8 λ 16 TXRU (4 V elems combined) |
| UE antenna configuration | 4 TX (M, N, P) = (2, 1, 2) co-pol dH = 0.5 λ | 2 TX (M, N, P) = (1, 1, 2) co-pol |  |
| Precoder | NR 4 Tx codebook (fully-coherent) Selection based on SRS | NR 2 Tx codebook (fully-coherent) Selection based on SRS |  |
| Power control | P0 = 8 dB over thermal α = 0.9 | P0 = 8 dB over thermal α = 0.9 | P0 = 17 dB over thermal α = 0.9 |
| Channel estimation | Non-ideal estimation for both channel and interference |  |  |
| TDD Config | DDSU D = 2 PDCCH, 12 PDSCH S = 2 PDCCH, 8 PDSCH, 2 Gap, 1 SRS, 1 PUCCH U = 12 PUSCH, 1 SRS, 1 PUCCH |  |  |
| Processing delay | UE grant processing delay: 1 slot; gNB processing delay: 3 slots |  |  |
| Scheduling | MU-MIMO P-Fair with subband scheduling |  |  |
| Bandwidth | System BW = 100 MHz, Simulated BW = 20 MHz (54 RBs at 30 KHz SCS) |  |  |
| UE Tx Power | 23 dBm max over system BW |  |  |
| Traffic | Full buffer, 10 UEs / cell |  |  |

The power control operating points were chosen such that the average effective interference-over-thermal was within the 10 dB limit specified in the IMT evaluation guidelines [2]. The spectral efficiency results are shown in the table below. We observe that the requirements specified in the ITU Report [1] for uplink can be achieved by NR TDD (FR1).

Table 4: Uplink TDD FR1 Spectral Efficiency Results

| Scenario | Average Cell SE (b/s/Hz/TRxP) |  | 5th %ile UE SE (b/s/Hz) |  |
| --- | --- | --- | --- | --- |
|  | Simulation | Requirement | Simulation | Requirement |
| Dense Urban (4 GHz) | 6.097 | 5.4 | 0.193 | 0.15 |
| Rural (4 GHz) | 10.18 | 1.6 | 0.122 | 0.045 |
| Rural (700 MHz) | 4.635 | 1.6 | 0.247 | 0.045 |

Observation 2: NR can meet the IMT average cell spectral efficiency and 5th percentile UE spectral efficiency requirements for uplink in the dense-urban and rural scenarios for TDD FR1 scenario.

# Downlink TDD FR2 Results

Table 5 provides the simulation assumptions used to evaluate the performance of downlink for FR2.

Table 5: Simulation Assumptions for Downlink TDD FR2

| Parameter | Dense Urban (30 GHz) | Indoor HotSpot (30 GHz) |
| --- | --- | --- |
| BS antenna configuration | 256 elements per pol (M, N, P) = (8, 32, 2) dH = 0.5 λ, dV = 0.8 λ | 128 elements per pol (M, N, P) = (8, 16, 2) dH = 0.5 λ, dV = 0.8 λ |
| UE antenna configuration | 16 elements per pol (M,N,P) = (4,4,2) dH = 0.5 λ |  |
| Precoder at gNB | CSI-RS based with non-ideal channel estimation at gNB |  |
| Channel Estimation | Non-ideal estimation for both channel and interference for both CSI and Data |  |
| TDD Config | DDDU: D = 1 PDCCH, 13 DMRS+PDSCH+CSI-RS 2 symbol gap for transition from D to U U = 12 PUSCH+DMRS, 1 SRS, 1 PUCCH |  |
| Processing delay | UE Decoding Delay: 1 slot gNB Processing Delay: 1 slot |  |
| Scheduling | SU-MIMO / MU-MIMO with Proportional Fair Scheduling |  |
| Traffic | Full Buffer w/10 UEs/cell |  |
| Bandwidth | System BW = 80 MHz at 95% efficiency |  |

The results are to be updated in the table below.

Table 6: Downlink TDD FR2 Spectral Efficiency Results

| Scenario | Average Cell SE (b/s/Hz/TRxP) |  | 5th %ile UE SE (b/s/Hz) |  |
| --- | --- | --- | --- | --- |
|  | Simulation | Requirement | Simulation | Requirement |
| Dense Urban (30 GHz) |  | To be updated |  |  |
| Indoor HotSpot (30 GHz) |  |  |  |  |

Observation: TBD.

# Uplink TDD FR2 Results

Table 7 provides the simulation assumptions used to evaluate the performance of uplink for FR2. The evaluation was performed for the following scenarios:

Table 7: Simulation Assumptions for Uplink TDD FR2

| Parameter | Dense Urban (30 GHz) | Indoor HotSpot (30 GHz) |
| --- | --- | --- |
| BS antenna configuration | 256 elements per pol (M, N, P) = (8, 32, 2) dH = 0.5 λ, dV = 0.8 λ | 128 elements per pol (M, N, P) = (8, 16, 2) dH = 0.5 λ, dV = 0.8 λ |
| UE antenna configuration | 16 elements per pol (M,N,P) = (4,4,2) dH = 0.5 λ |  |
| Precoder at gNB | SRS based with non-ideal channel estimation at gNB |  |
| Channel Estimation | Non-ideal estimation for both channel and interference for both CSI and Data |  |
| TDD Config | DDDU: D = 1 PDCCH, 13 DMRS+PDSCH+CSI-RS 2 symbol gap for transition from D to U U = 12 PUSCH+DMRS, 1 SRS, 1 PUCCH |  |
| Processing delay | UE Decoding Delay: 1 slot gNB Processing Delay: 1 slot |  |
| Scheduling | SU-MIMO with Proportional Fair Scheduling |  |
| Traffic | Full Buffer w/10 UEs/cell |  |
| Bandwidth | System BW = 80 MHz at 95% efficiency |  |

The results are to be updated in the table below.

Table 8: Uplink TDD FR2 Spectral Efficiency Results

| Scenario | Average Cell SE (b/s/Hz/TRxP) |  | 5th %ile UE SE (b/s/Hz) |  |
| --- | --- | --- | --- | --- |
|  | Simulation | Requirement | Simulation | Requirement |
| Dense Urban (30 GHz) |  | To be updated |  |  |
| Indoor HotSpot (30 GHz) |  |  |  |  |

Observation: TBD.

# Conclusion

Observation 1: NR can meet the IMT average cell spectral efficiency and 5th percentile UE spectral efficiency requirements for both downlink and uplink in the dense-urban and rural scenarios for TDD FR1.

# References

- Report ITU-R M.2410-0, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, 2017.

- Report ITU-R M.2412-0, “Guidelines for evaluation of radio interface technologies for IMT-2020”, 2017.