3GPP TSG RAN WG1 Meeting #95	R1-1812404

Spokane, USA, November 12th – 16th, 2018

Agenda Item:	7.2.2.3.2

Source: 	Fujitsu

Title: 	On UL signals and channels design for NR-U

Document for:	Discussion/Decision

- Introduction

In this contribution, we would like to discuss some issues related to PRACH formats, RO configuration and multiplexing of UL channels for NR-U. The following are some related agreements in previous meetings [1-2].

Agreement:

- Support for Rel-15 NR PRACH formats can be considered. Exclusion of the support of certain formats is to be identified.

  - Note: It is RAN1’s understanding that certain formats do not meet the minimum bandwidth requirement by regulation.

- It is identified that interlaced based PRACH can be beneficial.

- The following aspects can be considered for Interlace waveform based PRACH design for 4-step random access:

  - Interlacing based on PRB or REs

  - Targeted cell sizes

  - Targeted PRACH capacity

  - Targeted false alarm and detection rates

  - Targeted timing estimation accuracy

  - Number of formats

  - Multiplexing with other channels such as block interlaced PUCCH and PUSCH

Agreement:

- For scenarios in which a block-interlaced waveform is used for PUCCH/PUSCH, it has been identified that from FDM-based user-multiplexing standpoint it can be beneficial to have UL channels on a common interlace structure, at least for PUSCH, PUCCH, associated DMRS, and potentially PRACH

  - Note: This is only from a user-multiplexing perspective. Other aspects of PRACH design need to be considered, i.e., timing estimation accuracy, miss detection rate, PAPR, RACH capacity, transmission power

- For scenarios in which a contiguous allocation for PUSCH and PUCCH is used, it is beneficial to use contiguous resource allocation for PRACH

- FFS: Potential LBT blocking due to TA difference between FDM’d PUSCH, PUCCH, and PRACH.

- Discussion

PRACH format

According to agreements in RAN1#93 meeting, support for Rel-15 NR PRACH formats can be considered and exclusion of the support of certain formats is to be identified. NR Rel-15 supports both long preamble formats with sequence length L=839 and short preamble formats with sequence length L=139. The long preamble formats are designed for the use cases e.g. LTE refarming, large cell and high speed etc. And short preamble formats are for use cases e.g. small cell and normal cell. Some short preamble formats (e.g. A2/A3/B4/C0/C2) can support a maximum cell radius more than 2 km with 15 kHz SCS. So support short preamble formats only would be sufficient for NR-U scenario considering the limited cell range. And down selection could be further made e.g. considering PRACH SCS.

Proposal 1: NR-U only supports preamble formats with sequence length L=139.

RO conguration

For short preamble formats, time-domain RACH occasions (ROs) are configured continuously within a PRACH slot in NR Rel-15. One example is as illustrated in Figure 1. In addition, as shown in Table 1, if the LBT for PRACH takes at least 25 us, for most of the short preamble formats, there would be no enough extra space in the GT period for LBT between two neighboring time-domain ROs. Therefore, the transmission on the prior RO would block the later one, resulting in wasting resources, reducing PRACH capacity and increasing latency. RO configuration should be enhanced to avoid potential blocking between neighboring time-domain ROs.

Observation 1： Assume the LBT for PRACH takes at least 25 us, there would be no enough blank period for LBT between neighboring time-domain ROs.

Proposal 2: For NR-U, NR Rel-15 RO configuration should be enhanced to avoid potential blocking between neighboring time-domain ROs.

Figure 1. Example of ROs within a PRACH slot.

Table 1. Guard Time of short preamble formats

| Format |  |  |  | PRACH duration | Guard Time |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 15 kHz (us) | 30 kHz (us) | 60 kHz (us) |
| A1 |  |  |  | 2 | 0 | 0 | 0 | 0 |
| A2 |  |  |  | 4 | 0 | 0 | 0 | 0 |
| A3 |  |  |  | 6 | 0 | 0 | 0 | 0 |
| B1 |  |  |  | 2 | 72k∙2-μ | 2.34 | 1.17 | 0.59 |
| B2 |  |  |  | 4 | 216k∙2-μ | 7.03 | 3.52 | 1.76 |
| B3 |  |  |  | 6 | 360k∙2-μ | 11.72 | 5.86 | 2.93 |
| B4 |  |  |  | 12 | 792k∙2-μ | 25.78 | 12.89 | 6.45 |
| C0 |  |  |  | 2 | 1096k∙2-μ | 35.68 | 17.84 | 8.92 |
| C2 |  |  |  | 6 | 2912k∙2-μ | 94.79 | 47.40 | 23.70 |

FDM multiplexing of UL channels

As per agreements in RAN1#94 meeting, potential blocking due to TA difference between FDM’d PUSCH/PUCCH/PRACH was noted as FFS. According to [3], a slot duration (9 us) is idle if the power detected by the UE for at least 4 us within the slot duration is less than ED threshold, soup to 5 us could be reserved e.g. for propagation and Rx/Tx switching. As shown in Figure 2, assuming 4 us is reserved at the end of the slot duration, the PRACH would not be blocked if the PUSCH/PUCCH starts within the reserved duration. That is, for timing difference up to 4 us between PUSCH/PUCCH/PRACH, LBT blocking could be avoided by UE implementation. Considering small cell range of NR-U, the potential blocking due to TA difference may not be a big issue.

Observation 2: Considering small cell range of NR-U, LBT blocking due to TA difference between FDM’d PUSCH/PUCCH/PRACH could be avoided by implementation.

- Conclusion

Proposals in this contribution are summarized as following:

Proposal 1: NR-U only supports preamble formats with sequence length L=139.

Observation 1： Assume the LBT for PRACH takes at least 25 us, there would be no enough blank period for LBT between neighboring time-domain ROs.

Proposal 2: For NR-U, NR Rel-15 RO configuration should be enhanced to avoid potential blocking between neighboring time-domain ROs.

Observation 2: Considering small cell range of NR-U, LBT blocking due to TA difference between FDM’d PUSCH/PUCCH/PRACH could be avoided by implementation.

References

- Chairman's Notes RAN1_93_final, 3GPP TSG RAN WG1 Meeting #93, May 2018.

- Chairman's Notes RAN1_94_final, 3GPP TSG RAN WG1 Meeting #94, Aug. 2018.

- TS 37.213 f10, Physical layer procedures for shared spectrum channel access