3GPP TSG RAN WG1 Meeting #95	R1-1812410

Spokane, United States, November 12th – 16th, 2018

Source:	Fujitsu

Title:	Resource Allocation for NR V2X Sidelink Communication

Agenda item:	7.2.4.1.5

Document for:	Discussion and Decision

Introduction

Resource allocation modes for NR V2X sidelink communication were discussed in RAN1#94 and RAN1#94b meetings, and the following agreements were made:

RAN1#94

Agreements:

At least two sidelink resource allocation modes are defined for NR-V2X sidelink communication

  - Mode 1: Base station schedules sidelink resource(s) to be used by UE for sidelink transmission(s)

  - Mode 2: UE determines (i.e. base station does not schedule) sidelink transmission resource(s) within sidelink resources configured by base station/network or pre-configured sidelink resources

- Notes:

  - eNB control of NR sidelink and gNB control of LTE sidelink resources will be separately considered in corresponding agenda items.

  - Mode-2 definition covers potential sidelink radio-layer functionality or resource allocation sub-modes (subject to further refinement including merging of some or all of them) where

    - UE autonomously selects sidelink resource for transmission

    - UE assists sidelink resource selection for other UE(s)

    - UE is configured with NR configured grant (type-1 like) for sidelink transmission

    - UE schedules sidelink transmissions of other UEs

RAN1 to continue study details of resource allocation modes for NR-V2X sidelink communication

RAN1#94b

Agreements:

Continue studying NR sidelink resource allocation techniques by NR Uu for mode-1:

- Dynamic resource allocation

- Semi-persistent scheduling allocation or NR grant type-2 (activation/de-activation by physical layer signaling)

- Grant free transmission i.e., configured NR grant type-1

Agreements:

- Sidelink sensing and resource selection procedures are studied for Mode-2(a)

  - The following techniques are studied to identify occupied sidelink resources

    - decoding of sidelink control channel transmissions

    - sidelink measurements

    - detection of sidelink transmissions

    - other options are not precluded, including combination of the above options

  - The following aspects are studied for sidelink resource selection

    - how a UE selects resource for PSCCH and PSSCH transmission (or other sidelink physical channel/signal, if it is introduced)

    - which information is used by UE for resource selection procedure

In this contribution, we give our views on the design considerations of NR sidelink resource. Considering the stringent requirements of advanced V2X services, we present some potential solutions for sidelink resource allocation by means of mode 1 and mode 2.

Basic considerations on NR sidelink resource

NR sidelink operate on both NR licensed spectrum and unlicensed ITS spectrum

NR V2X study item [1] covers the scenarios of the NR sidelink operate on both NR licensed spectrum and unlicensed ITS spectrum also in both the low (i.e. sub-6 GHz) and the high frequency bands (i.e. above 6 GHz). Therefore, both unlicensed band and NR licensed band(s) need to be considered for NR sidelink resource configuration.

Proposal 1: Both unlicensed band and NR licensed band(s) need to be considered for NR sidelink resource configuration.

Sidelink resource allocation based on base station scheduling and autonomous selection

The base station scheduling method is feasible for the scenario of a V2X UE under network control. By this way, the base station can allocate the resource to achieve a better resource utilization and reduce the interference. One concern for this method is the latency, which can be increased due to the inevitable coordination between base station and V2X UE.

The autonomous selection based method, on the other hand, belongs to a distributed method with lower latency. UE autonomously selects its resource by sensing the resource pool.

Proposal 2: The details of base station scheduling and autonomous selection method need to be carefully studied.

Requirements of advanced V2X services

Based on the SID [1], NR-V2X is expected to support the advanced V2X services, which are classified into four use case groups: vehicle platooning, advanced driving, extended sensors and remote driving [2,3]. NR-V2X must support a variety of traffic: including periodic traffic characterized with quasi-deterministic parameters, aperiodic traffic with unpredictable packet arrival time and varied packet sizes.  Meanwhile, the performance requirements for NR-V2X are quite stringent. As shown in Table I, in some use cases, e.g., for advanced driving and extended sensors, the reliability requirement is 99.999% and the latency requirement is 3 ms, respectively. Hereinafter, the packets with the most stringent requirements are referred as critical packets.

Table I Performance Requirements of advanced V2X services

| Use case group | Max end-to-end latency (ms) | Payload (Bytes) | Reliability (%) | Data rate (Mbps) | Min required Communication range (meters) |
| --- | --- | --- | --- | --- | --- |
| Vehicle Platooning | 10 ~ 500 | 50 ~ 6000 | 90 ~ 99.99 | [50] ~ [65] | 80 ~ 350 |
| Advanced driving | 3 ~ 100 | 300 ~ 12000 | 90 ~ 99.999 | 10 ~ 50 | 360 ~ 500 |
| Extend sensors | 3 ~ 100 | [1600] | 90 ~ 99.999 | 10 ~ 1000 | 50 ~ 1000 |
| Remote driving | 5 | - | 99.999 | UL: 25 DL: 1 | - |

Observation 1. Both periodic traffic and event-driven traffic need to be supported in NR-V2X.

Observation 2. In some use cases, NR-V2X needs to meet reliability requirement of 99.999% under latency target of 3 ms.

Discussion on NR-V2X sidelink resource configuration

Considerations on NR sidelink slot structure

For NR V2X, Uu licensed spectrum should be supported for sidelink transmission. Considering the different application scenarios of NR, like URLLC and eMBB, NR accepts to use flexible frame structure. In NR, the downlink only, the uplink only, and the flexible slot are supported. For the flexible slot, it includes some symbols with ‘unknown’ direction, which can be overwritten on a per-UE basis, or a group-common DCI with SFI-RNTI.

Since the NR V2X needs to use Uu licensed spectrum, a multiplexing of Uu and NR sidelink should be supported. For flexibly using the Uu resource, the NR sidelink can use the similar slot structure for multiplexing Uu and slidelink. As shown in Fig. 1, both sidelink-only and multiplexed Uu & SL slot need to be supported for NR V2X (AGC symbol, gap symbol, etc. are included in SL).

Comparing with the NR slot structure, it needs to introduce a new part, e.g., ‘SL’ part for NR sidelink slot structure.  Here, the ‘SL’ part may include the UL resource allocated for sidelink using and/or also the unknown symbols which can be dynamically configured for sidelink uses.

Figure 1. NR sidelink slot structure

In NR, it supports dynamic TDD configuration for DL and UL by using the unknown symbols. Similarly, it needs to support dynamic SL resource configuration to accommodate Uu and SL resource in the same band for the scenario of the SL resource is configured on NR Uu licensed band. Based on the dynamic SL resource configuration, it can use the unknown symbols dynamically to assure the traffic requirements, like low latency.

Proposal 3: Both the sidelink-only slot structure and the multiplexed Uu & SL slot structure need to be supported for NR V2X.

Proposal 4: It needs to introduce a new part, e.g., ‘SL’ part for NR sidelink slot structure, which include the UL resource allocated for sidelink uses and/or the unknown symbols.

Proposal 5: It needs to support dynamic SL resource configuration, if SL resource is configured on NR licensed band.

Resource pool and Bitmap

In LTE V2X, the resource pool in time domain is defined by a repeating bitmap mapped to all subframes except downlink subframes and sidelink synchronization subframes. The bitmap length for LTE-V2X can be 16, 20, or 100. For the uplink subframes, the bitmap can indicate which subframes are used for LTE V2X. For example, in bitmap, the bit indicates 1 means that this subframe can be used for V2X transmission, while 0 means that it is used for uplink transmission, as shown in Figure 5. Thus, using this bitmap information, those uplink subframes used for LTE V2X are becoming the time resource of a resource pool. Then V2X UE can select resource from resource pool for its transmission.

Using the above bitmap scheme, LTE V2X support the broadcast by using flexible configuration of resource pools so that different resource pools in a carrier can be TDM, FDM or a mix of TDM/FDM. For NR Sidelink communication, not only broadcast but unicast and groupcast should also be supported. Thus, similar to LTE V2X, NR sidelink should use the resource pool as well, based on the bitmap scheme for resource selecting and SCI blind detecting.

Proposal 6: NR sidelink should use the resource pool as well based on the bitmap scheme for resource selecting and SCI blind detecting.

In Rel.13 D2D, multiple transmission resource pools (up to 8) can be (pre-)configured. Each pool can have one or more associated priority, i.e., PPPP. For transmission of a MAC PDU, the UE can select a transmission pool based on the PPPP identified in the MAC PDU. Considering that NR-V2X should support diverse traffic with different QoS requirements, it is beneficial to adopt similar concept to (pre-)configure multiple resource pools based on QoS levels. However, separate resource pools for different QoS levels might lead to significant waste of radio resource. To satisfy various QoS requirements without sacrificing resource efficiency, a QoS based resource pool selection and switching mechanism could be considered. The details of QoS based resource pool selection and switching mechanism will be presented in the next section.

Proposal 7: NR sidelink should support QoS based resource pool configuration.

NR V2X need to support low latency traffic, such as 3ms end to end latency. By using resource selection from bitmap based resource pool, it may result in extra latency considering the existing of the slots indicated as 0 from bitmap, which cannot be used for sidelink. As shown in Figure 2, when emergency V2X traffic exists, it has to wait for 3 slots since there are no available resource for NR sidelink uses within 3 slots as indicated by bitmap. Thus, the latency will be increased by 3 slots as shown in the figure. Therefore, the method to assure the low latency requirement should be further studied by using the bitmap based resource indication.

Figure 2 Example of bitmap indication

Proposal 8: The method to assure the low latency requirement should be further studied by using the bitmap based resource indication.

Discussion on NR-V2X sidelink resource allocation

Fast SL scheduling mechanism for NR-V2X Mode 1

In previous RAN1 meeting, it is agreed to continue studying the following resource allocation techniques for NR V2X mode 1: dynamic resource allocation, activation/deactivation based resource allocation and RRC (pre-)configured resource allocation. As discussed in [4], dynamic resource allocation is more capable of handling aperiodic packet characterized with unpredictable packet arrival time and varied packet size. In LTE-V2X, dynamic resource allocation follows the SR-BSR procedure. In other words, if there is no available resource/grant when V2X traffic arrives at the VUE side, the VUE shall send SR for requesting uplink (UL) resources first, and then transmit sidelink (SL) BSR according to the received UL grant. Based on the SL BSR, the eNB can schedule corresponding SL resources, accordingly. As pointed out in the previous section, NR-V2X is required to meet the latency target as low as 3 ms. According to the analysis [5], if NR-V2X follows the same resource request procedure as LTE-V2X, it is quite difficult to satisfy the stringent latency requirement.

To reduce latency, one candidate solution is a fast SL scheduling mechanism. A fast UL scheduling has already been supported for URLLC in Rel.15 NR. In this scheme, the base station can schedule UL resources for a UE after receiving a SR without the reception of a followed BSR. Similar concept can be adopted to sidelink to enable fast SL scheduling, as depicted in Figure 3. An uplink control information (UCI) can be introduced for sidelink resource requesting. Such an UCI can carry all or a part of the following information to help the base station to allocate SL resources: SR for SL resources; SL BSR; QoS attributes such as communication types (broadcast/groupcast/unicast), latency and reliability requirements, minimum communication range, etc. After receiving the UCI, the base station can schedule sidelink resources for a VUE without additional BSR transmission. As analysed in [4], the latency target of 3 ms can be achieved by the fast SL scheduling mechanism.

Figure 3. Fast sidelink scheduling mechanism

Proposal 9: For NR-V2X mode 1 dynamic resource allocation, it needs to consider the fast SL scheduling mechanism as a candidate-scheduling scheme. Details of the UCI design are FFS.

Preemption mechanism

The autonomous resource selection mechanism of LTE V2X is tailored for periodic traffic. In LTE V2X, selected resources based on long-term sensing results (RSRP and RSSI) are reserved for the packet of the next period. The information of resource reservation is encoded in SCI. Other UEs can use this information to perform resource exclusion and the resource reserved by packets with higher priorities will most likely be excluded. However, such mechanism is not suitable for critical packets. Due to unpredictable arrival time, it is difficult to reserve resource beforehand for critical packets. Beside, as discussed in [6], the latency reduction below 20ms for resource selection leads to increased probability of collision and reporting the low SINR resources for selection. Under such situation, there is no guarantee that adequate resource can be selected for critical packet.

For NR, the multiplexing of eMBB and URLLC was supported. It was accepted to use preemption indication method to allow URLLC traffic to preempt the eMBB traffic considering the uncertainty of URLLC traffic. Similarly, the NR V2X traffic has different types which may have different priorities and latency requirements. It is a feasible way to support the preemption indication method for NR V2X as well. Therefore, it may permit the UE with the higher priority V2X traffic to preempt the resource reserved by low priority traffic as the targeted resource if there are no available enough resources. Then, the related UEs with lower priority packet which has reserved the targeted resource will release its reservation. As shown in Fig. 4, it assures the critical V2X packet to be transmitted within its stringent latency. The UE with the critical V2X packet can send the preemption indicator to other UEs via multicast or broadcast. The recommended resources that the UE wants to preempt can be included in the preemption indicator signaling. The channel or resource used for transmitting the preemption indicator should be further studied.

One possible way is the preemption indicator can be included in the form of PSCCH, which could be decoded by other UEs. Another way is the preemption indicator can be transmitted in another special form with a dedicated channel using the dedicated resource. If the related UE have their own transmission as well in the slot with the preemption indication transmission based on PSCCH, they cannot decode the preemption indication considering the half-duplex constraint. This issue should be considered when design the transmission timing of preemption indication. For the case the preemption indicator transmitted in another special form, the resource configuration for the preemption indicator should be studied.

Therefore, the details of the preemption based method to indicate the UE with low priority packet to release its reservation needs to be further studied.

Proposal 10: The resource preemption method for NR V2X sidelink communications is supported. The details on the preemption method need to further study

Time

Frequency

Subframe

(1ms)

Critical packet

arrives

Latency deadline

Time

Frequency

Subframe

(1ms)

Latency deadline

Preempt the reserved resource

Figure 4. Example of releasing the reserved resource

QoS based resource pool selection and switching mechanism

In order to guarantee available resources for a critical V2X packet, an alternative solution is to (pre)configure a dedicated resource pool. Some collision avoidance mechanism, such as listen-before-talk (LBT), can be applied to avoid collision when multiple UEs compete for resources. However, it is highly inefficient since the critical packets are sporadic.

To meet the stringent requirements of the critical packets without sacrificing much resource efficiency, a QoS based resource pool selection and switching mechanism can be considered. To exemplify the working of this mechanism, let us assume there are two kinds of V2X packets: one is critical packets with QoS level-1 (reliability requirement of 99.999% and latency requirement of 3 ms); the other one is periodic packets with QoS level-2 (reliability requirement of 99.99% and latency requirement of 10 ms).  Accordingly, let us assume there are two resource pools configured. The resource pool-1 applies the sensing based SPS resource selection mechanism. The resource pool-2 applies the LBT based resource selection mechanism, as shown in Figure 5, a symbol-level sensing window are used before transmission to sense whether the resource is occupied or not. Different sizes of the symbol-level sensing window can be (pre-)configured based on QoS level. A smaller size is (pre-)configured for a higher QoS level to guarantee it can access the channel faster.

Figure 5. LBT procedure with symbol-level sensing window

For V2X packets with different QoS levels, the selection priorities of different resource pools are listed in Table II.

- For the critical packet with QoS level-1: it can use resource pool-1 and resource pool-2 with equal priority, i.e., it can select one randomly between these two resource pools.

- For the periodic packet with QoS level-2: it can use resource pool-1 and resource pool-2, with resource pool-2 being selected with higher priority than resource pool-1.

Table II QoS based selection priorities of resource pools

| QoS Selection priority | Priority 1 | Priority 2 |
| --- | --- | --- |
| QoS level-1 | Resource pool-1, resource pool-2 | - |
| QoS level-2 | Resource pool-2 | Resource pool-1 |

As illustrated in Figure 6, the V2X application running at UE-1 generates a critical packet with QoS level 1 and the V2X application of UE-2 generates periodic packets with QoS level 2. For UE-1, after the critical packet arrives, it has the freedom to choose randomly between the resource pool-1 and the resource pool-2. On the other hand, the UE-2 must select the resource pool 2 first. After resource exclusion, if there is no sufficient resource within the selection window of the resource pool 2, the UE-2 can switch to resource the pool-1 and perform LBT to select an idle resource for transmission. Other switching conditions can also be considered, such as the CBR of the current resource pool is higher than a (pre-)configured threshold, etc. The details of switching condition need to be further studied.

Figure 6. QoS based resource pool selection and switching mechanism

Proposal 11: The QoS based resource pool selection and switching mechanism can be supported to assure the UE with a low latency packet to timely obtain the resource for its critical packets. Details of switching condition are FFS.

Conclusions

In this contribution, some potential mechanisms for mode 1 and mode 2 NR-V2X resource allocation considering the stringent requirements on reliability and latency are discussed. The following observations and proposals are made:

Observation 1. Both periodic traffic and event-driven traffic need to be supported in NR-V2X.

Observation 2. In some use cases, NR-V2X needs to meet reliability requirement of 99.999% under latency target of 3 ms.

Proposal 1: Both the unlicensed band and NR licensed band(s) need to be considered for NR sidelink resource configuration.

Proposal 2: The details of base station scheduling and autonomous selection method need to be carefully studied.

Proposal 3: Both the sidelink-only slot structure and the multiplexed Uu & SL slot structure need to be supported for NR V2X.

Proposal 4: It needs to introduce a new part, e.g., ‘SL’ part for NR sidelink slot structure, which include the UL resource allocated for sidelink uses and/or the unknown symbols.

Proposal 5: It needs to support dynamic SL resource configuration, if SL resource is configured on NR licensed band.

Proposal 6: NR sidelink should use the resource pool as well based on the bitmap scheme for resource selecting and SCI blind detecting.

Proposal 7: NR sidelink should support QoS based resource pool configuration.

Proposal 8: The method to assure the low latency requirement should be further studied by using the bitmap based resource indication.

Proposal 9: For NR-V2X mode 1 dynamic resource allocation, it needs to consider the fast SL scheduling mechanism as a candidate-scheduling scheme. Details of the UCI design are FFS.

Proposal 10: The resource preemption method for NR V2X sidelink communications is supported. The details on the preemption method need to further study.

Proposal 11: The QoS based resource pool selection and switching mechanism can be supported to assure the UE with a low latency packet to timely obtain the resource for its critical packets. Details of switching condition are FFS.

- References

RP-181480, “New SID: Study on NR-V2X”, Huawei, CATT, LG Electronics, HiSilicon, China Unicom, La Jolla, USA, June 11th – 14th, 2018.

3GPP TS 22.186, “Enhancement of 3GPP support for V2X scenarios; Stage 1”, V14.2.1, March 2017.

3GPP TR 22.886, “Study on enhancement of 3GPP Support for 5G V2X Services”, V15.0.0, December 2016.

R1-1810594, “Discussion on Uu-based NR sidelink resource allocation”, Chengdu, China, Sep. 8th – 12th, 2018.

R1-1808300, “Discussion on latency performance of Uu-based sidelink resource allocation”, Gothenburg, Sweden, Aug. 20th – 24th , 2018.

R1-1810593, “resource allocation for NR V2X sidelink communication considering low latency requirement”, Chengdu, China, Sep. 8th – 12th, 2018.