3GPP TSG RAN WG1 Meeting #95                              	R1-1812431

Spokane, USA, November 12th – 16th, 2018

Source:       ZTE

Title:         Evaluation Results for NR-U

Agenda Item:  7.2.2.1

Document for: Discussion and Decision

# Introduction

Some conclusions on evaluations for NR-U were reached at RAN1#92bis and #94, which includes the following agreements [1][2][3]:

Agreement:

- For sub7 indoor simulation evaluation:

  - Scenario: Option 2 (3+3) with indoor mixed office model

    - Target to reach 10%-15% serving links below -72dBm

    - Further layout parameter fine tuning may be needed. An example procedure for fine tuning is the following sequence.

      - Currently a-b-a=15-20-15

      - If not reaching target, try a-b-a=15-30-15 and a-b-a=20-40-20

      - If not reaching target, apply a scaling factor to the layout with a-b-a=20-40-20

  - Other parameters: Default is NR parameters in 38.901 and 38.802 with the exception of the following

The final result of the email discussion is(a,b,c,d)=(20,40,20,40) .

Agreements:

- The base metrics for NR-U evaluation are the same as in LTE-LAA in TR 36.889.

- For coexistence evaluations below 7GHz, for parameters not covered by previous agreements, the evaluations assumptions specified for LTE (e)LAA coexistence evaluations apply.

  - For example, the minimum distance between a small cell and a UE, and between two UEs is three meters.

Agreement:

- For coexistence evaluation, WiFi+WiFi, WiFi+NR-U and NR-U+NR-U evaluations are baseline with equal priority.

Agreement:

A=1.0 for scenario 1 and A=1.5 for scenario 2

Conclusion:

For evaluation purpose, the previous agreement on dropping 5 UEs per gNB is interpreted as exactly 5 UEs are associated with each cell.

Agreement:

When submitting the evaluation result, the following has to be provided as well

- the TxOP assumptions of WiFi and NR-U

- Is RTS/CTS enabled for WiFi

- PD/ED threshold assumptions

- Max modulation order supported in each technology

- MIMO scheme and number of MIMO layers used for both technologies

- WiFi MAC layer A-MPDU/A-MSDU aggregation level, MPDU size

- NR-U SCS,

- WiFi guard interval

- NR UE processing time capability (#1 or #2)

- NR PDSCH/PUSCH mapping type, PDCCH monitoring configuration

- Link adaptation assumptions

- NR assumption on self scheduling or using cross carrier scheduling

Agreement:

For the coexistence evaluation of sub-7GHz bands other than 5GHz band, previously agreed to use technology neutral assumptions. Companies are encouraged to provide simulation results together with assumption on the technology neutral channel access mechanism.

Agreement:

The template in Annex B of TR 38.889 v0.1.1 is used to capture results in the TR at least for 5 GHz.

In this contribution, we present our simulation results for NR-U.

# Simulation Results for sub7 GHz NR-U Indoor Scenario

In this contribution, we evaluated an indoor deployment scenario according to [5]. In this contribution, the following three cases are evaluated to investigate the impact of NR-U deployment toward WiFi deployment:

- two operators of WiFi deployment;

- one operator of NR-U deployment coexist with another operator of WiFi deployment;

- two operators of NR-U deployment.

For each WiFi AP in both case 1 and 2, CCA-CS threshold and CCA-ED threshold are used respectively for intra-RAT and inter-RAT channel sensing. For NR-U gNB in case 2 and case 3, CCA-ED threshold is used as the LBT threshold.

For a fair comparison, WiFi AP and STA placement is using the same parameters and layout of NR gNB and UE placement. In each case, different operators’ sites are chosen independently (i.e. without network planning) but they are fixed during the evaluation of a case for a fair comparison.

The detailed simulation assumptions are listed in Appendix A.

The following tables 1 to 4 are the simulation results for the co-existence scenarios of WiFi+WiFi, WiFi+NR-U and NR-U+NR-U. For NR-U, we have DL+UL/DL-only/UL-only traffic in each case.

Table 1  Wi-Fi and NR-U with DL+UL coexistence evaluation with 20MHz and FTP traffic

| Tdoc /Source | Reported parameters |  | Low load BO range for Wi-Fi in WiFi+WiFi: 10%~25% |  |  |  | Medium load BO range for Wi-Fi in WiFi+WiFi: 35%~50% |  |  |  | High load BO range for Wi-Fi in WiFi+WiFi: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U |
| R1-1812431 | DL: UPT CDF [Mbps] | 5% | 40.958 | 50.921 | 12.749 | 20.9086 | 19.582 | 42.015 | 6.059 | 24.4146 | 7.768 | 42.348 | 2.854 | 24.2027 |
|  |  | 50% | 51.341 | 58.429 | 44.530 | 83.6427 | 32.045 | 51.229 | 30.037 | 78.1366 | 22.447 | 49.634 | 15.259 | 75.6554 |
|  |  | 95% | 61.063 | 64.120 | 63.951 | 114.0128 | 40.058 | 58.609 | 47.489 | 99.3320 | 28.934 | 55.007 | 36.573 | 100.7447 |
|  |  | Mean | 51.903 | 59.083 | 47.601 | 76.3185 | 32.107 | 52.505 | 32.781 | 72.6793 | 21.242 | 49.544 | 21.404 | 70.5092 |
|  | DL: Delay CDF [s] | 5% | 0.060 | 0.060 | 0.047 | 0.034 | 0.061 | 0.060 | 0.051 | 0.035 | 0.073 | 0.060 | 0.060 | 0.035 |
|  |  | 50% | 0.066 | 0.062 | 0.088 | 0.047 | 0.142 | 0.065 | 0.153 | 0.052 | 0.297 | 0.072 | 0.277 | 0.053 |
|  |  | 95% | 0.207 | 0.112 | 0.333 | 0.153 | 0.739 | 0.163 | 0.817 | 0.252 | 3.268 | 0.211 | 1.933 | 0.307 |
|  |  | Mean | 0.096 | 0.072 | 0.130 | 0.068 | 0.232 | 0.088 | 0.276 | 0.083 | 0.817 | 0.096 | 0.527 | 0.087 |
|  | UL: UPT CDF [Mbps] | 5% | 40.245 | 49.152 | 12.083 | 16.3911 | 23.012 | 48.127 | 4.081 | 15.4290 | 13.400 | 40.174 | 3.018 | 59.8555 |
|  |  | 50% | 49.537 | 58.413 | 42.110 | 74.4228 | 34.770 | 50.625 | 29.733 | 62.8715 | 23.108 | 49.411 | 17.757 | 16.4273 |
|  |  | 95% | 60.993 | 64.496 | 76.645 | 92.7418 | 39.682 | 54.735 | 55.973 | 90.0290 | 36.185 | 54.341 | 42.229 | 62.9272 |
|  |  | Mean | 50.859 | 58.871 | 49.220 | 68.3392 | 34.875 | 51.439 | 32.720 | 62.3984 | 23.966 | 48.391 | 22.963 | 84.8273 |
|  | UL: Delay CDF [s] | 5% | 0.060 | 0.060 | 0.042 | 0.041 | 0.062 | 0.060 | 0.042 | 0.041 | 0.065 | 0.060 | 0.047 | 0.041 |
|  |  | 50% | 0.068 | 0.062 | 0.082 | 0.054 | 0.127 | 0.066 | 0.162 | 0.061 | 0.211 | 0.076 | 0.248 | 0.064 |
|  |  | 95% | 0.192 | 0.132 | 0.363 | 0.259 | 0.454 | 0.174 | 0.792 | 0.274 | 0.864 | 0.187 | 1.392 | 0.290 |
|  |  | Mean | 0.092 | 0.074 | 0.128 | 0.097 | 0.173 | 0.089 | 0.282 | 0.106 | 0.306 | 0.096 | 0.448 | 0.113 |
|  | 𝜌DL |  | 1.00 | 1.00 | 1.00 | 1.00 | 0.99 | 0.93 | 1.00 | 99.8339 | 0.96 | 0.92 | 0.97 | 1.00 |
|  | 𝜌UL |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 0.93 | 0.98 | 99.8333 | 1.00 | 0.92 | 0.95 | 1.00 |
|  | BO |  | 23.677 | 36.875 | 14.704 | 5.260 | 41.023 | 40.448 | 45.818 | 11.154 | 59.295 | 45.374 | 72.597 | 15.094 |
|  | 𝜆 |  | DL:1Mbps; UL:1Mbps |  |  |  | DL:2Mbps; UL:2Mbps |  |  |  | DL:2.5Mbps; UL:2.5Mbps |  |  |  |
|  |  |  | Additional comments: Note: A fixed time slot ratio of downlink-uplink is used for NR-U, i.e. DL:UL = {5:5}. |  |  |  |  |  |  |  |  |  |  |  |

Table 2 Wi-Fi and NR-U with DL-only coexistence evaluation with 20MHz and FTP traffic

| Tdoc /Source | Reported parameters |  | Low load BO range for Wi-Fi in WiFi+WiFi: 10%~25% |  |  |  | Medium load BO range for Wi-Fi in WiFi+WiFi: 35%~50% |  |  |  | High load BO range for Wi-Fi in WiFi+WiFi: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U |
| R1-1812431 | DL: UPT CDF [Mbps] | 5% | 40.958 | 50.315 | 32.719 | 62.8229 | 19.582 | 44.334 | 18.189 | 54.2921 | 7.768 | 44.419 | 11.733 | 52.0858 |
|  |  | 50% | 51.341 | 61.638 | 109.690 | 179.0133 | 32.045 | 54.118 | 75.648 | 164.9158 | 22.447 | 49.424 | 62.421 | 156.4768 |
|  |  | 95% | 61.063 | 64.916 | 136.431 | 207.9165 | 40.058 | 59.754 | 105.746 | 202.6525 | 28.934 | 55.316 | 93.050 | 204.5259 |
|  |  | Mean | 51.903 | 60.768 | 96.879 | 162.1622 | 32.107 | 54.513 | 66.707 | 150.6907 | 21.242 | 50.293 | 57.011 | 145.9356 |
|  | DL: Delay CDF [s] | 5% | 0.060 | 0.060 | 0.026 | 0.019 | 0.061 | 0.060 | 0.026 | 0.019 | 0.073 | 0.060 | 0.026 | 0.019 |
|  |  | 50% | 0.066 | 0.062 | 0.047 | 0.022 | 0.142 | 0.063 | 0.080 | 0.024 | 0.297 | 0.069 | 0.097 | 0.024 |
|  |  | 95% | 0.207 | 0.110 | 0.156 | 0.077 | 0.739 | 0.146 | 0.457 | 0.131 | 3.268 | 0.177 | 0.662 | 0.139 |
|  |  | Mean | 0.096 | 0.069 | 0.063 | 0.034 | 0.232 | 0.081 | 0.137 | 0.039 | 0.817 | 0.093 | 0.197 | 0.041 |
|  | UL: UPT CDF [Mbps] | 5% | 40.245 | 50.386 | - | - | 23.012 | 46.349 | - | - | 13.400 | 45.585 | - | - |
|  |  | 50% | 49.537 | 60.834 | - | - | 34.770 | 50.113 | - | - | 23.108 | 50.104 | - | - |
|  |  | 95% | 60.993 | 65.066 | - | - | 39.682 | 57.205 | - | - | 36.185 | 55.314 | - | - |
|  |  | Mean | 50.859 | 60.523 | - | - | 34.875 | 52.000 | - | - | 23.966 | 51.155 | - | - |
|  | UL: Delay CDF [s] | 5% | 0.060 | 0.060 | - | - | 0.062 | 0.060 | - | - | 0.065 | 0.060 | - | - |
|  |  | 50% | 0.068 | 0.062 | - | - | 0.127 | 0.066 | - | - | 0.211 | 0.069 | - | - |
|  |  | 95% | 0.192 | 0.106 | - | - | 0.454 | 0.152 | - | - | 0.864 | 0.161 | - | - |
|  |  | Mean | 0.092 | 0.069 | - | - | 0.173 | 0.086 | - | - | 0.306 | 0.089 | - | - |
|  | 𝜌DL |  | 1.00 | 0.95 | 1.00 | 1.00 | 0.99 | 0.96 | 0.97 | 1.00 | 0.96 | 91.821 | 0.97 | 1.00 |
|  | 𝜌UL |  | 1.00 | 0.96 | - | - | 1.00 | 0.95 | - | - | 1.00 | 93.255 | - |  |
|  | BO |  | 23.677 | 36.156 | 6.923 | 3.952 | 41.023 | 39.764 | 26.174 | 8.783 | 59.295 | 44.171 | 41.198 | 11.427 |
|  | 𝜆 |  | DL:1Mbps; UL:1Mbps |  | DL:2Mbps; |  | DL:2Mbps; UL:2Mbps |  | DL:4Mbps; |  | DL:2.5Mbps; UL:2.5Mbps |  | DL:5Mbps; |  |
|  |  |  | Additional comments: |  |  |  |  |  |  |  |  |  |  |  |

Table 3 Wi-Fi and NR-U with UL-only coexistence evaluation with 20MHz and FTP traffic

| Tdoc /Source | Reported parameters |  | Low load BO range for Wi-Fi in WiFi+WiFi: 10%~25% |  |  |  | Medium load BO range for Wi-Fi in WiFi+WiFi: 35%~50% |  |  |  | High load BO range for Wi-Fi in WiFi+WiFi: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U | Wi-Fi in WiFi+ WiFi | Wi-Fi in WiFi+ NR-U | NR-U in WiFi+ NR-U | NR-U in NR-U+ NR-U |
| R1-1812431 | DL: UPT CDF [Mbps] | 5% | 40.958 | 47.276 | - | - | 19.582 | 44.392 | - | - | 7.768 | 41.229 | - | - |
|  |  | 50% | 51.341 | 60.190 | - | - | 32.045 | 51.535 | - | - | 22.447 | 48.156 | - | - |
|  |  | 95% | 61.063 | 64.231 | - | - | 40.058 | 57.314 | - | - | 28.934 | 50.807 | - | - |
|  |  | Mean | 51.903 | 59.268 | - | - | 32.107 | 51.647 | - | - | 21.242 | 47.494 | - | - |
|  | DL: Delay CDF [s] | 5% | 0.060 | 0.060 | - | - | 0.061 | 0.060 | - | - | 0.073 | 0.060 | - | - |
|  |  | 50% | 0.066 | 0.062 | - | - | 0.142 | 0.066 | - | - | 0.297 | 0.077 | - | - |
|  |  | 95% | 0.207 | 0.130 | - | - | 0.739 | 0.170 | - | - | 3.268 | 0.250 | - | - |
|  |  | Mean | 0.096 | 0.072 | - | - | 0.232 | 0.089 | - | - | 0.817 | 0.107 | - | - |
|  | UL: UPT CDF [Mbps] | 5% | 40.245 | 50.842 | 26.834 | 34.9247 | 23.012 | 42.962 | 14.354 | 33.1597 | 13.400 | 38.706 | 9.914 | 32.3721 |
|  |  | 50% | 49.537 | 61.343 | 114.239 | 141.0299 | 34.770 | 48.151 | 72.546 | 126.7055 | 23.108 | 48.265 | 57.714 | 126.1715 |
|  |  | 95% | 60.993 | 65.154 | 157.121 | 185.7409 | 39.682 | 55.809 | 112.481 | 178.2521 | 36.185 | 54.055 | 98.468 | 174.1342 |
|  |  | Mean | 50.859 | 60.017 | 102.352 | 135.9199 | 34.875 | 49.242 | 68.307 | 126.5704 | 23.966 | 48.418 | 53.314 | 123.3722 |
|  | UL: Delay CDF [s] | 5% | 0.060 | 0.060 | 0.022 | 0.021 | 0.062 | 0.060 | 0.022 | 0.021 | 0.065 | 0.060 | 0.022 | 0.021 |
|  |  | 50% | 0.068 | 0.062 | 0.040 | 0.027 | 0.127 | 0.071 | 0.089 | 0.030 | 0.211 | 0.077 | 0.128 | 0.031 |
|  |  | 95% | 0.192 | 0.117 | 0.193 | 0.111 | 0.454 | 0.193 | 0.474 | 0.125 | 0.864 | 0.187 | 0.738 | 0.137 |
|  |  | Mean | 0.092 | 0.071 | 0.067 | 0.045 | 0.173 | 0.095 | 0.151 | 0.052 | 0.306 | 0.098 | 0.224 | 0.056 |
|  | 𝜌DL |  | 1.00 | 95.070 | - | - | 0.99 | 0.96 | - | - | 0.96 | 0.93 | - | - |
|  | 𝜌UL |  | 1.00 | 95.620 | 1.00 | 1.00 | 1.00 | 0.95 | 0.98 | 1.00 | 1.00 | 0.92 | 0.93 | 1.00 |
|  | BO |  | 23.677 | 36.196 | 8.38 | 5.280 | 41.023 | 40.487 | 36.885 | 11.545 | 59.295 | 44.734 | 40.175 | 14.749 |
|  | 𝜆 |  | DL:1Mbps; UL:1Mbps |  | UL:2Mbps; |  | DL:2Mbps; UL:2Mbps |  | UL:4Mbps; |  | DL:2.5Mbps; UL;2.5Mbps |  | UL:5Mbps; |  |
|  |  |  | Additional comments: |  |  |  |  |  |  |  |  |  |  |  |

From the above simulation results, we observe that NR-U system (with suitable co-existence mechanisms such as LBT function) does not impact WiFi services more than an additional WiFi network on the same carrier. In fact, some performance improvements were observed when NR-U coexist with WiFi than two WiFi network coexistence case.

Observation 1: NR-U can coexist well with WiFi if LBT mechanism is adopted.

# Conclusion

In this contribution, some simulation results are provided. Based on these, we have the following observation:

Observation 1: NR-U can coexist well with WiFi if LBT mechanism is adopted.

# Reference

- Draft report of 3GPP TSG RAN WG1#92bis v0.1.0, Sanya, China, April 16th - 20th, 2018

- Draft_Minutes_report_RAN1#94_v010, Gothenburg, Sweden, August 20th – 24th, 2018

- Draft_Minutes_report_RAN1#94b_v010,Chengdu, China, October 8th – 12th, 2018

- 3GPP TR 36.889 V0.2.0(2018-10)

- R1-1807384,Email discussion summary for sub7 indoor evaluation calibration

# Appendix A: NR-U Indoor sub7 GHz Simulation Assumptions

Table A-1 NR-U evaluation assumptions

| Parameters | Indoor Sub-7GHz |
| --- | --- |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz |
| Number of carriers | 1 |
| Number of users per operator | 5 per gNB per 20MHz |
| Channel Model | NR InH Mixed Office model |
| BS/AP Tx Power | 23 dBm (total across all TX antennas) |
| UE/STA Tx Power | 18 dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0 dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5 dB |
| UE/STA Receiver Noise Figure | 9 dB |
| Minimum received power from serving cell for UE dropping | -82 dBm |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1 |
| TxOP | 4ms |
| ED threshold assumptions | -72 dBm |
| Max modulation order | 256QAM |
| MIMO scheme and number of MIMO layers | 2 layers |
| NR-U SCS, | 15kHz |
| NR UE processing time capability (#1 or #2) | Capability #1 |
| NR PDSCH/PUSCH mapping type, PDCCH monitoring configuration | type A monitoringSlotPeriodicityAndOffset：1slot; coreset duration： 2 symbol ; monitoringSymbolswithinSlot:[1 0 0 0 0 0 0 0 0 0 0 0 0 0] |
| Link adaptation assumptions | Enable |
| NR assumption on self scheduling or using cross carrier scheduling | cross carrier scheduling( feedback in licensed band) |

Table A-2 WiFi system evaluation assumptions

| Parameter |  | value |
| --- | --- | --- |
| TxOP |  | 4.096 ms |
| RTS/CTS |  | Not Enabled |
| PD/ED threshold assumption |  | ED detection CCA-CS:-82 dBm CCA-ED:-62 dBm |
| Max modulation order |  | 256QAM |
| MIMO scheme and number of MIMO layers |  | STBC,2 layers |
| MAC layer A-MPDU/A-MSDU aggregation level, MPDU size |  | Frame aggregation:A-MPDU, 10 frames aggregation MPDU:Fixed 1500B MPDU size |
| guard interval |  | Normal guard interval: 0.8us |
| TX Power |  | 23 dBm, same as NR-U 18 dBm TX power for WiFi STA |
| MAC | Coordination | EDCA |
|  | SIFS, DIFS | SIFS, DIFS |
|  | Detection | Energy detection |
|  | Contention window | Min : 16 slot, Max : 64 slot |
| Rate control |  | Proprietary algorithm |