3GPP TSG RAN1 Meeting #95		   R1-1813192

Spokane, USA, Nov 8-12, 2018

Agenda item:	7.2.2.3.1

Source: 	CableLabs

Title: 	DL Channels and Signals for NR-U

Document for:	Discussion/Decision

# Introduction

During the RAN#75, a study item (SI) has been approved [1] to study NR unlicensed (NR-U) . Objectives of the SI include studying coexistence methods within NR-based and between NR-based operation in unlicensed and LTE-based LAA and with other incumbent RATs in accordance with regulatory requirements. An effective way of achieving coexistence between NR-U and WiFi is to introduce a new NR-U preamble decodable by both technologies. A common signal or preamble can improve coexistence current and future 802.11 releases. In this contribution, we share our view on the benefits of a such signaling.

# Possible benefits of using a mutual preamble between NR-U and 802.11 access technologies

Introduction of a new preamble for NR-U has been discussed in multiple meetings recently. Some of the benefits of preamble based channel access include robust transmission detection, power savings on the receiver, mitigation of collisions due to hidden nodes, increased spatial re-use by directional LBT. In addition to these, the new preamble design can include  a portion decodable by WiFi to improve coexistence. As proposed by some companies, a straight forward way of achieving this objective is to include legacy WiFi preamble, i.e. 802.11a preamble in the new preamble. Since all WiFi generations implement the legacy preamble including 802.11ax, this approach can improve channel access between WiFi and NR-U in 5 GHz as well as in the 6GHz band.

## UE Power Savings

The main design element to the channel access in the unlicensed band is the listen-before-talk (LBT), which can be performed by sensing the energy level in the environment or searching for a preamble that includes a known training or synchronization signal. These approaches are called energy detection (ED) and preamble detection (PD), respectively. In ED based channel access, it is not possible to obtain information related to an ongoing transmission. Therefore, the node sensing the channel should continuously monitor the channel until it is released.

On the other hand, the preamble can include useful control information such as burst configuration, source and destination nodes. The new preamble design can indicate the burst duration in the length field of 802.11a portion in addition to including NR-U specific control information in the new fields.

When the number of devices trying to access the channel increases, the probability of getting an access to the channel decrease and the probability of collisions increases. Therefore, it is very critical to understand channel occupancy information of other devices in the environment. A good channel access scheme in the unlicensed band should not only ensure how much time is allocated between nodes but also when they can start transmission.

[5] shows the increasing collision probability for an increasing number of nodes using a CSMA-CA type of access (802.11) against the number of accessing nodes for different contention windows. Simulation results from [5] are included  in the appendix section.

If an NR-U node employs  the 802.11a preamble, it will also support two types of actions:

- An NR-U device employing an 802.11a preamble could understand the channel occupancy, triggered by 802.11 devices.

- Coexisting Wi-Fi devices, when listening to an NR-U device, can understand the channel occupancy by coexisting NR-U devices.

Accordingly, i) the amount of collisions over the air caused on the NR-U device by the coexistent Wi-Fi devices will be reduced and ii) the coexistent Wi-Fi devices would optimize their power savings.

Observation 1: Mutual preamble detection and transmission supports power savings for all cross-coexistent Wi-Fi and NR-U devices.

Observation 2: The higher is the number of coexisting devices attempting to access the network, the higher is the collision probability.

Observation 3: The higher is the collision probability over the air, the higher could be the UE power savings, if the UE could predict the channel occupancy.

Proposal 1: The 802.11 preamble (either 802.11a or ax) should be analysed in relationship with the UE power savings.

## Access Fairness

LAA LTE SI assumed that the 802.11ac nodes, which were rolled out on a mass scale, by the time LAA LTE specifications were discussed, are the incumbent access technology. In order to address this, [6] defined an adequate fairness coexistence criterion, equivalent to: another access technology node should not degrade the performance of a coexisting Wi-Fi node more than another Wi-Fi node.

LAA LTE WIs approved a compromised solution (LAA LTE ED -72 dBm), which allowed the LAA LTE nodes to take over the coexisting Wi-Fi nodes (received below -72 dBm) related traffic and also yielded the access to the Wi-Fi nodes with a RSSI received above -72 dBm.

Figure 1. LAA LTE ED solution

This solution didn’t provide an optimized channel access between LAA LTE and 802.11a/n/ac nodes due to the rationale provided above. One could conclude that the fairness criterion was only partially achieved, given the majority of measured RSSI traffic in public deployments resides below the -72 dBm mark. In [7], statistical data collected across hundreds of thousands of Wi-FI nodes were presented, showing the highest percentage of the traffic happening below the -72 dBm RSSI mark.

However, if an NR-U node could have the capability to transmit and receive 802.11 preamble, then the NR-U channel access can be enhanced  resulting into:

- A better coexistence NR-U with Wi-Fi nodes

- A better spectral efficiency for both access technologies under discussion

- Lower traffic latencies, given the fact the higher contention probability drives to a higher latency for the targeted nodes.

Observation 4: NR-U nodes employing the 802.11 preamble capability, would provide an optimized coexistence with the coexistence Wi-Fi nodes, resulting into an optimized spectral efficiency and traffic latency for both accessing technologies.

Proposal 2: Analyze the potential benefits and downgrades of employing the 802.11 preamble (either 802.11a or 802.11ax) on NR-U nodes, in order to optimize the coexistence with the Wi-Fi nodes in terms of spectral efficiency and traffic latency.

## 802.11a vs. 802.11ax preamble

The 802.11ax nodes have a dual type of behaviour

- Coexistence with other (legacy) 802.11a/ac/n nodes

In this case, an 802.1ax node would detect other legacy 802.11 nodes (11a/n/ac) and would operate with fixed ED/PD thresholds (-62/-82 dBm), thus preserving the coexistence with the legacy Wi-Fi devices.

- Coexistence with itself and other (non Wi-Fi) access technologies

In this case, an 802.11ax node has the ability to adjust its ED/PD thresholds in order to increase spatial reuse (see Fig 2).

Observation 5. An 802.11ax node could be more aggressive towards an NR-U node than another legacy 802.11 node (when no other legacy Wi-Fi traffic is present).

It should also be noted that the first 802.11ax deployments are expected to hit the market by late 2019, probably by 6-12 months earlier than the NR-U networks. Under this assumption, the 802.11ax may become an incumbent access technology for the NR-U access technology.

While the LAA LTE fairness criterion (also used for the actual 5 GHz NR-U design) may or may not be used for the coexistence of NR-U with 802.11ax, rolling out both these access technologies without any coexistence measures, would set these access technologies on a collision course.

Proposal 3. The NR-U SI should study the NR-U coexistence with 802.11ax.

Nevertheless, the 802.11ax preambles also have an additional capability vs. the legacy 802.11a preambles:

- Improved power savings due to the Target Wake-Up time capability,

- Adjustable ED/PD thresholds based on the dynamic selection of CCA thresholds,

- Support for spatial reuse.

Observation 6. An 802.11ax node could provide more functionality than a legacy 802.11 node.

Proposal 4. The NR-U should study the benefits and downgrades of employing an 802.11ax preamble.

Figure 2. The 802.11ac ED/PD behavior when coexisting with non Wi-Fi technologies.

# Conclusions

We outlined our views on spectrum sharing concerning the 5 GHz case. We recommend:

Observation 1: Mutual preamble detection and transmission supports power savings for all cross-coexistent Wi-Fi and NR-U devices.

Observation 2: The higher is the number of coexistent devices attempting to access the network, the higher is the collision probability.

Observation 3: The higher is the collision probability over the air, the higher could be the UE power savings, if the UE could predict the channel occupancy.

Observation 4: NR-U nodes employing the 802.11 preamble capability, would provide an optimized coexistence with the coexistence Wi-Fi nodes, resulting into an optimized spectral efficiency and traffic latency for both accessing technologies.

Observation 5. An 802.11ax node could be more aggressive towards a NR-U node than another legacy 802.11 node (when no other legacy Wi-Fi traffic is present).

Observation 6. An 802.11ax node could provide more functionality than a legacy 802.11 node.

Accordingly the following proposals should be considered.

Proposal 1: The 802.11 preamble (either 802.11a or ax) should be analysed in relationship with the UE power savings.

Proposal 2: Analyze the potential benefits and downgrades of employing the 802.11 preamble (either 802.11a or 802.11ax) on NR-U nodes, in order to optimize the coexistence with the Wi-Fi nodes in terms of spectral efficiency and traffic latency.

Proposal 3. The NR-U SI should study the NR-U coexistence with 802.11ax.

Proposal 4. The NR-U should study the benefits and downgrades of employing an 802.11ax preamble.

# Appendix

## Collision probability vs. the number of accessing nodes for different contention windows [5]

| Contention Window W=8 | Contention Window W=16 | Contention Window W=32 |
| --- | --- | --- |

Figure 3. Collision probability vs. the number of accessing nodes for CSMA/CA access for different contention window sizes

# References

[1] RP-170828, New SID on NR-based Access to Unlicensed Spectrum, Qualcomm, 3GPP TSG RAN Meeting #75, March 2017

[2] RP-181515, IEEE 802.11 WG LS to 3GPP RAN in relation to a proposed joint Coexistence Workshop, 3GPP TSG RAN1 Meeting #94, Aug 2018

[3] RP-182168, Reply LS on proposed joint Coexistence Workshop, 3GPP TSG RAN1 Meeting #94, Aug 2018

[4] R1-1813567, Broadcom, Cisco, “Preamble/Initial Signal for NR-U Unlicensed”, RAN1 #95, Nov 2018

[5] H. L. Vu, T. Sarai, “Collision Probability in Saturated IEEE 802.11 Networks”. Swinburne University of Technology, Australia, The University of Melbourne, Australia

[6] TR 36.889, v13.0.0, “Study on Licensed-Assisted Access to Unlicensed Spectrum (Release 13)”, 3GPP, June 2015

[7] R1-1805093, Broadcom, CableLabs, Charter, Cisco, HPE, “Discussion on network toology for sub-7 GHz coexistence evaluation between NR-U and Wi-Fi”