3GPP TSG-RAN WG1 #96							        R1-1901831

Athens, Greece, 25th February – 1st March, 2019

Source:	Mitsubishi Electric

Title:	Waveform evaluations for V2X sidelink

Agenda Item:	7.2.4.1.5	Others

Document for: 	Discussion/Decision

# Introduction

A new study item on NR V2X [1] has been approved at the RAN#80 plenary, which is intended to support advanced V2X services beyond services supported in LTE Rel-15 V2X. The stringent requirements on latency and reliability imposed in TS 22.186[2] in the context of advanced V2X services require enhancements to the current NR system. One of the main challenges is the definition of a new NR sidelink.

In RAN1#94 meeting [3], the following starting points for the study were identified:

- RAN1 to continue study on the physical channel considering at least the following aspects:

- Waveform

  - Candidates: CP-OFDM, DFT-s-OFDM

  - Proposals from companies:

    - CP-OFDM only

    - Support both

  - Consideration points:

    - Different channel can have different waveform?

    - Benefit and impact of supporting only one waveform and supporting both waveforms

In RAN1#95 meeting [4], the following waveform-related progress was made:

- At least CP-OFDM is supported.

- Continue study on whether to support DFT-S-OFDM including the potential issues and the following potential benefit:

  - Synchronization coverage enhancement

  - PSCCH coverage enhancement, e.g., with Option 2 of PSCCH/PSSCH multiplexing with the restriction that PSCCH and PSSCH use adjacent frequency resources

  - Feedback channel coverage enhancement

- A single waveform is used in all the sidelink channels in a carrier.

  - Note: A sequence based channel can be supported in any waveform.

  - (Pre-)configuration will be used to determine the used waveform if the specification supports multiple waveforms.

In RAN1 NR-AH#1901, there was no consensus for a step forward on the waveform issue, and it was decided [5] to conclude the discussion in the current RAN1#96 meeting.

In this contribution, we evaluate the performance advantages of DFTsOFDM for NR V2X. This contribution is revised from R1-1901147.

# Discussion

## Scenarios and use cases

SA1 has identified 25 use cases for advanced V2X services and they are categorized into four use case groups: vehicles platooning, extended sensors, advanced driving and remote driving. The normative requirements for each use case group, given in TS 22.186[2], depend on the characteristics of each use case group.

For Vehicles Platooning, where vehicles driving together in a same direction in a coordinated manner under the management of a leading vehicle, positioning accuracy is of utmost importance. The 3GPP system shall support relative longitudinal position accuracy of less than 0.5 m for UEs supporting V2X application for platooning in proximity.

For Extended Sensors, where vehicles can increase the perception of their environment beyond of what their own sensors can detect through exchanges of raw or processed data gathered by other members of the V2X environment, high data rate is one of the key characteristics. Support of a data rate of up to 1000Mbps is required in some cases with higher degree of automation, and high communication ranges (at least 1000m in some cases) need to be ensured.

For Advanced Driving, allowing vehicles to synchronize and coordinate their trajectories or maneuvers, and for Remote Driving, allowing to remotely control a vehicle, ultra-low latency and high reliability are required. It is required to support message exchange between a UE supporting V2X application and V2X application server for an absolute speed of up to 250 km/h and a minimum communication range up to 700m.

The requirements in TS 22.186 are dimensioned to enable advanced V2X services, beyond those already implemented in LTE Rel.15 V2X. Future vehicles will most likely be equipped with both LTE V2X and NR V2X, their respective services being complementary: basic safety applications may use LTE-V2X, while advanced applications would use NR-V2X.

The requirements in terms of latency, reliability and throughput are summarized in Table 1, by taking into account the considered scenario for each use case. An absolute speed up to 250 km/h needs to be supported. A sidelink communication must be possible both under coverage and out of coverage, similar to Modes 3 and 4 in LTE V2X.

Table 1 – Overview of V2X performance requirements in TS 22.186

| Use case | Max end-to-end latency (ms) | Reliability (%) | Payload (Bytes) | Data rate (Mbps) | Min. required communication range (meters) |
| --- | --- | --- | --- | --- | --- |
| Vehicles Platooning | 10-500 | 99.99 | 50-6000 | 50-65 | 80-350 |
| Extended Sensors | 3-100 | 95-99.999 | 1600 | 10-1000 | 200-1000 |
| Advanced Driving | 3-100 | 90-99.999 | 300-12000 | 10-53 | 360-700 |
| Remote Driving | 5 | 99.999 | - | UL:25/DL:1 | -- |

These requirements directly affect the sidelink design. A low latency, high reliability, high throughput, flexible and forward-compatible sidelink will rely on useful features from the NR toolkit such as flexible numerology, flexible slot design, multi-antenna techniques.

Observation 1: NR V2X will cover diverse use cases having different requirements in terms of throughput, coverage, reliability, latency.

NR sidelink is expected to offer high system capacity and extended coverage. It is desirable to have a unitary sidelink design for both unlicensed ITS bands and licensed bands in both FR1 and FR2. In LTE, sidelink connectivity takes place in the UL part of the spectrum for FDD, and in the UL subframes for FDD, mainly because of regulatory and implementation complexity reasons. LTE sidelink reuses the UL principles, having as baseline PUSCH design.

## Waveform assessment

### Considerations on previously conducted waveform comparison

In NR UL both DFTsOFDM and CP-OFDM are supported, being considered as complementary. CP-OFDM is oriented towards mid-cell throughput increase allowed by using MIMO techniques, while DFTsOFDM is targeted for link-budget limited scenarios. This decision was based on extensive simulation campaigns comparing DFTsOFDM and OFDM, taking into account SNR performance, behavior in the presence of a non-linear HPA, impact of PAPR reduction techniques, throughput, coverage, etc. The conclusions of those evaluations are still valid in a V2X sidelink scenario.

In [8] it is shown that in both narrowband and wideband allocation scenario, DFT-S-OFDM provides 2 dB improvement in link budget compared to OFDM with companding/clipping under the same emission requirements. Moreover, it is shown in [9] that applying PAPR reduction techniques does not change the advantage of DFTsOFDM.

Evaluations in [10] show that, with same PA, DFT-s-OFDM provides 1.5~2 dB coverage benefit over CP-OFDM at low spectral efficiency.

As stressed out in [11], it is known that with low PAPR waveforms, the power amplifier back-off can be reduced. Reducing the PA back-off is then conceptually equivalent to power boosting. Significant improvements in both mean and cell edge throughputs can be achieved with the power boosting that can be performed with a low PAPR waveform.

Observation 2: During NR UL evaluations, DFTsOFDM was reported to bring a gain of around 2dB over OFDM in link budget limited scenarios.

### PAPR advantage

For both control and data sidelink channels, robustness and coverage are design targets. Evaluations in a V2X scenario as described in the annex are aligned with the observations above and with results in [12]. As it can be seen from Figure 1, the PAPR advantage of DFTsOFDM is in the order of 2.7dB for QPSK, 2dB for 16QAM and 1.8dB for 64QAM respectively at target CCDF=10-4, and even higher at lower CCDF targets. The PAPR difference between DFTsOFDM with pi/2 BPSK and CP-OFDM with QPSK is around 4dB. The PAPR advantage is translated into increased robustness against non-linear distortion and thus into extended coverage range, particularly interesting for V2X communications.

Observation 3: The PAPR advantage of DFTsOFDM over CP-OFDM is in the order of 2.7dB for QPSK, 2dB for 16QAM and 1.8dB for 64QAM respectively. The PAPR difference between DFTsOFDM with pi/2 BPSK and CP-OFDM with QPSK is around 4dB.

### Performance on selective channels

The PAPR advantage of DFTsOFDM is significantly higher than the performance benefit of CP-OFDM, as it can be seen from Figure 2. In the ideal case of the absence of a HPA, CP-OFDM outperforms DFTsOFDM by around 0.6dB for 16QAM1/2 and 16QAM1/4, and the performance gap drops below 0.1dB for QPSK1/2 in the tested scenario. In the presence of a HPA, the performance of CP-OFDM degrades rapidly when driving the HPA closer to saturation due to the high PAPR of the CP-OFDM waveform, while DFTsOFDM displays a more robust behavior in the presence of a nonlinearity.

Observation 4: The performance advantage of CP-OFDM in the absence of a HPA is inferior to the PAPR advantage of DFTsOFDM.

Observation 5: DFTsOFDM is more robust to non-linearity.

### Total degradation: an approach to evaluate behavior in the presence of non-linear HPAs

The total degradation TD is defined as

TD=abs(OBO)+ΔSNR

where the OBO is the backoff with respect to the saturation value at the output of the HPA, and ΔSNR is the SNR loss at FER target 10-1 with respect to the ideal case where no HPA is present. The TD curve reflects the combined effect when jointly considering the HPA output back-off (OBO) and the SNR performance on a frequency selective channel. In regions far from the HPA saturation, ΔSNR≈0, but the TD is high because of the OBO loss. When close to saturation, the OBO loss is significantly reduced, but the SNR loss is high since the constellation is highly corrupted by the non-linear HPA. For each waveform and MCS, there is an optimum functioning point, where the TD is minimized.

Figure 3, Figure 5 and Figure 7confirm the fact that the PAPR advantage of DFTsOFDM is significantly more important than the performance difference due to behavior on frequency fading channels.

In Figure 7, for example, in the absence of any further constraint, the optimum functioning point of DFTsOFDM offers 1.7dB of TD gain over the optimum functioning point of CP-OFDM.

Taking things a step further, EVM and spectrum mask constraints need to be accounted for. Table 2 gives the minimum necessary backoff from saturation at the HPA output allowing complying with EVM and spectrum mask NR requirements in TS 38.101. These further constraints on the TD curves and give the functioning point for each waveform when the system is compliant with the NR requirements in TS 38.101.

DFTsOFDM waveform can be operated closer to saturation. The TD gain over OFDM is around 2dB for a given constellation.

Observation 6: For single layer transmission, when jointly considering the behavior in the presence of a non-linear HPA, the performance on frequency selective channel and the NR spectral normative constraints, the gain over OFDM is around 2dB.

Table 2 – Necessary OBO values in order to comply with EVM and spectrum mask NR requirements in TS 38.101

|  | EVM requirement | DFTsOFDM |  | CP-OFDM |  |
| --- | --- | --- | --- | --- | --- |
|  |  | OBO to reach EVM target | OBO to respect spectrum mask | OBO to reach EVM target | OBO to respect spectrum mask |
| QPSK | 17,5% | 3.9dB | 4.6dB | 6dB | 6.5dB |
| 16QAM | 12,5% | 5dB | 4.8dB | 6.75dB | 5.2dB |

### Considerations on coverage impact

For V2X waveform, the NR Uu interface, supporting both DFTsOFDM and CP-OFDM, should be the starting point.

As discussed in the previous section, NR V2X will cover a large number of use cases with various requirements. Vehicle platooning, for example, is mostly a proximity service, and the minimum required communication ranges scale from 80m to 350m. On the other hand, for advanced driving, a minimum communication range of 700m needs to be ensured, and at least 1000m are necessary for extended sensors, as required by TS 22.186.

As stated in the NR V2X SID [1], “NR system also expects to have higher system capacity and better coverage”. While applying MIMO techniques with CP-OFDM waveform for the sidelink data channel allows attaining higher system capacity, the use of DFTsOFDM is mandatory for achieving better coverage characteristics. Using CP-OFDM-based sidelink would result in the NR V2X system having a coverage inferior to the DFTsOFDM-based LTE V2X system.

Coverage is an important characteristic, as per the requirements of TS 22.186 and as justified in the SID. Coverage is a bottleneck when taking into considerations control and synchronization channels. Extended coverage is also an enabler for other use cases allowing the V2X device to communicate with UEs outside the vehicle (e.g. pedestrians).

DFTsOFDM provides important coverage gains over CP-OFDM. We have shown in the previous section that, at the same MCS, DFTsOFDM provides SNR gains around 2dB. Moreover, DFTsOFDM supports pi/2 BPSK, which can provide an extra 1.3dB with respect to DFTsOFDM with QPSK; the PAPR difference between DFTsOFDM with pi/2 BPSK and CP-OFDM with QPSK is around 4dB.

The link budget elements in TR 37.885 allow computing the required receiver sensibility depending on the distance between the transmitter and the receiver. To compute the required SNR per subcarrier we assumed a simulation setting as in Annex A and we assumed transmission of a packet of 200 bytes with QPSK1/2. Combined with the simulation results in Figure 2, it can be seen that CP-OFDM transmission range for achieving FER=10-1 with QPSK1/2 on a LOS/NLOSv highway channel is around 700m.

Table 3 – Required SNR per subcarrier for 222RB @5.9GHz for LOS/NLOSv Highway case

| Distance (m) | 100m | 200m | 300m | 400m | 500m | 600m | 700m | 800m | 900m | 1000m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Required SNR (dB) | 12.05 | 6.03 | 2.5 | 0.01 | -1.92 | -3.5 | -4.8 | -6.01 | -7.03 | -7.94 |

Observation 7: CP-OFDM coverage is insufficient with respect to NR V2X requirements expressed in TS 22.186.

In order to further evaluate the coverage difference between the two waveforms, we use the following procedure. We use the Max Coupling Loss (MCL) (Annex B.2 in TR 36.885) as indicated in the outcome of the e-mail discussion Draft R1-18XXXX summary 94-NR-04 v1.3:

MCL (dB) = maximum transmit power (23 dBm) + transmit antenna gain (3dBi) + receive antenna gain (3dBi) - thermal noise density (-174dBm/Hz) - receiver noise figure (9dB) - 10∙log10(occupied channel bandwidth (Hz)) - required SINR (dB))

The numerical values are given in Table 7.10.1-2 in TR 38.913 and others, such as the antenna gains and the occupied channel bandwidth, are specific to our simulation context. Required SNR value (including OBO) corresponds to a target frame error rate of 10%.

MCL (dB) = a + b*log10(MD) + c*log10(Fc)

where Fc is the carrier frequency in GHz and a, b, c are values that depend on the context (NLoS, Urban, Highway) according to TR 37.885 [15]:

Table 4 Parameters values of Path Loss

|  | a | b | c |
| --- | --- | --- | --- |
| NLoS | 36.85 | 30 | 18.9 |
| LOS/NLOSv Urban | 32.4 | 20 | 20 |
| LOS/NLOSv Highway | 38.77 | 16.7 | 18.2 |

From the two above equations of MCL, we obtain the maximum distance MD:

MD = 10(MCL-a-c*log Fc)b

Based on link budget computations and by assuming all other system parameters identical (antenna gains, carrier frequency, circuitry loss, receiver noise factors), a SNR difference can be directly related to a path loss difference,

ΔSNR=ΔPL=b*log10(MD)

where b is 20, 16.7 and 30 for LOS/NLOSv highway, LOS/NLOSv urban and NLOS cases respectively as defined in. As it can be seen from Table 5, the coverage extension provided by the SNR gains evaluated in the previous section is significant.

Table 5 – Estimated coverage improvement (in percentage) corresponding to different SNR gains

| ΔSNR (dB) | 1.5dB | 2dB | 3dB | 4dB |
| --- | --- | --- | --- | --- |
| LOS/NLOSv highway | 18.85% | 25.89% | 41.25% | 58.49% |
| LOS/NLOSv urban | 22.98% | 31.75% | 51.23% | 73.59% |
| NLOS | 12.20% | 16.59% | 25.89% | 35.94% |

Observation 8: A 2dB SNR gain ensures significant coverage extension estimated to range between 16.59% and 31.75% for different V2X typical channels.

In order to validate these estimates, Figure 4, Figure 6 and Figure 8 present a more thorough evaluation in several specific cases. We take into account the worst case scenario of a NLOS channel, for which a given SNR difference is translated in a minimal coverage difference (high multiplying factor b) and we compare the two waveforms for a same MCS, without taking into account the extra coverage gain that DFTsOFDM might achieve with e.g. BPSK pi/2.

The Maximum Distance (MD) is computed for the maximum Output Back-Off (OBO) value below the Error Vector Magnitude (EVM) limit (6.4.2.1 and Annex B.2 in TS 38.101-1) and the spectrum mask limit (Table 6.5.2.2-1 in TS 38.101-1). Table 2 summarizes the OBO values necessary to respect the EVM and spectrum mask limitations in the current simulation scenario.

Table 6 shows the relative coverage gain (MD) of DFT-s-OFDM over CP-OFDM corresponding to the simulations depicted in Figure 4, Figure 6 and Figure 8. In a worst case scenario, the minimum coverage gain achieved by DFTsOFDM over CP-OFDM is in the range of 17% with QPSK. With higher order modulations, the around 9% gain increases the range of those modulations, leading to system level throughput increase.

Table 6 - Relative coverage gain (MD) of DFT-s-OFDM over CP-OFDM

|  | 30kHz |  |  |
| --- | --- | --- | --- |
|  | 40kmph | 120kmph | 240kmph |
| QPSK 1/2 | 16,39% | 17,36% | 17,50% |
| 16QAM 1/2 | 9,82% | 8,93% | 9,09% |
| 16QAM 3/4 | 9,90% | 9,00% | 7,37% |

Observation 9: In a worst case scenario, with QPSK1/2 DFTsOFDM achieves a coverage extension of around 17% over CP-OFDM. Even higher coverage can be achieved with pi/2 BPSK.

### On common FR1/FR2 sidelink design

Another point to be considered is the SID objective to have a common sidelink design for both FR1 and FR2 [1]. While OFDM benefits from its good multiplexing capacity and better performance on highly frequency selective channels in FR1, DFTsOFDM has the advantage of a lower PAPR. Moreover, in FR2, the flatter nature of the channel and the fine beamforming highly diminish the advantages of OFDM, DFTsOFDM being more fit for the harsh propagation conditions.

From the perspective of specification effort, the design of an OFDM-based PSSCH from scratch is more challenging than a design based on DFTsOFDM, where elements from LTE PSSCH can be reused whenever possible.

Observation 10: Support of DFTsOFDM ensures a unified design between FR1 and FR2.

### Multiplexing options with DFTsOFDM

Some concerns about how to mix data with pilots in a PAPR-preserving manner for DFTsOFDM were voiced in the past meetings. The NR PTRS design, allowing pre-DFT RS insertion, offers a good starting point for a flexible DFTsOFDM-based RS design. We show in our companion contribution [14] that for equivalent DMRS density, pre-DFT RS insertion offers equivalent channel estimation performance with respect to OFDM-like scattered RS insertion, all in keeping the low PAPR property.

From a channel multiplexing point of view, the best option in order to strictly conserve the PAPR and to limit the decoding latency is to TDM PSSCH and PSCCH. A second option is the FDM multiplexing (in a LTE V2X-like manner), with increased latency and some PAPR penalty. The fact that the two channels are adjacent limits the PAPR loss, the overall PAPR of the transmitted signal being still significantly lower than with CP-OFDM.

Observation 11: Option 1b and Option 2 are appropriate PSCCH/PSSCH multiplexing options for DFTsOFDM.

### On DMRS design for DFTsOFDM

Regarding DFTsOFDM-based design, some concerns were voiced with respect to the possibility of multiplexing data and RS in the same symbol, without sacrificing the PAPR or the channel estimation performance.

The NR PTRS design, where data and RS are multiplexed before the DFT, offers a good starting point. The current PTRS patterns and densities are not fit for channel estimation due to their low density. But a PTRS-like design where a contiguous portion of the pre-DFT space carries RS with sufficient density opens the way to reliable channel estimation [16]. Preliminary results have shown that pre-DFT DMRS insertion with appropriate RS to data ratio yields very good channel estimation performance in a number of different cases [17].

It is confirmed in Figure 9 that, for equivalent DMRS density, pre-DFT RS insertion offers equivalent channel estimation performance with respect to OFDM-like scattered RS insertion, performance difference being under 0.05dB at FER target 10-1. Simulations in Figure 9 assume single DFTsOFDM symbol transmission with QPSK 1/3 and a DMRS overhead of 50%. In the case of scattered classical DM-RS insertion, the PAPR property is lost. In the case of pre-DFT RS insertion, the PAPR property is conserved.

Observation 12: Pre-DFT RS insertion for DFTsOFDM offers equivalent channel estimation performance with respect to OFDM-like scattered RS insertion without any PAPR loss.

To support high Doppler, dense time domain configurations are necessary, but care must be taken to the overall DMRS overhead. For DFTsOFDM, two examples on how to increase the time domain density of DMRS patterns without significantly changing the DMRS overhead with respect to a reference NR UL design are presented in Figure 10.

### Other general considerations

In the current state of the standing decisions, DFTsOFDM-based design and OFDM-based design can be developed in parallel, and DFTsOFDM design does not hinder in any way the progress on CP-OFDM, since it was decided that “single waveform is used in all the sidelink channels in a carrier” based on (pre-)configuration. The main reason for agreeing on CP-OFDM-based design was to allow MIMO processing based on FR1 considerations only. In FR2, where coverage and nonlinear behavior are critical, there is no technical reason for preferring CP-OFDM over DFTsOFDM, due to the flat nature of the channel and to the lack of need for any SU-MIMO techniques.

Proposal: Support DFTsOFDM at least in FR2.

# Conclusions

Based on the discussion in this contribution, we make the following observations:

Observation 1: NR V2X will cover diverse use cases having different requirements in terms of throughput, coverage, reliability, latency.

Observation 2: During NR UL evaluations, DFTsOFDM was reported to bring a gain of around 2dB over OFDM in link budget limited scenarios.

Observation 3: The PAPR advantage of DFTsOFDM over CP-OFDM is in the order of 2.7dB for QPSK, 2dB for 16QAM and 1.8dB for 64QAM respectively. The PAPR difference between DFTsOFDM with pi/2 BPSK and CP-OFDM with QPSK is around 4dB.

Observation 4: The performance advantage of CP-OFDM in the absence of a HPA is inferior to the PAPR advantage of DFTsOFDM.

Observation 5: DFTsOFDM is more robust to non-linearity.

Observation 6: For single layer transmission, when jointly considering the behavior in the presence of a non-linear HPA, the performance on frequency selective channel and the NR spectral normative constraints, the gain over OFDM is around 2dB.

Observation 7: CP-OFDM coverage is insufficient with respect to NR V2X requirements expressed in TS 22.186.

Observation 8: A 2dB SNR gain ensures significant coverage extension estimated to range between 16.59% and 31.75% for different V2X typical channels.

Observation 9: In a worst case scenario, with QPSK1/2 DFTsOFDM achieves a coverage extension of around 17% over CP-OFDM. Even higher coverage can be achieved with pi/2 BPSK.

Observation 10: Support of DFTsOFDM ensures a unified design between FR1 and FR2.

Observation 11: Option 1b and Option 2 are appropriate for PSCCH/PSSCH mux. for DFTsOFDM.

Observation 12: Pre-DFT RS insertion for DFTsOFDM offers equivalent channel estimation performance with respect to OFDM-like scattered RS insertion without any PAPR loss.

Proposal: Support DFTsOFDM at least in FR2.

# References

### RP-181429 Study on NR V2X, RAN#80, June 2018.

### 3GPP TS 22.186 Enhancement of 3GPP support for V2X scenarios.

### RAN1#94 Chairman’s notes, August 2018.

### RAN1#94b Chairman’s notes, October 2018.

### RAN1 NR-AH#1901 Chairman’s notes, January 2019.

### R1-1901147 Waveform evaluations for V2X sidelink, Mitsubishi Electric, RAN1 NR-AH#1901, January 2019.

### 3GPP TR 37.885 Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR.

### R1-1610113, Coverage analysis of DFT-s-OFDM and OFDM with low PAPR techniques, Qualcomm, RAN1#86, October 2016.

### R1-1610117, Common PAPR reduction techniques for OFDM / DFT-s-OFDM, Qualcomm, RAN1#86, October 2016.

### R1- 1609222, Discussion on DFT-s-OFDM and CP-OFDM for Uplink, LG Electronics, RAN1#86, October 2016.

### R1-1609598, Potential for system level gains with low PAPR waveforms, Nokia-ALSB, RAN1#86, October 2016.

### R1-1810476, On physical layer sidelink design for NR V2X, Mitsubishi Electric, RAN1#94b, October 2018.

### R1-1808203, On sidelink design for NR V2X, Mitsubishi Electric, RAN1#94, August 2018.

### R1-1901144, Physical layer design for NR V2X sidelink, Mitsubishi Electric, RAN1 NRAH#1901, January 2019.

### TR 37.885, Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR (v15.1.0, 2018-09).

### A. Bouttier, J. Guillet, A low-PAPR low-pilot overhead Reference Signal for the DFT-spread OFDM Modulation, ICT 2018.

### R1-1717759, On pre-DFT additional RS insertion for long PUCCH, Mitsubishi Electric, RAN1#90b, October 2017.

# Annex A – Simulation assumptions

Simulations are performed having as baseline the Rel-15 NR PUSCH design with following parameters:

| Carrier frequency | 5.9GHz |
| --- | --- |
| SCS/BW | 15kHz/50MHz; 30kHz/100MHz; 60kHz/100MHz:; 120kHz/200MHz |
| CP | Normal |
| Channel | CDL-A-based NLOS 160ns from Table 6.2.3-1, TR 37.885 |
| DMRS configuration | NR-like configuration type 1, PUSCH mapping type A (slot size 12), PUSCH mapping type B (slot size 4, 7) |
| Speed | 20kmph vs 20kmph (relative speed: 40kmph) 60kmph vs 60kmph (relative speed: 120kmph) 140kmph vs 140kmph (relative speed: 280kmph) |
| FEC | NR LDPC with 50 decoding iterations |
| CFO | 0.1ppm at Tx and -0.1ppm at Rx |
| HPA | Polynomial, variable IBO |
| Packet size | 200bytes, 400bytes, 800bytes, 1000bytes |

The packet size and coding rate are kept constant, the number of occupied RBs varies with MCS, slot size and DMRS configurations in the limit of the max bandwidth size as follows

Table 7 – Number of occupied RBs

| Slot | #DMRS | MCS | 200bytes | 400bytes | 600bytes | 800 bytes | 1000 bytes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 1 | QPSK ½ | 222 | - | - |  | - |
|  |  | 16QAM ½ | 112 | - | - | 268 | - |
|  |  | 16QAM ¾ | 44 | - | - | 178 | - |
| 7 | 1 | QPSK ½ | 112 | - | - | 268 | - |
|  |  | 16QAM ½ | 56 | - | - | 134 | - |
|  |  | 16QAM ¾ | 22 | - | - | 90 | - |
|  | 2 | QPSK ½ | 134 | 268 | - | - | - |
|  |  | 16QAM ½ | 68 | 134 | - | - | - |
|  |  | 16QAM ¾ | 28 | 54 | - | - | - |
| 12 | 1 | QPSK ½ | 62 | 122 | - | - | - |
|  |  | 16QAM ½ | 30 | 62 | - | - | - |
|  |  | 16QAM ¾ | 12 | 24 | - | - | - |
|  | 2 | QPSK ½ | 68 | 134 | - | - | - |
|  |  | 16QAM ½ | 34 | - | 60 | - | - |
|  |  | 16QAM ¾ | 14 | - | 40 | - | - |
|  | 4 | QPSK ½ | 84 | - | - | - | 250 |
|  |  | 16QAM ½ | 42 | - | - | - | 126 |
|  |  | 16QAM ¾ | 18 | - | - | - | 84 |

# Annex B – Simulation results

Figure 1 – CCDF of instantaneous normalized power of DFTsOFDM and OFDM for QPSK, 16QAM, 64QAM

Figure 2 – Comparative performance of DFTsOFDM and CP-OFDM, QPSK1/2, 16QAM1/2, 16QAM3/4, 200 bytes (back-off advantage not included)

| Figure 3 TD of QPSK ½, SCS=30kHz | Figure 4 MD of QPSK ½, SCS=30kHz |
| --- | --- |
| Figure 5 TD of 16QAM ½, SCS=30kHz | Figure 6 MD of 16QAM ½, SCS=30kHz |

| Figure 7 TD of 16QAM ¾, SCS=30kHz | Figure 8 MD of 16QAM ¾, SCS=30kHz |
| --- | --- |
| Figure 9 – 1-symbol DFTsOFDM with scattered DMRS vs pre-DFT DMRS |  |

Figure 10 – Dense time domain DMRS patterns for DFTsOFDM: reference NR configuration with 28.5% overhead (up), DMRS in every symbol with 33% overhead (middle), DMRS in every other symbol with 21.4% overhead (down).