# 3GPP TSG RAN WG1 Meeting #96		                                        R1-1901974

Athens, Greece, 25th February – 1st March, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | CRNum | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on SRS to TS38.211 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2019-1-30 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS38.211, it is specified that the SRS port index of an SRS resource is pi=1000+i , where i=0, …, NapSRS with NapSRS is the number of SRS antenna ports in the SRS resource. It means for a SRS resource with one SRS port, the SRS port is indexed as pi=1000 . In TS38.214, it is specified for non-codebook based UL transmission that ‘The UE shall transmit PUSCH using the same antenna ports as the SRS port(s) in the SRS resource(s) indicated by SRI(s) given by DCI format 0_1 or by configuredGrantConfig according to subclause 6.1.2.3, where the SRS port in (i+1)-th SRS resource in the SRS resource set is indexed as . ’ Therefore, the defination of SRS port index in TS38.211 and TS38.214 are not aligned for SRS resources in the SRS resource set with high layer parameter usage set to ‘nonCodebook’. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify in TS38.211 that the port index of SRS for a SRS resource in the SRS resource set with usage set to ‘nonCodebook’ is determined according to TS38.214. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The defination of SRS port index in TS38.211 and TS38.214 are not aligned for SRS resources in the SRS resource set with high layer parameter usage set to ‘nonCodebook’. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

- 6.4.1.4.1	SRS resource

An SRS resource is configured by the SRS-Resource IE and consists of

-	 antenna ports pii=0NapSRS-1, pi=1000+iwhere the number of antenna ports is given by the higher layer parameter nrofSRS-Ports, and pi=1000+i when the SRS resource is not in a SRS resource set with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', or pi is determined according to [6, TS 38.214] when the SRS resource is in a SRS resource set with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook'

-	 consecutive OFDM symbols given by the field nrofSymbols contained in the higher layer parameter resourceMapping

-	, the starting position in the time domain given by  where the offset counts symbols backwards from the end of the slot and is given by the field startPosition contained in the higher layer parameter resourceMapping and loffset≥NsymbSRS-1

-	, the frequency-domain starting position of the sounding reference signal