3GPP TSG RAN WG1 Meeting #96                                                                                      R1-1902016

Athens, Greece, 25th February – 1st March, 2019

Source:	CATT

Title:	Self-evaluation on Mobility for LTE

Agenda Item:	7.2.7.2

Document for:	Discussion and Decision

- Introduction

At RAN#75 meeting, Study Item on Self Evaluation towards IMT-2020 submission was approved [1]. Self-Evaluation will provide the performance towards all the ITU-R IMT-2020 requirements as defined in Report ITU-R M.2410 [2]. High-level assessment methods for these evaluation characteristics are given in § 6 of Report ITU-R M.2412 [3]. The evaluation results of mobility for eMBB (Indoor Hotspot-eMBB, Dense Urban-eMBB, and Rural-eMBB) in NR have been provided in RAN1#94[4].

In this contribution, we provide the partial preliminary mobility evaluation for rural-eMBB scenario in LTE (based on Rel-15), including UL pre-processing SINR CDF and preliminary self-evaluation results for mobility.

- Pre-processing SINR CDF and median SINR

At RAN1#92bis meeting, it was agreed that for SU-MIMO pre-processing SINR as defined in [5] is achieved in SLS. In this section, we provided the preliminary evaluation results of UL preprocessing SINR in Rural-eMBB.

The preliminary UL median pre-processing SINR results for LTE Rural-eMBB are listed in Table 1. System level simulation assumptions are listed in Appendix 1.

Figure 1: UL pre-processing SINR distribution for Rural-eMBB Config A with channel model A and B

Table 1. UL median pre-processing SINR

| Test environment | Configuration | Channel Model | 50%-tile UL pre-processing SINR |
| --- | --- | --- | --- |
| Rural-eMBB | Config A 120km/h | Model A | 12.87 |
|  |  | Model B | 12.78 |
|  | Config A 500km/h | Model A | 12.85 |
|  |  | Model B | 12.77 |

- Evaluation results for mobility

Based on the pre-processing SINR provided in the Table 1, the preliminary self-evaluation results for mobility in Rural-eMBB test environment for LTE are provided in Table 2. Besides ITU defined link level parameters of Mobility, some additional link level assumption parameters applied in this preliminary evaluation can be found in Table 3.

Table 2. Mobility traffic channel link data rates

| Test environment | Configuration | Channel Model | Median UL SINR[dB] | ITU Requirement [bps/Hz] | Spectrum efficiency of NLOS | Spectrum efficiency of LOS | Spectrum efficiency of NLOS | Spectrum efficiency of LOS | Fulfil the requirements |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | FDD |  | TDD |  |  |
| Rural-eMBB (120km/h) | Config A 700MHz | Model A | 12.87 | 0.8 | 2.7500 | 2.9052 | 2.6424 | 2.7915 | Y |
|  |  | Model B | 12.78 |  | 2.7342 | 2.9040 | 2.6272 | 2.7903 | Y |
| Rural-eMBB (500km/h) | Config A 700MHz | Model A | 12.85 | 0.45 | 1.8224 | 2.3877 | 1.7511 | 2.2943 | Y |
|  |  | Model B | 12.77 |  | 1.8161 | 2.3784 | 1.7450 | 2.2853 | Y |

Table 3. The additional link level simulation assumption for Rural-eMBB test environment in LTE

| Parameters | Value |
| --- | --- |
| Carrier frequency | 700 MHz |
| Waveform | DFT-S-OFDM |
| Duplexing | FDD， TDD: DSUUD |
| Simulation bandwidth | 10 MHz |
| Link-level Channel model | CDL-iii |
|  | CDL-v |
| UE speed | 120 km/h, 500km/h |
| Subcarrier spacing | 15kHz |
| Symbols number per slot | 14 |
| Antenna configuration at TRxP | 8R |
| Antenna configuration at UE | 1T |
| Transmission rank | 1 |
| TRxP receiver type | MMSE |
| Channel estimation | Real |
| Number of subcarriers per PRB | 12 |
| Data allocation | 14 symbol slots, with 12 RB allocated |
| Channel coding scheme | Turbo |
| Link adaptation | No link adaptation |
| HARQ | Max 4 HARQ tansmissions |
| DMRS configuration | 2 symbol DMRS |
| Other overhead | FDD: SRS： 1 complete symbols with 5ms period (except the PRB with PUCCH) PUCCH：2PRB pairs with 9 subframes and 4 PRB pair with 1 subframe TDD: SRS：2 complete symbols in UpPTS with 5ms period, PUCCH：2PRB pairs with 3 subframes and 4 PRB pair with 1 subframe |

According to above self-evaluation results, the achieved normalized traffic channel data rate in Rural-eMBB scenario for LTE are higher than the normalized traffic data rate for both LOS and NLOS. Thus, the IMT-2020 mobility requirements in Rural-eMBB test environment can be fulfilled in LTE system (based on Rel-15 LTE).

Observation: The achieved normalized traffic channel data rates in Rural-eMBB scenario are higher than the required spectrum efficiency values. Hence, the IMT-2020 mobility requirements are fulfilled in LTE.

- Conclusion

Based on the pre-processing SINR provided in the Table 1, the achieved normalized traffic data rate in Rural-eMBB scenario for LTE can fulfill the IMT-2020 mobility requirements for both LOS and NLOS.

Observation: The achieved normalized traffic channel data rates in Rural-eMBB scenario for LTE are higher than the required spectrum efficiency values. Hence, the IMT-2020 mobility requirements in Rural-eMBB scenario are fulfilled in LTE.

- References

- RP-170832, “New Study Item on Self Evaluation towards IMT-2020 submission”, Huawei, Ericsson.

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

- Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

- R1-1808428, “Self-evaluation on Mobility”, CATT

- R1-1805643, “Summary of discussion on mobility evaluation method and parameters”, Huawei

Appendix 1: Simulation assumption of SLS part for mobility

The simulation assumption of system level part for mobility evaluation is listed in Table A1-1.

Table A1-1. Simulation assumptions in SLS

|  | Rural - eMBB |
| --- | --- |
| Evaluation configuration | Configuration A |
| Carrier frequency for evaluation | 700 MHz |
| Multiple access | OFDMA |
| Duplexing | FDD |
| Transmission scheme | SU-MIMO with rank 1 |
| BS antenna height | 35 m |
| Total transmit power per TRxP | 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm |
| Percentage of high loss and low loss building type | 100% low loss (applies to Channel model B) |
| Inter-site distance | 1732 m |
| Number of antenna elements per TRxP | 64 Tx/Rx, (M,N,P,Mg,Ng) = (8,4,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) |
| Number of UE antenna elements | 1Tx/Rx, (M,N,P,Mg,Ng) = (1,1,1,1,1) |
| Device deployment | 50% indoor, 50% outdoor (in car) Randomly and uniformly distributed over the area |
| UE speeds of interest | 120 km/h, 500km/h |
| Traffic model | Full buffer |
| Simulation bandwidth | 10 MHz |
| UE density | 10 UEs per TRxP |
| UE antenna height | 1.5 m |
| Channel model variant | Alt. 1: Channel model A Alt. 2: Channel model B |
| TRxP number per site | 3 |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | 100° in LCS |
| Handover margin (dB) | 1 |
| TRxP boresight | 30 / 150 / 270 degrees |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Wrapping around method | Geographical distance based wrapping |
| Minimum distance of TRxP and UE | d2D_min=10m |
| Polarized antenna model | Model-2 in TR36.873 |
| Power control parameters | a = 0.8, P0 = -76 dBm |
| Numerology | 15 kHz SCS |
| Scheduling | PF |
| Receiver | MMSE-IRC |
| Pre-processing SINR calculation | Aligned with Section 2.1.1 in R1-1805643 |