3GPP TSG RAN WG1 #96	R1-1902322
Athens, Greece, 25th February – 1st March, 2019

Source:				Panasonic

Title:	NR-U PRACH design

Agenda item:	7.2.2.1.1

Document for:			Discussion and Decision

Introduction

This document is the update of [1], and discusses the following NR-U PRACH aspects:

PRACH format

PRACH interlace mapping

PRACH format

In NR Rel-15, PRACH sequence length L=139 and 839 are supported. In NR-U TR [2], L=839 is identified not to be beneficial for NR-U. We would propose to support L=139 only as we don’t see the benefit to support the other length.

Proposal 1:	PRACH sequence length L=139 is supported for NR-U.

PRACH interlace mapping

This chapter is the resubmission of [1].

In 3GPP TR38.889, following four potential design alternatives have been identified for frequency mapping of PRACH [2]. In this section, we compare the each interlace design, in viewpoint of timing estimation accuracy, CM, multiplexing with other signals channels, PSD and OCB requirement.

- Alt-1: Uniform PRB-level interlace mapping (B-IFDM+ uniform spacing)

- Alt-2: Non-uniform PRB-level interlace mapping (B-IFDM + non-uniform spacing) [1]

  - It has been identified that an irregular mapping (non-equal spacing of PRBs/REs) in frequency domain

- Alt-3: Uniform RE-level interlace mapping

- Alt-4: No interlacing mapping (contiguous only)

Timing estimation accuracy

The Figure 1 shows the evaluation results on the auto correlation properties using Zadoff-Chu sequence where a length-113 Zadoff-Chu sequence is cyclically extended to length-120. In the evaluation, 20MHz bandwidth and 30 kHz SCS are assumed and the resource mapping of B-IFDM as Figure 2 is used. In this document, B-IFDM + non-uniform spacing means the mapping method that the different interlace number can be set for each cluster. In the Figure 2 (b), interlace number {0, 2, 1, 0, 1, 2, 0, 2, 1, 0} is used for each cluster as an example of B-IFDM + non-uniform spacing (Alt-2). In the Figure 2 (a), the B-IFDM + uniform spacing (Alt-1) is also shown. RE interval is set to 5 for uniform RE-level interlace mapping (Alt-3) in the evaluation. From the evaluation results, Alt-4 shows better auto correlation property compared with the other methods. The correlation properties of PRACH preamble using the B-IFDM + uniform spacing (Alt-1) have some issues in timing estimation, because there are many false peaks in the correlation properties. Alt-2 (B-IFDM + non-uniform) shows better auto correlation property compared with Alt-1. In Alt-3, preamble is repeated by RE interval in time domain, therefore false peak occurs at the interval obtained by dividing the symbol length by the RE interval. For 30kHs SCS with short preamble format and RE interval as 5, the correlation peak of figure 1 are occurred in each 6.6 us. If the number of cyclic shift of PRACH preamble is 1, around 1 km cell size can be supported. If the number of cyclic shift of PRACH preamble is 6, around 150m cell size can be supported. These false peak can constrain or make the deployment more complex. Therefore, we suggest not to take Alt-3.

Figure 1 Auto correlation properties of PRACH preamble

| (a) B-IFDM + uniform spacing | (b) B-IFDM + non-uniform spacing |
| --- | --- |

Figure 2 Interlace mapping method of B-IFDM + uniform spacing and B-IFDM + non-uniform spacing in 30KHz SCS (the number of interlaces is 5, the nominal number of PRBs per interlace is 10PRB)

CM

The Table 1 shows the evaluation results of CM. In this evaluation, the resource mapping of B-IFDM as Figure 2 is used. According to this, Alt-3 and Alt-4 have better CM than Alt-1 and Alt-2. However, it would be not so big issue as the targeted cell size is relatively small in NR-U compared with licensed band operation.

Table 1 CM of PRACH preamble

| Interlace design | CM [dB] |
| --- | --- |
| Alt-1 (Uniform PRB-level interlace mapping) | 2.17 |
| Alt-2 (Non-uniform PRB-level interlace mapping) | 3.44 |
| Alt-3 (Uniform RE-level interlace mapping) | 1.80 |
| Alt-4 (No interlacing mapping) | 1.80 |

Multiplexing with other signals channels

In the scenario which B-IFDM is used for PUCCH/PUSCH, Alt-1 can multiplex with PUCCH/PUSCH with B-IFDM easily. In Alt-2, if nested resources are allocated for multiple PRACH occasions as Figure 3, Alt-2 can also multiplex with PUCCH/PUSCH with B-IFDM easily. For example, interlace#0-2 are used for PRACH, and other interlace numbers are used for PUCCH and PUSCH. Alt-3 and Alt-4 can’t multiplex PUCCH/PUSCH easily.

Figure 3 PRACH resource allocation for multiple PRACH occasions

PSD and OCB requirement

Alt-1, Alt-2 and Alt-3 can fulfil OCB requirement and increase transmission power more than Alt-4 under PSD limitation. Alt-4 can’t fulfil OCB requirement, therefore frequency domain repetition of preamble or temporal allowance of at least 2MHz OCB mechanism have to be applied.

Summary of the comparison of each interlace design

Based on the discussion above, the Table 2 shows the summary of the comparison of each interlace design in the scenario which B-IFDM is used for PUCCH/PUSCH. From the viewpoint of multiplexing with other channel and timing estimation accuracy, our proposal is the following.

Table 2 Summary of the comparison of each interlace design

| Interlace design | Multiplexing with other channel | Timing estimation accuracy | CM | PSD/OCB requirement |
| --- | --- | --- | --- | --- |
| Alt-1 (Uniform PRB-level interlace mapping) | Excellent | bad | good | Excellent |
| Alt-2 (Non-uniform PRB-level interlace mapping) | good | fair | fair | Excellent |
| Alt-3 (Uniform RE-level interlace mapping) | Bad | Good | Excellent | Excellent |
| Alt-4 (No interlacing mapping) | Bad | Excellent | Excellent | Bad |

Proposal 2:	For NR-U PRACH, support B-IFDM + non-uniform spacing in the scenario which B-IFDM is used for PUCCH/PUSCH.

Conclusion

For NR-U PRACH, we would have the proposals as follow:

| Proposal 1 | PRACH sequence length L=139 is supported for NR-U. |
| --- | --- |
| Proposal 2 | For NR-U PRACH, support B-IFDM + non-uniform spacing in the scenario which B-IFDM is used for PUCCH/PUSCH. |

Reference

R1-1900239, Panasonic, RAN1 Ad-Hoc Meeting 1901, Taipei, January 2019

3GPP TR 38.889 V16.0.0

Appendix – Agreements from prior Meetings

The following PRACH channel design for NR-based Access to Unlicensed Spectrum (NR-U) were captured in 3GPP TR38.889 [2]:

It has been identified that the long PRACH sequence length defined in NR Rel-15 (L = 839) is not beneficial for NR-U, since PRACH formats based on this length are tailored toward large cells not expected in an NR-U deployment. However, when it comes to shorter sequence lengths, some sources propose reusing the short sequence length (L = 139) defined in NR-Rel-15, whereas other sources propose defining new sequence lengths depending on which of the 4 alternatives above is supported.

Four potential design alternatives, including no interlacing, have been identified for the frequency mapping of PRACH sequences for NR-U, where consensus on which one(s) to support for NR-U has not yet been achieved: - Alt-1: Uniform PRB-level interlace mapping - In this approach a PRACH sequence for a particular PRACH occasion is mapped to all of the PRBs of one or more of the interlaces in the PRB-based block interlace structure. Within a PRB, either all or a subset of REs are used. Different PRACH occasions are defined using an orthogonal set of PRBs, or an orthogonal set of REs within the PRBs, from one or more same/different interlaces. - It has been identified that a uniform mapping (equal spacing of PRBs) in the frequency domain produces a zero-autocorrelation zone, of which the duration is inversely proportional to the frequency spacing between the PRBs. - Alt-2: Non-uniform PRB-level interlace mapping - In this approach a PRACH sequence for a particular PRACH occasion is mapped to some or all of the PRBs of one or more of the interlaces in the same PRB-based block interlace structure used for PUSCH/PUCCH. Within a PRB, either all or a subset of REs are used. Different PRACH occasions are defined using an orthogonal set of PRBs, or an orthogonal set of REs within the PRBs, from one or more same/different interlaces. - It has been identified that an irregular mapping (non-equal spacing of PRBs/REs) in the frequency domain reduces the false peaks in the PRACH preamble auto-correlation function. - Alt-3: Uniform RE-level interlace mapping - In this approach, a PRACH sequence for a particular PRACH occasion consists of a "comb-like" mapping in the frequency domain with equal spacing between all used REs. Different PRACH occasions are defined by way of different comb offsets. - Since this approach does not fit with the common PUSCH/PUCCH interlace structure, one source suggests that only TDM multiplexing of PUSCH/PUCCH and PRACH should be supported. Another source suggests that puncturing/rate matching PUSCH/PUCCH around the used PRACH REs may be used. - Alt-4: Non-interlaced mapping - In this approach, a PRACH sequence for a particular PRACH occasion is mapped to a number of contiguous PRBs, same or similar to NR Rel-15. - Some sources propose that to fulfill the minimum OCB requirement, that the PRACH sequence is mapped to a set of contiguous PRBs, and the PRACH sequence mapping is repeated across the frequency domain, potentially with guard RE(s)/PRB(s) between repetitions. For each repetition, a different cyclic shift or different base sequence may or may not be applied.