3GPP TSG RAN WG1 Meeting #96	R1-1903418

Athens, Greece, February 25th – March 1st, 2019

Agenda Item:	7.1.2

Source:	Huawei, HiSilicon

Title:	Maintenance for CSI-RS #1

Document for:	Discussion and Decision

Introduction

This paper summarizes the maintenance proposals for CSI-RS submitted to RAN1#96, as listed below:

R1-1901760	Draft CR for correction of ZP CSI-RS trigger in 38.212	ZTE

R1-1901764	Draft CR on description of ZP CSI-RS resource configuration	ZTE

R1-1902226	Draft CR on NR CSI-RS	Samsung

R1-1901615	Correction on AP-CSI-RS triggering offset in 38.214		Huawei, HiSilicon

R1-1901617	Correction on CSI-RS configuration in 38.214	Huawei, HiSilicon

Maintenance for CSI-RS

RRC parameter name for ZP CSI-RS

R1-1901760	Draft CR for correction of ZP CSI-RS trigger in 38.212	ZTE

Description:

It is reported in R1-1901760 that the RRC parameter name indicating the number of configured ZP CSI-RS resource sets in 382.212 is wrong. It is then proposed to refer to another RRC parameter as below.

Draft CR (for the convenience of reviewing):

For Section 7.3.1.2.2 in 38.212:

< Start of text proposal > - ZP CSI-RS trigger – 0, 1, or 2 bits as defined in Subclause 5.1.4.2 of [6, TS 38.214]. The bitwidth for this field is determined as bits, where is the number of ZP CSI-RS resource sets in the higher layer parameter aperiodic-ZP-CSI-RS-ResourceSetsToAddModList. < End of text proposal >

Views from companies:

| Huawei, HiSilicon | It seems incorrect to refer to aperiodic-ZP-CSI-RS-ResourceSetsToAddModList directly, as it stands for the ZP CSI-RS resource sets to be added in addition to previously configured ZP CSI-RS resource sets, instead of the ZP CSI-RS resource sets that is currently maintained by UE. One alternative solution is to remove ‘in the higher layer parameter zp-CSI-RS-Resource’. In addition, there is no analysis on isolated impacts, which should be added. |
| --- | --- |
| ZTE | We are fine to remove the incorrect description as follows where is the number of ZP CSI-RS resource sets If other companies are fine, I will update this tdoc |
| Samsung | Agree in principle. We suggest the following revision where is the number of ZP CSI-RS resource sets configured by higher layer. |
| ZTE2 | The updated tdoc is R1-1903390 including isolated impact. where is the number of aperiodic ZP CSI-RS resource sets. Isolated impact analysis: This CR corrects one error in 38.212 and no implementation change is introduced. |
| Ericsson | Also agree in principle, but would suggest the further refined wording, similar to SS comment: where is the number of aperiodic ZP CSI-RS resource sets configured by higher layer. |
| ZTE3 | OK with E///’s wording. The updated tdoc R1-1903390 will be based on E///’s wording. |

FL recommendation (to be updated if needed):

R1-1901760	Draft CR for correction of ZP CSI-RS trigger in 38.212	ZTE

The draft CR is to correct an RRC parameter name without functional change. The updated draft CR in “R1-1903390.docx.zip” has some unnecessary change marks. Further offline.

Description of parameters for ZP CSI-RS

R1-1901764	Draft CR on description of ZP CSI-RS resource configuration	ZTE

Description:

It is reported in R1-1901764 that the description of parameters for ZP CSI-RS in 38.214 can be better structured. It is then proposed to revise the description in 38.214 as below.

Draft CR (for the convenience of reviewing):

For Section 5.1.4.2 in 38.214:

< Start of text proposal > - zp-CSI-RS-ResourceId in ZP-CSI-RS-Resource determines ZP CSI-RS resource configuration identity. - - resourceMapping given by ZP-CSI-RS-Resource defines the number of CSI-RS ports, CDM values and pattern, and the OFDM symbol and subcarrier occupancy of the ZP-CSI-RS resource within a slot that are given in Subclause 7.4.1.5 of [4, TS 38.211]. The parameters NrofPorts, cdm-Type, density and freqBand are provided in resourceMapping as described in Subclause 5.2.2.3.1 < End of text proposal >

Views from companies:

| Huawei, HiSilicon | Seems not needed, as it is editorial restructuring and there won’t be misunderstanding if this CR is not approved. |
| --- | --- |
| ZTE | To Huawei, at least ‘CDMType’ should be ‘cdm-Type’. This should be corrected. Anyway, this is one CR which can also be used to include parameters density and freqBand. Suggest isolated impact: This CR just corrects the errors and make the description in 38.214 align with 38.331. No implementation change is introduced. |
| Ericsson | Although this is mainly editorial, we should correctly reference the RRC parameters. |

FL recommendation (to be updated if needed):

R1-1901764	Draft CR on description of ZP CSI-RS resource configuration	ZTE

The draft CR is to correct an RRC parameter name without functional change. The isolated impact analysis is missing. Further offline.

Transmission timing for CSI-RS for mobility

R1-1902226	Draft CR on NR CSI-RS	Samsung

Description:

It is reported in R1-1902226 that the transmission timing for CSI-RS for mobility is undefined, as the current description in 38.211 is applicable to CSI-RS with high-layer parameter resourceType configured, while this parameter does not exist for CSI-RS for mobility. It is then proposed to extend the description in 38.211 to define transmission timing for CSI-RS for mobility.

Draft CR (for the convenience of reviewing):

For Section 7.4.1.5.3 in 38.211:

< Start of text proposal > The UE shall assume that a CSI-RS is transmitted using antenna ports numbered according to where is the sequence index provided by Tables 7.4.1.5.3-2 to 7.4.1.5.3-5, is the CDM group size, and is the number of CSI-RS ports. The CDM group index given in Table 7.4.1.5.3-1 corresponds to the time/frequency locations for a given row of the table. The CDM groups are numbered in order of increasing frequency domain allocation first and then increasing time domain allocation. For a CSI-RS resource configured as periodic or semi-persistent by the higher-layer parameter resourceType or configured by the higher-layer parameter CSI-RS-CellMobility, the UE shall assume that the CSI-RS is transmitted in slots satisfying where the periodicity (in slots) and slot offset are obtained from the higher-layer parameter CSI-ResourcePeriodicityAndOffset or slotconfig. The UE shall assume that CSI-RS is transmitted in a candidate slot as described in clause 11.1 of [5, TS 38.213]. < End of text proposal >

Views from companies:

| Huawei, HiSilicon | There is no analysis on isolated impacts, which should be added. |
| --- | --- |
| Samsung | We think “Inconsistent specification between TS38.331 and TS38.211.” is enough for the analysis. Please let us know, if any other views. |
| Ericsson | OK |

FL recommendation (to be updated if needed):

R1-1902226	Draft CR on NR CSI-RS	Samsung

The draft CR is to clarify that the specified CSI-RS timing is applicable to CSI-RS for mobility. The isolated impact analysis is missing. Further offline.

Triggering offset for aperiodic CSI-RS

R1-1901615	Correction on AP-CSI-RS triggering offset in 38.214		Huawei, HiSilicon

Description:

It is reported in R1-1901615 that the value range of triggering offset for aperiodic CSI-RS in 38.214 is misaligned with 38.331, where in addition to {0, 1, 2, 3, 4}, two extra values of {16, 24} slots are added. It is then proposed to revise the description in 38.214 to align with 38.331.

Draft CR (for the convenience of reviewing):

For Section 5.2.1.5.1 in 38.214:

< Start of text proposal > When aperiodic CSI-RS is used with aperiodic reporting, the CSI-RS offset is configured per resource set by the higher layer parameter aperiodicTriggeringOffset. The CSI-RS triggering offset has the values of {0, 1, 2, 3, 4, 16, 24} slots. If all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states, the CSI-RS triggering offset is fixed to zero. The aperiodic triggering offset of the CSI-IM follows offset of the associated NZP CSI-RS for channel measurement. < End of text proposal >

Views from companies:

| Samsung | Ok |
| --- | --- |
| vivo | We have similar CR. Moreover, there are default QCL beam issues related to these offsets and may need to be discussed in beam management part. |
| Ericsson | Support, this correction is needed. |

FL recommendation (to be updated if needed):

R1-1901615	Correction on AP-CSI-RS triggering offset in 38.214		Huawei, HiSilicon

The draft CR is agreed, the final CR is in R1-19xxxxx.

Common configuration for resources within a set

R1-1901617	Correction on CSI-RS configuration in 38.214	Huawei, HiSilicon

Description:

It is reported in R1-1901617 that the agreement on having the same starting RB and number of RBs and CDM-type for CSI-RS resources within a set has not been captured. It is then proposed to capture this agreement in 38.214.

Agreement

The UE expects that all the CSI resources of a resource set are configured with the same starting RB and number of RBs and the same CDM-type.

Draft CR (for the convenience of reviewing):

Fro Section 5.2.2.3.1 in 38.214:

< Start of text proposal > All CSI-RS resources within one set are configured with same density and same nrofPorts, except for the NZP CSI-RS resources used for interference measurement. The UE expects that all the CSI-RS resources of a resource set are configured with the same starting RB and number of RBs and the same CDM-type. The bandwidth and initial common resource block (CRB) index of a CSI-RS resource within a BWP, as defined in Subclause 7.4.1.5 of [4, TS 38.211], are determined based on the higher layer parameters nrofRBs and startingRB, respectively, within the CSI-FrequencyOccupation IE configured by the higher layer parameter freqBand within the CSI-RS-ResourceMapping IE. Both nrofRBs and startingRB are configured as integer multiples of 4 RBs, and the reference point for startingRB is CRB 0 on the common resource block grid. If startingRB<NBWPstart, the UE shall assume that the initial CRB index of the CSI-RS resource is Ninitial RB=NBWPstart, otherwise Ninitial RB=startingRB. If nrofRBs>NBWPsize+NBWPstart-Ninitial RB, the UE shall assume that the bandwidth of the CSI-RS resource is NCSI-RSBW=NBWPsize+NBWPstart-Ninitial RB, otherwise NCSI-RSBW=nrofRBs. In all cases, the UE shall expect that NCSI-RSBW≥min⁡(24, NBWPsize). < End of text proposal >

Views from companies:

| ZTE | CDM-type should be cdm-Type |
| --- | --- |
| Samsung | Agree with ZTE |
| Ericsson | Support |

FL recommendation (to be updated if needed):

R1-1901617	Correction on CSI-RS configuration in 38.214	Huawei, HiSilicon

The draft CR is agreed, the final CR is in R1-19xxxxx, where CDM-type is changed as cdm-Type.

Agreements on CSI-RS since RAN1#91

Agreements in RAN1#95 in CSI-RS agenda

Agreement

Text proposal for Section 4.1 in 38.214:

< Start of text proposal >

TFor link recovery, the ratio of the PDCCH EPRE to NZP CSI-RS EPRE is assumed as 0 dB.

< End of text proposal >

Agreement

Send an LS to RAN2 to inform that powerControlOffset under NZP-CSI-RS-Resource IE is defined as power offset of PDSCH RE to NZP CSI-RS RE, and suggest revising the corresponding description in 38.331 accordingly.

LS is endorsed in R1-1813950

Agreement

CRB in 38.214 is undefined and the editor is respectfully suggested to either expand it into full spelling or define the abbreviation

Agreements in RAN1#94b in CSI-RS agenda

Agreement

Text proposal for Section 5.2.1.5.1 in 38.214:

< Start of text proposal >

The REs corresponding to aperiodic NZP CSI-RS triggered by the CSI request are available for PDSCH.

< End of text proposal >

Text proposal for Section 7.3.1.5 in 38.211:

< Start of text proposal >

-	not used for non-zero-power CSI-RS according to clause 7.4.1.5, except for non-zero-power CSI-RSs configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE, and aperiodic non-zero-power CSI-RS resources

< End of text proposal >

Agreement

Text proposal for Section 5.2.2.3.1 in 38.214:

< Start of text proposal >

The following parameters for which the UE shall assume non-zero transmission power for CSI-RS resource are configured via the higher layer parameter NZP-CSI-RS-Resource, CSI-ResourceConfig and NZP-CSI-RS-ResourceSet for each CSI-RS resource configuration:

< Unchanged parts are omitted >

-	CSI-RS-resourceMapping defines the number of ports, CDM-type, and OFDM symbol and subcarrier occupancy of the CSI-RS resource within a slot that are given in Subclause 7.4.1.5 of [4, TS 38.211].

-	nrofPorts in CSI-RS-RresourceMapping defines the number of CSI-RS ports, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211].

-	density in CSI-RS-RresourceMapping defines CSI-RS frequency density of each CSI-RS port per PRB, and CSI-RS PRB offset in case of the density value of 1/2, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211]. For density 1/2, the odd/even PRB allocation indicated in density is with respect to the common resource block grid.

-	cdm-Type in CSI-RS-RresourceMapping defines CDM values and pattern, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211].

< End of text proposal >

Agreement

Text proposal for Section 4.1 in 38.214:

< Start of text proposal >

- -	trs-Info in NZP-CSI-RS-ResourceSet is associated with a CSI-RS resource set and for which the UE can assume that the antenna port with the same port index of the configured NZP CSI-RS resources in the NZP-CSI-RS-ResourceSet is the same as described in Subclause 5.1.6.1.1 and can be configured when reporting setting is not configured or when the higher layer parameter reportQuantity associated with all the reporting settings linked with the CSI-RS resource set is set to 'none'.

< End of text proposal >

Agreement

Text proposal for Section 7.4.1.5.3 in 38.211:

< Start of text proposal >

he frequency-domain location is given by a bitmap provided by the higher-layer parameter frequencyDomainAllocation in the CSI-RS-ResourceMapping IE or the CSI-RS-ResourceConfigMobility IE with the bitmap and value of in Table 7.4.1.5.3-1 given by

-	, ki-1=fi  for row 1 of Table 7.4.1.5.3-1

-	, ki-1=fi for row 2 of Table 7.4.1.5.3-1

-     , ki-1=4fi for row 4 of Table 7.4.1.5.3-1

-     , ki-1=2fi for all other cases

where  is the bit number of the  bit in the bitmap set to one, repeated across every  of the resource blocks configured for CSI-RS reception by the UE when . The starting position and number of the resource blocks in which the UE shall assume that CSI-RS is transmitted are given by the higher-layer parameters freqBand and density in the CSI-RS-ResourceMapping IE for the bandwidth part given by the higher-layer parameter bwp-Id in the CSI-ResourceConfig IE or given by the higher-layer parameters csi-rs-MeasurementBW in the CSI-RS-CellMobility IE where the the startPRB given by csi-rs-MeasurementBW is relative to common resource block 0.

< End of text proposal >

Agreements in RAN1#94 in CSI-RS agenda

Agreement

Text proposal for Section 7.4.1.5.3 in 38.211:

< Start of text proposal >

The time-domain locations l0∈0, 1, 2, 3, …, 12, 13 and l1∈2, 3, …, 12 are provided by the higher-layer parameters firstOFDMSymbolInTimeDomain and firstOFDMSymbolInTimeDomain2, respectively, in the CSI-RS-ResourceMapping IE and defined relative to the start of a slot.

< End of text proposal >

Agreement

Text proposal for Section 5.1.4.2 in 38.214:

< Start of text proposal > < End of text proposal >

Text proposal for Section 5.1.4.2 in 38.214:

< Start of text proposal > - The ZP-CSI-RS time domain behavior can be configured to periodic, semi-persistent or aperiodic by higher parameters within PDSCH-config. All the resources in a ZP CSI-RS resource set are configured with the same time domain behavior ('periodic', 'semi-persistent', 'aperiodic'). < End of text proposal >

Text proposal for Section 5.1.4.2 in 38.214:

< Start of text proposal > For a UE configured with a list of ZP-CSI-RS-ResourceSet(s), provided by higher layer parameter sp-ZP-CSI-RS-ResourceSetsToAddModList, is configured < End of text proposal >

Agreement

Text proposal for Section 3.2 in 38.211:

< Start of text proposal > System frame number; see clause 4.3.1 < End of text proposal >

Agreement

Text proposal for Section 5.1.6.1.2 in 38.214:

< Start of text proposal > If a UE is configured with a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'on', the UE may assume that the CSI-RS resources, described in Subclause 5.2.2.3.1, within the NZP-CSI-RS-ResourceSet are transmitted with the same downlink spatial domain transmission filter, where the CSI-RS resources in the NZP-CSI-RS-ResourceSet are transmitted in different OFDM symbols. If repetition is set to 'off', the UE shall not assume that the CSI-RS resources within the NZP-CSI-RS-ResourceSet are transmitted with the same downlink spatial domain transmission filter. < End of text proposal >

Agreement

The possible agreement in section 2.14 of R1-1809749 is agreed (text proposal for Section 5.1.6.1.2 in 38.214).

< Start of text proposal >

Table 7.4.1.5.3-2: The sequences  and  for cdm-Type equal to 'no CDM'.

| Index | wf0 | wt0 |
| --- | --- | --- |
| 0 | 1 | 1 |

Table 7.4.1.5.3-3: The sequences  and  for cdm-Type equal to 'FD-CDM2'.

| Index | wf0wf1 | wt0 |
| --- | --- | --- |
| 0 |  | 1 |
| 1 |  | 1 |

Table 7.4.1.5.3-4: The sequences  and  for cdm-Type equal to 'CDM4'.

| Index | wf0wf1 | wt0wt1 |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

Table 7.4.1.5.3-5: The sequences  and  for cdm-Type equal to 'CDM8'.

| Index | wf0wf1 | wt0wt1wt2wt3 |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |

< End of text proposal >

Agreement

Text proposal for Section 5.2.2.3.1 in 38.214:

< Start of text proposal > repetition in NZP-CSI-RS-ResourceSet is associated with a CSI-RS resource set and defines whether UE can assume the CSI-RS resources within the NZP-CSI-RS ResourceSet are transmitted with the same downlink spatial domain transmission filter or not as described in Subclause 5.1.6.1.2. and can be configured only when the higher layer parameter reportQuantity associated with all the reporting settings linked with the CSI-RS resource set is set to 'cri-RSRP' or 'none'. - qcl-InfoPeriodicCSI-RS contains a reference to a TCI-State indicating QCL source RS(s) and QCL type(s). If the TCI-State is configured with a reference to an RS with 'QCL-TypeD' association, that RS may be an SS/PBCH block located in the same or different CC/DL BWP or a CSI-RS resource configured as periodic located in the same or different CC/DL BWP. - trs-Info in NZP-CSI-RS-ResourceSet is associated with a CSI-RS resource set and for which the UE can assume that the antenna port with the same port index of the configured NZP CSI-RS resources in the NZP-CSI-RS-ResourceSet is the same as described in Subclause 5.1.6.1.1 and can be configured when reporting setting is not configured or when the higher layer parameter reportQuantity associated with all the reporting settings linked with the CSI-RS resource set is set to 'none'. < End of text proposal >

Agreement

- For multi-slot and single-slot PDSCH scheduling, the triggered aperiodic ZP CSI-RS is applied to all the slot(s) of the scheduled PDSCH.

Agreement

- UE does not perform rate matching nor assumes puncturing on PDSCH REs overlapped with the REs for the triggered aperiodic NZP CSI-RS resource

- UE does not rate match PDSCH around the overlapped aperiodic ZP-CSI-RS resource scheduled by a DL DCI other than the one which scheduled this PDSCH

For further discussion until the next RAN1 meeting:

UE is not expected to be configured with NZP-CSI-RS resource(s) without functionality

- FFS on whether specification change is needed

For reference the following is provided for consideration until the next RAN1 meeting

A configured NZP-CSIRS resource falls under at least one of the cases below:

1) a NZP-CSI-RS resource is included in a configured NZP-CSI-RS resource set with a configured field “repetition” or “trs-info”

2) a NZP-CSI-RS resource is included in a configured NZP-CSI-RS resource set which is included by a configured CSI-ResourceConfig included in a configured CSI-ReportConfig

3) a NZP-CSI-RS resource is included in a configured “RadioLinkMonitoringRS” IE

4) a NZP-CSI-RS resource is included in a configured PUSCH-PathlossReferenceRS

5) a NZP-CSI-RS resource is included in a configured PUCCH-PathlossReferenceRS

6) a NZP-CSI-RS resource is included in a configured SRS-ResourceSet

7) a NZP-CSI-RS resource is included in a configured "SRS-SpatialRelationInfo"

Agreements in RAN1#93 in CSI-RS agenda

Agreement (Impact to RAN2 specifications)

Text proposal #3 for Section 5.2.2.3.1 in TS38.214

< Unchanged parts are omitted >

The bandwidth and initial CRB index of a CSI-RS resource within a BWP, as defined in Subclause 7.4.1.5 of [4, TS 38.211], are determined based on the higher layer parameters nrofRBs and startingRB, respectively, within the CSI-FrequencyOccupation IE configured by the higher layer parameter freqBand within the CSI-RS-ResourceMapping IE. Both nrofRBs and startingRB are configured as integer multiples of 4 RBs, and the reference point for startingRB is CRB 0 on the common resource block grid. If startingRB<NBWPstart, the UE shall assume that the initial CRB index of the CSI-RS resource is Ninitial RB=NBWPstart, otherwise Ninitial RB=startingRB. If nrofRBs>NBWPsize+NBWPstart-Ninitial RB, the UE shall assume that the bandwidth of the CSI-RS resource is NCSI-RSBW=NBWPsize+NBWPstart-Ninitial RB, otherwise NCSI-RSBW=nrofRBs. In all cases, the UE shall expect that NCSI-RSBW≥min24, NBWPsize.

< Unchanged parts are omitted >

Include as part of LS to RAN2.

Agreement (Impact to RAN2 specifications)

Change the value range of nrofRBs in CSI-RS bandwidth configuration from 24 to maxNrofPhysicalResourceBlocks + 1 and include as part of an LS to RAN2. Ask RAN2 to update the relevant description.

Agreement

UE expects to be configured with same BWP_info for all CSI-RS resources within one set.

UE expects to be configured with same density for all CSI-RS resources within one set.

Agreement

- For P/SP-CSI-RS, UE expects to be configured with same periodicity for all CSI-RS resources within one set (slot offset can still be different for different CSI-RS resources).

Agreement

Text proposal for Section 5.1.6.1.2 in 38.214

If the UE is configured with a CSI-ReportConfig with reportQuantity set to "cri-RSRP", or "none" and if the CSI-ResourceConfig for channel measurement (higher layer parameter resourcesForChannelMeasurement) contains a NZP-CSI-RS-ResourceSet that is configured with the higher layer parameter repetition  and configured without the higher layer parameter trs-Info, the UE can only be configured with the same number (1 or 2) of  ports with the higher layer parameter nrofPorts for all CSI-RS resources within the set,

Agreement

UE expects to be configured with same number of ports for all CSI-RS resources within one set.

Agreement

Text proposal for Section 7.4.1.5.2 in TS38.211

< Unchanged parts are omitted >

where the pseudo-random sequence  is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with

at the start of each OFDM symbol where  is the slot number within a radio frame,  is the OFDM symbol number within a slot, and  equals the higher-layer parameter scramblingID.

< Unchanged parts are omitted >

Note: Inside the equation, the number of 14 is replaced as N_symb^slot.

Agreement

Suggest RAN2 to change the value range of firstOFDMSymbolInTimeDomain2 as from 2 to 12 and include as part of an LS to RAN2.

Agreement

Text proposal for Section 7.4.1.5.1 in 38.211

< Unchanged parts are omitted >

Zero-power and non-zero-power CSI-RS are defined

-	for a non-zero-power CSI-RS configured by the NZP-CSI-RS-Resource IE, the sequence shall be generated according to clause 7.4.1.5.2 and mapped to resource elements according to clause 7.4.1.5.3

for a zero-power CSI-RS configured by the ZP-CSI-RS-Resource IE, the UE shall assume that the resource elements defined in clause 7.4.1.5.3 are not used for PDSCH transmission. UE performs the same measurement/reception on channels/RSs except PDSCH regardless of whether they collide with ZP CSI-RS or not.

< Unchanged parts are omitted >

Working Assumption

Text proposal for TS38.214

If the UE is configured with a CSI-RS resource and a search space set associated with a CORESET in the same OFDM symbol(s), the UE may assume that a PDCCH DM-RS transmitted in the all search space sets associated with CORESET CORESET are quasi co-located with ‘QCL-Type D’, if ‘QCL-Type D’ is applicable. Furthermore, the UE shall not expect to be configured with CSI-RS in PRBs that overlap those of the CORESET in the OFDM symbols occupied by the search space set(s).

Agreements in RAN1#92b in CSI-RS agenda

Agreement:

Confirm the WA on CSI-RS sequence mapping for the case with 0.5 density

Agreement:

A UE may assume all CSI-RS ports within one CSI-RS resource are quasi co-located with respect to ‘QCL type A’ and ‘QCL type D’ when applicable.

Agreement:

Text proposal for Section 5.2.2.3.1 in 38.214

----------------------------------------------------------------------------------------------------------------------------------

-	CSI-RS-FreqBand parameters configure the bandwidth and the initial RB index in the frequency domain, both in units of 4RBs, of a CSI-RS resource within a BWP as defined in Subclause 7.4.1.5 of [4, TS 38.211] , and the minimum bandwidth is the minimum between 24 and   resource blocks.

----------------------------------------------------------------------------------------------------------------------------------

Agreement

Text proposal for Section 5.1.6.1.2 in 38.214

----------------------------------------------------------------------------------------------------------------------------------

If the UE is configured with a CSI-ReportConfig with reportQuantity set to “CRI/RSRP”, or “No Report” and if the Resource Setting for channel measurement contains a CSI-RS Resource Set that is configured with the higher layer parameter CSI-RS-ResourceRep and configured without the higher layer parameter TRS-Info, the UE can only be configured with 1 or 2 ports with the higher layer parameter NrofPorts for all CSI-RS resources within the set, and the UE may be configured with the CSI-RS resource in the same OFDM symbol(s) as an SS/PBCH block, and the UE may assume that the CSI-RS and the SS/PBCH block are quasi co-located with ‘QCL-TypeD’ if ‘QCL-TypeD‘ is applicable. Furthermore, the UE shall not expect to be configured with the CSI-RS in PRBs that overlap those of the SS/PBCH block, and the UE shall expect that the same subcarrier spacing is used for both the CSI-RS and the SS/PBCH block.

Agreement

The number of ports in a resource set for L1-RSRP

- Always have the same number of ports (1 or 2) across all resources

Agreement

For density ½, odd/even PRB allocations is respect to the common resource block grid

Include in LS to RAN2 for CSI-RS Xi (Huawei)

Agreement

Text proposal for Section 5.1.4.2 in 38.214

----------------------------------------------------------------------------------------------------------------------------------

- ZP-CSI-RS-ResourceConfigId determines ZP CSI-RS resource configuration identity.

- NrofPorts defines the number of CSI-RS ports, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211].

- ZP-CSI-RS-Density defines the zero-power CSI-RS frequency density, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211].

- CDMType defines CDM values and pattern, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211].

----------------------------------------------------------------------------------------------------------------------------------

Agreement

Text proposal for Section 5.2.2.3.1 in 38.214

-	CSI-RS-FreqBand parameters configure the bandwidth and the initial RB index in the frequency domain, both in units of 4RBs, of a CSI-RS resource within a BWP as defined in Subclause 7.4.1.5 of [4, TS 38.211]. If the configured bandwidth is larger than the corresponding BWP, UE shall assume that the actual CSI-RS bandwidth is equal to the BWP size.

Agreement

- The UE expects that all the CSI resources of a resource set are configured with the same starting RB and number of RBs and the same CDM-type.

  - FFS: Other parameters

Agreement

Text proposal for Section 5.2.2.3.1 in 38.214

--------------------------------------------------------------------------------------------------------------------------

-	CSI-RS-Density defines CSI-RS frequency density of each CSI-RS port per PRB and CSI-RS PRB offset in case of the density value of 1/2, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211]. For density ½, odd/even PRB allocations is respect to the common resource block grid.

-	CDMType defines CDM values and pattern, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211].

-	CSI-RS-FreqBand parameters configure the bandwidth and the initial RB index in the frequency domain, both in units of 4RBs, of a CSI-RS resource within a BWP as defined in Subclause 7.4.1.5 of [4, TS 38.211].

-	Pc: which is the assumed ratio of PDSCH EPRE to NZP CSI-RS EPRE when UE derives CSI feedback and takes values in the range of [-8, 15] dB with 1 dB step size.

-	Pc_SS: which is the assumed ratio of SS/PBCH block EPRE to NZP CSI-RS EPRE.

-	ScramblingID: defines scrambling ID of CSI-RS with length of 10 bits.

-	CC_Info defines which component carrier the configured CSI-RS is located in.

- 	BWP_Info defines which bandwidth part the configured CSI-RS is located in.

--------------------------------------------------------------------------------------------------------------------------

Agreement

Text proposal for Section 5.1.4.2 in 38.214

----------------------------------------------------------------------------------------------------------------------------------

-	ZP-CSI-RS-ResourceMapping defines the OFDM symbol and subcarrier occupancy of the ZP-CSI-RS resource within a slot that are given in Subclause 7.4.1.5 of [4, TS 38.211].

----------------------------------------------------------------------------------------------------------------------------------

R1-1805740	[Draft] LS on NR CSI-RS	Huawei, HiSilicon

LS is endorsed in R1-1805626

Agreements in RAN1#92 in CSI-RS agenda

Agreement (RRC parameter update):

maxNrofCSI-IM-Resources                          INTEGER ::= 32        -- Maximum number of CSI-IM resources per CC. See CSI-IM-ResourceMax in 38.214.

Agreement (RRC parameter update):

Revise the higher-layer parameter CSI-RS-ResourceRep defined in TS 38.331 to be an optionally configurable parameter (not mandatory parameter as in the current specification), and include this into an LS to RAN2.

- CSI-RS-ResourceRep can only be configured for CSI-RS resource sets which are associated with CSI report with  report of L1 RSRP or “no report”

Agreement:

For the CSI-RS sequence mapping, the working assumption from RAN1 NR Ad Hoc 1801 for density 1 and 3 is confirmed

Agreement:

- By default, UE does not perform rate matching on REs overlapped with at least CSI-RS for mobility

  - Note: UE shall perform rate matching on REs overlapped with a CSI-RS for mobility only if ZP-CSI-RS covers the REs overlapped with the CSI-RS for mobility.

Conclusion:

UE performs the same measurement on NZP CSI-RS or CSI-IM regardless of whether they collide with ZP CSI-RS or not.

Agreement:

- The formula for SRS can be reused for CSI-RS:

It’s up to the editor how to reflect the above agreement into the specification.

Agreement (RRC parameter update):

maxNrofNZP-CSI-RS-Resources-1 is 191

Agreements in RAN1#AH1801 in CSI-RS agenda

Conclusion:

On the support of indicating two starting OFDM symbol positions for non-contiguous NZP CSI-RS mapping, include as part of an LS to RAN2 to inform them that the RRC signalling should support indication of up to two starting OFDM symbol positions and it is up to RAN2 to decide on the detailed signalling (relevant RRC parameter: CSI_RS_resourcemapping)

Agreement:

Remove Pc-PDCCH from RRC parameter list for NZP-CSI-RS and add clarification on 0dB power offset between CSI-RS for beam failure detection and PDCCH DMRS

Agreement:

- Indication mechanism of aperiodic ZP CSI-RS is RRC + DCI

- Each triggering state of “ZP CSI-RS trigger” in DCI is to trigger one ZP CSI-RS resource set

  - A state is reserved for not triggering aperiodic ZP CSI-RS

- All the resources in a ZP CSI-RS resource set are configured with the same ResourceConfigType (periodic or aperiodic)

- The bit-length of DCI field “ZP CSI-RS trigger” depends on the number of aperiodic ZP CSI-RS resource sets configured (up to 2 bits)

- The maximum number of aperiodic ZP CSI-RS resource sets configured per BWP is 3

Include as part of LS to RAN2

Agreement:

- Add the following RRC parameters for ZP CSI-RS

| PDSCH | 38.214 |  |  |  | ZP-CSI-RS-ResourceSetConfigList | New | ZP-CSI-RS-ResourceSetConfigList | Resource set configuration for ZP CSI-RS |  | NA | UE specific | 38.331 |  | Y |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDSCH | 38.214 |  |  |  | ZP-CSI-RS-ResourceSetConfigId | New | ZP-CSI-RS-ResourceSetConfigId | ZP CSI-RS Resource set configuration ID | 0 .. ZP-CSI-RS-ResourceSetMax-1 | NA | UE specific | 38.331 | Contained in ZP-CSI-RS-ResourceSetConfig | Y |

- Update the RRC parameter Aperiodic-ZP-CSI-RS-Resource-List as follows

| Aperiodic-ZP-CSI-RS-Resource-List | Contains list of ZP-CSI-RS resource set IDs for aperiodic triggering | Up to 3 ZP-CSI-RS resource set IDs per BWP |
| --- | --- | --- |

Include as part of LS to RAN2

Agreement:

- Introduce additional periodicities of {4,8,16,32,64} slots and the corresponding slot offsets to at least the following periodic/semi-persistent RS:

  - CSI-RS (includes ZP-CSI-RS and NZP-CSI-RS)

  - SRS

- FFS on whether and how to align to additional periodicities as agreed in the frame structure agenda

  - “The two periodicities form X ms + Y ms total periodicity, where X, and Y are from {0.5, 0.625, 1, 1.25, 2, 2.5, 5, 10} ms”

- FFS on CSI-IM periodicity of 1 slot

Include as part of LS to RAN2

Working Assumption

Agree to the following text for TS.38.211 with the understanding that the 1 port rate=1/2 case needs to be checked. If there is an issue with the working assumption for 1 port rate=1/2, technical details will be modified.

7.4.1.5.3	Mapping to physical resources

For each CSI-RS component configured, the UE shall assume the sequence  being mapped to physical resources according to

under the condition that the resource elements indexed by are within the resource blocks occupied by the CSI-RS for which the UE is configured. The reference point for  is subcarrier 0 in common resource block 0. The value of ρ is given by the higher-layer parameter CSI-RS-Density.

Agreement:

- By default, UE does not perform rate matching on REs overlapped with at least CSI-RS for mobility, which is not configured with serving cell ID

  - UE shall perform rate matching on REs overlapped with a CSI-RS for mobility only if ZP-CSI-RS covers the REs overlapped with the CSI-RS for mobility.

  - FFS, rate matching on REs overlapped with CSI-RS for mobility configured with serving cell ID

Agreements in RAN1#91 in CSI-RS agenda

Agreement

Support reusing the allowed CSI-RS-to-PDSCH power offset values in LTE for NR.

Agreement:

- Support including the OFDM symbol index within a slot in the formula for c_init.

- Support including slot index within a radio frame in the formula for c_init.

Agreement:

Length-31 Gold sequence is used for CSI-RS

- Same polynomial as in LTE

- QPSK sequence modulation is used

- N_c and c_init are to be discussed separately

Agreement:

For NR CSI-RS sequence,

- CSI-RS scrambling ID has a length of 10 bits

- There is no default value for the scrambling ID

Agreement

Introduce the following RRC parameter for CSI-RS:

| CC/BWP-Info | Indication of which CC/BWP the configured CSI-RS is located in This parameter belongs within a CSI-RS resource configuration or in a BWP configuration (up to editor) | FFS |
| --- | --- | --- |

Agreement

Introduce the following RRC parameter for CSI-RS:

- Comb offset for D=1/2

  - 1 bit to indicate between odd and even RBs

Agreement

UE rate matches PDSCH around ZP-CSI-RS

Agreement

| CSI-RS-ResourceMapping | Include parameters to capture OFDM symbol location(s) in a slot and subcarrier occupancy in a PRB of the CSI-RS resource FFS: how to configure CSI-RS in different slots for fine time/frequency tracking | Starting subcarrier: For 1 port CSI-RS, there is no restriction For Y=2, is constrained to be one among even subcarriers, in the given PRB (indexed from 0) For Y=4, is constrained to be one among subcarriers 0, 4, 8, in the given PRB (indexed from 0) Symbol location: {0,1,2,3,4,5,6,7,8,9,10,11,12,13}, where 2 is supported only when DL-DMRS-typeA-pos equals 3 - UE is not expected to receive CSI-RS and DMRS on overlapping REs - Only uniform RE pattern across all symbols for CSI-RS resource is supported |
| --- | --- | --- |
| CDMType | Includes parameters to capture CDM value (1, 2, 4, or 8), CDM pattern (freq only, time and freq, time only) | Agreed CDM types for different X and N {No CDM} for X = 1 and N = 1 {FD-CDM2} for X = 2 and N = 1 {FD-CDM2} for X = 4 and N = 1 {FD-CDM2} for X = 8 and N = 1 {FD-CDM2, CDM4 (FD2,TD2)} for X = 8 and N = 2 {FD-CDM2} for X = 12 and N = 1 {CDM4 (FD2,TD2)} for X = 12 and N = 2 {FD-CDM2, CDM4 (FD2,TD2)} for X = 16 and N = 2 {FD-CDM2, CDM4 (FD2, TD2), CDM8 (FD2, TD4)} for X = 24 and N = 4 {FD-CDM2, CDM4 (FD2, TD2), CDM8 (FD2, TD4)} for X = 32 and N = 4 |
| CSI-RS-FreqBand | Includes parameters to enable configuration of wideband and partial band CSI-RS | Combined indication methods from [90b-NR-19] Starting RB index and number of spanned RBs in the units of 4 - Minimum CSI-RS BW is min(24RBs, BWP for data) |
| Pc_SS | Power offset of NZP CSI-RS RE to SS RE Note: This parameter is optional | New parameter 2 bits in the range of [-3, 6] with step size of 3dB |
| ZP-CSI-RS-ResourceMapping | Include parameters to capture OFDM symbol and subcarrier occupancy of the ZP CSI-RS resource within a slot | A list of NZP-CSI-RS resource mapping(s) by explicit configuration of time and frequency domain information |
| ZP-CSI-RS-timeConfig | Contains periodicity and slot offset for periodic ZP-CSI-RS | Same as NZP-CSI-RS |
| ZP-CSI-RS-FreqBand | Includes parameters to enable configuration of wideband and partial band ZP-CSI-RS | Same as NZP-CSI-RS |
| Aperiodic-ZP-CSI-RS-Resource-List | Contains list of ZP-CSI-RS resource IDs for aperiodic triggering |  |
| CC-Info | Indication of which CC the configured CSI-RS is located in. This parameter belongs within a CSI-RS resource configuration or in a CC configuration (up to editor) | How to capture this in the specification is up to the editor. This parameter applies to both NZP-CSI-RS and ZP-CSI-RS. |
| BWP-Info | Indication of which BWP the configured CSI-RS is located in. This parameter belongs within a CSI-RS resource setting configuration | How to capture this in the specification is up to the editor. This parameter applies to both NZP-CSI-RS and ZP-CSI-RS. |

Agreement

Introduce parameter PC-PDCCH which has a fixed value of 0dB and indicates the power offset of PDCCH and CSI-RS

Agreement

In addition to agreed RRC parameters for ZP-CSI-RS, following RRC parameters are added for ZP CSI-RS configuration.

| ZP-CSI-RS-Density | Density of ZP CSI-RS resource in frequency domain = RE pattern existence per PRB (1 PRB = 12 subcarriers and 1 sym) | Same as NZP CSI-RS resources |
| --- | --- | --- |
| ZP-CSI-RS-ResourceConfigId | ZP-CSI-RS resource configuration ID | 0 .. ZP-CSI-RS-ResourceMax - 1 |
| ResourceConfigType | Time domain behavior of resource configuration | aperiodic or periodic |

Agreements

- Support mapping CSI-RS sequence to the resource grid at RE-level granularity

- Same sequence for all CSI-RS ports on one symbol within a CSI-RS resource

Agreements:

- Support assigning CSI-RS port index across CDM groups first in frequency domain and then in time domain

Agreement

For NR CSI-RS sequence initialization,

    - Similar to LTE, the sequence initialization values should be different for all OFDM symbols within a frame (10ms)

cinit=210×14ns+l+12NID+1+NID mod 231,

where ns denotes slot index within a radio frame, and l is OFDM symbol index within a slot and NID∈{0,1,…,210-1} is UE specifically configured scrambling ID

Agreement:

- For Rel-15, the number of APs configured for one NZP CSI-RS resource (P) is equal to number of APs configured for CSI acquisition and reporting, i.e., N1*N2*2 = P (analogous to LTE)

Agreements:

NR supports aperiodic ZP-CSI-RS for rate matching, with a separate DCI field for triggering aperiodic ZP-CSI-RS