3GPP TSG RAN WG1 Meeting #96	R1-1903514

Athens, Greece, February 25 – March 2, 2019

Agenda Item:	6.2.3.1

Source:	Huawei, HiSilicon

Title:	Remaining issues and proposals on SRS enhancements

Document for:	Discussion and decision

Introduction

In this contribution, the remaining issues on enhancing SRS capacity and coverage for LTE [2-23] are summarized and some suggested proposals are also provided.

Remaining issues and proposals

UE specific SRS configuration and transmission

Periodic and aperiodic of additional SRS symbols

The views of companies are listed as below:

| Company | Proposals and comments |
| --- | --- |
| Huawei, HiSilicon | Proposal 3: Support periodic SRS transmission for additional SRS symbols. - FFS semi-persistent SRS to achieve the tradeoff between overhead and performance Proposal 7: Support intra-subframe frequency hopping and repetition for periodic SRS in additional SRS symbols. |
| ZTE | Proposal 1: Not support periodic SRS in additional symbols. |
| Vivo | Proposal1: periodic transmission of additional SRS symbol(s) is not supported. |
| MTK | Proposal 6: Intra-subframe frequency hopping and repetition are not supported for periodical additional SRS symbols. |
| Lenovo, Moto | Proposal 2: Periodic SRS transmission for additional SRS symbol(s) should also be supported. Proposal 3: Frequency hopping and repetition should be supported for periodic SRS transmission for additional SRS symbol(s). |
| Samsung | Proposal 3: Periodic SRS transmission in the additional SRS symbols is not supported in Rel-16. |
| QC | Proposal 6: Support periodic SRS transmission in additional SRS symbols. - Intra-subframe antenna switching/frequency hopping/repetition are supported for periodic SRS in additional symbols. |
| Nokia, NSB | Proposal 4: Periodic SRS transmission is supported in the additional SRS symbols Proposal 5: UE drops SRS transmission if it collides with PUCCH/PUSCH/PRACH in the same carrier |
| Ericsson | Proposal 4 The trade-off between channel sounding accuracy from multi-symbol SRS transmissions of Rel-16 UEs and UL throughput impact on legacy UEs should first be carefully evaluated before specifying periodic multi-symbol SRS enhancements in Rel-16. |

On the support of periodic SRS transmission for additional SRS, the views of companies are summarized as below:

- Periodic:

  - Support: Lenovo, Moto, QC, Nokia, NSB, Huawei, HiSilicon, LGE

  - Not support: ZTE, Vivo, Samsung, MTK

  - FFS: Ericsson (trade-off between channel sounding accuracy and UL throughput impact)

The reasons to support periodic SRS transmission include further delay with only AP SRS in frequency hopping or antenna switching, configuration flexibility, DL intermittent traffic UEs. The reasons to not support periodic SRS transmission include impacts to uplink transmission, usage of legacy SRS, spec complexity. Based on the inputs, the following observation is made:

Proposal 1: Periodic SRS triggering for additional SRS is supported for additional SRS symbols.

Antenna switching for additional SRS

The views of companies are listed as below:

| Company | Proposals and comments |
| --- | --- |
| Huawei, HiSilicon | Proposal 4: Intra-subframe antenna switching and intra-subframe frequency hopping/repetition can be concurrently configured. Proposal 5: Introduce the guard period for antenna switching when additional SRS symbols are allocated for the same UE. |
| ZTE | Proposal 4: If both SRS antenna switching and frequency hopping are configured within one subframe, the gap should be minimized. |
| Vivo | Observation2: it is possible for a UE to transmit additional SRS with antenna switching within a sub-frame. |
| MTK | Proposal 5: Concurrent configuration of Intra-subframe antenna switching and intra-subframe frequency hopping/repetition for aperiodic SRS in additional SRS symbols is not supported. |
| Lenovo, Moto | Proposal 4: Intra-subframe antenna and intra-subframe frequency hopping cannot be concurrently configured. |
| QC | Proposal 4: Support intra-subframe antenna switching/frequency hopping/repetition with flexible configuration to enable the following - Intra-subframe AS with no repetition/frequency hopping on narrowband or wideband - Intra-subframe AS with frequency hopping - Intra-subframe AS with repetition |
| Intel | Proposal 5: Intra-subframe antenna switching and intra-subframe frequency hopping/repetition can be concurrently configured, where, frequency hopping/repetition is performed before switching antenna ports. |

On whether intra-subframe antenna switching and intra-subframe frequency hopping/repetition can be concurrently configured, the views are summarized as below:

- Support: ZTE, QC, Intel, Huawei, HiSilicon

- Not support: MTK, Lenovo, Moto, LGE

The reasons to support concurrently configuration include guaranteeing of SRS coverage, fast sounding of all antennas, flexibility. The reasons to not support concurrently configuration include difference use cases for frequency hopping/repetition and antenna switching, limited number of symbols.

Based on the majority view, the following is proposed to move forward:

Proposal 2: Intra-subframe antenna switching and intra-subframe frequency hopping/repetition can be concurrently configured.

Considering the RF impact due to antenna switching, the guard period was proposed and solutions to minimize the RF impact are proposed that frequency hopping/repetition is performed before antenna switching. Therefore, the following is proposed:

Proposal 3: Frequency hopping/repetition is performed before switching antenna ports if intra-subframe antenna switching and intra-subframe frequency hopping/repetition are concurrently configured to a UE (if supported).

Concurrent transmission of legacy SRS symbols and additional SRS symbols

The views of companies are listed as below:

| Company | Proposals and comments |
| --- | --- |
| ZTE | Proposal 5: UE can transmit legacy aperiodic SRS and additional SRS symbol(s) in the same subframes. |
| MTK | Observation 3: There is no significant benefit for UE to transmit aperiodic legacy SRS symbol and aperiodic additional SRS at the same time. Proposal 3: In case legacy SRS is aperiodic, UE cannot transmit legacy SRS and additional SRS symbol(s) in the same subframes. Proposal 4: one bit in a re-interpreted field in DCI format 0 can indicate if SRS request is for legacy SRS symbol or additional SRS symbols – It is FFS which re-interpreted field in DCI format 0. |
| LG Electronics | Proposal 5: Additional and legacy AP-SRS can be triggered in the same subframe by extending legacy AP-SRS triggering mechanism. |
| Lenovo, Moto | Proposal 5: Aperiodic legacy SRS transmission cannot be triggered in the subframes containing additional SRS symbol(s). |
| Samsung | Proposal 5: Support aperiodic SRS transmission across legacy and additional SRS symbols within a subframe. |
| QC | Proposal 1: In case of legacy SRS is aperiodic, UE can transmit legacy SRS and additional SRS symbol(s) in the same subframes. |
| Intel | Proposal 2: UE does not support concurrent transmission of legacy aperiodic SRS and additional SRS symbol(s) in the same subframe. |
| Mitsubishi | Proposal 5 : Allow aperiodic legacy and additional SRS transmission in the same subframe |
| Nokia, NSB | Proposal 3: Reuse existing SRS request field in the relevant DCI formats to trigger aperiodic SRS transmission in additional SRS symbols. |

For the concurrent triggering/transmission of aperiodic legacy SRS symbols and aperiodic additional SRS symbols, the views are summarized as:

In case of legacy SRS is aperiodic, UE can transmit legacy SRS and additional SRS symbol(s) in the same subframes

- Support: ZTE, LGE, Samsung, QC, Mitsubishi, Nokia, NSB

- Not support: MTK, Lenovo, Moto, Intel, Ericsson

Based on the inputs, the following is proposed:

Proposal 4: For further study on whether and the triggering on transmission of aperiodic legacy SRS and aperiodic additional SRS symbol(s) in the same subframes for a UE.

Sequence generation with introduction of virtual cell ID

And the details of group/sequence hopping of proponents are:

| Sourcing | Proposals |
| --- | --- |
| ZTE | NR introduces symbol level generation of the pseudo-random sequence for both the sequence-group number and the base sequence number because multiple symbols in one slot can be used for SRS transmission. Proposal 6: For SRS pseudo-random sequence in additional SRS symbol(s), support the same symbol-level generation of the pseudo-random sequence for both and as NR. To be more specific, we provide the modified formula based on proposal 4 as below. Where is the symbol number within the subframe . |
| QC | Proposal 2: When nIDRS is configured as cell ID or virtual cell ID, the time-varying SRS sequence for multiple SRS symbols in a UL normal subframe is defined by changing (u, v) for SRS ZC roots as - u=fghl0+l',ns+nIDRSmod 30, where the group hopping pattern fghl',ns is given by fghl',ns=i=07c8(l0+l'+ns∙Nsymbslot)+i∙2imod 30if grop hopping is enabled0 if grop hopping is disabled , - with l0+l'+ns∙Nsymbslot as the index of the starting SRS symbol, and c(i) is initialized with cinit=nIDRS 30 at the beginning of each radio frame. - v=c(l0+l'+ns∙Nsymbslot if sequence hopping is enabled and grop hopping is disabled0 otherwise where c(i) is initialized with cinit=nIDRS 30∙25+nIDRS+∆ssmod30 at the beginning of each radio frame, and ∆ss could be 0 or ∆ss∈{0,1,…29} configured by higher layers. |
| Ericsson | - Time-varying SRS sequences for multi-symbol SRS in normal UL subframes is not introduced in LTE Rel-16, unless the performance benefits of this feature are quantified via evaluations. |

Based on companies’ inputs, the following is proposed:

Proposal 5: The per-symbol group hopping or sequence hopping are supported. FFS details.

Application of virtual cell ID

On the application of virtual cell ID, there are following proposals:

| Sourcing | Proposal |
| --- | --- |
| Huawei, HiSilicon | Proposal 1: The eNB can configure to apply the virtual cell ID to: - Only legacy SRS - Or only additional SRS symbol(s) - Or both legacy and additional SRS symbol(s) |
| ZTE | Proposal 1: eNB has the flexibility to configure the virtual cell ID to legacy SRS symbol and/or additional SRS symbol(s). |
| Vivo | Proposal: alternative 1, i.e. when a virtual cell ID is configured, it applies to both legacy and additional SRS symbol(s), is supported. |
| LGE | Proposal 1: When a virtual cell ID is configured, it applies to both legacy and additional SRS symbol(s). |
| Samsung | Proposal: - We support Alt1: When a virtual cell ID is configured, it applies to both legacy and additional SRS symbol(s). - FFS: in which level the SRS symbols are to be initialized, in slot level or in symbol level. |
| QC | Proposal 1: When a virtual cell ID is configured, eNB has the flexibility to apply the virtual cell ID to - Only legacy SRS - Or only additional SRS symbol(s) - Or both legacy and additional SRS symbol(s) |
| Intel | Proposal 1: When a virtual cell ID is configured, it applies to both legacy and additional SRS symbol(s) |
| Nokia, NSB | Proposal 1: Alt 2 from the agreement in the last meeting is adopted: When a virtual cell ID is configured, eNB has the flexibility to apply the virtual cell ID to o Only legacy SRS o Or only additional SRS symbol(s) o Or both legacy and additional SRS symbol(s) |
| Ericsson | - When a virtual cell ID is configured, it applies to both legacy and additional SRS symbol(s). |

As agreed in last meeting, there are two options on applicability of virtual cell IDs:

Agreement

Down-select among the following alternatives in the next RAN1 meeting

- Alt1: When a virtual cell ID is configured, it applies to both legacy and additional SRS symbol(s)

- Alt2: When a virtual cell ID is configured, eNB has the flexibility to apply the virtual cell ID to

  - Only legacy SRS

  - Or only additional SRS symbol(s)

  - Or both legacy and additional SRS symbol(s)

If virtual cell ID is not configured, physical cell ID is used.

Based on the inputs, the views are summarized as:

- Alt 1: Vivo, LGE, Samsung, Intel, Ericsson

- Alt 2: ZTE, QC, Nokia, NSB, Huawei, HiSilicon

As the views are still diverse, the following is proposed:

Proposal 6: Down-select in this meeting among the following alternatives

- Alt1: When a virtual cell ID is configured, it applies to both legacy and additional SRS symbol(s)

- Alt2: When a virtual cell ID is configured, eNB has the flexibility to apply the virtual cell ID to

  - Only legacy SRS

  - Or only additional SRS symbol(s)

  - Or both legacy and additional SRS symbol(s)

If virtual cell ID is not configured, physical cell ID is used.

Conclusion

Proposal 1: Periodic SRS triggering for additional SRS is supported for additional SRS symbols.

Proposal 2: Intra-subframe antenna switching and intra-subframe frequency hopping/repetition can be concurrently configured.

Proposal 3: Frequency hopping/repetition is performed before switching antenna ports if intra-subframe antenna switching and intra-subframe frequency hopping/repetition are concurrently configured to a UE (if supported).

Proposal 4: For further study on whether and the triggering on transmission of aperiodic legacy SRS and aperiodic additional SRS symbol(s) in the same subframes for a UE.

Proposal 5: The per-symbol group hopping or sequence hopping are supported. FFS details.

Proposal 6: Down-select in this meeting among the following alternatives

- Alt1: When a virtual cell ID is configured, it applies to both legacy and additional SRS symbol(s)

- Alt2: When a virtual cell ID is configured, eNB has the flexibility to apply the virtual cell ID to

  - Only legacy SRS

  - Or only additional SRS symbol(s)

  - Or both legacy and additional SRS symbol(s)

If virtual cell ID is not configured, physical cell ID is used.

- References

RP-181485, “DL MIMO efficiency enhancements for LTE”, 3GPP TSG RAN Meeting #80, La Jolla, USA, June 11 - 14, 2018.

R1-1901585	Introduction of additional SRS symbols in normal UL subframe	Huawei, HiSilicon

R1-1901586	On virtual cell ID for SRS sequence generation	Huawei, HiSilicon

R1-1901587	Discussion on power control for additional SRS	Huawei, HiSilicon

R1-1901644	Discussion on additional SRS symbols	ZTE

R1-1901645	Discussion on virtual Cell ID for SRS	ZTE

R1-1901649	Further discussion on SRS enhancements	vivo

R1-1901650	On virtual cell ID configuration for SRS	vivo

R1-1901730	Additional SRS symbols in LTE MIMO Enhancements	MediaTek Inc.

R1-1902075	Enhancements on SRS resource allocation	LG Electronics

R1-1902076	Discussion on VCID for SRS sequence	LG Electronics

R1-1902164	Discussion of additional SRS symbols	Lenovo, Motorola Mobility

R1-1902221	Discussion on additional SRS symbols	Samsung

R1-1902222	Discussions on virtual cell ID for SRS	Samsung

R1-1902380	Additional SRS symbols	Qualcomm Incorporated

R1-1902381	Virtual cell ID for SRS	Qualcomm Incorporated

R1-1902461	Additional SRS symbols	Intel Corporation

R1-1902462	Virtual cell ID support for SRS	Intel Corporation

R1-1902832	Views on additional SRS designs for LTE MIMO	Mitsubishi Electric Co.

R1-1902855	Discussion on introduction of additional SRS symbols	Nokia, Nokia Shanghai Bell

R1-1902856	Remaining issues on virtual cell ID for SRS	Nokia, Nokia Shanghai Bell

R1-1902914	On Rel-16 LTE SRS enhancements	Ericsson

R1-1902915	On virtual cell ID for SRS	Ericsson