3GPP TSG RAN WG1 #96		R1-1903523

Athens, Greece, 25th February – 1st March, 2019

Source:	Sierra Wireless

Title:	LTE-M Preconfigured UL Resources remain issues #2

Agenda Item:	6.2.1.2

Document for:	Discussion and Decision

# Introduction

The following tdoc provides remain proposals for the PUR feature.

# PUR Search Space

For dedicated PUR in idle mode, the PUR search space configuration shall at least include:

RNTI

RAN2 Domain

Possible Proposal #1

Only one RNTI is assigned to the PUR search space

RAN2 will decide how the RNTI is signaled to the UE

Possible Proposal #2

The UE can assume the RNTI assigned to the PUR search space is not shared.

# PUR configuration

Possible Proposal #3

For dedicated PUR in idle mode, the PUR resource configuration includes at least the following

a frequency hopping indication to enable or disable legacy frequency hopping

# TA Validation

- Possible Observation #1:

- The change in RSRP is dependent on the distance from the eNB

The change in RSRP depends on whether the UE is moving towards or away from the eNB.

- Possible Agreement #4:

In idle mode, a UE can be configured such that TA is always valid within a given cell.

- FFS: up to RAN2 how to implement e.g. PUR Time Alignment Timer = infinity

# Appendix: Aggreements

## RAN1 #96

In idle mode, the TA validation configuration can include “PUR Time Alignment Timer”

- Where the UE considers the TA as invalid if the (current time – time at last TA update) > the PUR Time Alignment Timer

- Details on how to specify the “PUR Time Alignment Timer” is up to RAN2

In idle mode, when the UE validates TA, the UE considers the TA for the previous serving cell as invalid if the serving cell changes

- Above applies for the case where the UE is configured to use the serving cell change attribute

For dedicated PUR in idle mode, the Dedicated PUR ACK is at least sent on MPDCCH

- RAN2 can decide if a higher layer PUR ACK is also supported

For dedicated PUR in idle mode, the PUR search space configuration shall be included in the PUR configuration.

- PUR search space is the search space where UE monitors for MPDCCH

- FFS: Whether PUR search space is common or UE specific

When the TA is validated and found to be invalid and the UE has data to send, the UE can obtain a valid TA and may send data via legacy RACH or EDT procedures

FFS whether only TA is acquired and then data sent on PUR is supported

FFS other approaches to obtain a valid TA

When the UE is configured to use several TA validation criteria, the TA is valid only when all the configured TA validation criteria are satisfied.

For dedicated PUR, in idle mode, the PUR resource configuration includes at least the following

- Time domain resources including periodicity(s)

    - Note: also includes number of repetitions, number of RUs, starting position

- Frequency domain resources

- TBS(s)/MCS(s)

- Power control parameters

- Legacy DMRS pattern

In idle mode, at least the following PUR configurations and PUR parameters may be updated after a PUR transmission:

Timing advance adjustment

UE TX power adjustment

FFS: Repetition adjustment for PUSCH

FFS: Whether the above update is done in L1 and/or higher layer

In idle mode, the PUR search space configuration includes at least the following:

MPDCCH narrowband location

MPDCCH repetitions and aggregation levels

MPDCCH starting subframe periodicity (variable G)

Starting subframe position (alpha_offset)

## RAN1 #95

For dedicated PUR in idle mode, the UE may skip UL transmissions.

- FFS: Resource release mechanism

- FFS: Whether or not to support mechanism to disable skipping by eNB

The following evaluation parameters for CFS PUR link level simulation are agreed.

| Parameter | Value/Description |
| --- | --- |
| Channel model | ETU 1Hz |
| Resource Bandwidth(s) | 1, 2, 3 or 6 PRBs (contiguous) for eMTC Proponent to provide details on resource allocation Same bandwidth for all UEs |
| Traffic Pattern | Fixed (N UEs transmit in the same resource) or (Optional) Bursty (N UEs may transmit in the same resource, with activity factor p for each of them) Note: For all traffic models, it is assumed that a UEhas a dedicated DMRS Note: Final decision can be based on the results for both fixed and bursty traffic model |
| Number of repetitions in a resource | To be declared by proponent |
| Transport Block Size and Modulation Order | To be declared by proponent |
| Number of eNB Rx antennas | 2 as baseline, other values are not precluded |
| Metrics | BLER before HARQ Number of resources (PRB x subframes) used by the technique to serve a given number of UEs Baseline for comparison is dedicated PUR |
| MCL | 164, 144, 124dB MCL |
| UE grouping | All UEs in the same MCL level. |
| Processing Receiver at eNB | No SIC/joint decoding of UEs is assumed. MMSE single user decoding. Not NOMA. |
| DMRS collision | Each company to provide details on their DMRS assumption |
| Channel Estimation CFO Timing Error Power Imbalance | Realistic Uniform +/-50 Hz, +/-100 Hz (optional) +/-0.5 CP uniform, +/-1 CP uniform (optional) Uniform +/-[5 or 10] dB |

The evaluated proposals should be within the scope of the eMTC WI.

- If multi-TB grant is not enabled, a dedicated PUR allocation is associated to only a single TB and single HARQ process

- FFS: if multi-TB grant is enabled/supported

- In idle mode, at least the following TA validation attributes are supported:

- Serving cell changes (serving cell refers the cell that the UE is camping on)

- Time Alignment Timer for idle mode

- Serving cell RSRP changes (serving cell refers the cell that the UE is camping on)

  - Based on RSRP measurement definition in existing Rel-15 TS36.214

Include in LS to RAN2, RAN4 to consider in their work.

- The UE can be configured to use at least these TA validation attributes:

- Time Alignment Timer for idle mode

- Serving cell RSRP changes

- Note: the configuration shall support disabling of the TA validation attributes

Include in LS to RAN2, RAN4

For further study:

TA validation attributes:

- Subscription based UE differentiation (or Stationary indication in held in subscription)

- Cell specific indication where TA is valid within that cell

Include in LS to RAN2, RAN4:

RAN1 assumes that a UE transitioning from EDT/connected to idle mode can use the valid TA that was used while in EDT/connected mode.

For dedicated PUR in idle mode, UL grant for HARQ retransmission is transmitted in MPDCCH search space

- FFS: Details on the search space (for example USS, CSS)

- For dedicated PUR in idle mode, upon successful decoding by eNB of a PUR transmission, the UE can expect an explicit ACK

FFS: if ACK is sent on MPDCCH (layer 1) and/or PDSCH (layer 2/3)

Include in LS to RAN2, RAN4.

- For dedicated PUR in idle mode, upon unsuccessful decoding by eNB of a PUR transmission, the UE can expect

- an UL GRANT for retransmission on the MPDCCH, or

- FFS: a NACK, or

- FFS: no explicit ACK

Include in LS to RAN2, RAN4.

## RAN1#94bis

In idle mode, the UE will at least consider one or more of the following attributes when validating TA (combination of multiple attributes is allowed):

- Serving cell changes (serving cell refers the cell that the UE is camping on)

- Time Alignment Timer for idle mode

- Serving cell RSRP changes (serving cell refers the cell that the UE is camping on)

- FFS Other attributes:

  - Neighbour cell RSRP change

  - TDOA of >=2 eNBs

  - TA History

  - Subscription based UE differentiation

  - Others not precluded (for example, attributes that need to be considered for high mobility UEs)

Note that UE power consumption should be taken into account for the FFS attributes

Dedicated preconfigured UL resource is defined as an PUSCH resource used by a single UE

  - PUSCH resource is time-frequency resource

  - Dedicated PUR is contention-free

Contention-free shared preconfigured UL resource (CFS PUR) is defined as an PUSCH resource simultaneously used by more than one UE

  - PUSCH resource is at least time-frequency resource

  - CFS PUR is contention-free

Contention-based shared preconfigured UL resource (CBS PUR) is defined as an PUSCH resource simultaneously used by more than one UE

  - PUSCH resource is at least time-frequency resource

  - CBS PUR is contention-based (CBS PUR may require contention resolution)

In idle mode, dedicated PUR is supported.

- Support for CFS PUR is FFS.

- Support for CBS PUR is FFS.

In IDLE mode, HARQ is supported for transmission in dedicated PUR

- A single HARQ process is supported,

  - FFS whether more than one HARQ processes are supported

- FFS: The design of the corresponding MPDCCH search space

For UL transmission in preconfigured resource, fallback mechanism to RACH/EDT procedures is supported.

For transmission in preconfigured UL resources, an RRC idle UE may use the latest TA that passed the validation criteria

Pre-configured UL resources for transmission of data are indicated by RRC signaling. At least UE-specific RRC signaling is supported.

## RAN1 #94

Idle mode based pre-configured UL resources is supported for UEs in possession of a valid TA

- FFS: Validation mechanism for TA

- FFS: How the pre-configured UL resources is acquired

For transmission in preconfigured UL resources, the UE may use the latest TA of which its validity can be confirmed

Study both shared and dedicated resource for preconfigured UL resources. If both shared and dedicated resources are supported, strive for commonality in design of both resource types.

HARQ procedures for transmission in preconfigured UL resources should be studied and the following aspects should be considered:

  - Whether to support HARQ;

      - If supported, details of HARQ design including the number of HARQ processes;

  - Whether ACK/NACK is necessary

Fallback mechanisms should be considered, e.g. fallback to legacy RACH/EDT procedures.