3GPP TSG RAN WG1 Meeting #97	Tdoc R1-1906110

Reno, USA,  May 13 – May 17, 2019

Agenda Item:	7.2.6.7

Source:	Ericsson

Title:	Preliminary Evaluation Results for Uu QoS Characteristics Values for V2X Services

Document for:	Discussion, Decision

# Introduction

An LS has been sent from SA2 to RAN1 and RAN2 [1], regarding combination of QoS characteristics values for eV2X services. In essence, two new standardized 5QI values are under consideration for eV2X services. RAN1 and RAN2 are requested to study if the following two new combinations of QoS characteristics values are feasible or not:

- Case 1: PDB (Packet Delay Budget) = 5 ms, PER (Packet Error Rate) = 10-4  and MDBV (Maximum Data Budget Value) = 1354 bytes, for services Collision Avoidance and Platooning with high LoA

- Case 2: PDB ~ 1.5 ms, PER = 10-5  and MDBV ~ 1300 bytes, for services Emergency Trajectory and Sensors information Sharing with high LoA

In this contribution we provide preliminary system level simulation results for the feasibility of Case 1 and Case 2.

# Discussion

## Case 1 simulation results

The QoS characteristics PDB, PER and MDBV in the LS need to be transformed into RAN1 requirements for performance evaluation. PDB includes both latency in core network and air interface latency. We assume a 2 ms core network latency as is assumed for remote driving scenario [2]. For PER we use the same value for packet error rate over air interface. Thus, we assume latency and reliability requirements to be 3 ms and 99.99%.

The traffic for Case 1 designed for Collision Avoidance and Platooning with high LoA includes both periodic and a-periodic arrival of packets with sizes up to MDBV. For our simulations we assume period packet arrival with arrival interval of 5 ms and packets yielding a TBS of at least 1354 bytes. The actual TBS vary depending on scheduler and link adaptation choices of allocation size and MCS. To summarize we use the RAN1 requirements according to:

Table 1: Case 1 requirements for RAN1 evaluation

|  | Reliability | Air interface latency | Data packet size and traffic model |
| --- | --- | --- | --- |
| Case 1 | 99.99% | 3ms | Packet size 1354 bytes Periodic data arrival rate 200 packets per second |

The system level simulation assumptions are summarized in Table 4 in the appendix. Since 30 kHz SCS is being used, a 3-ms air interface latency budget corresponds to 6 slots. Within this time limit, there could be many transmission strategies proposed, including segmentation or slot aggregation. In this contribution we schedule half-slots and use adaptive HARQ re-transmissions. Since traffic is periodic UL scheduling can be grant-based without relying on SR (traffic is assumed to be predictable).

We assume Capability 2 processing capability and since N1 = 4.5 symbols we can use 1-symbol PDCCH and 1-symbol PUCCH which enables UE to send HARQ-ACK with n+1 timing for 7-OS TTI. PUSCH preparation time is 5.5 symbols which enable n+1 TTI timing from grant to PUSCH transmission. For gNB timing we assume n+3 re-transmission timing which leave 7-OS for gNB to decode HARQ-ACK and schedule and prepare a re-transmission assignment.

The simulation results are summarized in Table 2.

Table 2: Percentage of UEs fulfilling URLLC requirements

|  | Number of UEs per cell |  |  |
| --- | --- | --- | --- |
|  | 5 | 10 | 15 |
| Downlink | 100% | 100% | 99.9% |
| Uplink | 98.3% | 97.5% | 87.1% |

With the system level simulation assumption in Table 5 for Case1 more than 95% of UEs fulfill 3 ms latency and 99.99% reliability can be achieved with up to 10 UEs per cell.

## Case 2 simulation results

For Case 2, a core network latency of 1 ms give only 0.5 ms latency budget for RAN. For 30 kHz SCS even 7 monitoring occasions per slot and 2-OS PDSCH the worst-case DL latency is above 0.5 ms. According to agreed assumptions of UE and gNB processing time for the Rel-16 URLLC SI, the gNB processing for initial PDSCH is N2/2 + 2 symbols and UE decoding time for last PDSCH is N1+d_1,1, where d_1,1=1 since PDCCH and PDSCU overlaps in one symbol. The total worst-case latency including alignment delay for DL is N2/2 + 2 + 2*2 + N1+d_1,1 = 14.25 symbols which equals ~0.51 ms. For UL with configured grant, worst-case latency below 0.5 ms is possible, but considering that the packet size is ~1300 bytes it will be very challenging to meet the reliability requirements.

With 1 ms core network latency, 1.5 ms PDB cannot be fulfilled for DL while for UL the reliability requirement will be very challenging.

With a core network latency of 0.75 ms, then RAN also have 0.75 ms latency budget but still 7-OS TTI, i.e. 7-OS PDSCH and 2 PDCCH monitoring occasions per slot, cannot meet the latency requirement for both DL and UL. However, with 4-OS PDSCH and more PDCCH monitoring occasions it is possible to meet the latency requirement. If core network latency is only 0.5 ms, then RAN have a 1 ms latency budget and 7-OS TTI is possible to use.

With system level simulation assumptions in Table 5 the results are summarized in Table 2 and Table 3, with 4-OS TTI and with assumption that DL and UL transmissions may cross slot border without performance loss.

Table 3: Percentage of UEs fulfilling URLLC requirements with 4-OS TTI and 5 UEs/cell.

|  | RAN latency budget |  |  |
| --- | --- | --- | --- |
|  | 0.5 ms | 0.75 ms | 1 ms |
| Downlink | 13.3% | 98.4% | 100% |
| Uplink | 60.6% | 96.9% | 96.9% |

Table 4: Percentage of UEs fulfilling URLLC requirements with 4-OS TTI and 10 UEs/cell.

|  | RAN latency budget |  |  |
| --- | --- | --- | --- |
|  | 0.5 ms | 0.75 ms | 1 ms |
| Downlink | 11.5% | 94.3% | 100% |
| Uplink | 60.7% | 97.0% | 97.0% |

For Case2, with the system level simulation assumption in Table 5, 4-OS TTI, and the assumption of crossing-slot-border without performance loss:

- more than 95% of UEs fulfill 0.75 ms latency and 99.999% reliability can be achieved with up to 5 UEs per cell.

- more than 95% of UEs fulfill 1.0 ms latency and 99.999% reliability can be achieved with up to 5 UEs per cell.

# Conclusion

This contribution provides the evaluation results for the transport industry scenario. In section 2 we made the following observation.

Observation 1	With the system level simulation assumption in Table 5 for Case1 more than 95% of UEs fulfill 3 ms latency and 99.99% reliability can be achieved with up to 10 UEs per cell.

Observation 2	With 1 ms core network latency, 1.5 ms PDB cannot be fulfilled for DL while for UL the reliability requirement will be very challenging.

Observation 3	For Case2, with the system level simulation assumption in Table 5, 4-OS TTI, and the assumption of crossing-slot-border without performance loss:

- more than 95% of UEs fulfill 0.75 ms latency and 99.999% reliability can be achieved with up to 5 UEs per cell.

- more than 95% of UEs fulfill 1.0 ms latency and 99.999% reliability can be achieved with up to 5 UEs per cell.

# References

- S2-1813386 eV2X Uu QoS characteristics, TSG-SA2 meeting #129bis, November 2018

- “RAN1 Chairman’s notes”, 3GPP TSG RAN WG1 Meeting #94bis, October 2018

# Appendix. Simulation assumptions

Table 5: System level simulation assumption

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid 1 site with 3 sectors |
| Inter-BS distance | 500m |
| Carrier frequency | 4 GHz |
| Channel model | UMa in TR 38.901 |
| UE Tx power | 23dBm |
| BS antenna configurations | Case 1: 4Tx/4Rx ports; (M, N, P, Mg, Ng; Mp, Np) = (8, 2, 2, 1, 1; 1, 2), dH = 0.5λ, dV = 0.8λ; 3 degrees electrical antenna tilt Case 2: 16Tx/16Rx ports; (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 4), dH = 0.5λ, dV = 0.8λ; 3 degrees electrical antenna tilt |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 5dB |
| UE antenna configuration | 2 Tx/4 Rx antenna ports |
| UE antenna height | 3m |
| UE antenna gain | 0dBi |
| UE receiver noise figure | 9 dB |
| Total transmit power per TRxP | 49 dBm |
| Simulation bandwidth | 40 MHz DL and 40 MHz UL |
| Duplex mode | FDD |
| SCS | 30 kHz |
| UE distribution | 100% of users are outdoors |
| UE speed | 60 km/h |
| UE power control | alpha = 0.8, target SINR = 20 dB |
| HARQ/repetition | Case 1: Adaptive HARQ re-transmissions Case 2: Single-TX, configured-grant PUSCH |
| Channel estimation | Ideal |
| Traffic | Case 1: Periodic, 200 packets/sec, TBS 1354 bytes Case 2: Periodic, 200 packets/sec, TBS 1300 bytes |