3GPP TSG RAN WG1 #98	R1-1909102

Prague, CZ, August 26th – 30th, 2019

Source:	Sharp

Title:	Discussion on enhancements to scheduling/HARQ

Agenda Item:	7.2.6.4

Document for:	Discussion and Decision

Introduction

In RAN#83 meeting, enhancement for NR eURLLC as Rel-16 work item (Rel-16 URLLC) [1] approved that includes following detailed objectives for specification of enhancements to scheduling/HARQ [RAN1]:

- Out-of-order HARQ-ACK associated with PDSCHs with different HARQ process IDs

- Out-of-order PUSCH scheduling associated with different HARQ process IDs, including overlapping PUSCHs and non-overlapping PUSCHs in time-domain

- Methods to handle DL data/data resource conflicts for overlapping PDSCHs in time-domain, scheduled by dynamic DL assignments.

How to support out-of-order HARQ-ACK have been discussed in the previous meetings, and relating agreements and conclusions were reached as below [2] [3]:

RAN1#96 Agreements: For a Rel. 16 eURLLC UE and dynamic downlink scheduling, on the active BWP of a given serving cell, the HARQ-ACK associated with the second PDSCH with HARQ process ID x received after the first PDSCH with HARQ process ID y (x != y) can be sent before the HARQ-ACK of the first PDSCH. Specify based on the following solutions: - Solution 1: The UE always processes the second PDSCH. The UE may or may not drop the processing of the first channel. - Solution 2: The UE processes both the first and second PDSCHs as a UE capability with no condition. - Solution 3: The UE processes both the first and second channels under some conditions, e.g. using the CA capability. The conditions are reported as a UE capability. If the conditions are not satisfied, the UE behavior is not defined. - FFS: The details of the UE capability. - Solution 4: - A UE drops (terminates) the processing of the first PDSCH. - Alt1: The UE always drops the first PDSCH. - Alt2: Some scheduling conditions should be defined. If not satisfied, the UE drops the processing of the first channel. - FFS how to define the scheduling conditions, e.g., based on the number of RBs, TBS, number of layers, the gap between the first and second PDSCHs, the gap between the two PUCCHs carrying HARQ-ACK, etc. - The UE behavior, e.g., decision on dropping the first channel and timing capability associated with the second channel, is determined, and is fixed, after decoding the PDCCH associated with the first and the second PDSCH. - When the UE drops the processing of the first channel, increasing the minimum PDSCH processing procedure time (N1) of the second PDSCH by d symbols can be considered. - FFS the value of d. - Dropping the processing of the first PDSCH can be done in one of the two ways: - Alt1: dropping the processing of the first PDSCH on the same serving cell - Alt2: dropping the processing of a PDSCH(s) on the same cell or a different serving cell. - The UE only expects a maximum of one OOO PDSCH-to-HARQ-ACK flow on the active BWP of a given serving cell when applicable FFS whether or not, out-of-order operation is allowed across PDSCHs with PDSCH-to-HARQ gap compatible with PDSCH processing time (N1) for capability X. Agreements: For a Rel. 16 UE, on the active BWP of a given serving cell, the UE can be scheduled with a second PUSCH associated with HARQ process x starting earlier than the ending symbol of the first PUSCH associated with HARQ process y (x != y) with a PDCCH that does not end earlier than the ending symbol of first scheduling PDCCH. Specify based on the following solutions: - Solution 1: The UE always processes the second scheduled PUSCH. The UE may or may not drop the processing of the first schedeuled PUSCH. - If the first scheduled and second scheduled PUSCHs are not colliding in the time domain: - Solution 2: The UE processes both the first scheduled and second scheduled PUSCHs as a UE capability with no condition. - Solution 3: The UE processes both the first scheduled and second scheduled PUSCHs under some conditions. The conditions are reported as a UE capability. - FFS: The details of the UE capability. - Solution 4: - A UE drops (terminates) the processing of the first scheduled PUSCH. - Alt1: The UE always drops the first scheduled PUSCH. - Alt2: Some scheduling conditions should be defined. If not satisfied, the UE drops the processing of the first scheduled PUSCH. - FFS how to define the scheduling conditions, e.g., based on the number of RBs, TBS, number of layers, the gap between the first and the second PUSCHs, etc. - The UE behavior, e.g., decision on dropping the first scheduled PUSCH and timing capability associated with the second scheduled PUSCH, is determined, and is fixed, after decoding the PDCCH associated with first and the second scheduled PUSCHs. - When the UE drops the processing of the first scheduled PUSCH, increasing the minimum PUSCH preparation procedure time (N2) of the second PUSCH by d symbols can be considered. - FFS the value of d. - Dropping the processing of the first scheduled PUSCH can be done in one of the two ways: - Alt1: dropping the processing of the first scheduled PUSCH on the same serving cell - Alt2: dropping the processing of a PUSCH(s) on the same cell or different serving cell. - The UE only expects a maximum of one OOO PDCCH-to-PUSCH flow on the active BWP of a given serving cell when applicable. - FFS whether or not out-of-order operation is allowed across PUSCHs with PDCCH-to-PUSCH gap compatible with PUSCH processing time (N2) for capability X. - If the first scheduled PUSCH and the second scheduled PUSCH are colliding in the time domain, the UE drops the processing and the transmission of the first scheduled PUSCH. - For dropping, the scheduling limitations do not apply. The UE always drops the first scheduled PUSCH. - Other details of dropping are as those of the solution 4. RAN1#97 Conclusion: Study further whether/how to support the following scenarios for the handling of two unicast PDSCHs: - When different DL processing times are associated with different PDSCHs on the same serving cell, and the two PDSCHs are non-overlapping. - Note: The PDSCH-to-PUCCHs can be out-of-order or in-order. - Note: The solution(s) should address the UE processing pipelining issue. - Two PDSCHs follow DL processing timing capability #1 and #2, respectively, on the same serving cell. - FFS if any different solutions are necessary to address different scenarios when the above condition occurs - When the same DL processing time is configured on the same serving cell, and the two PDSCHs are non-overlapping, and the PDSCH-to-PUCCHs are out of order. - Note: There is no UE processing pipelining issue. - Note: the in-order PDSCH-to-PUCCHs are already handled in Rel-15. - The two unicast PDSCHs are overlapping at least in the time domain, regardless of whether the same or different DL processing times is configured on the same serving cell. - Note: The solution(s) should address the UE processing pipelining issue in this case.

In this contribution, we show our views on the details for out-of-order HARQ-ACK and out-of-order PUSCH scheduling, respectively.

Discussions

Out-of-order HARQ-ACK

At the RAN1#97 meeting [3], a conclusion has been drawn to study further whether/how to support the following scenarios for the handling of two unicast PDSCHs.

Conclusion: Study further whether/how to support the following scenarios for the handling of two unicast PDSCHs: - When different DL processing times are associated with different PDSCHs on the same serving cell, and the two PDSCHs are non-overlapping. - Note: The PDSCH-to-PUCCHs can be out-of-order or in-order. - Note: The solution(s) should address the UE processing pipelining issue. - Two PDSCHs follow DL processing timing capability #1 and #2, respectively, on the same serving cell. - FFS if any different solutions are necessary to address different scenarios when the above condition occurs - When the same DL processing time is configured on the same serving cell, and the two PDSCHs are non-overlapping, and the PDSCH-to-PUCCHs are out of order. - Note: There is no UE processing pipelining issue. - Note: the in-order PDSCH-to-PUCCHs are already handled in Rel-15. - The two unicast PDSCHs are overlapping at least in the time domain, regardless of whether the same or different DL processing times is configured on the same serving cell. - Note: The solution(s) should address the UE processing pipelining issue in this case.

Handling of two unicast PDSCHs in the scenarios would involve two unicast PDSCHs associated with different DL processing times. That is, the current Rel.15 DL processing capability restriction due to additional DMRS configuration should be removed. In Rel.15, mix of processing capabilities only happen for a UE reporting its capability of pdsch-ProcessingType2-Limited in a given serving cell. For the UE processing capability #2 with scheduling limitation for 30 kHz, capability #1 for a PDSCH is applied to the UE if the scheduled RBs of the PDSCH exceeds the 136 RBs, as a kind of fallback operation. Another PDSCH of which the scheduled RBs does not exceed the 136 RBs will follow the processing capability #2. Other than the scheduling limitation for 30kHz, in Rel.15, whether additional DMRS is configured for a given serving cell or not would restrict PDSCH processing capability applied in the serving cell. As known, if additional DMRS is configured for a serving cell, PDSCH processing capability #2 will not be applied to the serving cell. That is, processing capability #2 can only be applied to the serving cell for which additional DMRS is not configured. Due to the restriction of the additional DMRS configuration, only one processing capability can be applied to a given serving cell and mix of processing capabilities will not occur. For Rel.16 eURLLC, there is a possible deployment that the UE will support both eMBB and URLLC services on a serving cell. In this case, it is beneficial that additional DMRS can be configured especially for eMBB data transmission with long duration. The additional DMRS can facilitate the UE to make more accurate channel estimation and thereby improve the detection performances for the eMBB data with long duration. In the meanwhile, a UE expects to use the processing capability #2 to decode the URLLC data with front-loaded DMRS and can quickly transmit the feedback of URLLC data.

Observation 1:

- Due to the restriction of the additional DMRS configuration, only one processing capability is applied to a serving cell for a UE in Rel.15.

Proposal 1:

- For Rel.16 DL URLLC, Rel.15 DL processing capability restriction due to the additional DMRS configuration should be removed so that two different processing capabilities can be allowed to be applied to a same serving cell.

Regarding scenario 1, four detailed cases can be considered and illustrated in the Fig.1. PDSCH-to-PUCCHs in cases (a) and (c) are in-order, while PDSCH-to-PUCCHs in cases (b) and (d) are out-of-order.

Figure.1: four cases of scenario 1.

Firstly, the case (d) can be precluded given reasonable network would not schedule a such case. A reasonable scheduling is that the HARQ-ACK for an earlier PDSCH with high priority would be surely transmitted in advance. Regarding case (c), although the first PDSCH and the second PDSCH are corresponding to different processing capabilities, there is no impact on the UE’s processing pipelining. As in case (a) and (b), regardless of whether the PDSCH-to-PUCCH is in-order or out-of-order, UE’s pipeline operation may be impacted. For handling of two unicast PDSCH associated with different processing capability, UE’s pipelining issue would occur in such case that a latter PDSCH with capability #2 follows an earlier PDSCH with capability#1. In these cases, solutions are necessary to address the UE’s processing pipelining issue. Solutions to address the processing pipelining issue should accommodate two aspects where UE should guarantee the processing of the second PDSCH with high priority and UE should also maximally strive to process the first PDSCH with low priority if possible. Hence, the critical point is how to handle the first PDSCH in the premise of assuring the processing of the second PDSCH in the cases (a) and (b).

Solution 4 with Alt2 is a simple way to address the processing pipelining issue for the cases (a) and (b). The solution 4 with Alt2 has clearly denoted the UE behaviour regarding whether UE processes or drops the first PDSCH with capability #1. If scheduling conditions are satisfied, UE processes the first PDSCH. Otherwise, the UE drop the first PDSCH. A simple and reason scheduling conditions can be the time interval between the first and second PDSCHs corresponding to the different processing capabilities. As in Rel.15, for UE processing capability #2 with scheduling limitation for 30kHz in Rel.15, if time interval between two PDSCH respectively with capability 1 and capability 2 is within 10 symbols, UE may skip decoding the previous PDSCHs. Here 10 symbols are the PDSCH decoding time N1 of PDSCH processing capability 1 for SCS=30kHz. The situation is similar to the above-mentioned cases (a) and (b). Therefore, the Rel.15 mechanism can be reused to define the scheduling condition for the solution 4 with alt2 without costing much standardization efforts. The scheduling condition can be defined as whether a time interval between the first PDSCH and second PDSCH exceed a value, for example, a processing time for the first PDSCH as like the Rel.15 mechanism. The specific value can be further discussed. In the cases in which processing pipelining issue need to be addressed, if the time interval between the first PDSCH and second PDSCH is within the value, UE drops the first PDSCH. Otherwise, UE can also process the first PDSCH.

Regarding scenario 2 that two non-overlapping unicast PDSCHs correspond to same processing capability and the PDSCH-to-PUCCHs are out of order, UE can both process the first PDSCH and second PDSCH and store their corresponding HARQ-ACKs. What different from the Rel-15 is the transmission order of the HARQ-ACK. No additional solution is need for such scenario 2.

Regarding scenario 3 that two PDSCHs are overlapping at least in the time domain on a same serving cell, if network schedules such a scenario, it means that a latter PDSCH is emergency. In such scenario, a simple solution is that UE may process the latter PDSCH and drop the processing of the earlier PDSCH. Solution 4 with alt 2 is also applied by reinterpreting the scheduling conditions.

Base on above discussion, solution 4 with Alt2 is a preferred solution for the scenarios in which the UE processing pipelining issue need to be addressed.

Proposal 2:

- For scenarios in which the UE processing pipelining issue need to be addressed, solution 4 with Alt2 is preferred solution.

  - For two non-overlapping PDSCHs, the scheduling conditions can be defined as whether time interval between the first PDSCH and second PDSCH exceed a processing time of first PDSCH. The specific value is FFS.

  - For two overlapping PDSCHs, the scheduling conditions can be defined as whether two PDSCHs are overlapping or not.

Out-of-order PUSCH scheduling

For out-of-order PUSCH scheduling, two scheduled PUSCHs can be overlapping or non-overlapping in the time domain. If two scheduled PUSCHs are overlapping in the time domain, UE has to ensure the transmission of the second PUSCH scheduled by a later UL grant. UE drops the processing and transmission of the first PUSCH scheduled by an earlier UL grant as agreed in the last meeting.

For the case two PUSCHs are not overlapping in the time domain, similar discussion and solution for out-of-order HARQ can be considered for out-of-order PUSCH scheduling. One key discussion is whether to always drop the first PUSCH or conditionally drop the first PUSCH. The Rel-15 specification has defined a PUSCH preparation behavior that if the time offset between the UL grant and the scheduled PUSCH is equal to or larger than the PUSCH preparation procedure time, UE will perform the PUSCH processing/transmission. The PUSCH preparation procedure time defined in Rel-15 can be only several symbols, e.g. the timing capability 2. On the other hand, slot offset K2 in Rel-15 between the UL grant and corresponding PUSCH can be configured from 0 and up to 32. Taking into account the configurability of the smaller PUSCH preparation time and larger slot offset value, always dropping the first scheduled PUSCH is not a sensible way to the UE. When out-of-order PUSCH scheduling occurs, a UE can perform the processing of the second scheduled PUSCH. After completion of processing of the second scheduled PUSCH, the UE can perform first scheduled PUSCH processing if there is enough time for the first scheduled PUSCH. For an extreme case, the first scheduled PUSCH preparation can be discarded when a DCI for the second scheduled PUSCH is detected. After the transmission of the second scheduled PUSCH, the UE can start the first scheduled PUSCH preparation again even from the beginning. If the time is enough for UE to start the first scheduled PUSCH preparation again, UE can also transmit the first scheduled PUSCH without dropping. Therefore, if the time for the first scheduled PUSCH is large enough to accommodate the preparation time of the second scheduled PUSCH and then the first scheduled PUSCH, the UE can complete the processing/transmission of second scheduled PUSCH and also be capable of transmitting the first scheduled PUSCH. A key point whether or not a UE drops the first scheduled PUSCH lies in whether or not the UE can have sufficient time to continue to prepare and transmit the first scheduled PUSCH after completion of processing of the second scheduled PUSCH. Therefore, the scheduling conditions used for determining whether to drop the processing of first PUSCH can be defined as whether the time interval between the first PDCCH and corresponding first PUSCH can accommodate the preparation time of both second PUSCH and first PUSCH.

Observation 2:

- Even when the first scheduled PUSCH preparation is suspended due to the second scheduled PUSCH, if there is sufficient time for the first scheduled PUSCH preparation, UE can still perform the first scheduled PUSCH transmission without dropping.

Taking above aspects into account, we would like to propose followings.

Proposal 3:

- For out-of-order PUSCH scheduling, if two scheduled PUSCHs are not overlapping in time-domain, solution 4 with Alt2 is preferred.

  - The scheduling conditions can be defined as whether time interval between the first PDCCH and corresponding first PUSCH can accommodate the preparation time of both second PUSCH and first PUSCH. The specific value is FFS.

Conclusion

In this contribution, we have the following observations and proposals:

Observation 1:

- Due to the restriction of the additional DMRS configuration, only one processing capability is applied to a serving cell for a UE in Rel.15.

Proposal 1:

- For Rel.16 DL URLLC, Rel.15 DL processing capability restriction due to the additional DMRS configuration should be removed so that two different processing capabilities can be allowed to be applied to a same serving cell.

Proposal 2:

- For scenarios in which the UE processing pipelining issue need to be addressed, solution 4 with Alt2 is preferred solution.

  - For two non-overlapping PDSCHs, the scheduling conditions can be defined as whether time interval between the first PDSCH and second PDSCH exceed a processing time of first PDSCH. The specific value is FFS.

  - For two overlapping PDSCHs, the scheduling conditions can be defined as whether two PDSCHs are overlapping or not.

Observation 2:

- Even when the first scheduled PUSCH preparation is suspended due to the second scheduled PUSCH, if there is sufficient time for the first scheduled PUSCH preparation, UE can still perform the first scheduled PUSCH transmission without dropping.

Proposal 3:

- For out-of-order PUSCH scheduling, if two scheduled PUSCHs are not overlapping in time-domain, solution 4 with Alt2 is preferred.

  - The scheduling conditions can be defined as whether time interval between the first PDCCH and corresponding first PUSCH can accommodate the preparation time of both second PUSCH and first PUSCH. The specific value is FFS.

References

RP-190726 “New WID: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication(URLLC)”, RAN#83,Huawei, HiSilicon

“RAN1 Chairman’s Notes”, RAN1#96

“RAN1 Chairman’s Notes”, RAN1#97