3GPP TSG RAN WG1 Meeting #98                                      R1-1909158

Prague, CZ, August 26th – 30th, 2019

Agenda item:	6.2.2.2

Source:			Institute for Information Industry (III)

Title:			Discussion on transmission in preconfigured UL resources

Document for:	Discussion and Decision

# 1 Introduction

In RAN1 #97, support for transmission in preconfigured UL resources in NB-IoT was discussed and the following agreements were made [1].

Agreement For dedicated PUR in idle mode and for HD-FDD UEs, the start of the PUR SS Window is [x] subframes after the end PUR transmission - FFS: Value of x, and if x is fixed or signaled - FFS: Support for monitoring of PUR SS Window before PUR transmission Note: The PUR SS Window is the time period where the UE monitors the NPDCCH for at least a time period after a PUR transmission Agreement NPDCCH candidates are determined by USS like search space - FFS: Other details on the USS like search space - Type2-CSS can also be discussed as part of the FFS

There are still two working assumptions made in RAN1 #96bis as the following [2]. Whether to confirm the working assumptions depends on RAN2 progress.

- Working Assumption#1 - In idle mode, updating PUR configurations and/or PUR parameters via L1 signalling after a PUR transmission is supported - FFS: Which PUR configurations and PUR parameters will be signaled via L1 - FFS: Definition of PUR configurations and PUR parameters - The working assumption will be automatically confirmed if for some cases L2/L3 signaling is not needed. If RAN2 decides that L2/L3 signaling is needed for all cases, the working assumption will be reverted. - Working Assumption#2 - For dedicated PUR - During the PUR search space monitoring, the UE monitors for DCI scrambled with a RNTI assuming that the RNTI is not shared with any other UE - Note: It is up to RAN2 to decide how the RNTI is signaled to UE or derived - FFS if the UE monitors any additional RNTI which may be shared with other UEs. - Note: The same RNTI may be used over non-overlapping time and/or frequency resources - Send an LS to RAN2 to include two above working assumptions. Ask whether the first bullet in working assumption #2 is feasible. If it is concluded that working assumption #2 feasible, the working assumption #2 will be automatically confirmed. (LS is endorsed in eMTC agenda item)

In this contribution, we share our views on transmission in preconfigured UL resources in NB-IoT.

# 2 Discussion

## 2.1 TA maintenance in RRC_IDLE

In RRC_CONNECTED, eNB can adjust UE’s TA by a TA command, and TAT is restarted after new TA is applied. In RRC_IDLE, a downlink signaling should also be used to adjust UE’s TA based on UL transmission in preconfigured UL resource (PUR). The downlink signaling for TA adjustment can be transmitted to UE together with HARQ-ACK or a DCI eNB transmits to the UE for scheduling a retransmission in response to a PUR transmission, which can be taken into account when designing the DCI for HARQ-ACK and retransmission. Alternatively, TA command can be transmitted with L2/L3 response. As for the uplink transmission for eNB to estimate TA, besides UL transmission in PUR, NPRACH transmission can also be considered. To be more specific, a NPRACH resource and preamble indices dedicated for the purpose of TA update can be configured, so that a 2-step RACH for TA update can be performed when UE determines that the TA have become invalid. The 2-step RACH for TA update comprises 2 signaling exchanges, the first signaling is UE transmitting NPRACH preamble to eNB, and the second signaling is eNB responds UE with a RAR with only TA value and no uplink grant.

When TA is updated, the TAT should be restarted, so that the UE can use PUR when TAT is running and other configured TA validation criteria are satisfied. Therefore, when TAT expires or when other configured TA validation criteria are not satisfied, UE can first attempt to update the TA. When TA is successfully updated, UE keeps the PUR. And when the TA update is not successfully performed, UE fallback to legacy RACH or EDT procedure.

Proposal 1: UE can attempt to update TA after TAT expired.

Proposal 2: Dedicated NPRACH resource can be configured with the PUR for TA update purpose only.

## 2.2 NPDCCH monitoring

After transmission in PUR, UE expects eNB to respond with either L1 HARQ-ACK or L2/L3 signaling, which still depends on RAN2 progress. UE can also be scheduled to perform retransmission if the PUR transmission was not successfully decoded by eNB.

In any case, UE will monitor NPDCCH after PUR transmission. It was agreed the UE will monitor NPDCCH in a PUR SS window, and the PUR SS window starts at [x] subframes after the end of PUR transmission, where [x] is still FSS.

Whether L1 or L2/L3 response will be used will affect the time it takes for eNB to transmit the response.

For example, if only L1 HARQ-ACK is used, eNB can respond UE immediately after it successfully decodes PUR transmission. In this case, [x] can be a small value, for example, 3. On the other hand, when L2/L3 response is used, it is possible that the response carries DL data. Since it takes longer time for eNB to determine whether there is DL data from MME or S-GW, L2/L3 response may be transmitted to UE hundreds or thousands of milliseconds after PUR transmission. In this case, [x] should be a larger value so that UE can start NPDCCH monitoring later. Furthermore, in case eNB can know whether there is higher layer ACK for the UL data, eNB can release the UE earlier, so that the response can be transmitted earlier. This is related to the discussion on whether to introduce AS release assistance indication (RAI) in RAN2. Therefore, the value of [x] should wait for RAN2 progress on the above issues.

Observation 1: [x] for start time of PUR SS window depends on whether L1 or L2/L3 response is used.

It was agreed that UE can skip a PUR transmission. To save UE power, UE should not monitor NPDCCH if it skipped the PUR occasion.

Proposal 3: UE does not monitor NPDCCH in a PUR SS window if the associated PUR occasion is skipped.

## 2.3 HARQ

In case eNB did not successfully decodes the PUR transmission, it was agreed that eNB can use a DCI to schedule UE to perform retransmission. On the other hand, if eNB successfully decodes the PUR transmission, in some cases, HARQ-ACK can be used to respond the UE.

It was not decided whether HARQ-NACK should be introduced. The purpose of HARQ-NACK should be similar to DCI for retransmission. The difference is that HARQ-NACK may not include the resource allocation information for retransmission. Therefore, resource for retransmission of a PUR transmission should be preconfigured if HARQ-NACK is to be introduced. However, it is not resource efficient since the preconfigured resource for retransmission may not be used. Alternatively, retransmission can utilize the PUR in the subsequent periods, but it will cause the new UL data in subsequent periods to be delayed. Therefore, there benefit of introducing HARQ-NACK is not clear.

Proposal 4: HARQ-NACK is not introduced for PUR transmission.

For a PUR transmission, after monitoring NPDCCH in the PUR SS window, if no L1 or L2/L3 response or UL grant for retransmission is received, UE should fallback to legacy RACH or EDT procedure for UL data transmission.

Proposal 5: UE fallbacks to legacy RACH or EDT procedure if nothing is received in PUR SS window.

Since it was agreed that only one HARQ process is supported for PUR, retransmission of UL data by RACH or EDT procedure should be completed before the next PUR occasion. Otherwise, the next PUR occasion cannot be used for transmission of new UL data.

## 2.4 Activation and Release of preconfigured resource

To maintain the eNB scheduling flexibility and resource utilization efficiency, it should be able to activate and release the PUR for UE(s) after resource configuration signaling is delivered to UEs. It is also beneficial for eNB to release PUR for certain UEs if it detected that the UE’s link quality has degraded. Doing so can lower the interference the UE causes to other uplink transmission from other UEs, and can also lower the power consumption because the transmission in PUR which are not likely to be successfully decoded can be avoided. Possible signaling for eNB to indicate to UEs in RRC_IDLE to release the PUR can be through system information, or paging message, or transmitted by dedicated RRC message which is agreed in RAN2.

On the other hand, methods for UE to automatically release the PUR should also be designed to avoid possible interference to other uplink transmission. It was agreed in RAN2 that PUR is released when the eNB doesn’t detect “m” consecutive UE transmissions.

How to realize the agreement has not been decided. That is, how UE determines that eNB has not detected “m” consecutive UE transmissions should be discussed. In our view, HARQ-ACK can be utilized. For example, UE can autonomously release the PUR if it does not receive HARQ-ACK for “m” consecutive PUR occasions, if only L1 HARQ-ACK is expected. It can be further discussed whether a DCI for retransmission should be counted. If “detection” means that eNB knows the UE has transmitted in the PUR, regardless of the decoding result, then UE receiving a DCI for retransmission should not be counted towards “m”.

Since whether L1 or L2/L3 will be used and in what condition will they be used is still under discussion in RAN2, the following can be agreed first.

Proposal 6: UE releases the PUR when no L1 or L2/L3 response from eNB has been received for “m” consecutive PUR occasions.

On the other hand, if “detection” means that eNB does not successfully decodes the PUR transmission, then UE receiving a DCI for retransmission can be counted towards “m”. However, the link quality may quite different for the case where eNB does not know the UE’s presence from the case where eNB knows the UE’s presence but did not decode the transmission correctly. Therefore, it may be better not to mix the two cases together.

Nonetheless, in case eNB schedules UE for retransmission of PUR transmission for “m” consecutive PUR occasions, it is clear that there is misconfiguration for resource allocation or TBS of the PUR. In this case, it can be further discuss whether UE should autonomously release the PUR, or should UE initiate legacy RACH or EDT procedure to request reconfiguration of PUR. Alternatively, it can be left to eNB implementation to reconfigure the PUR when this case happens. Note that “m” in this case may be different than the “m” in the above case.

Observation 2: There is PUR misconfiguration when “m” consecutive PUR transmission require retransmission.

# 3 Conclusions

Based on the discussion, we have the following observations and proposals.

Observation 1: [x] for start time of PUR SS window depends on whether L1 or L2/L3 response is used.

Observation 2: There is PUR misconfiguration when “m” consecutive PUR transmission require retransmission.

Proposal 1: UE can attempt to update TA after TAT expired.

Proposal 2: Dedicated NPRACH resource can be configured with the PUR for TA update purpose only.

Proposal 3: UE does not monitor NPDCCH in a PUR SS window if the associated PUR occasion is skipped.

Proposal 4: HARQ-NACK is not introduced for PUR transmission.

Proposal 5: UE fallbacks to legacy RACH or EDT procedure if nothing is received in PUR SS window.

Proposal 6: UE releases the PUR when no L1 or L2/L3 response from eNB has been received for “m” consecutive PUR occasions.

# 4 Reference

[1] RAN1 Chairman’s Notes, RAN1 #97, Reno, USA, 13th – 17th May, 2019

[2] RAN1 Chairman’s Notes, RAN1 #96bis, Xi’an, China, 8th – 12th April, 2019