3GPP TSG RAN WG1 #98bis                                 	       R1-1910001

Chongqing, China, October 14th - 20th, 2019

Agenda item:		7.2.6.4

Source: 		Idaho National Laboratory

Title:   	Non-Periodic Scheduling Request Transmitter for NR Release 16: Additional Results and Considerations

Document for:    	Discussion/Decision

# Introduction

In a previous contribution [1], we presented the Non-Periodic Scheduling Request (NP-SR) transmitter structure, some preliminary results in terms of SR missed detection as a function of SNR for a varying number of URLLC users/cell and a comparison with the periodic NR SR method highlighting the advantages of NP-SR. In this contribution we present additional performance results taking into account interference from an NR signal sharing the same spectrum with the NP-SR signal.

The contribution is organized as follows:

- Section 2 presents additional simulation results for NP-SR performance in the presence of NR signals sharing the same spectrum as NP-SR.

- Section 3 presents some conclusions.

# Additional NP-SR Simulation Results

In this section we update the NP-SR performance results presented in [1] to include the case of the NP-SR sharing the same spectrum as an UL NR. Interference is modelled as a set of Resource Blocks (RBs) randomly located in frequency within the occupied total bandwidth, with the total RB bandwidth amounting to a percentage of the occupied total bandwidth by the NP-SR signal (see [1] and table of simulation assumptions in the Appendix), which is made a parameter in the simulations.  The interfering UL NR signal power is assumed to be 15 dB above the noise power level.

Figure 1 shows the NP-SR missed detection performance as a function of SNR for varying levels of interfering bandwidth ranging from 0% to 70% for 10 URLLC users/cell for the factory automation cell layout considered in [1]. The figure shows the expected performance degradation as the bandwidth of the interfering signal increases. For 10 users/cell and for a missed detection probability under 1.0e-4, this degradation amounts to approximately 0.2 dB, 2.4 dB and 4.2 dB, as the bandwidth of the UL NR interfering signal varies to 20%, 50% and 70% of the total bandwidth of the NP-SR signal of 40 MHz, respectively. Despite the degradation in detection performance due to the increased bandwidth of the interference, the required SNR for this SR missed detection probability remains under -10 dB, which is unlikely to cause significant interference to the UL NR signal.

Figure 1 NP-SR performance results (NP-SR missed detection) for 10URLLC UEs

BW = 40 MHz. Factory Automation Cell Layout considered in [1]  (10 and 20 UEs per cell).

# Conclusions

Based on the above and on the results previously presented in [1], the NP-SR method has the following advantages over the periodic SR method adopted in NR Rel-15:

- It eliminates the wait time incurred by the periodic SR method, which becomes a non-negligible latency component in the stringent overall latency target for some applications, such as Factory Automation.

- It eliminates spectrum overhead, as the NP-SR signal is transmitted over spectrum already occupied by other resources and is spread at a sufficiently low spectral density to cause negligible interference.

- NP-SR performance (in terms of missed SR detection probability) is relatively insensitive to the number of users per cell for the range of users required for a factory automation scenario contemplated in [1].

- NP-SR detection performance remains robust in the presence of a strong UL NR interfering signal, requiring SNR levels for acceptable performance that are unlikely to cause interference concerns.

Proposal: The NP-SR method should be considered for NR Release 16.

# References

[1] R1-1908290, “Non-Periodic Scheduling Request Transmitter for NR Release 16,” Idaho National Laboratory, RAN1#98, August 2019, Prague, Czech Republic.

# Appendix: Simulation Assumptions

See [1], Annex A.2.2 Simulation Assumptions for Factory Automation

| Parameter | Value | Notes |
| --- | --- | --- |
| Number of cells | 12 | As per agreed Factory Automation cell layout in TR 38.824 |
| Number of URLLC UEs per cell | 10 |  |
| Channel model | TDL-A | Rms delay spread = 11 ns. Pathloss and LOS/NLOS probability according to 3GPP TR 38.901. |
| Zadoff-Chu root sequences | 12 | One root sequence per cell is used for 10 and 20 users/cell |
| Packet arrival distribution and packet arrival rate | Poisson distribution λ = 500 packets/sec | NP-SRs are transmitted in the OFDM symbol immediately following packet arrival. |
| NR interfering signal power | 15 dB above noise power level |  |
| Carrier frequency | 4 GHz |  |
| Simulation bandwidth | 40 MHz |  |
| Subcarrier Spacing | 30 kHz |  |
| Number of subcarriers | 2048 |  |
| Cyclic Prefix Length | 149 | Channel impulse response is 140 taps. Normal CP is assumed. |