3GPP TSG RAN WG1 #98-Bis                                                             R1-1910683

Chongqing, China 14th – 20th October, 2019

Source: 	Intel Corporation

Title:	Discussions on aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH

Agenda item:	7.2.13.4

Document for:	Discussion and Decision

# Introduction

At the RAN #85 meeting it was agreed to revise the scope of Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements work item [1]. The following objective was added to WID:

- Introduce support for aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH [RAN1]

In this contribution solutions to support aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH are discussed.

# Discussion

The main motivation to support aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH is to enable transmission of a CSI report on FR1 carrier which corresponds to CSI-RS measured on FR2 carrier. Thus, coverage issue of UL transmission for FR2 can be avoided for CSI reporting. In addition to the use case described above cross-numerology CSI-RS triggering may be beneficial for other cases. For example, it can be used to transmit “heavy” Type II CSI feedback measured on FR1 carrier on FR2 carrier in order to avoid high UL resource utilization and interference level on FR1 carrier. Thus, in our view it is beneficial to support both cases  μCSIRS < μPDCCH and μCSIRS > μPDCCH.

Proposal 1:

- Support aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH for μCSIRS < μPDCCH and μCSIRS > μPDCCH

  - μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH respectively

In order to determine the slot for CSI-RS reception in the numerology of CSI-RS, reference slot with index n’ can be defined. The slot index for CSI-RS transmission is derived based on the equation n’+X, where X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset. The reference slot index is determined based on the slot n corresponding to the triggering DCI in the numerology of the PDCCH containing the triggering DCI. The following equations can be considered for the reference slot index determination.

- Alt. 1: n'=(n+1)2μCSIRS2μPDCCH

- Alt. 2: n'=n2μCSIRS2μPDCCH

The reference slot location is schematically represented in figure 1 and figure 2 for μCSIRS > μPDCCH and μCSIRS < μPDCCH respectively for the considered alternatives.

Figure 1. Location of reference slot n’ for μCSIRS > μPDCCH

Figure 2. Location of reference slot n’ for μCSIRS < μPDCCH

Alt. 1 has higher scheduling flexibility comparing to Alt. 2. However, additional timing requirements for DCI decoding and UE beam switching may be needed for Alt. 1. Thus, we propose to support Alt. 2 for the reference slot index determination. The text proposal for the Rel. 15 specification corresponding to Alt. 2 is captured in the Appendix for reference.

Proposal 2:

- For aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH, the slot index for CSI-RS transmission in the numerology of CSI-RS is derived based on the following equation (n+1)2μCSIRS2μPDCCH+X

  - n is the slot index corresponding to the triggering DCI in the numerology of the PDCCH containing the triggering DCI

  - X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset

  - μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively

# Conclusions

In this contribution support of aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH is discussed. The following proposals were made.

Proposal 1:

- Support aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH for μCSIRS < μPDCCH and μCSIRS > μPDCCH

  - μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH respectively

Proposal 2:

- For aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH, the slot index for CSI-RS transmission in the numerology of CSI-RS is derived based on the following equation (n+1)2μCSIRS2μPDCCH+X

  - n is the slot index corresponding to the triggering DCI in the numerology of the PDCCH containing the triggering DCI

  - X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset

  - μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively

# References

- RP-1922323 Revised WID: Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements, CMCC

- RP-190632 Correction to aperiodic CSI-RS triggering with different numerology between PDCCH and CSI-RS, Ericsson, AT&T, T-Mobile USA, Sprint, Nokia, Nokia Shanghai Bell, CHTTL, Deutsche Telekom

# Appendix

START OF TEXT PROPOSAL FOR SECTION 5.2.1.5.1 OF 38.214

##### 5.2.1.5.1	Aperiodic CSI Reporting/Aperiodic CSI-RS

For CSI-RS resource sets associated with Resource Settings configured with the higher layer parameter resourceType set to 'aperiodic', 'periodic', or 'semi-persistent', trigger states for Reporting Setting(s) (configured with the higher layer parameter reportConfigType set to 'aperiodic') and/or Resource Setting for channel and/or interference measurement on one or more component carriers are configured using the higher layer parameter CSI-AperiodicTriggerStateList. For aperiodic CSI report triggering, a single set of CSI triggering states are higher layer configured, wherein the CSI triggering states can be associated with any candidate DL BWP. A UE is not expected to receive more than one DCI with non-zero CSI request per slot. A UE is not expected to be configured with different TCI-StateId's for the same aperiodic CSI-RS resource ID configured in multiple aperiodic CSI-RS resource sets with the same triggering offset in the same aperiodic trigger state. A UE is not expected to receive more than one aperiodic CSI report request for transmission in a given slot. A UE is not expected to be triggered with a CSI report for a non-active DL BWP. A trigger state is initiated using the CSI request field in DCI.

-	When all the bits of CSI request field in DCI are set to zero, no CSI is requested.

-	When the number of configured CSI triggering states in CSI-AperiodicTriggerStateList is greater than , where  is the number of bits in the DCI CSI request field, the UE receives a subselection indication, as described in subclause 6.1.3.13 of [10, TS 38.321], used to map up to  trigger states to the codepoints of the CSI request field in DCI.  is configured by the higher layer parameter reportTriggerSize where . When the HARQ/ACK corresponding to the PDSCH carrying the subselection indication is transmitted in the slot n, the corresponding action in [10, TS 38.321] and UE assumption on the mapping of the selected CSI trigger state(s) to the codepoint(s) of DCI CSI request field shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ.

-	When the number of CSI triggering states in CSI-AperiodicTriggerStateList is less than or equal to , the CSI request field in DCI directly indicates the triggering state.

-	For each aperiodic CSI-RS resource in a CSI-RS resource set associated with each CSI triggering state, the UE is indicated the quasi co-location configuration of quasi co-location RS source(s) and quasi co-location type(s), as described in Subclause 5.1.5, through higher layer signaling of qcl-info which contains a list of references to TCI-State's for the aperiodic CSI-RS resources associated with the CSI triggering state. If a State referred to in the list is configured with a reference to an RS associated with 'QCL-TypeD', that RS may be an SS/PBCH block located in the same or different CC/DL BWP or a CSI-RS resource configured as periodic or semi-persistent located in the same or different CC/DL BWP.

-	If the scheduling offset, in the numerology of the aperiodic CSI-RS, between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without the higher layer parameter repetition is smaller than the UE reported threshold beamSwitchTiming, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}.

-	if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48}, periodic CSI-RS, semi-persistent CSI-RS;

-	else, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored.

-	If the scheduling offset, in the numerology of the aperiodic CSI-RS, between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the UE reported threshold beamSwitchTiming when the reported value is one of the values of {14,28,48}, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI.

-	A non-zero codepoint of the CSI request field in the DCI is mapped to a CSI triggering state according to the order of the associated positions of the up to 2NTS-1 trigger states in CSI-AperiodicTriggerStateList with codepoint '1' mapped to the triggering state in the first position.

For a UE configured with the higher layer parameter CSI-AperiodicTriggerStateList, if a Resource Setting linked to a CSI-ReportConfig has multiple aperiodic resource sets, only one of the aperiodic CSI-RS resource sets from the Resource Setting is associated with the trigger state, and the UE is higher layer configured per trigger state per Resource Setting to select the one CSI-IM/NZP CSI-RS resource set from the Resource Setting.

When aperiodic CSI-RS is used with aperiodic CSI reporting, the CSI-RS triggering offset X is configured per resource set by the higher layer parameter aperiodicTriggeringOffset. The CSI-RS triggering offset has the values of {0, 1, 2, 3, 4, 16, 24} slots. The aperiodic CSI-RS is transmitted in slot n+X if the PDCCH SCS is equal to the CSI-RS SCS and n+12μCSIRS2μPDCCH+X otherwise, where n is the slot with the triggering DCI in the numerology of the PDCCH containing the triggering DCI, X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset, and μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively. If all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states, the CSI-RS triggering offset X is fixed to zero. The aperiodic triggering offset of the CSI-IM follows offset of the associated NZP CSI-RS for channel measurement.

The UE does not expect that aperiodic CSI-RS is transmitted before the OFDM symbol(s) carrying its triggering DCI.

If interference measurement is performed on aperiodic NZP CSI-RS, a UE is not expected to be configured with a different aperiodic triggering offset of the NZP CSI-RS for interference measurement from the associated NZP CSI-RS for channel measurement.

If the UE is configured with a single carrier for uplink, the UE is not expected to transmit more than one aperiodic CSI report triggered by different DCIs on overlapping OFDM symbols.

END OF TEXT PROPOSAL FOR SECTION 5.2.1.5.1 OF 38.214