3GPP TSG RAN WG1 Meeting #99	R1-1912912

Reno, USA, November 18–22, 2019

Agenda Item:	7.2.1.3

Source:	Huawei, HiSilicon

Title:	Further discussion and evaluation results for 2-step RACH

Document for:	Discussion and Decision

Introduction

In RAN#82, a new work item to specify 2-step RACH in NR was agreed [1]. After RAN1#96, there was an email discussion on potential LLS assumptions for 2-step RACH [2], and some simulation parameters have been agreed as in Table A.1 of the Appendix. In our companion contributions [3][4], the channel structure and the procedure of 2-step RACH have been discussed. In this contribution, we present further discussion and evaluations for 2-step RACH, including the preamble detection, payload size, resource size and MCS, intra-slot hopping, and mapping between preambles and PRUs,.

PRACH Configuration

Preamble Detection

In 2-step RACH, preamble detection is applied by gNB to detect whether there is any transmission of MsgA, and there can be miss-detection and false-alarm for preamble detection. In this contribution, the miss-detection rate Pm is defined as the ratio between the number of transmitted preambles that are not detected or detected as a different preamble and total number of transmitted preambles within an observation interval, and the false-alarm rate Pf is defined as the ratio between the number of detected but not transmitted preambles and total number of not transmitted preambles within an observation interval.

Figure 1 presents the preamble detection performance for different preamble formats. In the simulation, the carrier frequency is 4GHz, the subcarrier spacing for the short preambles (format A1, A2, A3) is assumed to be 30 kHz, and the number of active UEs can be 1 or 2. For all the curves, the target false-alarm rate is kept the same, i.e., Pf=1%, or 0.1%. The detailed simulation parameters are given in the Table A.1 and Table A.2 of Appendix. The evaluation results show that the miss-detection rate will decrease with increasing SNR values, lower number of active UEs, or higher false-alarm rate. For single active UE case,

- the required SNR values to meet the given performance requirements are summarized as Table 1 if the target Pm=1%;

- the link budget for different preamble formats is further analyzed in Table 2, showing that the MCL for preamble format 0, A1, A2, A3 are 143.6dB, 130.5dB, 133.4dB, and 135.4dB, respectively with Pm=1%, Pf=0.1%.

| (a). Format 0, SCS=1.25 kHz |  | (b). Format A1, SCS=30 kHz |
| --- | --- | --- |
| (c). Format A2, SCS=30 kHz |  | (d). Format A3, SCS=30 kHz |

Figure 1. Miss-detection rate and false-alarm rate of preamble detection

Table 1. Minimum SNR values to meet the requirements of Pm and Pf (single active UE)

| Preamble format | (Pm,Pf) | Minimum SNR value (dB) |
| --- | --- | --- |
| 0 | (1%, 1%) | -13.2 |
|  | (1%, 0.1%) | -11.8 |
| A1 | (1%, 1%) | -6.8 |
|  | (1%, 0.1%) | -4.7 |
| A2 | (1%, 1%) | -9.7 |
|  | (1%, 0.1%) | -7.6 |
| A3 | (1%, 1%) | -11.8 |
|  | (1%, 0.1%) | -9.6 |

Table 2. Link budget calculation of different preamble formats (single active UE)

| Preamble Format | 0 | A1 | A2 | A3 |
| --- | --- | --- | --- | --- |
| (1) Tx Power (dBm) | 23 | 23 | 23 | 23 |
| (2) Thermal noise density (dBm/Hz) | -174 | -174 | -174 | -174 |
| (3) eNB receiver noise figure (dB) | 5 | 5 | 5 | 5 |
| (4) Interference margin (dB) | 0 | 0 | 0 | 0 |
| (5) Occupied channel bandwidth (Hz) | 1.04875e6 | 4.17e6 | 4.17e6 | 4.17e6 |
| (6) Effective noise power = (2)+(3)+(4)+10log((5)) (dBm) | -108.8 | -102.8 | -102.8 | -102.8 |
| (7) Required SINR (dB) (Pm=1%, Pf=0.1%) | -11.8 | -4.7 | -7.6 | -9.6 |
| (8) Receiver sensitivity = (6)+(7) (dBm) | -120.6 | -107.5 | -110.4 | -112.4 |
| (9) Receiver processing gain | 0 | 0 | 0 | 0 |
| (10) MCL = (1)-(8)+(9) (dB) | 143.6 | 130.5 | 133.4 | 135.4 |

Based on the above evaluations, we have the following observation:

Observation 1: The performance of preamble detection depends on preamble formats, and the number of active UEs for a given false-alarm rate.

PUSCH Configuration

Payload Size

In this section, link-level simulation is applied to evaluate the payload size of MsgA. Table 3 lists the simulation cases with different # of PRBs, modulation size, and TBS. In the evaluation, the time duration of PUSCH occasion is one slot, with 12 symbols for data and 2 symbols for DMRS, i.e., there are 144 REs per PRB for data transmission. Since the 2-step RACH is mainly for latency reduction compared to 4-step RACH, the reliability of the initial transmission should be enhanced to avoid frequent retransmission. Otherwise, the latency can be worse than 4-step RACH if the initial transmission fails. In the following evaluation, the target BLER of initial transmission is set to be 1%. For all the simulation cases, inter-cell interference is not considered yet.

Table 3. Simulation cases for payload size

| Case | # of PRBs | Modulation | TBS |
| --- | --- | --- | --- |
| 0 | 1, 2, 3 | QPSK | 56 |
| 1 | 1, 2, 3 | QPSK | 72 |
| 2 | 3, 6, 12 | QPSK | 200 |
| 3 | 6, 12 | QPSK | 1000 |

Single UE

Figure 2 presents the BLER performance for the single UE case. The results show that in general the required SNR values decrease when there are more PRBs, i.e., more resources. When TBS is 56 bits, target BLER of 1% can be achieved by all the cases, and the BLER curves for different SCSs are close to each other. When TBS is 1000 bits, target BLER of 1% can only be achieved when the number of PRBs and the SCS is large enough. This is mainly due to the imperfect channel conditions. When the resource size is large, the code rate becomes low enough to tolerate the imperfect channel conditions.

With the required SNR values, the detailed link budget analysis for all the simulated cases are given in Table A.3 to A.6 of the Appendix. For each payload size, the simulation case with highest MCL are listed in Table 4. The results show that when the TBS is 56 or 72 bits, the MCL is close to that of short preambles (format A1, A2, A3), and about 10dB worse than the long preamble (format 0). When payload size is 1000 bits, the MCL is more than 10dB worse than that of short preambles (format A1, A2, A3), and more than 22dB worse than that of long preamble (format 0). The large difference on MCL means large payload size is not applicable for many cases. The results also show that to achieve the highest MCL, the resource overhead of large payload size (200, 1000 bits) is 12 PRBs, which is much larger than that of 72 bits payload size (3 PRBs).

Observation 2: When payload size is 56 or 72 bits, the MCL is close to that of short preambles (format A1, A2, A3), and more than 10dB worse than that of long preamble (format 0).

Observation 3: When payload size is 1000 bits, the MCL is more than 10dB worse than that of short preambles (format A1, A2, A3), and more than 22dB worse than that of long preamble (format 0), which potentially require more repetitions for payload transmission thus likely larger latency compared to Rel-15 4-step RACH.

Observation 4: The resource overhead of large payload size (200, 1000 bits) is much larger than that of 56 or 72 bit payload size.

| (a). TBS=56 bits | (b). TBS=72 bits |
| --- | --- |
| (c). TBS=200 bits | (d). TBS=1000 bits |

Figure 2. BLER performance of different cases for single UE case

Table 4. Link budget calculation of different payload sizes

| Case | 0 | 1 | 2 | 4 |
| --- | --- | --- | --- | --- |
| TBS | 56 | 72 | 200 | 1000 |
| SCS (kHz) | 30 | 30 | 30 | 30 |
| # of PRBs | 3 | 3 | 12 | 12 |
| (1) Tx Power (dBm) | 23 | 23 | 23 | 23 |
| (2) Thermal noise density (dBm/Hz) | -174 | -174 | -174 | -174 |
| (3) eNB receiver noise figure (dB) | 5 | 5 | 5 | 5 |
| (4) Interference margin (dB) | 0 | 0 | 0 | 0 |
| (5) Occupied channel bandwidth (Hz) | 1.08e6 | 1.08e6 | 3.24e6 | 3.24e6 |
| (6) Effective noise power = (2)+(3)+(4)+10log((5)) (dBm) | -108.7 | -108.7 | -103.9 | -103.9 |
| (7) Required SINR (dB) | -1.1 | 0.2 | -2.6 | 5.2 |
| (8) Receiver sensitivity = (6)+(7) (dBm) | -109.8 | -108.5 | -106.5 | -98.7 |
| (9) Receiver processing gain | 0 | 0 | 0 | 0 |
| (10) MCL = (1)-(8)+(9) (dB) | 132.8 | 131.5 | 129.5 | 121.7 |

Multiple UEs

When multiple UEs share the same PO, they can be differentiated either by orthogonal DMRS ports, or different DMRS sequences. In the evaluation of Section 4, we show that if the resource size is aligned, one-to-one mapping with different DMRS sequences is never worse than multiple-to-one mapping scheme with single DMRS sequences. Figure 3 shows the results when there are one or multiple UEs in one PUSCH occasion and there is no PUR collision. When the payload size is 72 bits, there can be 2 UEs multiplexed in the same PO without much performance degradation. When the payload size is 1000 bits, the performance degradation is much larger. As more UEs can be multiplexed in the same PO, the resource utilization will be higher for small payload size.

Observation 5: With small payload size, more UEs can be multiplexed in the same PO, and higher resource utilization can be achieved.

| (a). TBS=72bits, 3 PRBs, SCS=30 kHz | (b). TBS=1000bits, 12 PRBs, SCS=30 kHz |
| --- | --- |

Figure 3. BLER performance for multiple UE cases

Resource Size and MCS

The payload size is determined by the MCS level and resource size. For MsgA PUSCH, as the payload size is usually small, the value range of resource size and MCS can be different from normal PUSCH, so that the signaling overhead can be reduced. For UEs in RRC_IDLE/INACTIVE state, the payload size is 56 or 72 bits. For UEs in RRC_CONNECTED state, the payload size can be larger, up to 1000 bits. Assume the MCS table of ‘qam64’ is applied, and the number of symbols of PUSCH can be 2, 4, 7, or 14. Then, the TBS under different resource size and MCS level is shown in Figure 4.

For RRC_IDLE/INACTIVE state, the value range of MCS index and bandwidth of MsgA PUSCH can be smaller than that of normal PUSCH. For example, MCS index of 0~7 and bandwidth of 1~8 PRBs can be enough. For RRC_CONNECTED state, the value range of MCS index and bandwidth can be larger.

Observation 6: For RRC_IDLE/INACTIVE state, MCS index of 0~7 and bandwidth of 1~8 PRBs can be enough to support the payload size of 56 or 72 bits.

| (a). 2 symbol PUSCH with 1 symbol DMRS | (b). 4 symbol PUSCH with 1 symbol DMRS |
| --- | --- |
| (c). 7 symbol PUSCH with 1 symbol DMRS | (d). 14 symbol PUSCH with 2 symbol DMRS |

Figure 4. TBS for different MCS and resource size

Intra-slot hopping

In RAN1#98bis, it was agreed that intra-slot hopping is supported. Due to the asynchronous transmission of MsgA PUSCH, intra-slot hopping can introduce additional inter-UE interference. As shown in Figure 5(a), when there is time offset between UE1 and UE2, the PUSCH transmissions will overlap. Guard period between the hops can be introduced to reduce the collision between UEs, as shown in Figure 5(b). Assume the bandwidth is 6 PRBs, the SCS is 30 kHz, and the TBS is 200 or 1000, bits. As shown in Figure 6, intra-slot hopping will cause performance degradation. When cell radius is large, the potential overlap between PUSCH transmission will be larger, the performance of intra-slot frequency hopping can be even worse than that without frequency hopping. When additional guard period is applied, the performance can be much improved, and the diversity gain by frequency hopping can be achieved.

Observation 7: For intra-slot hopping of MsgA PUSCH, guard period between hops can reduce the potential collision between UEs.

| - Intra-slot hopping without guard period | - Intra-slot hopping with guard period |
| --- | --- |

Figure 5. Examples of intra-slot hopping

| (a). 6 PRBs, TBS=200 bits, SCS = 30 kHz | (b). 6 PRBs, TBS=1000 bits, SCS = 30kHz |
| --- | --- |

Figure 6. Evaluation results with intra-slot hopping

Mapping between Preamble and PRU

Collision Analysis

For contention-based RACH procedure, if more than one UE choose the same preamble, collision happens. In this case, gNB can only detect one UE at most and the other UEs will be dropped. Specifically, the collision probability can be defined as the ratio between the total number of collided preambles and the total number of transmitted preambles within an observation interval. In order to have good access efficiency, the collision probability should be kept low.

Assume UE arrival follows Poisson process, each PRACH occasion (RO) has K contention-based preambles, and the traffic density is λ UEs/RO, then the collision probability can be expressed as: Pc=1-e-λ/K [5]. If there are multiple preamble groups, and each preamble group is applied for different purpose, e.g., different payload size, then the collision probability can also be expressed with Pc,n=1-e-λn/Kn with Kn representing the number of preambles for the target group n, and λn representing the traffic density of the target group. Assume the number of preamble groups is N and each preamble group has the same number of preambles, i.e. Kn=KN, n=1,…, N, and for the nth group, the arrival rate is λn=anλ, and nan=1, n=1,…, N. In the following, we give some preliminary collision probability analysis and the impact of K, N, and an.

In Figure 7, we assume K=64 and N=2 or 4, and different an for each group (but only one group is shown). As can be observed from the figure, the collision probability increases with the traffic density, and the larger the traffic density (observed from larger an), the larger the collision probability. Moreover, comparing between the figures with N=2 and N=4, with the same traffic density for a given group, the collision probability is much higher in the case of N=4 than that of N=2, e.g., for an=0.6, the collision probability is less than 0.01 for N=2 and about doubled to 0.02 in the case of N=4.

| (a). N=2 | (a). N=4 |
| --- | --- |

Figure 7. Collision probability for multiple preamble groups (an: load portion of current group)

Observation 8: For the case with multiple preamble groups, the collision probability increases if the traffic load with respect to the available PRACH resource is not matched.

Regarding the dimensioning for 2-step RACH, collision probability should be low enough to guarantee the BLER performance would not have error floor. Assume the UE activity pattern follows a Poisson process, each PRACH occasion (RO) has 64 contention-based preambles all for 2-step RACH transmission, and the traffic density λ is defined as UEs/RO, then the collision probability can be approximated as 1-e-λ/64 if the traffic load is not very high. It can be simply verified that when the collision probability is requested to be less than 1%, the average traffic load should be no larger than 0.643 UEs/RO. This means in the realistic deployment scenarios, the number of UEs simultaneously transmitting on the same RO should be small.

Observation 9: The number of UEs performing contention-based 2-step RACH on the same RO needs to be small to avoid large collision probability and error floor in BLER performance.

Resource Utilization

For contention-based 2-step RACH, PUSCH occasions (POs) are configured for the transmission of MsgA payload, and will not be used by other transmissions. There can be different mapping relations between preambles and POs. When there are K contention-based preambles, and  L preambles are mapped to one PO, then K/L POs will be needed. When  L is larger, the reserved resource for MsgA will be less.

As shown above, when K=64 and the collision probability is 1%, the average traffic load should be no larger than 0.643 UEs/RO, i.e., the average utilization of preambles per RO is about 1%. If only one preamble is mapped to a specific PO, i.e., L=1, the utilization of POs will also be 1%, which is too low and large amounts of time and frequency resources will be wasted. More resources will be wasted, if the pre-reserved resource size of the PO is larger. Therefore, to have a reasonable resource utilization, mapping multiple preambles to one PO should be considered. For example, when L=8 preambles are mapped to one PO, the utilization will grows to 8%. When multiple preambles are mapped to one PO, different preambles can be associated with different DMRS ports to avoid full collision between any two active UEs. In this case, the multiplexing of multiple active UEs in the same PO should also be considered.

PUSCH resource unit (PRU) has been defined as PUSCH occasion and DMRS port/sequence. For one PO, there can be multiple PRUs, i.e., multiple preambles can be mapped to different PRUs in the same PO. However, the number of preambles can still be larger than the number of PRUs in one PO. Multiple DMRS sequences can further increase the number of distinguishable PRUs, and the collision probability of PRUs can be reduced. As shown above, the performance can be improved with multiple DMRS sequences.

Observation 10: The resource utilization of PUSCH resource is usually low for 2-step RACH.

Detection Performance

To evaluate the performance of different mapping schemes, we first consider the case when two UEs select preamble with index 0 and 16 for 2-step RACH. With previous mapping rule, if the mapping is 4-to-1 or 2-to-1, the mapped PRU is the same for the two UEs, and if the mapping is 1-to-1, the mapped PRUs are different. As shown in Figure 8, the PUSCH decoding performance without PRU collision is much better than that with PRU collision. As the design of multiple DMRS sequences can reduce the PRU collision probability and improve PUSCH decoding performance, it should be considered for 2-step RACH.

For contention-based 2-step RACH, the UEs will randomly select the preambles, both preamble collision and PRU collision can happen. As preamble collision has similar impact on one-to-one or multiple-to-one mapping, we focus on the cases without preamble collision. Among the 64 preambles, 2 or 4 preambles are randomly selected, i.e., random active. The PUSCH decoding performance with random active preambles are presented in Figure 9 and Figure 10.

The results show that for all the cases simulated, 1-to-1 mapping is better than 2-to-1 mapping or 4-to-1 mapping. Therefore, the necessity to support multiple-to-one mapping should be further verified.

Observation 11: With more DMRS sequences, the collision probability of PUSCH resource units can be reduced, and the decoding performance of PUSCH can be improved.

Observation 12: In the case that PRU collision may happen, the PUSCH decoding performance of 1-to-1 mapping is much better than that of the multi-to-one mapping.

| (a). TDLA-30ns, 1T2R, 2 preambles, 72 bits | (b). TDLC-300ns, 1T2R, 2 preambles, 72 bits |
| --- | --- |
| (c). TDLA-30ns, 1T2R, 2 preambles, 200 bits | (b). TDLC-300ns, 1T2R, 2 preambles, 200 bits |

Figure 8. PUSCH decoding performance with and without PRU collision

| (a). TDLA-30ns, 1T2R, 4 preambles, 72 bits | (b). TDLC-300ns, 1T2R, 4 preambles, 72 bits |
| --- | --- |
| (c). TDLA-30ns, 1T2R, 4 preambles, 200 bits | (d). TDLC-300ns, 1T2R, 4 preambles, 200 bits |

Figure 9. PUSCH decoding performance with 4 random active preambles

| (a). TDLA-30ns, 1T2R, 2 preambles, 72 bits | (b). TDLC-300ns, 1T2R, 2 preambles, 72 bits |
| --- | --- |
| (c). TDLA-30ns, 1T2R, 2 preambles, 200 bits | (d). TDLC-300ns, 1T2R, 2 preambles, 200 bits |

Figure 10. PUSCH decoding performance with 2 random active preambles

Conclusions

In this contribution, we presented preliminary results for 2-step RACH evaluations. Based on the evaluation results, we have the following observations.

Observation 1: The performance of preamble detection depends on preamble formats, and the number of active UEs for a given false-alarm rate.

Observation 2: When payload size is 56 or 72 bits, the MCL is close to that of short preambles (format A1, A2, A3), and more than 10dB worse than that of long preamble (format 0).

Observation 3: When payload size is 1000 bits, the MCL is more than 10dB worse than that of short preambles (format A1, A2, A3), and more than 22dB worse than that of long preamble (format 0), which potentially require more repetitions for payload transmission thus likely larger latency compared to Rel-15 4-step RACH.

Observation 4: The resource overhead of large payload size (200, 1000 bits) is much larger than that of 56 or 72 bit payload size.

Observation 5: With small payload size, more UEs can be multiplexed in the same PO, and higher resource utilization can be achieved.

Observation 6: For RRC_IDLE/INACTIVE state, MCS index of 0~7 and bandwidth of 1~8 PRBs can be enough to support the payload size of 56 or 72 bits.

Observation 7: For intra-slot hopping of MsgA PUSCH, guard period between hops can reduce the potential collision between UEs.

Observation 8: For the case with multiple preamble groups, the collision probability increases if the traffic load with respect to the available PRACH resource is not matched.

Observation 9: The number of UEs performing contention-based 2-step RACH on the same RO needs to be small to avoid large collision probability and error floor in BLER performance.

Observation 10: The resource utilization of PUSCH resource is usually low for 2-step RACH.

Observation 11: With more DMRS sequences, the collision probability of PUSCH resource units can be reduced, and the decoding performance of PUSCH can be improved.

Observation 12: In the case that PRU collision may happen, the PUSCH decoding performance of 1-to-1 mapping is much better than that of the multi-to-one mapping.

References

- RP-182894, “New work item: 2 step RACH for NR”, RAN#82, Sorrento, Italy, Dec. 10-13, 2018.

- R1-1903834, “Email discussion on potential link-level simulation assumption for 2-step RACH”, ZTE, Xi’an, China, April 8-12, 2019.

- R1-1911871, “Discussion on channel structure of 2-step RACH”, Huawei, HiSilicon, Reno, USA, November 18–22, 2019.

- R1-1911872, “Discussion on 2-step RACH procedure”, Huawei, HiSilicon, Reno, USA, November 18–22, 2019.

- R1-061369, “LTE random access capacity and collision probability”, RAN1#45, Ericsson, Shanghai, China, May 8-12, 2006.

# Appendix

Approved by email [2]:

- Adopt the link-level simulation assumptions in the following table for the initial evaluation of feasible payload size, and for the potential down-selection of schemes, e.g.

  - whether to have shared ROs and/or preambles between 2-step RACH and 4-step RACH, and if yes the percentage for partitioning

  - whether to have the guard time between PRACH and PUSCH and/or guard period within PUSCH, and if yes the length of GT/GP

  - the mapping scheme between preamble and PUSCH+DMRS, .e.g 1-to-1, multiple-to-1, or 1-to-multiple

  - appropriate power offset(s) between preamble and PUSCH, and whether to support repetition of MsgA PUSCH

  - whether to have UCI in msgA PUSCH, if a PUCCH transmission overlaps the PUSCH part of MsgA

  - whether to dynamically adapt the payload size and indicate by UCI in msgA PUSCH, and if yes the content and structure of UCI

  - FFS other schemes, e.g. whether guard band is included

- Additional system-level simulations or analytical evaluations can be considered for the following analysis:

  - Latency

  - Signalling overhead

  - Resource reservation overhead

  - PUSCH collision, with definition FFS, e.g. overlapped PUSCH occasion, with shared or separate DMRS port, and with same or different scrambling ID

  - Rx detection complexity

- Note 1: the supported/recommended payload size from RAN1 perspective may also need to take other factors into account, e.g. use cases, resource utilization.

- Note 2: the WID scope should be strictly followed when using the evaluation results for the comparison of schemes.

Table A.1 Link-level evaluation assumptions by email discussion [2]

| Parameters | Values |
| --- | --- |
| Scenario | - For evaluation of schemes: 200m, UMi, 4 GHz. FFS: 500m, UMa, 4 GHz. - For evaluation of payload size: 200m, UMi, 4 GHz; or 500m, UMa, 4 GHz; or 1732m, RMa, 700 MHz; or 25km, RMa, 700 MHz. Other values can be reported if applicable. Note: this does not restrict preamble format selection. |
| Preamble format | Company report |
| Waveform (data part) | CP-OFDM, or DFT-s-OFDM |
| Subcarrier spacing for PUSCH | 15kHz at 700MHz, 30/60kHz at 4 GHz, 120kHz at 30GHz |
| TBS | - 56 bits as starting point for minimum payload size, other values are not precluded - Company report for the evaluation of payload size |
| MCS and Resource size | Company report the MCS, time/frequency resource size, and DMRS overhead. Strive to agree to some common values in RAN1#96bis. |
| Number of UEs | 1 as a starting point; FFS: 2 or more for evaluation of shared PUSCH occasion or interference from the adjacent PUSCH resource, including how to model relative SINR, timing, etc. |
| Traffic model | Full buffer |
| UE antenna configuration | 1Tx. FFS: 2 Tx |
| gNB antenna configuration | 2Rx or 4Rx, 8Rx as optional |
| Propagation channel & UE velocity | TDL/CDL-A 30ns, or TDL/CDL-C 300ns, 3km/h or 30km/h |
| Timing offset | Uniform [0, RTT]. |
| Frequency offset | 0.05ppm (fixed) at TRP, and 0.1 ppm (fixed) at UE |
| Max number of HARQ transmission | 1 as starting point, other values are not precluded and company should report the details of HARQ |
| Receiver | MMSE-IRC as baseline |
| Channel estimation | Realistic for both channel estimation and TO/FO estimation. Ideal can be considered for calibration, if needed. |
| Target BLER | [10%, 1%] for 1st transmission of msgA as starting points. |
| Performance metrics | 1) Missed detection probability vs. SNR for a given false alarm rate, e.g. 0.1%; 2) BLER vs. SNR; MCL can be reported using link budget calculations. 3) Optional: False alarm probability vs. SNR |

Table A.2 Parameters for simulation results in Figure 2

| Parameters | Values |
| --- | --- |
| Scenario | 200m, UMi, 4 GHz; 1732, RMa, 700 MHz(For preamble detection) |
| Preamble format | 0, A1, A2, A3 (30KHz SCS for 4GHz, 15KHz SCS for 700MHz) |
| Waveform (data part) | CP-OFDM |
| Subcarrier spacing for PUSCH | 30/60kHz at 4 GHz |
| TBS | 72, 1000 bits |
| MCS and Resource size | 1PRB, 2PRB, 6PRB in frequency domain 14 symbols in time domain, 2 for DMRS |
| Number of UEs | 1, 2 |
| Traffic model | Full buffer |
| UE antenna configuration | 1Tx |
| gNB antenna configuration | 2Rx |
| Propagation channel & UE velocity | TDL-A 30ns |
| Timing offset | Uniform [0, RTT]. |
| Frequency offset | 0.05ppm (fixed) at TRP, and 0.1 ppm (fixed) at UE |
| Max number of HARQ transmission | 1 |
| Receiver | MMSE-IRC as baseline |
| Channel estimation | Realistic for both channel estimation and TO/FO estimation. |
| Target BLER | 1% for 1st transmission of msgA as starting points. |
| Performance metrics | 1) Missed detection probability vs. SNR for a given false alarm rate, e.g. 0.1%; 2) BLER vs. SNR; MCL can be reported using link budget calculations. |

Table A.3 MCL analysis for TBS=56 bits

| SCS (kHz) | 30 | 30 | 30 | 60 | 60 | 60 |
| --- | --- | --- | --- | --- | --- | --- |
| # of PRBs | 1 | 2 | 3 | 1 | 2 | 3 |
| (1) Tx Power (dBm) | 23 | 23 | 23 | 23 | 23 | 23 |
| (2) Thermal noise density (dBm/Hz) | -174 | -174 | -174 | -174 | -174 | -174 |
| (3) eNB receiver noise figure (dB) | 5 | 5 | 5 | 5 | 5 | 5 |
| (4) Interference margin (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
| (5) Occupied channel bandwidth (Hz) | 3.6e5 | 7.2e5 | 1.08e6 | 7.2e5 | 1.44e6 | 2.16e6 |
| (6) Effective noise power = (2)+(3)+(4)+10log((5)) (dBm) | -113.4 | -110.4 | -108.7 | -110.4 | -107.4 | -105.7 |
| (7) Required SINR (dB) | 4.0 | 0.8 | -1.1 | 3.3 | 0 | -1.5 |
| (8) Receiver sensitivity = (6)+(7) (dBm) | -109.4 | -109.6 | -109.8 | -107.1 | -107.4 | -107.2 |
| (9) Receiver processing gain | 0 | 0 | 0 | 0 | 0 | 0 |
| (10) MCL = (1)-(8)+(9) (dB) | 132.4 | 132.6 | 132.8 | 130.1 | 130.4 | 130.2 |

Table A.4 MCL analysis for TBS=72 bits

| SCS (kHz) | 30 | 30 | 30 | 60 | 60 | 60 |
| --- | --- | --- | --- | --- | --- | --- |
| # of PRBs | 1 | 2 | 3 | 1 | 2 | 3 |
| (1) Tx Power (dBm) | 23 | 23 | 23 | 23 | 23 | 23 |
| (2) Thermal noise density (dBm/Hz) | -174 | -174 | -174 | -174 | -174 | -174 |
| (3) eNB receiver noise figure (dB) | 5 | 5 | 5 | 5 | 5 | 5 |
| (4) Interference margin (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
| (5) Occupied channel bandwidth (Hz) | 3.6e5 | 7.2e5 | 1.08e6 | 7.2e5 | 1.44e6 | 2.16e6 |
| (6) Effective noise power = (2)+(3)+(4)+10log((5)) (dBm) | -113.4 | -110.4 | -108.7 | -110.4 | -107.4 | -105.7 |
| (7) Required SINR (dB) | 5.6 | 2.0 | 0.2 | 4.6 | 1.5 | -0.6 |
| (8) Receiver sensitivity = (6)+(7) (dBm) | -108.0 | -108.4 | -108.5 | -105.8 | -105.9 | -106.3 |
| (9) Receiver processing gain | 0 | 0 | 0 | 0 | 0 | 0 |
| (10) MCL = (1)-(8)+(9) (dB) | 131.0 | 131.4 | 131.5 | 128.8 | 128.9 | 129.3 |

Table A.5 MCL analysis for TBS=200 bits

| SCS (kHz) | 30 | 30 | 30 | 60 | 60 | 60 |
| --- | --- | --- | --- | --- | --- | --- |
| # of PRBs | 3 | 6 | 12 | 3 | 6 | 12 |
| (1) Tx Power (dBm) | 23 | 23 | 23 | 23 | 23 | 23 |
| (2) Thermal noise density (dBm/Hz) | -174 | -174 | -174 | -174 | -174 | -174 |
| (3) eNB receiver noise figure (dB) | 5 | 5 | 5 | 5 | 5 | 5 |
| (4) Interference margin (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
| (5) Occupied channel bandwidth (Hz) | 1.08e6 | 2.16e6 | 3.24e6 | 2.16e6 | 4.32e6 | 6.48e6 |
| (6) Effective noise power = (2)+(3)+(4)+10log((5)) (dBm) | -108.7 | -105.7 | -103.9 | -105.7 | -102.7 | -100.9 |
| (7) Required SINR (dB) | 4.8 | 1.2 | -2.6 | 3.8 | -0.2 | -4.3 |
| (8) Receiver sensitivity = (6)+(7) (dBm) | -103.9 | -104.5 | -106.5 | -101.9 | -102.9 | -105.2 |
| (9) Receiver processing gain | 0 | 0 | 0 | 0 | 0 | 0 |
| (10) MCL = (1)-(8)+(9) (dB) | 126.9 | 127.5 | 129.5 | 124.9 | 125.9 | 128.2 |

Table A.6 MCL analysis for TBS=1000 bits

| SCS (kHz) | 30 | 30 | 60 | 60 |
| --- | --- | --- | --- | --- |
| # of PRBs | 6 | 12 | 6 | 12 |
| (1) Tx Power (dBm) | 23 | 23 | 23 | 23 |
| (2) Thermal noise density (dBm/Hz) | -174 | -174 | -174 | -174 |
| (3) eNB receiver noise figure (dB) | 5 | 5 | 5 | 5 |
| (4) Interference margin (dB) | 0 | 0 | 0 | 0 |
| (5) Occupied channel bandwidth (Hz) | 2.16e6 | 3.24e6 | 4.32e6 | 6.48e6 |
| (6) Effective noise power = (2)+(3)+(4)+10log((5)) (dBm) | -105.7 | -103.9 | -102.7 | -100.9 |
| (7) Required SINR (dB) | / | 5.2 | 8.4 | 2.7 |
| (8) Receiver sensitivity = (6)+(7) (dBm) | / | -98.7 | -94.3 | -98.2 |
| (9) Receiver processing gain | 0 | 0 | 0 | 0 |
| (10) MCL = (1)-(8)+(9) (dB) | / | 121.7 | 117.3 | 121.2 |