3GPP TSG RAN WG1 Meeting #99   		 R1-1913246

Reno, USA, November 18th – November 22nd, 2019

Agenda item:	6.2.1.3

Source: 	Qualcomm Incorporated

Title: 	Scheduling of multiple DL/UL transport blocks

Document for:	Discussion/Decision

# Introduction

In RAN1#98bis, the following agreements were reached regarding scheduling of multiple TBs in eMTC:

Agreement

The unicast multi-TB feature is enabled separately for DL and UL

Agreement

For unicast, scheduling gaps can be configured separately for DL and UL by RRC

- Dynamic activation/deactivation of scheduling gaps via DCI is FFS.

Agreement

For UEs that support multi-TB scheduling with HARQ-ACK bundling, the bundling is enabled/disabled/configured by RRC and the actual bundle size is indicated by DCI

Agreement

- For UEs that support multi-TB scheduling with HARQ-ACK bundling, the maximum bundle size is 4.

- Strive to reuse Rel-14 HARQ-ACK bundling feature as baseline at least for the non-interleaving case

Agreement

- For unicast in CE mode A, at least for 1-2 TBs, scheduling with a single DCI is supported without new restrictions on MCS.

  - FFS for > 2TBs

- For unicast in CE mode A, at least for 1-2 TBs, scheduling with a single DCI is supported without new restrictions on RA.

  - FFS for > 2TBs

- FFS for CE mode B

Agreement

For DCI contents for scheduling multiple DL/UL transport blocks for CE mode A, down select one from following options in R1-1911381 for HARQ ID, NDI and number of scheduled HARQ processes.

- Option 1

- Option 3/5

Agreement

For unicast in CE mode A, for the purpose indicating the number of TBs, select Option 2, i.e.:

- A new or repurposed field(s) in DCI indicates implicitly or explicitly to indicate 1 TB or multiple TBs

  - FFS: Details on how to indicate the exact number of TBs in case it is multiple

Agreement

For unicast in CE mode B, for the purpose indicating the number of TBs, select Option 2, i.e.:

- A new or repurposed field(s) in DCI indicates implicitly or explicitly to indicate 1 TB or multiple TBs

  - FFS: Details on how to indicate the exact number of TBs in case it is multiple

Conclusion

For multicast, interleaving is not supported

Agreement

For multicast, the scheduling gap configuration indicates:

- Scheduling gap duration with granularity(FFS)

- FFS: Scheduling gap periodicity

- FFS: Other scheduling gap properties

Agreement

For unicast, the scheduling gap configuration indicates:

- Scheduling gap duration with granularity(FFS)

- FFS: Scheduling gap periodicity

- FFS: Scheduling gap time offset

- FFS: Threshold for enabling scheduling gap

Agreement

For unicast, a scheduling gap containing an MPDCCH transmission can be used for indication of early termination of ongoing PUSCH transmission(s).

- FFS: Whether a UE is required to monitor MPDCCH during the scheduling gap

- FFS: Whether the above also applies for PDSCH

Agreement

Unicast multi-TB scheduling can be configured and used together with at least the following other features:

- Rel-14 feature for 2984 bits max UL TBS in 1.4 MHz in CE mode A

FFS till RAN1#99 whether unicast multi-TB scheduling can be configured and/or used together with the following other features:

- Rel-14 feature for new numbers of repetitions for PUSCH and modulation restrictions for PDSCH/PUSCH in CE mode A

- Rel-14 feature for modulation restrictions for PDSCH/PUSCH in CE mode A

- Rel-14 feature on HARQ-ACK bundling in HD-FDD in CE mode A

- Rel-14 features for 5 or 20 MHz max PDSCH/PUSCH channel bandwidths in CE mode A/B

- Rel-14 feature for 10 downlink HARQ processes in FDD in CE mode A

- Rel-14 feature for dynamic HARQ-ACK delay for HD-FDD in CE mode A

- Rel-15 feature for PUSCH sub-PRB allocation in CE mode A/B

- Rel-15 feature for 64QAM for non-repeated unicast PDSCH in CE mode A

- Rel-15 feature for uplink HARQ-ACK feedback in DCI in CE mode A/B

- Rel-15 features for flexible starting PRB for PDSCH/PUSCH in CE mode A/B

- Any other feature that was supported since Rel-13 LTE eMTC

In this contribution we present our views on the remaining issues for multi-TB scheduling for eMTC.

# DCI Design for CE Mode B

In this section we detail the DCI design for CE Mode B. Our design comprises the following key ingredients:

- The design ensures “full flexibility” for when 1 and 2 TBs are scheduled—i.e., there are no restrictions placed on the ranges of values indicated by the DCI fields in legacy DCI Format 6-1B.

- HARQ Processes scheduled and their corresponding NDIs are jointly encoded

- The design introduces a modest restriction on the MCS values permitted for large values of repetition numbers when 3 or 4 TBs are scheduled—this is a reasonable trade-off, considering that the largest repetitions for CE Mode B are mainly for UEs in poor coverage, wherein larger MCS values are not of much relevance.

- The design adds a total of 4 bits to legacy DCI Format 6-1B—this, keeping in mind the fact that legacy DCI Format 6-1B only supports scheduling 1 TB, versus up to 4 that is agreed to be supported within the ambit of this work item for CE Mode B.

## DCI Encoding

Table 1 outlines the encoding for the 13-bit DCI chunk across HARQ processes, NDIs, MCS and repetition number. Depending on how many TBs are scheduled, combinatorial numbers rHARQi (where i denotes the number of TBs scheduled) are used to index the HARQ processes scheduled. These numbers are formally defined below:

- rHARQi is the combinatorial index mapping the HARQ processes Hkk=0i-1, 1≤Hk≤4, Hk<Hk+1 scheduled, and is given by rHARQi= k=0i-14-Hki-k, where xy=xy x>y0     x<y

As evidenced in Table 1, our DCI design can be encoded in the form of a simple bit-table, which is amenable to straightforward decoding at the UE by interpreting the bits. The joint encoding of MCS and repetition number when 3 or 4 TBs are scheduled, is further outlined in Table 1(a).

Table 1: DCI Encoding across HARQ, NDI, MCS and repetition number for CE Mode B

| #TBs | a1 | a2 | a3 | a4 | a5 | a6 | a7 | a8 | a9 | a10 | a11 | a12 | a13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | HARQ ID |  | NDI0 | MCS |  |  |  | Repetition Number |  |  |
| 2 | 0 | rHARQ2 |  |  | NDI1 | NDI0 | MCS |  |  |  | Repetition Number |  |  |
| 3 | 1 | 0 | rHARQ3 |  | NDI2 | NDI1 | NDI0 | Joint MCS and Repetition Number |  |  |  |  |  |
| 4 | 1 | 1 | 0 | NDI3 | NDI2 | NDI1 | NDI0 | Joint MCS and Repetition Number |  |  |  |  |  |

Table 1(a): Jointly encoded bits (a8, ...,a13) across MCS and repetition number when 3 or 4 TBs are scheduled

| Repetition Number | a8 | a9 | a10 | a11 | a12 | a13 |
| --- | --- | --- | --- | --- | --- | --- |
| n1 | 0 | 0 | MCS |  |  |  |
| n2 | 0 | 1 | 0 | MCS |  |  |
| n3 | 0 | 1 | 1 | MCS |  |  |
| n4 | 1 | 0 | 0 | MCS |  |  |
| n5 | 1 | 0 | 1 | MCS |  |  |
| n6 | 1 | 1 | 0 | MCS |  |  |
| n7 | 1 | 1 | 1 | 0 | MCS |  |
| n8 | 1 | 1 | 1 | 1 | MCS |  |

## Proposals for CE Mode B

Proposal 1: For the DCI design for scheduling multiple transport blocks in CE Mode B:

- Retain same scheduling flexibility as legacy DCI when the number of TBs scheduled ∈{1,2}

- Retain full scheduling flexibility across all combinations of (up to 4) HARQ processes for all values of number of TBs scheduled

- Jointly encode HARQ processes scheduled and NDIs for all values of number of TBs scheduled

- For number of scheduled TBs ∈{3,4}, restrict the MCSs allowed when number of repetitions is large

Proposal 2: Adopt the DCI design for scheduling multiple transport blocks in CE Mode B outlined in Section 2.

# DCI Design for CE Mode A

For CE Mode A, the DCI has the following additional (and potentially significant to the eventual size of the DCI) aspects, that is different from the design for CE Mode B

- The legacy DCI for CE Mode A has associated with it a redundancy version (RV) index for every TB, that can take four potential values. Without restrictions and/or joint-encoding of fields, this has the potential to increase the signalling required exponentially in the number of TBs scheduled.

- The DCI for CE Mode A has associated with it a Frequency Hopping (FH) indicator—this indicator is designed for CE Mode A (on top of the RRC configurations for hopping) to allow opportunistic scheduling in frequency, when good quality CSI is available to the base station.

- Importantly, the maximum number of TBs that can be scheduled by a single DCI, per previous agreements, is 8 for CE Mode A, versus 4 for CE Mode B

In keeping with our DCI design methodology for CE Mode B, our design for CE Mode A has the following key ingredients:

- The design ensures “full flexibility” for when 1 TB is scheduled—i.e., there are no restrictions placed on the ranges of values indicated by the DCI fields in legacy DCI Format 6-1A. This includes full-flexibility for per-TB RV index signalling as well as for the FH indicator.

- For 2 TBs scheduled, full-flexibility is retained except:

  - We allow 2 choices of starting RV per scheduled TB (independent across the 2 TBs)

- For number of TBs ∈{3,4,5,6,8} the starting RV across the set of TBs is fixed to RV0.

- For number of TBs ∈{1,2,3,4,8}, the design allows for scheduling all possible combinations of HARQ IDs, while for 5, 6 or 7 TBs, continuous HARQ IDs (with wrap-around) may be scheduled; for 7 TBs the start HARQ ID is 0, i.e., the set of HARQ processes is fixed to {0,…,6}.

- The Frequency Hopping (FH) indicator is present in DCI only for number of TBs ∈{1,2,8}

  - For other values of number of TBs, the RRC indication is followed

- The design adds a total of 5 bits to legacy DCI Format 6-1A.

Table 2 summarizes the constraints for different number of scheduled TBs:

Table 2: Scheduling constraints for multi-TB scheduling in CE Mode A

| Number of TBs | HARQ process constraint | RV constraint | Frequency hopping |
| --- | --- | --- | --- |
| 1 | None | None | Indicated in DCI |
| 2 | None | {0,2}, separate for each TB | Indicated in DCI |
| 3 | None | Fixed to 0 | Follow RRC |
| 4 | None | Fixed to 0 | Follow RRC |
| 5 | Contiguous HARQ with wrap-around | Fixed to 0 | Follow RRC |
| 6 | Contiguous HARQ with wrap-around | Fixed to 0 | Follow RRC |
| 7 | Only processes {0,…,6} | Fixed to 0 | Follow RRC |
| 8 | None | Fixed to 0 | Indicated in DCI |

The areas in which our design differs from the one presented in Section 2 for CE Mode B are as follows:

- We don’t impose any MCS restriction for CE Mode A. This is because CE Mode A is designed for UEs with relatively good coverage (compared to CE Mode B), and the higher MCS values are more useful.

## DCI Encoding

Table 3 outlines the encoding for the 12-bit DCI chunk across HARQ processes, NDIs, RVs (when present), and FH indicator (when present). The approach is similar in spirit to the encoding for CE Mode B, outlined in section 2. Depending on how many TBs are scheduled, combinatorial numbers rHARQi (where i denotes the number of TBs scheduled) are used to index the HARQ processes scheduled. These numbers are formally defined below:

- rHARQi is the combinatorial index mapping the HARQ processes Hkk=0i-1, 1≤Hk≤8, Hk<Hk+1 scheduled, and is given by rHARQi= k=0i-18-Hki-k, where xy=xy x>y0     x<y

As evidenced from Table 3, our DCI design can be encoded in the form of a simple bit-table, which is amenable to straightforward decoding at the UE by interpreting the bits. The only subtle nuance happens in the case 2 or 4 TBs are scheduled—in these cases, the DCI encoding may not always be “prefix-free” as in the case of number of TBs ∈{1,7,5,3,6,8}. The DCI is still uniquely decodable, owing to the “shift by 3” applied to the combinatorial HARQ index rHARQ2, when 2 TBs are scheduled. We explain this further in the following subsection on DCI decoding.

## DCI Decoding of 12-bit jointly encoded chunk

### Number of TBs ∈{1,7,5,3,6,8}

The decoding for when the number of scheduled TBs ∈{1,7,5,3,6,8} is straightforward: from the prefix (first few bits), the UE can uniquely decode the number of TBs scheduled, and then proceed to decode the other parameters by reading off the corresponding bits from the designated bit locations in Table 3.

### Number of TBs ∈{2,4}

When the number of scheduled TBs is 2 or 4, the bits {b0,b1} may not (by themselves) uniquely determine whether 2 or 4 TBs are scheduled. To that end, the following simple sub-cases are used by the UE to determine the encoding uniquely

#### b0,b1={1,0}

In this case, the encoding is prefix-free, since this can only occur when 4 TBs are scheduled

#### b0,b1={1,1}

This is the only non-trivial case, where the encoding is not prefix free. However, due to the shift of 3 applied to the combinatorial HARQ index rHARQ2 for 2 TBs, the encoding is still unique. The UE simply checks whether b2,b3,b4,b5,b6∈{0,0,0,0,0,0,0,0,0,1,0,0,0,1,0}. If so, 4 TBs are scheduled (since rHARQ2+3>2); else 2 TBs.

This way of encoding the HARQ indices is unique since for 4 TBs with b0,b1={1,1}, rHARQ4∈{64,…,69}, wherein {b2,b3,b4}={0,0,0} and b5,b6∈{0,0,0,1,{1,0}}. Such a combination of bits {b2,b3,b4,b5,b6} can never occur for 2 TBs, per our encoding rule.

Table 3: 12-bit DCI encoding table across HARQ, NDI, RV and FH for CE Mode A

| #TBs | b0 | b1 | b2 | b3 | b4 | b5 | b6 | b7 | b8 | b9 | b10 | b11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 0 | HARQ ID |  |  | RV0 |  | NDI0 | FH |
| 7 | 0 | 1 | 0 | 1 | 1 | NDI0 | NDI1 | NDI2 | NDI3 | NDI4 | NDI5 | NDI6 |
| 5 | 0 | 1 | 0 | 0 | Starting HARQ ID |  |  | NDI0 | NDI1 | NDI2 | NDI3 | NDI4 |
| 3 | 0 | 1 | 1 | Combinatorial HARQ Index rHARQ3 |  |  |  |  |  | NDI0 | NDI1 | NDI2 |
| 6 | 0 | 0 | 1 | Starting HARQ ID |  |  | NDI0 | NDI1 | NDI2 | NDI3 | NDI4 | NDI5 |
| 8 | 0 | 0 | 0 | NDI0 | NDI1 | NDI2 | NDI3 | NDI4 | NDI5 | NDI6 | NDI7 | FH |
| 2 | 1 | 1 | Combinatorial HARQ Index + 3 (rHARQ2+3) |  |  |  |  | RV0 | RV1 | NDI0 | NDI1 | FH |
| 4 | 1 | Combinatorial HARQ Index (rHARQ4) |  |  |  |  |  |  | NDI0 | NDI1 | NDI2 | NDI3 |

## Proposals for CE Mode A

Proposal 3: For the DCI design for scheduling multiple transport blocks in CE Mode A:

- Retain same scheduling flexibility as legacy DCI when 1 TBs scheduled

- Retain same scheduling flexibility for 2 TBs scheduled except:

  - Allow 2 possible choices of RV index for each TB independently

- Retain scheduling flexibility across all HARQ ID combinations of 1, 2, 3, 4 or 8 TBs

- Allow scheduling of 5 or 6 TBs using continuous HARQ IDs with arbitrary start HARQ ID

- Allow scheduling of 7 TBs using continuous HARQ IDs with start HARQ ID equal to 0

- Jointly encode HARQ processes, NDIs, RV indices (when present) and FH indicator (when present)

- For number of scheduled TBs ∈{3,…8}, the starting RV for each TB across a set of repetitions is RV0

- Retain FH indication in DCI only for number of scheduled TBs ∈{1,2,8}; for other values of number of TBs scheduled, the RRC indication is followed.

Proposal 4: Adopt the DCI design for scheduling multiple transport blocks in CE Mode A outlined in Section 3.

## Non-Full Buffer Performance of DCI Design

In this subsection, we consider a non-full buffer traffic scenario, i.e., when there isn’t a continuous stream of 8 HARQ processes worth of data to transmit. This may happen with a bursty traffic arrival pattern, which is typical for IoT-type settings. Under such settings, we compare the percentage of times that a given multi-TB DCI design fails to schedule a non-full buffer traffic loading. The two schemes we compare are the proposed scheme in this contribution and the alternative scheme in [2]. In our simulations, we start with a full buffer that we want to empty, before another burst of traffic fills the buffer later. We assume enough elapsed time between successive bursts of traffic, such that the buffer typically will be emptied before a new burst of packets arrive

It can be seen from Table 3(a) below that owing to the restrictions on HARQ processes that can be scheduled in [2], in bursty traffic scenarios, the design in [2] encounters more “scheduling failures” than [1], i.e., in every such case, multiple DCIs are required to clear out the buffer, resulting in unnecessary DCI monitoring and throughput loss. In fact, as can be seen from the table below, without the provision for “full HARQ flexibility for 1 and 2 TBs”, the performance of [2] is even further hampered in bursty traffic scenarios.

Table 3(a): Scheduling failures in non-full buffer traffic for different DCI encoding schemes

| DCI Encoding Scheme | Percentage of scheduling failures (Target BLER = 10%) | Percentage of scheduling failures (Target BLER = 20%) |
| --- | --- | --- |
| Proposed scheme in this contribution | 0.02 | 0.5 |
| Scheme in [2] (with full-flex for 1 and 2 TBs) | 1.6 | 6.7 |
| Scheme in [2] (without full-flex for 1 and 2 TBs) | 5.1 | 12.36 |

Observation 1: The DCI design for CE Mode A proposed in this contribution outperforms the design in [2] considerably under bursty (non-full buffer) traffic patterns.

# HARQ-ACK Bundling Configurations for CE Mode A

For multiple TB scheduling by a single DCI, the “bundling configuration”—i.e., how the transmitted TBs are grouped into bundles, need to be signaled to the UE. Simply signaling a single bundling size is not enough. As an example, consider the case when Nsched=8 TBs are scheduled by a single DCI. The TBs can be grouped in multiple ways—e.g., (2,2,2,2), (3,3,2), (4,4) are possible bundling configurations. Note that for different values of Nsched, the “valid” bundling configurations will vary. For example, when Nsched=3, the valid bundling configurations are (2,1) and (3)—very different from the possibilities mentioned above for Nsched=8. In addition, to keep the legacy functionality of HARQ ACK bundling flag in DCI, we propose to always have the possibility to have bundling disabled from DCI, even if bundling is configured by RRC.

With the above in mind, we propose the following interpretation of a 2-bit HARQ ACK bundling configuration field as shown in the Table 4 below.

Table 4: DCI Indication of HARQ ACK Bundling Configuration for Multi-TB scheduling

| Nsched (Determined from DCI) | HARQ-ACK-Bundling Configuration |  |  |  |
| --- | --- | --- | --- | --- |
|  | Value = ‘00’ | Value = ‘01’ | Value = ‘10’ | Value = ‘11’ |
| 1 | ‘Unbundled’ | ‘Unbundled’ | ‘Unbundled’ | ‘Unbundled’ |
| 2 | ‘Unbundled’ | ‘Unbundled’ | ‘Unbundled’ | (2) |
| 3 | ‘Unbundled’ | ‘Unbundled’ | (3) | (2,1) |
| 4 | ‘Unbundled’ | (4) | (3,1) | (2,2) |
| 5 | ‘Unbundled’ | (4,1) | (3,2) | (2,2,1) |
| 6 | ‘Unbundled’ | (4,2) | (3,3) | (2,2,2) |
| 7 | ‘Unbundled’ | (4,3) | (3,2,2) | (2,2,2,1) |
| 8 | ‘Unbundled’ | (4,4) | (3,3,2) | (2,2,2,2) |

Proposal 5: Use 2 bits in DCI to signal the bundling configuration used when multiple TBs are scheduled by a single DCI, where the 2-bit field is interpreted jointly with the number of scheduled TBs according to Table 4 in Section 4, and one entry always signals unbundled transmission.

# HARQ-ACK Timeline Determination

Figure 1: Illustration of HARQ ACK timeline for multiple TBs scheduled by a DCI

We now outline an efficient “pipelined” transmission of HARQ ACKs for the multiple TBs scheduled by a single DCI. In the multi-TB setting, the UE may be able to process one TB while it is still receiving another TB of the multiple TBs scheduled by the DCI. Our method below works irrespective of whether interleaving is enabled or not.

## Timeline determination with no HARQ ACK bundling

Denote by miPDSCH, the last (absolute) subframe index for the ith TB, i∈{0,1,…,Nsched-1}, where Nsched denotes the number of TBs scheduled by the DCI. Accounting for the 3ms processing time for PDSCH, and the 1ms switch time (only for HD-FDD), we can express the starting (absolute) subframe siPUCCH for the ACK transmission corresponding to TBi (in the absence of HARQ ACK bundling) as follows:

- For HD-FDD

  - s0PUCCH=max m0PDSCH+4, mNsched-1PDSCH+2

  - siPUCCH=max {miPDSCH+4, mi-1PUCCH+Nabs,i-1 PUCCH},  i∈{1,2,…,Nsched-1}

- For FD-FDD and TDD

  - s0PUCCH=max m0PDSCH+4,  mNsched-1PDSCH+1

  - siPUCCH=max {miPDSCH+4, si-1PUCCH+Nabs,i-1 PUCCH},  i∈{1,2,…,Nsched-1}

where Nabs,iPUCCH denotes the number of absolute subframes required to transmit the ACK for the ith TB.

Proposal 6: The starting (absolute) subframe siPUCCH for the ACK transmission corresponding to TBi for TBi, i∈0,1,..,Nsched in the absence of HARQ ACK bundling is determined as:

- s0PUCCH=max m0PDSCH+4, (mNsched-1PDSCH+2)×Δ

- siPUCCH=max {miPDSCH+4, mi-1PUCCH+Nabs,i-1 PUCCH},  i∈{1,2,…,Nsched-1}

where miPDSCHdenotes the last (absolute) subframe index for the ith TB, i∈{0,1,…,Nsched-1}; Nabs,iPUCCH denotes the number of absolute subframes required to transmit the ACK for the ith TB; Δ=1 for HD-FDD and Δ=0 otherwise

## Timeline determination with HARQ ACK bundling

When HARQ ACK bundling is enabled, denote by Mk=miPDSCHTBi∈Bk, where Bk denotes the kth bundle in the configuration.

As an example, consider the case of N=8 and the bundling configuration (3,3,2). In this case, there are 3 bundles, with the bundle B0 comprising {TB0,TB1,TB2}, the bundle B1 comprising {TB3,TB4,TB5} and the bundle B2 comprising {TB6,TB7}. In this case, we have M0=m0PDSCH,m1PDSCH,m2PDSCH, M1=m3PDSCH,m4PDSCH,m5PDSCH, M2={m6PDSCH,m7PDSCH}

With the above notation in mind, in the presence of HARQ ACK bundling, the ACK timelines are given by:

- For HD-FDD

  - sB0PUCCH=max{mjPDSCH∈M0} mjPDSCH+4,  mNsched-1PDSCH+2

  - sBiPUCCH=max{mjPDSCH∈Mi} {mjPDSCH+4,  sBi-1PUCCH+Nabs,Bi-1 PUCCH}

- For FD-HDD and TDD

  - sB0PUCCH=max{mjPDSCH∈M0} m0PDSCH+4, mNsched-1PDSCH+1

  - sBiPUCCH=max{mjPDSCH∈Mi} {mjPDSCH+4,  sBi-1PUCCH+Nabs,Bi-1 PUCCH}

Where sBiPUCCH denotes the starting (absolute) subframe index for the HARQ ACK for the ith TB bundle, and Nabs,BiPUCCH denotes the number of absolute subframes required to transmit the HARQ ACK for the ith TB bundle.

Proposal 7: The starting (absolute) subframe sBiPUCCH for the ACK transmission corresponding to TB bundle Bi in the is determined as:

- sB0PUCCH=max{mjPDSCH∈M0} mjPDSCH+4,  mNsched-1PDSCH+2×Δ

- sBiPUCCH=max{mjPDSCH∈Mi} {mjPDSCH+4,  sBi-1PUCCH+Nabs,Bi-1 PUCCH}, i≠0

Where Mk=miPDSCHTBi∈Bk; sBiPUCCH denotes the starting (absolute) subframe index for the HARQ ACK for the ith TB bundle; Nabs,BiPUCCH denotes the number of absolute subframes required to transmit the HARQ ACK for the ith TB bundle; Δ=1 for HD-FDD and Δ=0 otherwise.

# Hopping-aware interleaver design

While it has been agreed to support inter-TB interleaving (across TB repetitions) for multiple TBs scheduled by a single DCI, the exact design of the interleaver—and more critically, how the interleaving pattern is also able to equitably distribute all TBs across all the frequency narrowbands when frequency hopping is enabled—has not been discussed thus far.

As we will motivate with an example, this problem is not trivial—a simple row-column interleaving of TBs may lead to cases where some of the TBs only see a subset of the narrowbands that may be configured. Before we provide the example, and indeed present our solution, it is instructive to setup notation that we will be using throughout the Section.

## Parameters and Corresponding Notation

- NNBch,DL: This denotes the hopping interval—i.e., the number of contiguous subframes in the same NB before the transmission hops to the next narrowband. This parameter is defined in TS 36.211 Section 6.4.1

- NNB,hopch,DL : This denotes the number of hopping narrowbands that may be configured. This parameter too is defined in TS 36.211 Section 6.4.1

- NIL: This parameter denotes the interleaving granularity that may be configured/specified for multi TB transmission. Interleaving granularity is defined as the number of repetitions of a TB that form the “base unit” of interleaving. For example, if NIL=1, the interleaved TBs would be of the form {0,1,2,3,0,1,2,3}, if NIL=2, the interleaved output will be {0,0,1,1,2,2,3,3,0,0…}, and so on.

- Nsched: This parameter denotes the actual number of TBs scheduled by the DCI. This is distinct from the maximum number of TBs configured-NMax. As we will illustrate in this Section, it is critical to have the interleaving pattern depend on NSched, to ensure equitable distribution of TBs across narrowbands for all possible scheduling situations.

## Motivating example

We now present an example, that illustrates why a simple row-column interleaving in time, without regard to the hopping pattern in frequency, leads to some TBs experiencing only a limited (and suboptimal) subset of frequency narrowbands configured.

In our running example, the configuration parameters are as follows: Nsched=4, NIL=2,NNB,hopch,DL=4, NNBch,DL=4. With simple row-column interleaving in time, across the repetitions of the 4 TBs (indexed in the figure and having TB identifiers ∈{0,1,2,3}), we see in the illustration below, that TBs 0 and 1 only see narrowbands NB0 and NB2, while TBs 2 and 3 only see narrowbands NB1 and NB3. Clearly, this is undesirable, and we want a pattern that ensures that for all possible configurations, every TB has its repetitions equitably distributed across all scheduled narrowbands.

## Design of hopping-aware, interleaved TB mapping, ITB(i)

### Sub-unit determination

Our design first identifies “sub-units” in time that encompass:

- Nsched×NIL subframes: This ensures that at least NIL repetitions of each of the NSched TBs are covered within the sub-unit

- NNBch,DL×NNB,hopch,DL subframes: This ensures that a sub-unit spans all the configured narrowbands

From the quantities above, we determine our sub-unit size to be equal to the Lowest Common Multiple (LCM) of Nsched×NIL and NNBch,DL×NNB,hopch,DL.

The sub-unit index in which a subframe i∈{0,1,…} lies is therefore given by iLCMNsched×NIL,    NNBch,DL×NNB,hopch,DL.

### Cyclic shifts across sub-units

We now need to check if the sub-unit(s) thus obtained already distribute all the NSched TBs equitably across all narrowbands—the condition to check this is whether the quantities Nsched and NNBch,DL×NNB,hopch,DLNIL are co-prime—i.e., whether or not the Greatest Common Factor (GCF) of the two quantities is equal to 1. If the GCF is 1, then each sub-unit provides adequate frequency diversity, and we can simply repeat the subunits in time. If not, however, we need to introduce cyclic shifts to the TB indices across consecutive sub-units, so that any concentration of TBs to a subset of narrowbands in a given sub-unit is spread out across sub-units. The value of the cyclic shift that ensures the minimum number of shifts required before we cycle back to the original sub-unit structure is given by NNB,hopch,DLNIL. Putting the conditions together, we obtain the cyclic shift that needs to be applied to each sub-unit with respect to its immediately preceding sub-unit as 1 mod GCFNsched, (NNBch,DL×NNB,hopch,DL)NIL×NNB,hopch,DLNIL. Note that the term 1 mod GCFNsched, (NNBch,DL×NNB,hopch,DL)NIL is equal to 0 is the quantities in the GCF are co-prime.

Combining the above entities (sub-unit index determination, and cyclic shift determination) we have, for a given subframe index i, the cyclic shift is given by (up to modulo additive operations with Nsched)

iLCMNsched×NIL,    NNBch,DL×NNB,hopch,DL×1 mod GCFNsched, (NNBch,DL×NNB,hopch,DL)NIL×NNB,hopch,DLNIL

### Putting it all together

Putting together all the key ingredients described above, we have the TB index ITB(i) at subframe i as:

ITBi=iNIL+iLCMNsched×NIL,    NNBch,DL×NNB,hopch,DL×1 mod GCFNsched, (NNBch,DL×NNB,hopch,DL)NIL×NNB,hopch,DLNILmod Nsched,

where the operation iNIL ensures that NIL consecutive subframes have the same TB index.

## Example Revisited

We now apply our design of ITB(i) as derived above to our motivating example in the beginning of the Section. For this example, the derived parameters relevant to our design of ITB(i) are as follows:

Nsched×NIL=8; NNBch,DL×NNB,hopch,DL=16,

LCMNsched×NIL,  NNBch,DL×NNB,hopch,DL=16; GCFNsched×NIL,   NNBch,DL×NNB,hopch,DL=8

Our equation in this case becomes

ITBi=i2+i8×2mod 4∈{0,1,2,3}

The resulting time-frequency mapping of the TBs is shown is illustrated below. As is evident, the distribution of TBs is now equitable across all the configured narrowbands.

Proposal 8: For subframe index i∈{0,1,2,…} the TB index mapping ITB(i), which implicitly involves the design of the time-domain interleaver, should be designed such that all TBs have an equitable distribution of repetitions across all configured narrowbands

Proposal 9: With frequency hopping enabled, for subframe index i∈{0,1,2,…} the TB index mapping ITB(i) should depend on the actual number of TBs scheduled, and the relationship may be determined by

ITBi=iNIL+iLCMNsched×NIL,    NNBch,DL×NNB,hopch,DL×1 mod GCFNsched, (NNBch,DL×NNB,hopch,DL)NIL×NNB,hopch,DLNILmod Nsched,

where the parameters are as defined in this contribution.

# Interleaving Granularity

The purpose of interleaving is to provide time diversity to the TB transmissions when multiple TBs are scheduled by a single DCI. Given that a large number (up to 8) TBs may be scheduled by a single DCI, it is important to note that even with a small interleaving granularity, significant time diversity gains may be realized by interleaving the TB repetitions across time. We thus believe that to facilitate maximum time diversity gains, the interleaving granularity should be 1 subframe for CE Mode A and 4 subframes for CE Mode B (to preserve the RV cycling repetitions together).

In Table 5 below, we show that even for a small number of repetitions per TB, interleaving TBs with a small interleaving granularity has the potential to improve performance. We see that with 8 repetitions per TB and 8 HARQ processes interleaved among each other, going from an interleaving granularity of NIL=8 to NIL=1 provides a 1.2 dB gain. Increasing NIL from 1 to 4 incurs a 0.2 dB relative loss in performance.

Table 5: Effect of interleaving granularity

| Scenario Index | TBS | Repetitions per TB (subframes) | Interleaving granularity (subframes) | Number of HARQ Processes (TBs) Interleaved | SNR (@ 10% BLER/1PRB) |
| --- | --- | --- | --- | --- | --- |
| 1 | 328 | 8 | 8 (No Interleaving) | Results independent of number of HARQ Processes | -1.55 |
| 1(a) | 328 | 8 | 1 | 8 | -2.75 |
| 1(b) | 328 | 8 | 4 | 8 | -2.55 |

The main point we wish to highlight here is that NIL should not be made arbitrarily large—for example, as argued in [2] and others, to match NIL to the frequency hopping interval NNBch,DL, which may take values as large as 8. In the examples shown in Table 5, if NNBch,DL was configured to be equal to 8, and NIL=NNBch,DL, it would be impossible to harness the 1.2 dB interleaving gain otherwise possible with our joint time-frequency interleaving solution proposed in Section 6 (in conjunction with NIL=1).

Observation 2: Restricting the interleaving granularity NIL to be equal to the hopping interval NNBch,DL may incur significant (e.g., 1.2 dB) performance penalty. Such penalties may be avoided with the hopping-aware interleaver design in Section 6 of this contribution, in conjunction with small interleaving granularities.

Both with and without frequency hopping, the interleaver design proposed in Section 6 above can be applied to derive the TB-to-subframe mapping, with an NIL=1 for CE Mode A, and NIL=4 for CE Mode B. When frequency hopping is not configured, the equation collapses to a simple row-column interleaving of TBs across subframes

Proposal 10: The interleaving granularity is 1 subframe for CE Mode A and 4 subframes for CE Mode B with and without frequency hopping.

# Applicability of other features

In RAN1#96b, bundling multi-TB scheduling with other features was discussed. In the following below we provide our views on supporting configuration of other features together with multi-TB scheduling:

- Rel-14 feature for new numbers of repetitions for PUSCH and modulation restrictions for PDSCH/PUSCH in CE mode A:

  - The configuration of this increases the size of the “repetition number” field and adds an additional bit in the DCI.

  - This case was mainly for VoLTE (align number of repetitions with VoLTE packet arrivals), for multi-TB this case does not seem to be relevant.

  - Do not support new repetitions, support modulation restrictions.

- Rel-14 feature for 2984 bits max UL TBS in 1.4 MHz in CE mode A

  - The configuration of this feature does not affect the DCI size, only the interpretation of its contents.

  - This feature is supported.

- Rel-14 feature on HARQ-ACK bundling in HD-FDD in CE mode A

  - There is agreement to support configured bundling for multi-TB, so this feature should not apply.

  - Do not support.

- Rel-14 features for 5 or 20 MHz max PDSCH/PUSCH channel bandwidths in CE mode A/B

  - This feature has a big impact on the DCI design. Additionally, the benefit of reduced overhead of MPDCCH may be reduced in this case, where the TB can span more than 1.4MHz.

  - Do not support.

- Rel-14 feature for 10 downlink HARQ processes in FDD in CE mode A

  - With the presence of bundling, it is unclear if we would actually need more than 10 HARQ processes to achieve “full rate scheduling”.

  - Revisit after bundling decision.

- Rel-14 feature for dynamic HARQ-ACK delay for HD-FDD in CE mode A

  - The delays for HARQ-ACK should be designed in such a way that the desired behaviour is obtained without the need to explicitly signal HARQ-ACK.

- Rel-15 features for flexible starting PRB for PDSCH/PUSCH in CE mode A/B

  - This feature only changes the resource allocation field (which is common for all TBs).

  - This feature is supported.

- Rel-15 feature for PUSCH sub-PRB allocation in CE mode A/B.

  - Adding sub-PRB may complicate the design, since there is bundling across multiple fields.

  - If DCI design allows for accommodating this easily, it should be supported.

  - This feature is deprioritized.

- Rel-15 feature for 64QAM for non-repeated unicast PDSCH in CE mode A

  - This feature bundles together the MCS field with the FH flag, which may complicate the DCI design.

  - Depending on the DCI design, this feature can be added.

  - This feature is deprioritized.

- Rel-15 feature for uplink HARQ-ACK feedback in DCI in CE mode A/B

  - For HD-FDD: Support if there are spare states in DCI.

  - For TDD and FD-FDD (early termination): Need further study.

Proposal 11: For optional features:

- Supported: Modulation restrictions, 2984 bits, flexible PRB, HARQ-ACK feedback (except early termination)

- Deprioritized/FFS: 10 HARQ processes, sub-PRB, 64-QAM, early PUSCH termination.

- Not supported: New repetitions for PDSCH/PUSCH, larger BW, dynamic HARQ-Ack delay

# Summary

Proposal 1: For the DCI design for scheduling multiple transport blocks in CE Mode B:

- Retain same scheduling flexibility as legacy DCI when the number of TBs scheduled ∈{1,2}

- Retain full scheduling flexibility across all combinations of (up to 4) HARQ processes for all values of number of TBs scheduled

- Jointly encode HARQ processes scheduled and NDIs for all values of number of TBs scheduled

- For number of scheduled TBs ∈{3,4}, restrict the MCSs allowed when number of repetitions is large

Proposal 2: Adopt the DCI design for scheduling multiple transport blocks in CE Mode B outlined in Section 2.

Proposal 3: For the DCI design for scheduling multiple transport blocks in CE Mode A:

- Retain same scheduling flexibility as legacy DCI when 1 TBs scheduled

- Retain same scheduling flexibility for 2 TBs scheduled except:

  - Allow 2 possible choices of RV index for each TB independently

- Retain scheduling flexibility across all HARQ ID combinations of 1, 2, 3, 4 or 8 TBs

- Allow scheduling of 5 or 6 TBs using continuous HARQ IDs with arbitrary start HARQ ID

- Allow scheduling of 7 TBs using continuous HARQ IDs with start HARQ ID equal to 0

- Jointly encode HARQ processes, NDIs, RV indices (when present) and FH indicator (when present)

- For number of scheduled TBs ∈{3,…8}, the starting RV for each TB across a set of repetitions is RV0

- Retain FH indication in DCI only for number of scheduled TBs ∈{1,2,8}; for other values of number of TBs scheduled, the RRC indication is followed.

Proposal 4: Adopt the DCI design for scheduling multiple transport blocks in CE Mode A outlined in Section 3.

Observation 1: The DCI design for CE Mode A proposed in this contribution outperforms the design in [2] considerably under bursty (non-full buffer) traffic patterns.

Proposal 5: Use 2 bits in DCI to signal the bundling configuration used when multiple TBs are scheduled by a single DCI, where the 2-bit field is interpreted jointly with the number of scheduled TBs according to Table 4 in Section 4 and one entry always signals unbundled transmission.

Proposal 6: The starting (absolute) subframe siPUCCH for the ACK transmission corresponding to TBi for TBi, i∈0,1,..,Nsched in the absence of HARQ ACK bundling is determined as:

- s0PUCCH=max m0PDSCH+4, (mNsched-1PDSCH+2)×Δ

- siPUCCH=max {miPDSCH+4, mi-1PUCCH+Nabs,i-1 PUCCH},  i∈{1,2,…,Nsched-1}

where miPDSCHdenotes the last (absolute) subframe index for the ith TB, i∈{0,1,…,Nsched-1}; Nabs,iPUCCH denotes the number of absolute subframes required to transmit the ACK for the ith TB; Δ=1 for HD-FDD and Δ=0 otherwise

Proposal 7: The starting (absolute) subframe sBiPUCCH for the ACK transmission corresponding to TB bundle Bi in the is determined as:

- sB0PUCCH=max{mjPDSCH∈M0} mjPDSCH+4,  mNsched-1PDSCH+2×Δ

- sBiPUCCH=max{mjPDSCH∈Mi} {mjPDSCH+4,  sBi-1PUCCH+Nabs,Bi-1 PUCCH}, i≠0

Where Mk=miPDSCHTBi∈Bk; sBiPUCCH denotes the starting (absolute) subframe index for the HARQ ACK for the ith TB bundle; Nabs,BiPUCCH denotes the number of absolute subframes required to transmit the HARQ ACK for the ith TB bundle; Δ=1 for HD-FDD and Δ=0 otherwise.

Proposal 8: For subframe index i∈{0,1,2,…} the TB index mapping ITB(i), which implicitly involves the design of the time-domain interleaver, should be designed such that all TBs have an equitable distribution of repetitions across all configured narrowbands.

Proposal 9: With frequency hopping enabled, for subframe index i∈{0,1,2,…} the TB index mapping ITB(i) should depend on the actual number of TBs scheduled, and the relationship may be determined by

ITBi=iNIL+iLCMNsched×NIL,    NNBch,DL×NNB,hopch,DL×1 mod GCFNsched, (NNBch,DL×NNB,hopch,DL)NIL×NNB,hopch,DLNILmod Nsched,

where the parameters are as defined in this contribution.

Observation 2: Restricting the interleaving granularity NIL to be equal to the hopping interval NNBch,DL may incur significant (e.g., 1.2 dB) performance penalty. Such penalties may be avoided with the hopping-aware interleaver design in Section 6 of this contribution, in conjunction with small interleaving granularities.

Proposal 10: The interleaving granularity is 1 subframe for CE Mode A and 4 subframes for CE Mode B with and without frequency hopping.

Proposal 11: For optional features:

- Supported: Modulation restrictions, 2984 bits, flexible PRB, HARQ-ACK feedback (except early termination)

- Deprioritized/FFS: 10 HARQ processes, sub-PRB, 64-QAM, early PUSCH termination.

- Not supported: New repetitions for PDSCH/PUSCH, larger BW, dynamic HARQ-Ack delay

# References

[1] R1-1908827 “Scheduling of multiple DL/UL transport blocks”, by Qualcomm Incorporated, in RAN1 98.

[2] R1-1911919 “Scheduling of multiple transport blocks”, by Huawei, HiSilicon, in RAN1 99.