3GPP TSG RAN WG1 Meeting #99								R1-1913396

Reno, NV, U.S.A., November 18 - 22, 2019

Title:	[Draft] Reply LS on uplink TDM pattern for LTE DAPS based enhanced make-before-break HO

Response to:	R1-1911802(R2-1913996)

Release:	Rel-16

Work Item:	LTE_feMob-core

Source:	Intel Corp. [RAN1]

To:	RAN2

Cc:	RAN3, RAN4

Contact Person:

#### Name:	Daewon Lee

E-mail Address:	daewon.lee (at) intel (dot) com

Attachments:

1. Overall Description:

RAN1 would like to thank RAN2 for the LS on feasibility of TDM pattern for DAPS based LTE handover. The following are RAN1 responses to the questions asked.

Q1: RAN2 respectfully requests feedback from RAN1 on the feasibility and need to specify uplink TDM pattern for DAPS based LTE handover.

A1: RAN1 concludes that TDM scheduling (pattern) realized only the network side is feasible at the sacrifice of DL throughput. Also the TDM scheduling (pattern) realized only the network may have implication in HARQ retransmission in some cases. TDM pattern that is provided to the UE may help alleviate this issue, but there may be additional interruption due to RRC configuration unless deterministic time for application of the TDM pattern at the UE is known at the eNB. TDM pattern realization only in the network side and also TDM pattern provided to the UE are feasible from the RAN1 perspective.

In both TDM realization methods, transmission of certain signals/channel (e.g. PRACH) at the source cell during DAPS based LTE handover may collide with target cell transmission. For handling of these cases, RAN1 agrees that UEs prioritize transmission of the signals/channels to the target cell.

Q2: If found feasible and needed by RAN1, RAN2 respectfully requests RAN1 to specify uplink TDM pattern for DAPS based LTE handover.

A2: RAN1 has discussed the possibility TDM framework defined for EN-DC for both FS1 (R15) and FS2 (R16) for source eNB during DAPS based LTE handover.

Alt1) However, RAN1 has not reached consensus on whether the TDM framework defined for EN-DC can be applicable for the DAPS based LTE HO. Given the lack of consensus on applicable TDM pattern for DAPS based LTE HO and given that it is feasible to support network side TDM scheduling pattern realization with performance loss, RAN1 was also not able reach consensus on need for the TDM pattern.

Alt2) RAN1 has agreed to reuse the TDM framework defined for EN-DC for the source eNB.

RAN1 would like to kindly ask RAN2 to consider the above in their further work.

2. Actions:

To RAN2: 	RAN1 would like to kindly ask RAN2 to consider the above in their further work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #99 	18 – 22 Nov 2019			Reno, Nevada, US

TSG-RAN WG1 Meeting #100 	24 – 28 Feb 2020			Athens, Greece