3GPP TSG-RAN WG2 #100 	Tdoc R2-1712340

Reno, Nevada, 27th November – 1st December 2017                    (Revision of R2-1710450)

Agenda Item:	10.4.4.5

Source:	Ericsson

Title:	Cell reselection measurement rules

Document for:	Discussion, Decision

# Introduction

This paper discusses the cell reselection measurement rule for a camped UE in RRC_IDLE and RRC_INACTIVE mode. In NR, the following are agreed in the previous meetings regarding the SS Burst Set periodicity and SS Block based RRM measurement timing configuration for RRC_IDLE mode.

Agreements - Same set of configuration values for SS periodicity for CONNECTED/IDLE & non-standalone cases. Values for configuration set for CONNECTED/IDLE & non-standalone case - {5, 10, 20, 40, 80, 160} ms - SS burst set periodicity default value for initial cell selection: 20/20 msec. - The transmission of SS blocks within SS burst set is confined to a 5 ms window regardless of SS burst set periodicity. - Regarding the SS block based RRM measurement timing configuration (SMTC) i.e., measurement window periodicity/duration/offset information for UE RRM measurement per frequency carrier, - For IDLE mode measurements, only single SMTC is configured per frequency carrier

This paper is a revision of R2-1710450 and the same proposals for the RRC_IDLE mode have been applied to the RRC_INACTIVE mode in this revised version.

# Discussion

Based on the agreements, it is still an open issue whether a camped UE in RRC_IDLE mode or RRC_INACTIVE mode can be configured by the network to know the SS Block based RRM measurement window for the serving cell.

Observation 1	It is an open issue in NR whether a camped UE in RRC_IDLE or RRC_INACTIVE mode can derive the SS Block based RRM measurement window for the serving cell.

In LTE, a camped UE in RRC_IDLE or RRC_INACTIVE mode is required to measure the serving cell at least every DRX cycle, while the minimum measurement periodicity of intra-frequency measurements can be larger (1, 2, or 4 DRX cycles depending on the DRX cycle length). Furthermore, a camped UE in RRC_IDLE or RRC_INACTIVE mode may choose not to perform intra-frequency measurements if the serving cell fulfils Srxlev > SintraSearchP and Squal > SIntraSearchQ shown in the table below.

Following rules are used by the UE to limit needed measurements (TS 36.304): - If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements. - Otherwise, the UE shall perform intra-frequency measurements.

In our view, NR should have similar schemes as in LTE for UE in RRC_IDLE or RRC_INACTIVE mode to conserve the energy. Specifically speaking, the minimum measurement periodicity of the intra-frequency SS Block based measurement for a camped UE in RRC_IDLE or RRC_INACTIVE mode in NR can be larger than the minimum measurement periodicity of the serving cell measurement. A camped UE in RRC_IDLE or RRC_INACTIVE mode in NR can choose not to perform the intra-frequency measurements if the serving cell fulfils a certain condition.

Given the above discussions, we have the following proposal.

Proposal 1	The network should provide information to a camped UE in RRC_IDLE or RRC_INACTIVE mode, so that the UE can choose to only perform the measurement during a time window that covers the SS Blocks of an SS Burst Set in the serving cell if the serving cell fulfils a certain condition. For example, the number of SS Blocks transmitted by the serving cell in an SS Burst Set can be included in the system information.

# Conclusion

In this paper, we have revisited some of the energy-conservation measurement concepts in LTE for UE in RRC_IDLE or RRC_INACTIVE mode and discussed an open issue for SS Block based measurement in RRC_IDLE and RRC_INACTIVE mode. Based on that, the following has been proposed:

Observation 1	It is an open issue in NR whether a camped UE in RRC_IDLE or RRC_INACTIVE mode can derive the SS Block based RRM measurement window for the serving cell.

Proposal 1	The network should provide information to a camped UE in RRC_IDLE or RRC_INACTIVE mode, so that the UE can choose to only perform the measurement during a time window that covers the SS Blocks of an SS Burst Set in the serving cell if the serving cell fulfils a certain condition. For example, the number of SS Blocks transmitted by the serving cell in an SS Burst Set can be included in the system information.