3GPP TSG-RAN WG2 #100	R2-1712359

Reno, USA, November 27th - December 1st 2017	R2-1710255

Agenda item:	10.2.19

Source:	Rapporteur (Nokia)

Title:	QoS Update

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

This contribution suggests an update to the Stage 2 to reflect the latest agreements on QoS from SA2 in 23.501, especially around reflective QoS.

# 12	QoS

The QoS architecture in NG-RAN, both for NR connected to 5GC and for E-UTRA connected to 5GC, is depicted in the Figure 12-1 and described in the following:

-	For each UE, 5GC establishes one or more PDU Sessions.

-	For each UE, the NG-RAN establishes one or more Data Radio Bearers (DRB) per PDU Session. The NG-RAN maps packets belonging to different PDU sessions to different DRBs. Hence, the NG-RAN establishes at least one default DRB for each PDU Session.

-	NAS level packet filters in the UE and in the 5GC associate UL and DL packets with QoS Flows.

-	AS-level mapping rules in the UE and in the NG-RAN associate UL and DL QoS Flows with DRBs.

# Figure 12-1: QoS architecture

At NAS level (see 3GPP TS 23.501 [3]), the QoS flow is the finest granularity of QoS differentiation in a PDU session. A QoS flow is identified within a PDU session by a QoS Flow ID (QFI) carried in an encapsulation header over NG-U.

NG-RAN and 5GC ensure quality of service (e.g. reliability and target delay) by mapping packets to appropriate QoS Flows and DRBs. Hence there is a 2-step mapping of IP-flows to QoS flows (NAS) and from QoS flows to DRBs (Access Stratum).

At NAS level, a QoS flow is characterised by a QoS profile provided by 5GC to NG-RAN and QoS rule(s) provided by 5GC to the UE. The QoS profile is used by NG-RAN to determine the treatment on the radio interface while the QoS rules dictates the mapping between uplink User Plane traffic and QoS flows to the UE. A QoS flow may either be "GBR" or "Non-GBR" depending on its profile. The QoS profile of a QoS flow contains QoS parameters, for instance:

-	For each QoS flow:

-	A 5G QoS Identifier (5QI); and.

-	An Allocation and Retention Priority (ARP).

-	In case of a GBR QoS flow only:

-	Guaranteed Flow Bit Rate (GFBR) for both uplink and downlink;

-	Maximum Flow Bit Rate (MFBR) for both uplink and downlink.

Notification Control (which when included indicates that notification is requested when the QoS of some flows can no longer be met) is pending clarifications from SA2.

-	In case of Non-GBR QoS only:

-	Reflective QoS Attribute (RQA): the RQA, when included, indicates that some (not necessarily all) traffic carried on this QoS flow is subject to reflective quality of service (RQoS) at NAS.

At Access Stratum level, the data radio bearer (DRB) defines the packet treatment on the radio interface (Uu). A DRB serves packets with the same packet forwarding treatment. In the downlink, the QoS flow to DRB mapping by NG-RAN is based on QFI and the associated QoS profiles. Separate DRBs may be established for QoS flows requiring different packet forwarding treatment, or several QoS Flows can be multiplexed in the same DRB.

In the uplink, the NG-RAN may control the mapping of QoS Flows to DRB in two different ways:

-	Reflective mapping: for each DRB, the UE monitors the QFI(s) of the downlink packets and applies the same mapping in the uplink; that is, for a DRB, the UE maps the uplink packets belonging to the QoS flows(s) corresponding to the QFI(s) and PDU Session observed in the downlink packets for that DRB. To enable this reflective mapping, the NG-RAN marks downlink packets over Uu with QFI.

-	Explicit Configuration: besides the reflective mapping, the NG-RAN may configure by RRC an uplink "QoS Flow to DRB mapping".

-	The UE shall always apply the latest update of the mapping rules regardless of whether it is performed via reflecting mapping or explicit configuration.

In the downlink, the QFI is only signalled by NG-RAN over Uu for the purpose of RQoS and if neither NG-RAN, nor the NAS (as indicated by the RQA) intend to use reflective mapping for the QoS flow(s) carried in a DRB, no QFI is signalled for that DRB over Uu. In the uplink, the QFI is only signalled by the UE over Uu when several QoS flows are multiplexed by NG-RAN in the same DRB i.e. when NG-RAN cannot set the value of the QFI of the encapsulation header over NG-U on its own and requires the UE to mark uplink packets of that DRB.

For each PDU session, a default DRB is configured. If an incoming UL packet matches neither an RRC configured nor a reflective "QoS Flow ID to DRB mapping", the UE shall map that packet to the default DRB of the PDU session.

Within each PDU session, it is up to NG-RAN how to map multiple QoS flows to a DRB. The NG-RAN may map a GBR flow and a non-GBR flow, or more than one GBR flow to the same DRB, but mechanisms to optimise these cases are not within the scope of standardization. The timing of establishing non-default DRB(s) between NG-RAN and UE for QoS flow configured during establishing a PDU session can be different from the time when the PDU session is established. It is up to NG-RAN when non-default DRBs are established.

In DC, the QoS flows belonging to the same PDU session can be mapped to different bearer types (see subclause 4.5.2) and as a result there can be two different SDAP entities configured for the same PDU session: one for MCG and another one for SCG (for instance when one MCG bearer and one SCG bearer are used for two different QoS flows).