3GPP TSG-RAN WG2#101 	R2-1801776

Athens, Greece, 26th February – 2nd March 2018

Agenda Item:	9.7.2

Source:	OPPO

Title:	Discussion UE Identifier Definition in RRC Signalling in eLTE

Document for:	Discussion, Decision

Introduction

In R2-1712068, RAN2 send LS to SA2 to ask the following questions to SA2:

RAN2 would like to know if the 40 bit S-TMSI address space is partitioned into two for EPC and 5GC (i.e., no overlap)? Or, is there is a possibility that EPC and 5GC may assign the same value for S-TMSI and 5G-S-TMSI identifiers for different UEs if both identifiers have same size? Also, does SA2 plans to have some level of coordination between EPC and 5GC in terms of S-TMSI and 5G-S-TMSI allocation?

In LS to RAN2 (S2-181262), SA2 replied as follows:

SA2 does not assume partitioning or coordination for the 5G-S-TMSI/S-TMSI number space between MMEs and AMFs serving an eNB connected to both EPC (MME) and 5GC (AMF). As a result EPC and 5GC are allowed to assign the same value for S-TMSI and 5G-S-TMSI identifiers for two different UEs accessing EPC and 5GC, respectively. SA2 would therefore like to ask RAN2 to distinguish the S-TMSI type (S-TMSI or 5G-S-TMSI) in RRC signalling.

In this contribution, we discuss how to distinguish S-TMSI and 5G-S-TMSI in RRC signaling and propose TP to running 36.331 CR.

Discussions and Proposals

In current TS 36.331, S-TMSI is used in the following RRC signaling messages including

RRCConnectionRequest, RRCConnectionSetupComplete, RRCConnectionReestablishmentRequest.  Meanwhile S-TMSI is also used in Paging message.  Some of these messages are for uplink and some are for uplink.  While the potential confusion between S-TMSI and 5G-S-TMSI may happen in uplink messages when ng-eNB needs to route the S1 or NG messages to MME or AMF.  For downlink RRC messages, there seems to be no risk for collision.

However, since S-TMSI and 5G-S-TMSI have different structure, we think in downlink RRC messages, S-TMSI and 5G-S-TMSI should also be indicated explicitly although there is no collision risk.

The structure of S-TMSI is defined as follows:

S-TMSI information element

-- ASN1START

S-TMSI ::=							SEQUENCE {

mmec								MMEC,

m-TMSI								BIT STRING (SIZE (32))

}

-- ASN1STOP

In TS 23.501, the structure of 5G-S-TMSI is defined as follows:

<5G-S-TMSI> := <AMF Set ID> <AMF Pointer> <5G-TMSI>

Basically, we think that similar to S-TMSI which is allocated by EPC, in TS 36.331 there should also be definition for 5G-S-TMSI for the case when UE is connected to 5GC.

- RAN2 should specify 5G-S-TMSI in TS 36.331 for both uplink and downlink RRC messages as proposed above.

In RAN2#99bis, there is one agreement about CN type indication in Msg 5.

Agreements

1-	Msg 5 is used to indicate the CN type. eNB shall initially configure SRB1 with LTE PDCP. Upon receiving CN Type Selection = 5GCN in Message 5, eLTE eNB reconfigures SRB1 with NR-PDCP

FFS: Whether the reconfiguration to NR PDCP is required before SMC.

2-	If it is found during further work that changes are required in Message 3 for other reasons, then this decision can be revisited (a solution where eNB initially configures SRB1 with NR-PDCP can be adopted)

We think that if 5G-S-TMSI is included in MSG5 together with 5G-S-TMSI, there is no ambiguity between S-TMSI and 5G-S-TMSI.  Thus in this case, either CN type indication or the indicator in RRC signalling would be redundant.  To solve this issue, RAN2 need to revisit previous agreement.

- RAN2 need to revisit the agreement and decide that in MSG5 whether the redundancy between previous agreed CN type in MSG5 and the proposed S-TMSI/5G-S-TMSI indicator can be avoided.

TP to Running 36.331 CR

The TP to running 36.331 CR is provided below.

==================First Change Start=========================

- 5.3.3.3	Actions related to transmission of RRCConnectionRequest message

The UE shall set the contents of RRCConnectionRequest message as follows:

1>	set the ue-Identity as follows:

2>	if upper layers provide an S-TMSI or 5G-S-TMSI:

3>	set the ue-Identity to the value received from upper layers;

2>	else:

3>	draw a random value in the range 0 .. 240-1 and set the ue-Identity to this value;

NOTE 1:	Upper layers provide the S-TMSI or 5G-S-TMSI if the UE is registered in the TA of the current cell.

1>	if the UE supports mo-VoiceCall establishment cause and UE is establishing the RRC connection for mobile originating MMTEL voice and SystemInformationBlockType2 includes voiceServiceCauseIndication:

2>	set the establishmentCause to mo-VoiceCall;

==================First Change Stop=========================

==================Second Change Start=========================

- 5.3.3.4	Reception of the RRCConnectionSetup by the UE

NOTE:	Prior to this, lower layer signalling is used to allocate a C-RNTI. For further details see TS 36.321 [6];

The UE shall:

1>	if the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest:

2>	discard the stored UE AS context and resumeIdentity;

2>	indicate to upper layers that the RRC connection resume has been fallbacked;

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

1>	if stored, discard the cell reselection priority information provided by the idleModeMobilityControlInfo or inherited from another RAT;

1>	if stored, discard the dedicated offset provided by the redirectedCarrierOffsetDedicated;

1>	stop timer T300;

1>	stop timer T302, if running;

1>	stop timer T303, if running;

1>	stop timer T305, if running;

1>	stop timer T306, if running;

1>	stop timer T308, if running;

1>	perform the actions as specified in 5.3.3.7;

1>	stop timer T320, if running;

1>	stop timer T350, if running;

1>	perform the actions as specified in 5.6.12.4;

1>	release rclwi-Configuration, if configured, as specified in 5.6.16.2;

1>	stop timer T360, if running;

1>	stop timer T322, if running;

1>	enter RRC_CONNECTED;

1>	stop the cell re-selection procedure;

1>	consider the current cell to be the PCell;

1>	set the content of RRCConnectionSetupComplete message as follows:

2>	if the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest:

3>	if upper layers provide an S-TMSI or 5G-S-TMSI:

4>	set the s-TMSI or 5G-S-TMSI to the value received from upper layers;

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1 (or SystemInformationBlockType1-NB in NB-IoT);

==================Second Change Stop=========================

==================Third Change Start=========================

PagingUE-Identity ::=				CHOICE {

s-TMSI								S-TMSI,

5G-S-TMSI                                5G-S-TMSI,

imsi								IMSI,

...

}

==================Third Change Stop=========================

==================Fourth Change Start=========================

InitialUE-Identity ::=				CHOICE {

s-TMSI								S-TMSI,

5G-S-TMSI						5G-S-TMSI,

randomValue							BIT STRING (SIZE (40))

}

==================Fourth Change Stop=========================

==================Fifth Change Start=========================

5G-S-TMSI information element

-- ASN1START

5G-S-TMSI ::=							SEQUENCE {

amf-set-id								AMF-SET-ID,

amf_pointer								BIT STRING (SIZE (4)),

5G-tmsi									5G-TMSI,

}

-- ASN1STOP

==================Fifth Change Stop=========================

- RAN2 agree the proposed TP to running 36.331 CR.

Conclusion

In this contribution, we discuss how UE determines the CN type and indicate to the ng-eNB and we have the following:

Proposal 1	RAN2 should specify 5G-S-TMSI in TS 36.331 for both uplink and downlink RRC messages as proposed above.

Proposal 2	RAN2 need to revisit the agreement and decide that in MSG5 whether the redundancy between previous agreed CN type in MSG5 and the proposed S-TMSI/5G-S-TMSI indicator can be avoided.

Proposal 3	RAN2 agree the proposed TP to running 36.331 CR.

- References

R2-1800036	Reply LS to CN node selection for LTE features when E-UTRA is connected to 5G CN (S2-179618; contact: Nokia)	SA2	LS in	Rel-15	5GS_Ph1	To:RAN2, CT1	Cc:RAN3

R2-1710182 Further Discussion on UE Preference and CN Selection, OPPO