3GPP TSG-RAN WG2 Meeting #101			                            R2-1802018

Athens, Greece, 26th February - 2nd March 2018

Source: 			ZTE Corporation, Sanechips

Title: 	Discussion on the indication of band information

Agenda item:		10.4.1.3.1

Document for:	    Discussion and Decision

# Introduction

LS on channel raster and SS raster from RAN4 was received at last RAN2 meeting [1]. The channel raster is defined by NR-ARFCN and the sync raster is defined by GSCN. Meanwhile, RAN4 has specified the supported frequency range, the applicable NR-ARFCN and the applicable SS raster per operating band in their specifications [2][3].

In this contribution, we discuss the indication of band information in RAN2.

# Discussion

In E-UTRA, the frequency ranges allocated for some of the bands may overlap with each other. When an IDLE UE detects a cell, it is unable to determine the band of the cell if the frequency of the cell belongs to more than one overlapped bands. So in E-UTRA, a frequency band indicator (freqBandIndicator) is broadcasted in SIB1 [4].

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n2 | 1850 MHz – 1910 MHz | 1930 MHz – 1990 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| … | … | … | … |

Table 1. NR operating bands in FR1

Similar as E-UTRA, in NR, there are also bands with overlapped frequency ranges, e.g. n20 and n28 as illustrated in Table 1. So, a similar frequency band indicator should also be introduced in SIB1 to indicate the operating band of the cell.

Proposal 1: A frequency band indicator should be introduced in SIB1 to indicate the operating band of the cell.

In E-UTRA, although some of the bands may overlap with each other, however, the EARFCN values are uniquely allocated for each bands. In other words, the EARFCN values allocated for one band will never overlap with another band. Given that, when configure a cell for a CONNECTED UE in E-UTRA, e.g. configure the target PCell during handover, add the PSCell for the SCG or add a SCell for CA, the EARFCN is used to uniquely identify the operating frequency and operating band of the cell.

| Frequency range [MHz] | ΔFGlobal [kHz] | FREF-Offs [MHz] | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |
| 3000 – 24250 | 15 | 3000 | 600000 | 600000 – 2016666 |
| 24250 – 100000 | 60 | 24250.08 | 2016667 | 2016667 – 3279167 |

Table 2. NR-ARFCN parameters for the global frequency raster

| Frequency range | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 2700 MHz | N * 900kHz + M * [5 kHz] N=1:3000, M=-1:1 | 3N + M - 1 | 1 – 8999 |
| 2400 – 24250 MHz | 2400 MHz + N * 1.44 MHz N= 0:15173 | 9000 + N | 9000 – 24173 |
| 24250 – 100000 MHz | 24250.08 MHz + N * 17.28 MHz N= 0:4383 | 24174+ N | 24174– 28557 |

Table 3. GSCN parameters for the global frequency raster

| NR Operating Band | ΔFRaster [kHz] | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 100 | 384000 – <20> – 396000 | 422000 – <20> – 434000 |
| n2 | 100 | 370000 – <20> – 382000 | 386000 – <20> – 398000 |
| n3 | 100 | 342000 – <20> – 357000 | 361000 – <20> – 376000 |
| n5 | 100 | 164800 – <20> – 169800 | 173800 – <20> – 178800 |
| n7 | 100 | 500000 – <20> – 514000 | 524000 – <20> – 538000 |
| n8 | 100 | 176000 – <20> – 183000 | 185000 – <20> – 192000 |
| n20 | 100 | 166400 – <20> – 172400 | 158200 – <20> – 164200 |
| n28 | 100 | 140600 – <20> – 149600 | 151600 – <20> – 160600 |
| … | … | … | … |

Table 4. Applicable NR-ARFCN per operating band in FR1

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 15 kHz | Case A | 7039 – <1> – 7224 |
| n2 | 15 kHz | Case A | 6439 – <1> – 6624 |
| n3 | 15 kHz | Case A | 6022 – <1> – 6258 |
| n5 | 15 kHz | Case A | 2902 – <1> – 2973 |
|  | 30 kHz | Case B | 2911 – <1> – 2964 |
| n7 | 15 kHz | Case A | [8740 – <1> – 8958] |
| n8 | 15 kHz | Case A | 3091 – <1> – 3192 |
| n20 | 15 kHz | Case A | 2644 – <1> – 2727 |
| n28 | 15 kHz | Case A | 2533 – <1> – 2667 |
| … | … | … | … |

Table 5. Applicable SS raster entries per operating band (FR1)

However in NR, the NR-ARFCN values for channel raster numbers and the GSCN values for sync raster numbers are defined as global [2][3], illustrated in Table 2 and Table 3 respectively. In this way, for the same particular frequency positions, the corresponding NR-ARFCN values are always the same no matter which band they belong to. For instance, as illustrated in Table 4, the NR-ARFCN values allocated for the overlapping band n20 and n28 are overlapped too. The same occurs for the GSCN (illustrated in Table 5). In other words, the operating band of a cell can’t be determined from the NR-ARFCN or GSCN in case of band overlapping.

Observation 1: In NR, the NR-ARFCN values for the same frequency positions are always the same no matter which band they belong to.

Observation 2: In NR, the GSCN values for the same frequency positions are always the same no matter which band they belong to.

Observation 3: The operating band of a cell can’t be determined from the NR-ARFCN or GSCN in case of band overlapping.

In the current 38.331[5], when configure a cell for a CONNECTED UE, e.g. configure the PSCell and SCell on SCG, only the DL frequency is indicated, for instance the GSCN of the SSB in current 38.331. However, given the above analysis, the UE can’t distinguish the exact operating band from the NR-ARFCN or GSCN only in the case of band overlapping. So a frequency band indicator should also be introduced when configuring the DL frequency of a cell in the CONNECTED state, e.g. in FrequencyInfoDL.

Proposal 2: A frequency band indicator should be introduced when configuring the DL frequency of a cell in the CONNECTED state, e.g. in FrequencyInfoDL.

In E-UTRA, if a cell supports the operating of multiple bands, a multi-band information list (multiBandInfoList) and an additional frequency band indicator priority (freqBandIndicatorPriority) are introduced in SIB1 to conduct the UE to select an operating band according to UE capability. With the absence of freqBandIndicatorPriority, if the UE supports the frequency band in the freqBandIndicator field it shall apply that frequency band. Otherwise, the UE shall apply the first listed band which it supports in the multiBandInfoList field. If the the freqBandIndicatorPriority is present, the UE shall prioritize the frequency bands in the multiBandInfoList field in decreasing priority order. Only if the UE does not support any of the frequency band in multiBandInfoList, the UE shall use the value in freqBandIndicator field.

Considering that there are also overlapped freqeuncy bands in NR, so there can also be cells supports the operating of multiple bands. To conduct the operating band selection behavior for the IDLE/INACTIVE UE, the similar parameters like multi-band information list (e.g. multiBandInfoList) and frequency band indicator priority (e.g. freqBandIndicatorPriority) should be introduced in SIB1 too.

Proposal 3: The E-UTRA similar parameters like multi-band information list (e.g. multiBandInfoList) and frequency band indicator priority (e.g. freqBandIndicatorPriority) should be introduced in NR SIB1.

# Conclusion and proposals

In this contribution, we discuss the indication of band information in RAN2 with the following observations and proposals:

Observations:

Observation 1: In NR, the NR-ARFCN values for the same frequency positions are always the same no matter which band they belongs to.

Observation 2: In NR, the GSCN values for the same frequency positions are always the same no matter which band they belongs to.

Observation 3: The operating band of a cell can’t be determined from the NR-ARFCN or GSCN in case of band overlapping.

Proposals:

Proposal 1: A frequency band indicator should be introduced in SIB1 to indicate the operating band of the cell.

Proposal 2: A frequency band indicator should be introduced when configuring the DL frequency of a cell in the CONNECTED state, e.g. in FrequencyInfoDL.

Proposal 3: The E-UTRA similar parameters like multi-band information list (e.g. multiBandInfoList) and frequency band indicator priority (e.g. freqBandIndicatorPriority) should be introduced in NR SIB1.

# References

- R2-1800021LS on channel raster and SS raster

- R4-1800265 DraftCR 38104-f00 Combined updates from R4 AH-1801

- R4-1800403, R4-1800404 (38.101 User Equipment (UE) radio transmission and reception)

- 36.331 Radio Resource Control (RRC)

- Draft CR 38331 v15.0.1