3GPP TSG-RAN WG2#101	R2-1802149

Athens, Greece, 26thFebruary-2ndMarch 2018	Revision of R2-1800170

Source:	CATT

Title:	Duplication activation/deactivation MAC CE

Agenda Item:	10.3.1.11

Document for:	Discussion and Decision

# Introduction

We got many agreements on packet duplication in previous meetings. We need to complete the duplication activation/deactivation MAC CE design based on them. In this contribution, we discuss the duplication activation/deactivation MAC CE from below aspects:

- Duplication activation/deactivation MAC CE format;

- CA and DC duplication for UEs connected to both MCG and SCG

# Discussion

## Duplication activation/deactivation MAC CE format

In RAN2#99, we agreed:

Agreements

1.	For DC, when DRB duplication is deactivated via MAC CE, the UE falls back to the split bearer operation.  Once de-activated we rely on split bearer operation and configuration.

2.	1 byte bitmap could be used as duplication activation/deactivation MAC CE

3. 	The mapping between DRB and the MAC bitmap is based on order of DRB ID(s) of the duplicate configured DRB(s)

This yields below duplication activation/deactivation MAC CE format:

- MAC subheader

One LCID can be used to indicate the duplication activation/deactivation MAC CE, the MAC subheader is shown in Figure 1. It is the same as other fixed-sized MAC CE subheaders.

Figure 1: R/LCID MAC subheader

- MAC CE format

Above agreement states that, “the mapping between DRB and the MAC bitmap is based on order of DRB ID(s) of the duplicate configured DRB(s)”. Because the MAC PDU bit string regulation is that “In the figures in clause 6, bit strings are represented by tables in which the most significant bit is the leftmost bit of the first line of the table, the least significant bit is the rightmost bit on the last line of the table”, and the number of DRBs configured with duplication is variable, then the duplicate DRB with the highest DRB ID should be mapped to the leftmost bit, and the following duplicate DRBs are mapped to the bitmap from left to right in decreasing DRB ID order. It is consistent to the statement in 38.321.

Different from the SCellActivation/Deactivation MAC CE, the R bit is not needed in the duplication activation/deactivation MAC CE. The SCellActivation/Deactivation MAC CE has 1 byte format for 8 cells and 4 byte format for 32 cells, but there is no activation/deactivation for PCell, then R bit is reserved. However, there is no similar reason for R bit in duplication activation/deactivation MAC CE.

The MAC CE format is shown in Figure 2 based on above analysis.

Figure 2 Duplication activation/deactivation MAC CE

Proposal 1: RAN2 confirms the duplication activation/deactivation MAC subheader and MAC CE format as depicted in Figure 1 and Figure 2.

## CA and DC duplication for UEs connected to both MCG and SCG

We agreed in RAN2#99 that:

Agreements

1	We will not support MAC CE activation/deactivation of duplication within LTE MAC.

2	We will not support the CA duplication in LTE

3	CA duplication is supported for all non-split UM DRBs if the bearer uses NR-PDCP, for all architecture options (apart from cases excluded by 1 and 2)

FFS: for AM DRBs and SRBs

4	DC duplication is supported for all split DRB and SRBs if the bearer uses NR-PDCP, for all architecture options

5	We will not introduce new bearer type changes into RRC, but user plane session can discuss and decide terminology for DC duplication, CA duplication, split bearer operation, etc (Some clarification is needed for how to handle CA duplication within the current bearer type change discussion)

From the agreements 1 and 2, we can confirm that duplication activation/deactivation MAC CE will not be introduced in LTE MAC. From the agreement 4, DC duplication is supported in EN-DC.

Proposal 2: Confirm that the duplication activation/deactivation MAC CE is not introduced in LTE MAC, and DC duplication in EN-DC can only be activated/deactivated by NR node.

From the agreements 3 and 4, when a UE is connected to both MCG and SCG, both CA duplication and DC duplication can be configured for the UE.

All possible duplicate DRB types are shown in Figure3. DRB1 is a CA duplicate DRB in MCG, and DRB4 is a CA duplicate DRB in SCG. DRB2 is a DC duplicate DRB via MCG split, and DRB3 is a DC duplicate DRB via SCG split.

Figure 3all possible duplicate RB types

We need to clarify the meaning and usage of duplication activation/deactivation MAC CE in these cases.

Firstly, In RAN2 AH#1702, we agreed:

Agreements:

2	UE acts on MAC CEs received from MCG and SCG. No UE behaviour will be specified to manage a conflict between the commands received from MN and SN.

FFS Whether UL packet duplication for spit bearer applies for EN-DC.

From the agreement, any node can activate/deactivate DC duplicate DRBs. Then the DC duplicate DRBs should be mapped to the duplication activation/deactivation MAC CEs of both nodes.

Proposal 3: Confirm that the DC duplicate DRBs should be mapped to the duplication activation/deactivation MAC CEs of both nodes.

Then the remaining question is how to map the CA duplicate DRBs existing in one node.

There are two options:

- Option 1: unified duplication activation/deactivation MAC CEs in MCG and SCG

In option 1, gNBs and UE construct/interpret the duplication activation/deactivation MAC CEs based on all duplicate DRBs (including CA and DC) configured for the UE.

When a network node sends a duplication activation/deactivation MAC CE, it includes all bits corresponding to all duplicate DRBs configured for the UE, including the CA duplicate DRBs of the other network node. Since it does not make sense that SCG controls the activation/deactivation of CA duplicated DRBs of MCG (and vice-versa) UE should always disregard those bits. UE can identifywhich bits are relevant according to the MAC entity transmitting the MAC CE.

Figure 4 Unified MAC CE bitmapin the two nodes

In RAN2#97bis, we got below agreement. Then the DC duplication (via MCG split or SCG split) should be decided and configured by MCG because all DRB IDs are allocated by MCG.

Agreements

1: When both MCG and SCG reconfiguration is required due to coordination, the SCG reconfiguration message must be encapsulated in an MCG RRC message that also carries the corresponding MCG reconfiguration that ensures that the combined configuration is valid.

The question of option 1 is SCG should get the configuration of CA duplicate DRBs in MCG. It is unnecessary for SCG to know the information of MCG bearers. Option 1 will increase the overhead in Xn interface.

- Option 2: separate duplication activation/deactivation MAC CEs in MCG and SCG

Figure 6Different MAC CE bitmapsin the two nodes

In option 2, gNB and UE construct/interpret the duplication activation/deactivation MAC CE according to the duplicate DRBs within the link between the gNB and UE. UE maintains two duplication activation/deactivation MAC CEs (different bitmaps) from different NW nodes. Note Option 2 also respects the agreement that “mapping between DRB and the MAC bitmap is based on order of DRB ID(s) of the duplicate configured DRB(s)”, with the addition that it is restricted to the DRB ID(s) of the duplicate configured DRB(s) in each CG.

The two options can achieve the same effect. The differences between the two options are signaling overhead in Xn and complexity. In option 1, SCG needs to know the CA duplicate DRBs in MCG. It will increase unnecessarily signaling overhead between MCG and SCG. In option 2, UE should maintain two duplication activation/deactivation MAC CEs and identify the MAC CE format based on the NW node transmitting the MAC CE. Since there are two separate MAC entities in the DC case and UE should maintain the two MAC entities simultaneously, it seems the complexity of maintaining two duplication activation/deactivation MAC CEs is acceptable. Then we prefer the option 2.

Proposal 4: When UE is connected to both MCG and SCG, UE maintains two separate duplication activation/deactivation MAC CEs from MCG and SCG. Each MAC CE bitmap only indicates the duplicate DRBs in the corresponding network node.

# Conclusion

In this contribution, we discuss the design of duplication activation/deactivation MAC CE and give below proposals. Proposal 2 and proposal 3 are further clarificationof previous agreements. Below TP is related to proposal 1 and proposal 4.

Proposal 1: RAN2 confirms the duplication activation/deactivation MAC subheader and MAC CE format as depicted in Figure 1 and Figure 2.

Proposal 2: Confirm that the duplication activation/deactivation MAC CE is not introduced in LTE MAC, and DC duplication in EN-DC can only be activated/deactivated by NR node.

Proposal 3: Confirm that the DC duplicate DRBs should be mapped to the duplication activation/deactivation MAC CEs of both nodes.

Proposal 4: When UE is connected to both MCG and SCG, UE maintains two separate duplication activation/deactivation MAC CEs from MCG and SCG. Each MAC CE bitmap only indicates the duplicate DRBs in the corresponding network node.

# Reference

- R2-1801546, General corrections on TS 38.321

# Text proposal to 38.321

| Reason for change: | The duplication activation/deactivation MAC CE format is not decided and the operation of the duplication activation/deactivation MAC CE in DC is not clear. |  |  |  |
| --- | --- | --- | --- | --- |
| Summary of change: | -Complete the duplication activation/deactivation MAC CE format. - Add the statement that the duplication activation/deactivation MAC CE in one MAC entity only includes the DRB IDs in its corresponding NW node. |  |  |  |
| Consequences if not approved: | The duplication activation/deactivation MAC CE format only include 7 DRB IDs and the duplication activation/deactivation MAC CE in DC case can’t work. |  |  |  |
| Clauses affected: | 6.1.3.11 |  |  |  |
|  | Y | N |  |  |
| Other specs |  | X | Other core specifications | TS/TR ... CR ... |
| affected: |  | X | Test specifications | TS/TR ... CR ... |
| (show related CRs) |  | X | O&M Specifications | TS/TR ... CR ... |
| Other comments: |  |  |  |  |

#### 6.1.3.11	Duplication Activation/Deactivation MAC CE

The Duplication Activation/Deactivation MAC CE of one octet is identified by a MAC PDU subheader with LCID as specified in Table 6.2.1-1. It has a fixed size and consists of a single octet containing eightD-fields and one R-field. The Duplication Activation/Deactivation MAC CE is defined as follows (Figure 6.1.3.11-1).

-	Di: this field indicates the activation/deactivation status of the PDCP duplication of DRB i where i is the ascending order of DRB IDs configured with duplication within the node which sends theDuplication Activation/Deactivation MAC CE. The Di field is set to one to indication that the PDCP duplication of DRB i shall be activated. The Di field is set to zero to indication that the PDCP duplication of DRB i shall be deactivated.

Figure 6.1.3.11-1: Duplication Activation/Deactivation MAC CE