3GPP TSG-RAN WG#101	Tdoc R2-1802329

Athens, Greece, Feb 26 – Mar 2, 2018	Revision of R2-1800303

Agenda Item:	10.4.1.6.1

Source:	Ericsson

Title:	Size of remaining minimum system information

Document for:	Discussion, Decision

# Introduction

Before completing the design of SIB structure for minimum system information for NR, it is important to evaluate whether the size of the minimum system information is small enough to be transmitted in all realistic deployments. Both the maximum size of system information that can be transmitted and the size of system information that needs to be transmitted are likely to vary significantly due to factors such as beam sweeping, network sharing, access barring etc.

In this contribution, we analyze the expected size of the minimum system information.

# Discussion

As highlighted in [1]-[4], by several companies during ASN.1 review and in RAN2 email discussion [NR-AH1801#11] (System information content/structure), the size of SIB1 could be quite large.

In order to make a rough estimate of the size of the minimum system information, we have calculated the size including most (if not all) of the IEs currently proposed to be included. The detailed calculation is provided in Section 5, but some of the main assumptions are

- The minimum system information will include most of the information currently included in RRC ServingCellConfigCommon

- Network sharing of up to 6 PLMNs will need to be supported

- Value tags are provided per SIB

- Access barring information is provided

- There are three SI messages and 8 SIBs, SI1 = SIB2,3,4,5 (NR and LTE reselection), SI2 = SIB6,7,8 (ETWS, CMAS), SI3 = 16 (GPS and UTC).

Our estimate for the size of the minimum system information is roughly between 400 and 1300 bits. Note that even though this estimate is unlikely to be extremely accurate, we do feel it represents the order of magnitude for remaining minimum system information reasonably well. The IEs with largest number of bits are

- Serving cell config common: between 100 and 1150 bits

- PLMN information for network sharing: ~150 bits

- Access barring: ~100 bits (note that we have not assumed PLMN specific ACB parematers)

- SI scheduling info: ~50 bits

- Value tags: ~50 bits

In our opinion, even 400 bits is a relatively large size for information that may need to be beam swept at least in some deployments. It would be possible to reduce the size by moving some of the information, such as some of the PLMN information in network sharing situations (proposed in [5]) or access barring information (proposed in [3]) to other system information. Such optimizations would increase the system access time, and would contradict the RAN2 desire to have all information needed to access the cell in the minimum system information, but might be justified in some deployments.

However, since the L1 configuration is by far the largest contributor to the SIB1 size, we would prefer to check with RAN1 if the expected minimum system information size is an issue and if yes, whether they could consider slightly less flexibility in L1 parameter configuration.

- Check with RAN1 whether remaining minimum system information size of roughly 1000 bits can be supported in all deployments.

A draft LS to RAN1 is provided in [6].

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	Check with RAN1 whether remaining minimum system information size of roughly 1000 bits can be supported in all deployments.

# References

R2-1712483	System information structure and contents	Ericsson

R2-1713284	Detailed design of the contents of System Information	Huawei, HiSilicon

R2-1713433	Some SIB1 contents and clean-up of Section 7.3 in Stage-2	Samsung R&D Institute India

R2-1713636	Organization of NR System Information	Qualcomm Incorporated

R2-1800291	System information content at network sharing	Ericsson

R2-1800304	Draft LS to RAN1 on size of remaining minimum system information	Ericsson

# Estimated IE sizes

Note: in case there are sub-fields (such as the IEs of a SEQUENCE), they are listed with 2 space indentation, and the overall size is marked in bold.

| Estimated total size: 426-1298 bits |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| IE |  |  | IE size | L1 IE size | L2 IE size | Comment |
| ServingCellConfigCommon |  |  | 114-1156 |  |  |  |
|  | physCellId OPTIONAL |  |  | 1 |  | Encoded in MIB in physical layer, not explicitly needed in SIB |
|  | frequencyInfoDL FrequencyInfoDL OPTIONAL |  |  | 34-106 |  |  |
|  |  | FrequencyInfoDL ::= SEQUENCE { absoluteFrequencySSB GSCN-ValueNR, ssb-subcarrier-offset INTEGER (1..15) OPTIONAL, absoluteFrequencyPointA ARFCN-ValueNR OPTIONAL, scs-SpecificCarriers SEQUENCE (SIZE (1..ffsValue)) OF SCS-SpecificVirtualCarrier, ... } |  |  | 34-106 15-16 1-5 1-24 16-60 1 | Assuming at most 2 scs-specific carriers |
|  |  | GSCN-ValueNR ::= CHOICE { lowCarrierFrequency SEQUENCE { frequencyIndex INTEGER (1..2944), frequencyOffsetSSB FrequencyOffsetSSB OPTIONAL }, midCarrierFrequency INTEGER (0..15173), highCarrierFrequency INTEGER (0..4383) } |  |  | 15-16 12 1-2 14 13 | FrequencyOffsetSSB ::= ENUMERATED { minus5kHz, plus5kHz } |
|  |  | ARFCN-ValueNR ::= CHOICE { lowCarrierFrequency INTEGER (0..599999), midCarrierFrequency INTEGER (600000..1999999), highCarrierFrequency INTEGER (2000000..3266667) } |  |  | 21-23 19 21 21 |  |
|  |  | SCS-SpecificVirtualCarrier ::= SEQUENCE { offsetToVirtualCarrier INTEGER (0..2199), subcarrierSpacing SubcarrierSpacing OPTIONAL, k0 FFS_Value OPTIONAL, carrierBandwidth INTEGER (1..maxNrofPhysicalResourceBlocks) OPTIONAL, ... } |  |  | 16-30 12 1-3 1-4? 1-10 1 | SubcarrierSpacing ::= ENUM {kHz15, kHz30, kHz60, kHz120} Assuming 3 bits for k0 maxNrofPhysicalResourceBlocks INTEGER ::= 275 |
|  | initialDownlinkBWP DownlinkBWP-Common OPTIONAL, |  |  | 28-371 |  |  |
|  |  | DownlinkBWP-Common ::= SEQUENCE { genericParameters BWP, pdcch-ConfigCommon SetupRelease { PDCCH-ConfigCommon } OPTIONAL, ... } |  |  | 22 1-344 1 |  |
|  |  | BWP ::= SEQUENCE { bwp-Id BWP-Id, locationAndBandwidth INTEGER (1..65536), subcarrierSpacing ENUMERATED {n0, n1, n2, n3, n4, n5}, cyclicPrefix ENUMERATED { extended } OPTIONAL } |  |  | 22 2 16 3 1 | BWP-Id ::= INTEGER (0..maxNrofBandwidthParts-1) maxNrofBWPs INTEGER ::= 4 |
|  |  | PDCCH-ConfigCommon ::= SEQUENCE { initialControlResourceSet ControlResourceSet OPTIONAL, initialSearchSpace SearchSpace OPTIONAL, searchSpaceOtherSystemInformation FFS_Value OPTIONAL, pagingSearchSpace FFS_Value OPTIONAL, ra-ControlResourceSet ControlResourceSetId OPTIONAL, ra-SearchSpace SearchSpace OPTIONAL } |  |  | 6-343 1 1 1-112 1-112 1-5 1-112 | Provided via MIB Provided via MIB Assuming common search space configuration, which is roughly 33-111 bits ControlResourceSetIndex ::= INTEGER (0..11) |
|  |  | SearchSpace ::= SEQUENCE { searchSpaceId SearchSpaceId , controlResourceSetId ControlResourceSetId, monitoringSlotPeriodicityAndOffset CHOICE { sl1 NULL, sl2 INTEGER (0..1), sl4 INTEGER (0..3), sl5 INTEGER (0..4), sl8 INTEGER (0..7), sl10 INTEGER (0..9), sl16 INTEGER (0..15), sl20 INTEGER (0..19) } OPTIONAL, monitoringSymbolsWithinSlot BIT STRING (SIZE (14)) OPTIONAL, nrofCandidates SEQUENCE { aggregationLevel1 ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}, aggregationLevel2 ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}, aggregationLevel4 ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}, aggregationLevel8 ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}, aggregationLevel16 ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8} }, searchSpaceType CHOICE { common SEQUENCE { format0-0-AndFormat1-0 SEQUENCE { ... } OPTIONAL, format2-0 SEQUENCE { slotFormatIndicatorSFI SlotFormatIndicatorSFI OPTIONAL, ... } OPTIONAL, format2-1 SEQUENCE { downlinkPreemption DownlinkPreemption OPTIONAL, ... } OPTIONAL, format2-2 SEQUENCE { ... } OPTIONAL, format2-3 SEQUENCE { ... } OPTIONAL }, ue-Specific SEQUENCE { formats ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1}, ... } } OPTIONAL } |  |  | 37-115 6 4 6-9 1-15 15 5-66 | SearchSpaceId ::= INTEGER (1..40) ControlResourceSetIndex ::= INTEGER (0..11) Assuming common search space for all SIB1 configurations Assuming INT-RNTI not relevant for SIB1 |
|  |  | SlotFormatIndicatorSFI ::= SEQUENCE { sfi-RNTI RNTI-Value OPTIONAL, nrofCandidates ENUMERATED {n1, n2}, aggregationLevel ENUMERATED {n1, n2, n4, n8, n16}, dci-PayloadSize INTEGER (1..maxSFI-DCI-PayloadSize), slotFormatConfigurations SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF SlotFormatCombinationsPerCell OPTIONAL, ... } |  |  | 18-59 1-17 1 2 6 7-32 1 | RNTI-Value ::= BIT STRING (SIZE (16)) maxSFI-DCI-PayloadSize INTEGER ::= ffsValue, assuming 64 assuming no aggregation in SIB1 |
|  |  | SlotFormatCombinationsPerCell ::= SEQUENCE { servingCellId ServCellIndex, slotFormatCombinations SEQUENCE (SIZE (1..maxNrofSlotFormatCombinationsPerSet)) OF SlotFormatCombination OPTIONAL, positionInDCI INTEGER(0..maxSFI-DCI-PayloadSize-1) OPTIONAL } |  |  | 6-31 4 1-20 1-7 | ServCellIndex ::= INTEGER (0..15) Assuming 1 slotFormatCombination maxSFI-DCI-PayloadSize INTEGER ::= ffsValue, assuming 64 |
|  |  | SlotFormatCombination ::= SEQUENCE { slotFormatCombinationId SlotFormatCombinationId, slotFormats SEQUENCE (SIZE (1..maxNrofSlotFormatsPerCombination)) OF INTEGER (0..255), subcarrierSpacing SubcarrierSpacing, subcarrierSpacing2 SubcarrierSpacing OPTIONAL } |  |  | 17-19 6 8 2 1-3 | SlotFormatCombinationId ::= INTEGER (0..FFS), assuming 64 Assuming only 1 slot format |
|  | uplinkConfigCommon UplinkConfigCommon |  |  | 35-225 |  |  |
|  |  | UplinkConfigCommon ::= SEQUENCE { frequencyInfoUL FrequencyInfoUL OPTIONAL, initialUplinkBWP UplinkBWP-Common OPTIONAL |  |  | 35-292 6-97 29-195 |  |
|  |  | FrequencyInfoUL ::= SEQUENCE { absoluteFrequencyPointA ARFCN-ValueNR OPTIONAL, scs-SpecificCarriers SEQUENCE (SIZE (1..ffsValue)) OF SCS-SpecificVirtualCarrier OPTIONAL, aSE AdditionalSpectrumEmission OPTIONAL, p-Max P-Max OPTIONAL, frequencyShift7p5khz ENUMERATED {true} OPTIONAL, ... } |  |  | 6-97 1-24 1-60 1-4 1-7 1 1 | (see downlink) Assuming 2 subcarrier spacings (see downlink for values) AdditionalSpectrumEmission ::= INTEGER (0..7) From LTE: P-Max ::= INTEGER (-30..33) |
|  |  | UplinkBWP-Common ::= SEQUENCE { genericParameters BWP, rach-ConfigCommon SetupRelease { RACH-ConfigCommon } OPTIONAL, pusch-ConfigCommon SetupRelease { PUSCH-ConfigCommon } OPTIONAL pucch-ConfigCommon SetupRelease { PUCCH-ConfigCommon } OPTIONAL ... } |  |  | 29-195 22 2-106 2-16 2-50 1 | (see downlink) |
|  |  | RACH-ConfigCommon ::= SEQUENCE { rach-ConfigCommonGeneric RACH-ConfigCommonGeneric, groupBconfigured SEQUENCE { ra-Msg3SizeGroupA ENUMERATED {b56, b144, b208, b256, b282, b480, b640, b800, b1000, spare7, spare6, spare5,spare4, spare3, spare2, spare1}, messagePowerOffsetGroupB ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18}, numberOfRA-PreamblesGroupA FFS_Value } OPTIONAL, ra-ContentionResolutionTimer ENUMERATED { sf8, sf16, sf24, sf32, sf40, sf48, sf56, sf64}, rsrp-ThresholdSSB RSRP-Range OPTIONAL, sul-RSRP-Threshold RSRP-Range OPTIONAL, prach-ConfigurationIndex INTEGER (0..255), prach-RootSequenceIndex CHOICE { l839 INTEGER (0..837), l139 INTEGER (0..137) }, msg1-SubcarrierSpacing SubcarrierSpacing, msg1-FDM ENUMERATED {one, two, four, eight}, msg1-FrequencyStart INTEGER (0..maxNrofPhysicalResourceBlocks-1), restrictedSetConfig ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB}, cb-preamblesPerSSB FFS_Value OPTIONAL, ssb-perRACH-OccasionAndPreamblesPerSSB CHOICE { oneEighth INTEGER (4..64), oneFourth INTEGER (4..64), oneHalf INTEGER (4..64), one INTEGER (4..64), two INTEGER (4..32), four INTEGER (1..16), eight INTEGER (1..8), sixteen INTEGER (1..4) } OPTIONAL, msg3-SubcarrierSpacing SubcarrierSpacing, msg3-transformPrecoding ENUMERATED {enabled} OPTIONAL } |  |  | 67-104 19 1-14 3 1-8 1-8 8 9-11 2 2 9 2 1-5 6-10 2 1 | Assuming 64 preambles This has been replaced by new RAN1 signalling RSRP-Range ::= INTEGER(0..124) SubcarrierSpacing ::= ENUM {kHz15, kHz30, kHz60, kHz120} maxNrofPhysicalResourceBlocks INTEGER ::= 275 RAN1 agreement: 4 bits Assuming 128 preambles per beam Assuming 64 beams From LTE: ENUMERATED { sf2, sf3, sf4, sf5, sf6, sf7, sf8, sf10}, Assuming 8 control set resources Assuming 8 search spaces and 8 control resources (see DL) |
|  |  | RACH-ConfigCommonGeneric ::= SEQUENCE { zeroCorrelationZoneConfig INTEGER(0..15), preambleReceivedTargetPower ENUMERATED { dBm-120, dBm-118, dBm-116, dBm-114, dBm-112, dBm-110, dBm-108, dBm-106, dBm-104, dBm-102, dBm-100, dBm-98, dBm-96, dBm-94,dBm-92, dBm-90, dBm-88, dBm-86, dBm-84,dBm-82, dBm-80, dBm-78, dBm-76, dBm-74, dBm-72, dBm-70, dBm-68, dBm-66, dBm-64, dBm-62, dBm-60, dBm-58, dBm-56, dBm-54, dBm-52, dBm-50, dBm-48, dBm-46, dBm-44, dBm-42, dBm-40, dBm-38, dBm-36, dBm-34, dBm-32, dBm-30,dBm-28, dBm-26, dBm-24, dBm-22, dBm-20, dBm-18, dBm-16, dBm-14, dBm-12, dBm-10, dBm-8, dBm-6, dBm-4, dBm-2, dBm-0, dBm2, dBm4, dBm6 }, preambleTransMax ENUMERATED {n3, n4, n5, n6, n7, n8, n10, n20, n50, n100, n200}, powerRampingStep ENUMERATED {dB0, dB2, dB4, dB6}, ra-ResponseWindow ENUMERATED {sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80} } |  |  | 19 4 6 4 2 3 |  |
|  |  | PUSCH-ConfigCommon ::= SEQUENCE { groupHoppingEnabledTransformPrecoding ENUMERATED {enabled} OPTIONAL, msg3-DeltaPreamble FFS_Value OPTIONAL, p0-NominalWithGrant INTEGER (-202..24) OPTIONAL, ... } |  |  | 4-15 1 1-4 1-9 1 | From LTE: deltaPreambleMsg3 INTEGER (-1..6) |
|  |  | PUCCH-ConfigCommon ::= SEQUENCE { pucch-ResourceCommon BIT STRING (SIZE (4)) OPTIONAL, pucch-GroupHopping ENUMERATED { neither, enable, disable }, hoppingId BIT STRING (SIZE (10)) OPTIONAL, p0-nominal INTEGER (-202..24) OPTIONAL, deltaF-PUCCH-f0 FFS_Value OPTIONAL, deltaF-PUCCH-f1 FFS_Value OPTIONAL, deltaF-PUCCH-f2 FFS_Value OPTIONAL, deltaF-PUCCH-f3 FFS_Value OPTIONAL, deltaF-PUCCH-f4 FFS_Value OPTIONAL, ... } |  |  | 11-48 1-5 2 1-11 1-9 1-4 1-4 1-4 1-4 1-4 1 | Assuming 8 values for all deltaFs |
|  | supplementaryUplinkConfig UplinkConfigCommon OPTIONAL |  |  | 1-293 |  |  |
|  | subcarrierSpacingCommon SubcarrierSpacing, ssb-PositionsInBurst CHOICE { shortBitmap BIT STRING (SIZE (4)), mediumBitmap BIT STRING (SIZE (8)), longBitmap BIT STRING (SIZE (64)) } OPTIONAL, ssb-periodicityServingCell ENUM { ms5, ms10, ms20, ms40, ms80, ms160 } OPTIONAL, dmrs-TypeA-Position ENUMERATED {pos2, pos3}, subcarrierSpacingSSB SubcarrierSpacingSSB OPTIONAL, tdd-UL-DL-configurationCommon TDD-UL-DL-ConfigCommon OPTIONAL, tdd-UL-DL-configurationCommon2 TDD-UL-DL-ConfigCommon OPTIONAL, ss-PBCH-BlockPower INTEGER (-60..50) |  |  | 2 1 1-4 1 1 1-38 1-38 7 |  | Not needed, known from MIB. Agreed to be signaled only with dedicated signaling. Not needed, known from MIB Not needed, known from SSB detection or band |
|  |  | TDD-UL-DL-ConfigCommon ::= SEQUENCE { refereceSubcarrierSpacing SubcarrierSpacing OPTIONAL, dl-UL-TransmissionPeriodicity ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10} OPTIONAL, nrofDownlinkSlots INTEGER (0.. maxNrofSlots) OPTIONAL, nrofDownlinkSymbols INTEGER (0.. maxNrofSymbols-1) OPTIONAL, nrofUplinkSlots INTEGER (0.. maxNrofSlots) OPTIONAL, nrofUplinkSymbols INTEGER (0.. maxNrofSymbols-1) OPTIONAL } |  |  | 6-37 1-3 1-4 1-10 1-5 1-10 1-5 | maxNrofSlots-1 INTEGER ::= 319 maxNrofSymbols-1 INTEGER ::= 13 |
| cellAccessRelatedInfo |  |  | 94-244 |  |  | From LTE |
|  | plmn-IdentityList PLMN-IdentityList, trackingAreaCode TrackingAreaCode, cellIdentity CellIdentity, ranNotificationAreaCode RANnotificationAreaCode |  |  | 6-156 16 36 36 |  | From LTE: PLMN-IdentityList ::= SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo PLMN-IdentityInfo ::= SEQUENCE { plmn-Identity PLMN-Identity, cellReservedForOperatorUse ENUMERATED {r, notR} } PLMN-Identity ::= SEQUENCE { mcc MCC OPTIONAL, mnc MNC } MCC ::= SEQUENCE (SIZE (3)) OF MCC-MNC-Digit MNC ::= SEQUENCE (SIZE (2..3)) OF MCC-MNC-Digit MCC-MNC-Digit ::= INTEGER (0..9) maxPLMN-r11 INTEGER ::= 6 TrackingAreaCode ::= BIT STRING (SIZE (16)) Notet that TAC might need to be provided per PLMN. (R3-174229) RAN3 has reached the following agreements related to NCI and its structure: 1) The NCI has fixed length of 36 bits. 2) The leftmost bits of the NCI correspond to the gNB ID. 3) The gNB ID has flexible length between 22 bits and 32 bits in 1-bit steps (11 values), to accommodate different deployment scenarios. |
| cellSelectionInfo |  |  | 1-10 |  |  |  |
|  | SEQUENCE { q-RxLevMin Q-RxLevMin, q-RxLevMinOffset INTEGER (1..8) OPTIONAL }, |  |  | 6 1-4 |  | From LTE: Q-RxLevMin ::= INTEGER (-70..-22) |
| SchedulingInfo |  |  | 45++ |  |  |  |
|  | si-WindowLength ENUMERATED {ms1, ms2, ms5, ms10, ms15, ms20, ms40}, schedulingInfoList SchedulingInfoList, + notification of on demand availability (1 bit per SIB) |  |  | 3 42 |  | From LTE: SchedulingInfoList ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo SchedulingInfo ::= SEQUENCE { si-Periodicity ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512}, sib-MappingInfo SIB-MappingInfo } SIB-MappingInfo ::= SEQUENCE (SIZE (0..maxSIB-1)) OF SIB-Type SIB-Type ::= ENUMERATED {sibType3, sibType4, sibType5, sibType6, sibType7, sibType8, sibType9, sibType10, sibType11, sibType12-v920, sibType13-v920, sibType14-v1130, sibType15-v1130, sibType16-v1130, sibType17-v1250, sibType18-v1250, ..., sibType19-v1250, sibType20-v1310, sibType21-v1430} maxSIB INTEGER ::= 32 maxSIB-1 INTEGER ::= 31 maxSI-Message INTEGER ::= 32 Assuming: SI1 = SIB2,3,4,5 (NR and LTE reselection) SI2 = SIB6,7,8 (ETWS, CMAS) SI3 = 16 (GPS and UTC) |
| ValueTag |  |  |  |  |  |  |
|  | systemInfoValueTag per SIB INTEGER (0..31) |  | 40 |  |  | Assuming 8 SIBs |
| Additional parameters for requesting other SI |  |  |  | 5 |  |  |
|  | Msg1vsMsg3 preamblesGroupBConfig SEQUENCE { sizeOfRA-PreamblesGroupB SetOfPreambles, } OPTIONAL |  |  | 1 4? |  |  |
| Access Barring |  |  | 68 |  |  |  |
|  | acBarringInfo SEQUENCE (SIZE (1..64)) OF AC-BarringConfig Assuming only one bitmap and AC 0..8 |  |  | 63 5 |  | From LTE: AC-BarringConfig ::= SEQUENCE { ac-BarringFactor ENUMERATED {p00, p05, p10, p15, p20, p25, p30, p40, p50, p60, p70, p75, p80, p85, p90, p95}, ac-BarringTime ENUMERATED {s4, s8, s16, s32, s64, s128, s256, s512}, ac-BarringForSpecialAC BIT STRING (SIZE(5)) Note that more information may need to be provided due to Unified Access Barring mechanism. |
| Service identification |  |  |  | ? |  |  |